STARFONT is a conversion of the original Gemfont files written for the

GEMINI 10X and now converted over to the STAR SG10 download format.

(I  would  like to point out that these will work just  as  easily

with the EPSON FX80).

Only the main 96 characters are reconfigured (CHR$(32-127), which are:

!"#$%&'()*+,-./0123456789:;<=>?

@ABCDEFGHIJKLMNOPQRSTUVWXYZ[\]^_

`abcdefghijklmnopqrstuvwxyz{|}~

Please ensure that DIP Switch 1-5 is turned off.(Dip switch 1-4 on

the EPSON). STARFONT files are recognizable by their SGF prefix

unlike the PFN prefix found on the GEMFONT files.

The original Gemfont was written on an ATARI 800 for the Gemini 10X.

The architecture of Gemfont is such that it may be used by any

computer that utilizes the STAR SG10 printer with its 96 downloadable

characters. The file consists of nothing but printer escape codes!!!

In order to use the file you must be able to print it directly to

the STAR SG10 from disk. There are numerous ways to go about this.

I will use the Atari options as those are the ones I am most familiar

with.

8-bit Atari:

Use your DOS 2 or 2.5 menu. And choose option C

at the prompt COPY -- From, To?

answer    STYLISH.PFN,P:

DOS will then print the file onto your STAR SG10 for you (assuming

of course that you had it turned on). You may then use what ever

wordprocessor or any other text lister you may so desire with the

new downloaded character set.

16-bit Atari:

Place the arrow on the particular printer font that you want to down-

load to your STAR SG10. Click the mouse twice and you will get the

option of viewing the file, printing the file on the printer, or

cancelling the operation. Choose the printer option and click again.

The file will be automatically sent to the printer for you. You may

then use what ever wordprocessor/text generator you desire and your

new character set will be printed out on the printer.

Others:

This will depend on your type of system. There is probably a way of

utilizing your operating system to send the file to your STAR SG10

in much the same manner as the two ATARI examples above. If worse

comes to worse you may have to build a little GET and PUT routine

to access the file and physically transfer it over to the printer

for you.

I hope you enjoy using these printer fonts as much as I enjoyed

creating them.

Rick Beetham

ter.  There are three files;

BRO10PT.HEX  (10 point pitch)

BRO12PT.HEX  (12 point pitch)