SPECTRUM SCREEN$ LOADER

=======================

If you have upgraded from a  Sinclair Spectrum and have had to

leave behind a cassettee  full  of  pictures,  fear  not,  this is the

program for you. Most  Speccy  picture  transfer  programs rely on you

having to transfer the data over the serial port, which can take ages.

This program overcomes  this  by  leting  you  load  the  SCREEN$ file

directly off the tape.

To do this, a lead has to be made up, which will plug into the

tape's earphone socket and the ST's printer port.

What you need is :-

1 x  25-way D type plug (male)

1 x  3.5 mm Jack Plug

a suitable length of wire

Connect the centre pin of the jack plug to pin 8 of the 25 way

plug. Connect the outer pin of the jack  plug  to pin 21 of the 25 way

plug.

Plug in the 25 way plug into  the  printer port of your ST and

the other end into the earphone socket of your tape player.

Run SPECPIC.PRG and play the tape containing the SCREEN$ file.

The border will flash  in  the  familiar  way  while loading. No image

appears on the screen until  the  screen  has  loaded,  and then it is

saved to disk in Neochrome format.

You can now load it into  your favourite art package and touch

it up.