Thermal.doc

The    self   generated   heat    of

transistors  used for small  signals

such  as  low power  amplifiers  and

oscillators  rarely  cause  problems

due  to overheating but as  soon  as

appreciable  power is involved  then

all  sorts  of problems  may  arise.

This  is very true of  power  supply

units  that  may  need  to   provide

output currents of several  amperes.

To  avoid  destruction of  a  series

pass transistor used in such a power

unit  the  generated  heat  in   the

transistor  must  be  dissipated  to

prevent the transistor case and thus

the   junction   rising  above   the

manufacturers  maximum  rating.  For

long  life it is desirable that  the

transistor   'runs'   as   cool   as

possible.

The  programme will ask for  various

parameters  to be entered  and  some

of  these can be obtained  from  the

transistor manufacturers data sheet.

The  others  are determined  by  the

characteristics of the power  supply

circuit and components.  For example

consider  a power unit  designed  to

output  12 volts at  1  ampere.  The

input   voltage  under   full   load

conditions will need to be 12  volts

plus the  volts  dropped  across the

series pass transistor,  typically 2

volts.  In  addition some  allowance

must  be made for variations in  the

mains  power  supply.  These  rarely

exceed  5%.  Therefore the 12  volts

must be increased by at least 5%  to

12.6 volts. The final design voltage

is  now 12.6 + 2 = 14.2  volts.  But

this  assumes  that  the   rectifier

filter  capacitor is  sufficient  to

'hold  up'  the voltage  during  the

trough  of  the  voltage  from   the

rectifiers.   Again  the  rectifiers

will  create a volts drop  of  about

0.6  volts  each and  this  must  be

allowed    for.     Under    typical

conditions    the   rectified    and

filtered  input to the  series  pass

transistor, under full load, will be

in the region of 15 volts at best.

As  an  example consider  a  popular

transistor  type  2N3055 used  as  a

series     pass     device.      The

manufacturers absolute maximum  case

temperature is 160 degrees  Celsius.

At  that  temperature  there  is  no

leeway  for temporary overloads  and

it  is  preferable  that  a  maximum

working  temperature of 100  degrees

be used.

So  far things are straight  forward

but now the tricky part is assessing

the  specification of the heat  sink

that  will be required to  dissipate

the  transistor  heat  so  that  the

maximum  temperature of 100  degrees

is  not exceeded.  Fortunately  heat

sink manufacturers normally  specify

their heat sinks with a degrees  per

watt figure. For example a heat sink

specified  as 4 degrees C  per  watt

means  that  it's  temperature  will

rise  4  degrees above  ambient  for

each watt dissipated:  e.g.  4 watts

results  in  16 degrees  rise  above

ambient.

If  the thermal contact between  the

transistor and heat sink is  perfect

then  the  transistor  will  operate

near  the  same temperature  as  the

heat  sink BUT this is an  imperfect

world and some temperature  gradient

will  exist between  the  transistor

and heat sink. The magnitude of this

gradient  is,  to  a  large  extent,

under  your control.  The heat  sink

surface   should  be  as   flat   as

possible  and the transistor  should

be  firmly  bolted to it  so  as  to

provide the maximum surface  contact

and lowest thermal  resistance.  The

addition  of a  proprietory  thermal

grease  between the mating  surfaces

will make a substantial improvement.

In  most  instances  the  electronic

circuit  arrangement   neccessitates

the  body  of the  transistor  being

electrically insulated from the heat

sink  and recourse must be  made  to

fit a suitable insulator between the

transistor and sink.  The choice  is

limited  to  a  thin mica shim, hard

anodised    aluminium (this being an

insulator  so long  as  the anodised

surface  is  not  damaged) and  heat

conductive    plastic/fibre    glass

compounds. Thermal grease must still

be   used   to maintain good thermal

contact. The Thermal.Prg  allows for

various transistor/heat sink types.

It is assumed that the heat sink  is

mounted with it's fins vertical  and

with free air circulation.

Example of programme use:-

Transistor:     2N3055

Case Size :     TO3

Input Volts:     18

Output Volts:    12

Output Amps:      3

Case Temp:      100 degrees C

Sink Insulator: Mica with grease

Ambient Temp:   30 degrees C

Putting  the above  parameters  into

the  programme will specify  a  heat

sink  rated at 3.39 degrees  Celsius

per watt.  Use the nearest  standard

sink  available  that  has  a  lower

figure eg 3 degrees per watt.

- - - - - - - - -