MYMONO - The STE Monochrome Emulator

MYMONO.PRG is an STE  monoemulator  that  makes  extensive  use  of  the

machine's enhanced  capabilities to produce  the fastest  mono  emulator

available. You'll  see  from the timings  below that  it is considerably

faster than version 5.0 of MICK WEST's mono  emulator  and  as  such  is

actually quite a  serious  alternative  to  buying a mono monitor if you

just use mono applications occaisionally.

I've tried it with everything I have  including  Calamus and  some music

packages and there seems to be no problems whatsoever.   I had to resort

to a trick to make TOS think there was no blitter in the machine  and as

such the menu option to turn the blitter on or off no  longer appears on

the GEM desktop.  This does mean however that YOU MUST SAVE YOUR DESKTOP

WITH THE BLITTER TURNED OFF OR THE SOFTWARE WILL NOT WORK.

There are quite a few options and 3 different emulation modes available.

The software must run from the Auto folder,  but I don't believe it  has

to be the first program run,  unless  other  things in this folder  make

decisions based on  the  resolution  of the  machine,  in  which case it

should run before those applications.   When the emulator boots you have

the  option  of  hitting <RETURN> and getting  your  saved  defaults  or

choosing all the options manually.

SPEED:

This controls the speed of update.   0 will give  you a perfectly smooth

emulation, just like looking at a real  mono monitor, the mouse  doesn't

jerk and the graphics don't ghost.  The down side is of course that this

slows down the operation of the machine considerably, look at the timing

table below to see the facts and figures.  9 is very slow update but the

machine runs almost as if no emulation was happenning.

ALIASING:

This is the same type of emulation used by existing  emulators  although

considerably faster, it uses a special palette that allows the screen to

be antialiased, subsequently increasing the apparent resolution but also

giving a slightly blurred look to screen.

INTERLACE:

A new way of cramming  400 lines  of data into  only 200, the idea is of

course used on the Amiga to do just that.   This,  while  being  a  very

faithful representation of the high rez screen, is very annoying to most

people, I personally don't  mind  it that  much but use it  at  your own

risk.

SCROLLING

This method is the clearest of all modes but you only get the 200  lines

nearest your mouse pointer shown at any time.  The display automatically

scrolls with the cursor.

SYNC

You can choose to  put your machine and monitor into 50  or  60Hz  mode.

This is for the sake of choice.  Some  people  prefer  60Hz on interlace

mode as the flicker is faster and slightly less noticable.

SAVE

Once you have made all  your  choices  you  can  save  the  settings  as

defaults so that each time  you  boot  hitting return will produce  your

favorite setup.  An info file will be created on the  disk  and you will

be warned of any problems (like write protect).

TIMINGS

All these timings were produced with the  excelent QINDEX1.5 all numbers

are percent of normal speed (100%).  In each column the left hand number

is using the fastest emulation (screen  update)  the  middle  using  the

initial default settings, and the right most number is using the slowest

emulation.

MYMONO                  MONO_EMU

CPU:___________________________________________________________________

Memory  |        57      89      96   |     46      70      92

Register|        57      89      96   |     48      70      92

Divide  |        92      93      97   |     48      70      92

Shifts  |        93      96      97   |     48      70      92

BIOS:           |                             |

Text    |        56      87      94   |     47      69      90

String  |        57      88      95   |     48      69      91

Scroll  |        57      89      96   |     48      70      92

GEM:            |                             |

Draw    |        60      94      101  |     51      74      97

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Legal Stuff

The MYMONO Program was written by Tony Barker/Moving Pixels.  The MYMONO

Program  is  Copyright  (C)  1986-1995  Tony  Barker/Moving  Pixels  and

Copyright (C) 1995  Crawly  Crypt Corporation.   All  MYMONO  Binary and

Source files are intended for use only by  owners of  the  Moving Pixels

Collection CD.  They may not in any way be  copied, sold, given away  or

transmitted  by modem.  Tony Barker, Moving Pixels, and the Crawly Crypt

Corporation make no  warranties, expressed or  implied,  with respect to

the software contained  in  this  package,  it's  quality,  performance,

merchantability, or  fitness  for  any particular purpose.  Tony Barker,

Moving Pixels, and the Crawly Crypt Corporation shall not be responsible

for any direct, indirect, incidental, or  consequental  damages  brought

about by the use, misuse, abuse, or inability  to  use this product even

if we have been advised of the possibility of such damages.   The entire

risk as to  the  performance  and  quality  of  the software is with the

user.  Please refer to the file "COPYRITE.TXT"  for  complete  copyright

information.

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Please direct all correspondence about this product to:

The Crawly Crypt Corporation

PO Box 23

Webb City, MO  64870-0023

United States