06601030305800

F0110030

9[...................................................]001

************************************************************************

*                                                                      *

*         DigiTape Effect Module Construction Kit                      *

*         ---------------------------------------                      *

*                                                                      *

*           How to build loadable Modules for                          *

*                 DigiTape Release 2.0                                 *

*                   7. Juli 1993                                       *

*                                                                      *

************************************************************************

DigiTape untersttzt ab der Version 1.03  freie Effektmodule.

Das heit, da jeder, der sich zu einem bichen C- und

etwas mehr DSP-Assembler-Programmierung in der Lage sieht,

nachladbare Module mit Klageffekten erstellen kann, die in

DigiTape eingebunden werden.

Wir stellen anbeiEntwicklerinformationen ber das Softwareinterface

und die Verbindung zu DigiTape zur Verfgung.

Weitere Informationen, die bentigt werden, finden sich im

Motorola Entwicklerhandbuch zum DSP 56k sowie im Falcon

Entwicklerkit von Atari, das Sie bei Atari Computer GmbH in

Schwalbach bestellen knnen.

Beachten Sie bitte die Artikelserie "DSP-Programierung" in der

ST Comuter, Heim Verlag.

Weiterhin mchten wir auf eine Artikelserie mit dem Titel

"Klangwunder" in der Zeitschrift c't verweisen, die auf

Soundprogrammierung fr den DSP 56k / Falcon 030

gezielt eingeht:

"Klangwunder" in c't Heise Verlag, Heft  9/93 S. 226ff

Heft 10/93 S. 212ff

Heft 11/93 S. 260ff

External Digitape modules contain DSP56000 code, GEM Recources and the

user-interface code in one file. The modules are loadable from within the

Digitape desktop and can be handled like interanal ones. If an external

module is used together with a recording, it is automatically loaded

next time the recording tape is opened.

As an Example we supply a complete module wich implements a reverb effect:

Sourcefiles:

-------------

resource:      HALL.RSC

HALL.H

HALL.DFN

DSP assembler: HALL.ASM

User interface:HALL.C

DT_DEFIN.H

DT_LOCAL.H

projectfiles:  HALL.PRJ

MAKEFILE

Tools:

------

You need:      C Programming Environment

Resource Construction Set

DSP56000 Assembler

(a Make Programm, not nessesary but simplifies the calls

the other tools)

We supply:     CLN_TO_D.TTP   builds a C source file out of a DSP

assembler object file.

HALL.CLN becomes HALL.D and HALL.DH.

RSC_TO_C.TTP   GEM resource to C converter.

HALL.RSC becomes HALL.R

Building procedure:

-------

*---------------*       *----------*  *----------*  *-----------*

|   HALL.ASM    |       | HALL.RSC |  |  HALL.H  |  | HALL.C    |

|               |       |          |  |          |  |           |

| DSP assembler |       |   GEM    |  | resource |  |  C file   |

|   source      |       | resource |  |  defines |  | interface |

*---------------*       *----------*  *----------*  *-----------*

|                     |             |             |

V                     V             |             |

=================    ==================     |             |

= ASM56000.TTP  =    = RSC_TO_C.TTP   =     |             |

= DSP assembler =    =   resource     =     --------> #include

=               =    =   converter    =                   |

=================    ==================                   |

|                     |                           |

V                     V                           |

*---------------*      *-------------*                    |

|   HALL.CLN    |      |   HALL.R    |                    |

|               |      |             |                    |

|  DSP object   |      |  resource   |                    |

|               |      | as C source |                    |

*---------------*      *-------------*                    |

|                     |                           |

V                     |                           |

===================            |                           |

=  CLN_TO_D.TTP   =            |                           |

=  convert tool   =            ----------------------> #include

=                 =                                        |

===================                                        |

|          |                                           |

V          V                                           |

*-------------*  *-------------*                                |

|   HALL.D    |  |   HALL.DH   |                                |

|             |  |             |                                |

| DSP object  |  | DSP defines |                                |

| as C source |  | as C source |                                |

*-------------*  *-------------*                                |

|                |                                       |

|                ----------------------------------> #include

|                                                        |

---------------------------------------------------> #include

|

DT_DEFIN.H --------------------> #include

|

DT_LOCAL.H --------------------> #include

|

V

=========================

=  C-Compiler & Linker  =

=     Make HALL.PRJ     =

=                       =

=========================

|

V

*------------------*

|*----------------*|

||   HALL.DTM     ||

||                ||

|| DigiTape-Module||

|*----------------*|

*------------------*

A few restrictions to your DSP code:

------------------------------------

Identifiers have to be at most 7 characters long.

ORG-Directives are not permitted.

Calculations in Directtives are not permitted.

Example:  move  X1,X:DATUM   ; OK

move  X1,X:DATUM+1 ; not OK

Modules should return with RTS.

Access to L-Memory (X- and Y-Memory together) is not permitted.

Following registers are used internally and you should absolutely

NOT use them within your code: R3,M3,N3

Registers M0-M2 and M4-M7 have to contain 0xFFFF after each module call,

that means, if you change them, set them back to $FFFF

All other registers are at your disposal.