A1000 Audio Lowpass filter bypass

=====================================================

Ok... here's the deal: if you're an A1000 owner and would like to get

the same high fidelity sound that can be achieved in other amigas by

turning off their lowpass filter, this file documents how you can do it.

Just a word of warning.. if you're not comfortable opening up your

computer and removing boards, etc., I would rather recommend some

computer technician (or hacker friend) to do it for you. I won't say

anything about voiding warranties (at least I didnt get a 6 yr.

manufacturer's warranty on MY A1000 :) ) If you have one of those

refurbished models which still have valid warranties... consult

your amiga dealer!

The newer amigas (500, 2000, 3000, 3000T?) have the same audio chip and

surrounding circuitry as our A1000s, except that theirs provides for

a software-selectable audio filter bypass, which can be toggled at

will. The fix I present here is permanent (see below), but I doubt that

you will ever want to switch the filter back on!

Now... on with the instructions:

Instructions:

=====================================

+---------------------------------------------------------------------+

| DISCLAIMER::: Ok, I hold no responsability for damaged computer     |

|               components or project-related fatalities. This, after |

|               all, is a hack, and although I am quite sure it will  |

|               work for you the way it has for me, accidents do      |

|               happen, words are misspelled/misread, etc.            |

|               Although this is a fairly simple modification (it only|

|               involves soldering two wires), beware:                |

|              ***MODIFY YOUR COMPUTER AT YOUR OWN RISK!***           |

+---------------------------------------------------------------------+

First, before you go prying your computer open, here's a list of what

you'll need:

Philips Screwdriver

22 Gauge Wire (or 20)

Wire cutter or wire stripper

Small Pliers

Soldering Iron (20-25 Watts)

Some solder (For PC board work)

Ok.. now after removing all pheripheral connections, 256k memory

expander (you DO have one, don't you?), bus devices and power

connections, flip the computer over and remove all the screws that hold

the case together. I suggest you find some place to put and organize all

the screws since there are lots of them, and they're easy to confuse

and maybe even have some left after closing the case (trust me.)

Ok, now carefully put the computer face up again. Now remove the top

cover by prying each corner open, first the ones on the back, and then

the front ones. A small flat screwdriver often helps in this procedure,

just be careful and don't force the cover off.

Now that you have the top cover in your hands, look at the inside

portion. See all the signatures? Neat, huh? :)

Now remove the RF shield by first using your pliers to straighten the

metal tabs that hold it and then removing the screws around it. You

should now see the motherboard, kickstart daughterboard, floppy drive and

power supply. We shall now concentrate on the daughterboard.

The daughterboard is a small board that stands about an inch above the

main motherboard, components facing down. This board is connected to

the main board by several gold posts located at each end. Since the

audio buffer/filter chip is below this board, we must first remove it.

First, make sure you ground yourself to avoid zapping any components

with static electricity. If any of the floppy connectors are in your

way, push them to the side, or remove them carefully, noting their

orientation so you can put them back in correctly.

To remove the daughterboard, first remove the screws that hold it in

place, and then pull up on each corner of it just a bit at a time,

making sure that the gold posts aren't being bent by keeping the board

as level as possible. Take your time-- it'll come off eventually.

When you finally pull the board off, set it aside on a clean static-free

surface. Now, look for the audio buffer chip. It is located at position

U5G, right below the main power supply connector on the motherboard

(refer to the diagram in the IFF file.) Now, for the fun part...

Cut 2 lengths of wire, strip the ends of each one. Don't tin the ends

of the wires, just twist them... it'll be easier to afix them this way.

One of the wires connects pins 14 and 3 on the chip, and the other one

connects pins 5 and 8. Wrap each end to the appropiate pin on the chip,

and after making sure that the wire doesn't touch any other pins, solder

it in place, aplying only the necessary heat. The chip may be damaged if

you place too much heat on the pins. Also make sure that the solder

doesnt touch any oother pins, either. If it does, remove it with solder

wick or a vacuum bulb, and try again. (Refer to diagram for pin numbers

and jumper location.)

That's it! Now, we must put the daughterboard back in place. Do so by

aliging each of the red connectors on the daughterboard with the gold

posts from the motherboard, and then gently pushing down on each corner,

a bit at a time, following the same precautions we took when removing

it. Keep pushing down until the plastic parts from the gold posts and

the connectors on the daughterboard come together completely. Replace

the board's screws. Also, if you removed any of the floppy connectors,

replace them, being sure they're correctly connected.

At this point, connect the monitor and power cable to the computer, and

power it up. If the kickstart 'hand' picture doesn't appear on the

screen, then you didn't mount the kickstart board correctly. Power off

and check that it is seated correctly.

Load kickstart and then any sound demo of your choice (game, program,

etc.) to test the computer's sound. There should be a noticable change

in audio quality. When you're ready, power back down and remove all

cables. Screw the RF shield back in, and twist the metal tabs to secure

its ground connection. Replace the top cover, flip the computer over,

and replace the remaining screws. Now you can re-connect all your

peripherals and power cord. Now you're ready to Rock and Roll!

And now......

=====================

There it is. Simple, isn't it? And its worth the trouble... my A1000 has

had the modification for about 1 1/2 years now, and there have been no

problems. One thing I did put in was a switch- just in case I wanted to

get the old sound back; but I've never used it once! But if you want to

get your old sound back, just place a DPDT switch between the jumper

cables. One drawback to this method is that there's a bit more noise

due to the longer wires, but not much. I would rather have you NOT put

a switch in-- maybe a defective switch could ruin some components, etc.

(I wouldn't know WHY someone would even care to go back to having lesser

audio fidelity...)

I guess some of you who aren't too techinaclly inclined have their

doubts or maybe some of the instructions aren't very clear to you...

so here are my addresses (electronic and real-world) just in case you

want to ask me something, or just to chat!

Alejandro Garza Gonz�lez        pa250225@mtevc2.mty.itesm

Rio Rhin 514 pte.                       @131.178.1.5

Colonia Del Valle                    (Internet address)

Garza Garc�a, N.L. 66220

MEXICO                          JanusMan@IRC

(phone 38-46-86)              (Internet Relay Chat)

Before I sign off, I'd very much like to thank Howard Bassem, who did

the original article back in '88 on Amazing Computing Magazine V3.4

about bypassing the audio filter. Thanks, Howard!

Also, thanks to everyone at +Amiga! (IRC) for all the tech support,

Teijo Kinnuen (for MED) and Psyke-nose-is!

If you ever come to Mexico, gimme a call! Glad to show any amiga pal

around! (Of course, if you're from around here, don't bother 8^) )

--------------------------------------------------------------------25/Oct/91

============================================================================

| Alejandro Garza Gonz�lez/38-46-86/Monterrey, Mexico::  ^^^^^�            |

| JanusMan@IRC::pa250225@mtecv2.mty.itesm.mx         ::  (..) � (the hell?)|

|                                                    ::  /_   �   o        |

|    // Only amiga, folks!                           ::  |_- _� ��         |

|  \X/  "What the hell is THAT, boy??"               ::    ==        J'91! |

============================================================================