The Multi-Kickstart Board Project.

Revision 3.1

Copyright 1990, 1991.  Neil Coito and Michael Cianflone.

Amiga and Kickstart are registered trademarks of Commodore-Amiga.

**********************************************************************

Three Sentence Overview Of What This Thing Is:

The Multi-Kickstart Board Project allows the user of an Amiga 500,

2000, or 2500 to switch between different versions of the operating

system, called Kickstart.  Some older software, and even some new

software, are not compatible with Commodore's latest version of

the operating system. This hardware project will let you use the new

version of the operating system (it is *tons* better than 1.3 and

below), and still remain compatible with the software that won't work

under the new operating system.

**********************************************************************

The files below, and the information contained with them, are NOT

public domain. They are copyrighted by Neil Coito and Michael

Cianflone. What this means is that you cannot take the design and

information within the archive and make your own boards and sell them

without prior written permission from us.  What it does mean is that

you are free to make your own board from this information whether you

buy the board and parts from us or whether you wire-wrap your own.

This archive may be freely copied and distributed as long as all the

files remain unmodified and intact, and as long as there is no charge

for them except a reasonable copying and media fee.

The files included within this archive are:

TMKBP31.doc           The file you are reading right now. This file

contains all the ordering information, assembly

instructions, and everything else you will need

to know to build The Multi-Kickstart Board

Project, except pictures.

TMKBP31Schematic.iff  Schematic diagram of The Multi-Kickstart Board

Project

TMKBP31Parts.iff      Diagram of what each part looks like

TMKBP31Layout.iff     Mechanical drawing of The Multi-Kickstart Board

Project

TMKBP31Ribbon.iff     Diagram, and measurements of the 40 pin ribbon

cable

TMKBP31Switch.iff     Diagram of how to hook up the 3 position switch

TMKBP31Board.iff      Picture of the printed circuit board showing

where the optional cutting of a jumper needs to

be done, and how to determine if you need the

jumper cut.

TMKBP31Probes.iff     Diagram of the probes needed to test and

troubleshoot the  Multi-Kickstart Board Project.

----------------------------------------------------------------------

Disclaimer:

Although every effort has been made to eliminate errors and insure

compatibility, these files come with no warranty. If there is a

typing error... sorry, it's not my fault it's yours :-) Also, you

are at your own risk to build The Multi-Kickstart Board Project

(TMKBP). If you blow up your computer or burn your house down then

it's not our fault. You assume all responsibility for all information

taken from this archive, and for building the Multi-Kickstart Board

Project. Basically, it's not our fault if you have any problems,

though we will try to help; if you have access to a modem we can be

reached at:

UUCP: {hplabs!hp-sdd ucsd nosc}!crash!pnet01!neil

ARPA: crash!pnet01!neil@nosc.mil

INET: neil@pnet01.cts.com

There, if you're still with us, and if I haven't scared you off then

continue reading and maybe you can build one of these nifty projects

to almost ensure compatibility with your older software, and even a

few new ones!

----------------------------------------------------------------------

Compatibility:

Works with all versions of the Amiga 500.

Works with all versions of the Amiga 2000 including models A and B.

----------------------------------------------------------------------

General Information:

The Multi-Kickstart Board Project allows the user of an Amiga 500,

2000, or 2500 to switch between different versions of the operating

system, called Kickstart.  These versions of Kickstart are contained

on ROM chips that are normally plugged into the computer mother board.

The reason for the switching ability is due to the incompatibility of

older software programs, and perhaps a few new ones, with the newest

version of the operating system, particularly Kickstart version 2.04.

The Multi-Kickstart Board Project will enable the user to access an

older version of Kickstart so that 2.04-incompatible software in the

user's library may be used.  The Multi-Kickstart Board Project will

also allow the user to switch to the latest version of Kickstart to

gain the advantages it offers with newer, more compatible software.

The user removes the ROM chip from the computer mother board, inserts

it into any empty socket on The Multi-Kickstart Board Project, and

then inserts the 40 pin DIP connector from The Multi-Kickstart Board

Project into the empty ROM socket on the mother board.  When a newer

version of Kickstart is purchased, that ROM is simply plugged into

another empty socket on The Multi-Kickstart Board Project.  The user

may then, with a flick of a switch, select either version of

Kickstart: the older version that is more compatible with the user's

collection of older software, or the newer version with its many

enhancements.  A third ROM socket is present on The Multi-Kickstart

Board Project to allow for another, future version of Kickstart.

You can build this project by obtaining all the parts yourself and

wire wrapping it together on a perfboard, following the schematics, or

you can order The Multi-Kickstart Board Project in kit form which

includes the printed circuit board and all the parts needed to put it

together. See the parts list for a detailed breakdown of the parts

needed.  You can also order just a printed circuit board from us if

you have your own parts.

----------------------------------------------------------------------

The Multi-Kickstart Board Project Parts List:

If you order The Multi-Kickstart Board Project printed circuit board

with parts, then it comes with everything listed here except those

marked OPTIONAL. If you just order the printed circuit board with no

parts then, you guessed it, it just comes with a printed circuit

board.

COMPONENTS:

REF# - DESCRIPTION

R1  - 330 Ohm 1/4W Resistor

R2  - 330 Ohm 1/4W Resistor

R3  - 330 Ohm 1/4W Resistor

C1  - .01uF 25V Ceramic Capacitor

C2  - .01uF 25V Ceramic Capacitor

C3  - .01uF 25V Ceramic Capacitor

U1  - 74LS32 quad two input OR gate

ROM1 - Any version Amiga Kickstart ROM (OPTIONAL)

ROM2 - Any version Amiga Kickstart ROM (OPTIONAL)

ROM3 - Any version Amiga Kickstart ROM (OPTIONAL)

SW1  - Single Pole 3 Position Switch with 4 wire ribbon cable

CN1  - 40 conductor ribbon cable with 40 pin DIP connector

MISCELLANEOUS PARTS:

QTY - DESCRIPTION

1  - The Multi-Kickstart Board Project printed circuit board

3  - 40 pin DIP Sockets

1  - 14 pin DIP Socket

1  - 2.5 feet 4 conductor ribbon cable

4  - Rubber feet or plastic spacers (OPTIONAL)

All parts shown above are included in our "The Multi-Kickstart Board

Project" kit except for the parts marked (OPTIONAL).  We do not supply

the parts marked (OPTIONAL).

----------------------------------------------------------------------

Ordering Information:

To order The Multi-Kickstart Board Project either print out the

following order form or make your own just like it and mail it, along

with your payment, (make the check payable to Neil Coito) to:

The Multi-Kickstart Board Project

P.O. Box 60414

San Diego, CA 92166

USA

......................................................................

Please send:

____ Board(s) and parts at $14.95 each for a total of    $_____

____ Board(s) WITHOUT parts at $9.95 each for a total of $_____

if in California add 8.25% tax                           $_____

shipping and packaging (see below)                       $_____

Total                                                    $_____

TO: ____________________________________

name

____________________________________

address

____________________________________

city           state     zip code

____________________________________

country

.....................................................................

note: DO NOT SEND CASH. Only send a check or money order. We will

accept personal checks. All checks must be in US funds drawn on a US

bank.

********************************************************************

************* MAKE THE CHECK PAYABLE TO: Neil Coito ****************

********************************************************************

---------------------------------------------------------------------

Shipping and Packaging Prices:

All shipping within the United States is done First Class, and all

shipping done outside the United States is done by Air Mail.

Shipping and Packaging:

Prices are for EACH board with parts ordered. Same shipping and

packaging price for each board ordered without parts.

United States -  $1.16

Canada        -   1.22

Mexico        -   1.25

Germany       -   2.53

England       -   2.53

France        -   2.53

Australia     -   3.11

These are all the main locations I could think of to ask the post

office for shipping rates to. Shipping and packaging to Europe appears

to be $2.53 so if you're in Europe and we don't have a shipping price

for your area then just make it $2.53 for shipping. The weight of the

package is 3.9 ounces so if you are somewhere else that we don't have

a price for then you can go to the post office there and see what the

shipping would be and add 18 cents for packaging. If you can't get to

the post office and your country is not listed then make the shipping

and packaging $3.50 and that should cover it.

----------------------------------------------------------------------

Theory Of Operation:

The Multi-Kickstart Board Project is simply an extension of the

Amiga's ROM socket with switching ability.  The circuit board consists

of three ROM sockets (ROM1, ROM2, and ROM3), three 330 ohm resistors

(R1, R2, and R3), three .01uF capacitors (C1, C2, and C3), and a

74LS32 quad two input OR gate (U1).

The ROM chip of the Amiga is a Read Only Memory chip that contains the

operating system of the Amiga Computer called Kickstart.  In order for

the computer to read the contents of any memory chip, the processor

has to somehow tell the memory chip that it would like to read its

contents.  Many memory chips have a pin call Output Enable (*OE). This

pin is used by the computer system to tell the ROM chip that it is the

device being selected to be read from by the processor. By decoding

the address lines of the processor, a signal called *ROMEN is produced

by the computer. This signal is then fed into the Output Enable pin of

the Kickstart ROM.  The Output Enable pin of the Amiga's ROM chip is

an active low line. This means that the ROM will only respond to the

address bus if its Output Enable line is low, or 0 volts. If this line

is high, or 5 volts, the ROM chip will effectively take itself off the

data bus and will not respond to the address bus. By taking control of

the Output Enable line and steering it to one of three ROM sockets,

The Multi-Kickstart Board Project is able to control which one of

three ROMs will be used by the computer.  When one of the ROMs is

enabled, the other two ROMs remain off the data bus. They will stay

off the data bus as long as their Output Enable pins stay high. The

logic circuitry of The Multi-Kickstart Board Project was designed to

only allow one ROM to be enabled at a time, allowing for only one

operating system to be running, for example Kickstart 1.3, Kickstart

2.04 or any other Kickstart version installed in the board. Other than

the Output Enable lines, all the other lines of the ROMs on the board

are connected in parallel as you can see in the schematic.  If for any

reason all the ROMs were all enabled at the same time bus contention

would exist on the data bus and the machine would crash.

At the heart of The Multi-Kickstart Board Project is a 74LS32 two

input quad OR gate. This device is configured as a logical switch that

will only allow one Output Enable line to be activated at a time. The

logic circuit is controlled by a three position switch which is used

to select between the ROMs. All the lines from the Amiga's ROM socket

are placed in parallel with all the lines of the ROM sockets on The

Multi-Kickstart Board Project, except for the Output Enable lines of

both the Amiga's ROM socket and The Multi-Kickstart Board Project's

ROM sockets.  The Output Enable line from the Amiga's ROM socket is

sent into one of the gates of the logic chip that is configured as a

buffer. This buffered signal is then fed into one of the inputs of

each of the remaining three gates.  The other input of the remaining

gates is connected to one of the switches three positions. At each of

these points a pull-up resistor is added (R1, R2, and R3).  The

purpose  of the pull-up resistor is to keep the input of the gate at a

high when it is not being pulled low by the switch. The switch

activates a gate by connecting the gate's input to ground or taking it

low. This condition allows only one gate to pass the Output Enable

signal through its output to the ROM socket and then to the ROM

sitting in that particular socket. All the other Output Enable signals

stay high.

On The Multi-Kickstart Board Project printed circuit board, pin 1 and

pin 31 are jumpered together.  This was done because earlier versions

of the Amiga's motherboard had the address line A17 at pin 1, the

newer versions of the motherboard have A17 on pin 31 of the ROM

socket.  For People that have the newer versions of the mother board,

they must simply cut the trace at JMP1 on the printed circuit board.

On the newer versions of the mother board pin 1 must NOT be connected

to pin 31. Having these lines connected together on the newer boards

will cause  the computer to crash. The jumper was added to The Multi-

Kickstart Board Project printed circuit board for the convenience of

the people that have the older version of the mother board, and to

make switching between different board versions an easy task.

The purpose of the capacitors on this board is for filtering the power

supply line.

----------------------------------------------------------------------

Tools Required To Put The Multi-Kickstart Board Project Together:

The following items are required in order to put the Multi-Kickstart

Board Project together.

Soldering Iron

Solder

Wire Cutters

Wire Strippers

Chip Extractor or Straight-Edge Screwdriver

The following items are NOT required but will make it easier to put

certain components together, for example to cut the 40 pin cable down

to a size that best fits your computer you should measure the length

first, however you do not need a ruler if you want to eyeball it and

cut what you think should fit.

X-acto Knife (optional)

Needle Nose Pliers (optional)

Ruler (optional)

Masking Tape, or Scotch Tape (optional)

Permanent Marker Pen (optional)

----------------------------------------------------------------------

Assembly Of The Multi-Kickstart Board Project:

The assembly and testing of The Multi-Kickstart Board Project should

not take very long to do.  Although it is a very simple project, I can

not stress it enough that you take your time and be neat with putting

it together and testing it.  The more time you take with putting this

board together the less mistakes you are going to make and the happier

you will be with the results.  The less mistakes you make, the less

time you will spend banging your head against the wall for making

those stupid mistakes.  Before starting, read all instructions and

check to make sure that all the parts in the parts list NOT marked

(OPTIONAL) are in the kit.

For those of you that have never soldered on a printed circuit board

before, this paragraph will give you a few tips to help you through

this task.  First of all let the soldering iron heat up for at least

ten minutes prior to soldering.  When soldering, it is always a good

idea to keep the tip of your iron clean. Clean the tip with a damp

cloth before each pass with the iron.  When soldering  on a board like

this, it is best to use a small and pointed soldering iron tip.  The

most important thing to remember is what I call the "Ten Second Rule."

This rule simply states that you do not leave your iron touching the

printed circuit board for more than ten seconds.  The reason for this

is that if you let the hot iron sit on the board for more than ten

seconds, you will probably lift a trace or a pad from the board

because of the heat.  Ten seconds is more than enough time to let the

solder flow.  If you must resolder a point on the board, let the board

cool down before reapplying the iron.  Remember, whenever you solder,

treat the board gently.  When you solder, it is a good idea to use a

flux core solder.  Another good tip to remember is to first place the

iron at the point to be soldered THEN add solder to that point. DO NOT

put too much solder at each point, put enough just to get the job

done.

There are some drawings included within this archive that may help you

put this project together (see the first section of this file for

their names and what they are pictures of).  You may look at them with

any program that display standard Amiga IFF pictures, and you may

print them out so that you have them at hand.

Now we will begin putting the Multi-Kickstart Board Project together.

You may check off each number as you perform each step so when you

take a break you can easily continue where you left off.

[ ] 1.  Read this entire document before going on to step 2! Never

work on a project until you have read all the instructions at least

once!

[ ] 2.  If you would like to test the blank board for shorts prior to

putting it together, you can assemble the continuity tester shown in

the drawing "TMKBP31Probes.iff". If you do not have access to the

supplies to build the continuity tester, or if you do not want to test

the board for shorts, then check off steps 2 and 3, and go on to step

4. If you do want to test the board for shorts, then using the

continuity tester shown in the drawing, check to make sure that the +5

volt line and ground line are not shorted together.  To do this touch

the (+) probe of the continuity tester to the pin 14 location of U1 on

the board and the (-) probe to the pin 7 location of U1 on the board.

If the LED lights, that means there is a short.  If this condition

exists, check the board for a short and remove that short.

[ ] 3.  If you've built the continuity tester, then check and test the

board for open lines and shorts by using the continuity tester and the

schematic in the drawing "TMKBP31Schematic.iff"; check to see that all

the lines on the printed circuit board are connected as they should

be.

[ ] 4.  Now we are going to solder the four sockets to the printed

circuit board one at a time.  Place the three 40 pin IC sockets on the

top side of the printed circuit board so that pin 1 of the socket is

in the top left corner of each of the socket locations marked ROM1,

ROM2, and ROM3.  Place the 14 pin IC socket in the top right corner of

the board in the location marked U1.  Pin 1 of this socket is at the

bottom left.  When placing the sockets on the board and prior to

soldering, make sure you do not bend any of the pins.  The top side of

the printed circuit board is the side with the silkscreen stating the

name and revision number of the board.  Refer to tips 5, 6, and 7 (in

the "Tips For Building The Multi-Kickstart Board Project" section) for

a few helpful hints.

[ ] 5.  Solder the three capacitors (C1, C2, and C3) to the printed

circuit board.  The capacitors are non-polarized and can go in either

direction.  They go in the locations marked C1, C2, and C3.  The

capacitors are .01uF capacitors.  Refer to tips 5, 6, and 7.

[ ] 6.  Solder the three resistors (R1, R2, and R3) to the printed

circuit board.  The resistors go in the locations on the printed

circuit board marked R1, R2, and R3.  Just like the capacitors the

resistors can go in the board in either direction.  All three

resistors are 330 ohm resistors.  Refer to tips 5, 6, and 7.

[ ] 7.  If you have an Amiga 2000, see "Installation Of The Multi-

Kickstart Board Project" to determine which motherboard revision you

have. This is necessary in order to determine the length and

orientation of the 40 wire ribbon cable used in step 8.

[ ] 8.   You will eventually need to cut the 40 wire ribbon cable to a

length that you feel will fit in your computer properly.  We give you

a six inch cable so that you can cut it down to the size that best

fits your system.  On a newer revision Amiga 2000 a length of about

1.75 inches works good, but double check to make sure.  On the older

revision 2000 the ROM was put into the machine reverse of how it is in

the 500 and newer version 2000; on the old version, pin 1 of the ROM

is facing forward. We have never seen the old revision Amiga 2000

except in pictures.  We figure that if you have the old version you

will have to mount The Multi-Kickstart Board Project upside down with

the red stipe facing forward, keep this in mind when measuring the

length of your cable.  On a 500 the length you choose will depend on

what other items you may have stuffed under your 500's cover.  After

you get a good length, add about a half an inch for soldering

purposes.

[ ] 9.  Now cut the cable.

[ ] 10.  Once the cable is cut, peel the wires away from each other

about a half inch.  See cable drawing "TMKBP31Ribbon.iff".

[ ] 11.  Strip the ends of all the wires about 1/8 of an inch.

[ ] 12.  Tin the ends of the wires with solder.

[ ] 13.  Pin forty of the cable is the wire with the red stripe.  See

cable drawing "TMKBP31Ribbon.iff" for wire and hole numbering.  We

will call the top of the cable the part that faces up when the teeth

of the connector points down.  Starting with the red wire, bend every

other wire up at a forty five degree angle and the other wires down at

a forty five degree angle.  This zig-zag pattern will allow the cable

to fit in the holes labeled CN1.

[ ] 14.   Place the wires in the holes by matching the numbers in the

drawing "TMKBP31Ribbon.iff" and then solder the wires to the board.

Refer to tips 8, 9, and 10.

[ ] 15.  The last thing you need to do is assemble the switch harness.

The switch harness consists of the three position switch and the four

wire ribbon cable that connects to it.  Peel each wire on the four

wire switch cable at both ends about half an inch.

[ ] 16.  Strip those wires.

[ ] 17.  Tin those wires.

[ ] 18.  Following the drawing "TMKBP31Switch.iff", solder the wires,

as shown, to the switch.  In operation the switch should allow only

one wire at a time to be connected to ground (GND).

[ ] 19.  Now that the switch harness is finished, solder its wires to

the board using the numbers on the board and the drawing

"TMKBP31Switch.iff" as a guide.  Refer to tips 8, and 9.

[ ] 20.  Place the 74LS32 (U1) into the U1 socket.  Be VERY careful

that you do not bend any of the pins.  Make sure that you insert the

chip in the right direction.  Pin 1 at the bottom left.  Pin 1 is at

the end of the chip that is marked with a notch.

We are now finished with assembly of The Multi-Kickstart Board

Project.

----------------------------------------------------------------------

Testing Of The Multi-Kickstart Board Project:

Before you place your ROMs into any of the sockets it is a good idea

that you run through this little test to make sure that the logic

circuitry on the board is operating correctly.  Running through this

test will help pin point and solve any problems before they happen.

For this test you will need to obtain the parts and construct a simple

logic probe as shown in the drawing "TMKBP31Probes.iff". If you are

not able to get the parts to build the logic probe, or you do not want

to test what you have just built, then skip the rest of this section

and go on to the section "Installation Of The Multi-Kickstart Board

Project"

[ ] 1.  Apply +5 volts to the left side of one of the capacitors and

ground to the right side of one of the capacitors.  This will energize

the board.

[ ] 2.  Place the (+) probe of the logic probe on pin 14 of U1 and the

(-) probe on pin 7 of U1.  The LED should light.  If the LED does not

light then U1 is not receiving power.  If this is the case disconnect

power and look for an open or short circuit and fix it.  Once you pass

this test go to the next step.

[ ] 3.  Connect pin 12 of the ribbon cable to +5 volts.  Next take the

(-) probe and touch pin 12 and take the (+) probe and touch pin 21 of

each ROM socket.  The LED should stay off at each of these points.

Repeat this test for each switch position.  If the LED turns on at any

of these points re-check the switch wiring and board traces.  If all

the wiring is okay, the 74LS32 is bad, replace the chip and test

again.  Once you pass this test go to the next step.

[ ] 4.  Disconnect the +5 volts from pin 12 of the ribbon cable and

connect pin 12 of the ribbon cable to ground.  Take the (-) probe and

touch it to pin 12 and the (+) probe and touch pin 21 of each ROM

socket.  With the switch in position one, pins 12 and 21 of socket

ROM1 should be the only ones to light the LED.  Place the switch to

position two, touch pins 12 and 21 of all the ROM sockets, and now

pins 12 and 21 of ROM2 should be the only ones to light the LED.

Place the switch to position three, touch pins 12 and 21 of all the

ROM sockets, now pins 12 and 21 of ROM3 should be the only ones to

light the LED.  Repeat this test several times with the switch in

different positions to insure proper operation.  If the board failed

this test recheck the wiring of the switch and check the board for

opens and shorts, and fix the problem.  If you do not find any opens,

shorts or other problems replace the 74LS32 (U1) and retest. Once you

pass this test, disconnect the ground from pin 12 of the ribbon cable.

Your project is now ready to install in your Amiga computer.

----------------------------------------------------------------------

Installation Of The Multi-Kickstart Board Project:

The instructions below are based on the instructions that Commodore

provides with the 2.04 Kickstart ROM.  We added more steps to the

installation instructions so that they could be used for The Multi-

Kickstart Board Project.  Installation instructions for both the 500

and the 2000 are provided.

- - - - - - - - - - - - - - - - - - - -

Installation Instructions For Amiga 500 Systems:

[ ] 1. With power disconnected and the 500 upside down, remove the six

screws holding the case together with a #10 torx driver.

[ ] 2. Place the 500 right side up and carefully remove the top cover.

[ ] 3. Disconnect the keyboard harness and the ground strap.

[ ] 4. Using a pliers, bend up the four tabs that hold the shield,

then remove the four screws that also hold the shield.  The shield

should come out by lifting it straight up.

[ ] 5. The Kickstart ROM will be located to the right of the 68000,

in the U6 position.  It should be marked 315093-01 or 315093-02.

[ ] 6. Remove the Kickstart ROM from the motherboard and place it in

any one of the ROM sockets on The Multi-Kickstart Board Project.  Also

place your 2.04 Kickstart ROM in any one of the other ROM sockets.

The placement of the ROMs is up to you.  Place them in the positions

that you feel comfortable with.  The ROM positions 1, 2, and 3 match

the switch positions 1, 2, and 3.  Refer to tips 11, and 12.

[ ] 7. There are many different versions of the Amiga 500 motherboard.

You will have to determine which mother board you have and make

modifications accordingly.  The revision number of the mother board is

printed on the top right front corner of the mother board.

[ ] 8. If you have a Rev 3 or a Rev 5 mother board you must have

pins 1 and 31 of the ROMs jumpered together.  In this case you do not

have to do anything because The Multi-Kickstart Board Project already

has this done at JMP1.

[ ] 9. If you have a Rev 6A or higher motherboard you must cut the

jumper at JMP1 on The Multi-Kickstart Board Project with an X-acto

knife or something equivalent (refer to the drawing "TMKBP31Board.iff"

to find the jumper).  If the 2.04 Kickstart ROM comes with a jumper

connecting pins 1 and 31 you must also cut or remove this jumper.

[ ] 10. Place the board gently inside the Amiga.  Take the 40 pin DIP

connector and insert it into the ROM socket of the mother board.  The

red stripe on the cable should be to the back of the computer and The

Multi-Kickstart Board Project should be sitting to the right of the

68000.  Pin one of the ROM socket is in the top left corner.  When

inserting the DIP connector make sure that you do not bend any of the

pins on the connector or misalign the connector.  After inserting the

DIP connector examine the connection to make sure it was done properly

and that no bent pins exist.  Refer to tips 11 and 12.

[ ] 11. Now comes  the time for you  to mount the board.  On one of

our computers we simply glued a piece of nonconductive and nonstatic

foam underneath The Multi-Kickstart Board Project and simply let the

board sit on top of the mother board.  On another computer we glued

plastic feet to the board and let it sit on the mother board.  It is

okay to mount the board any way that is acceptable to you as long as

you do not short any lines on the mother board or The Multi-Kickstart

Board Project.

[ ] 12. The next thing you need to do is run the cable for your switch.

It is best to run the cable through one of the holes in the back of

the computer.  For the most part any way of running the cable should

be okay as long as you don't pinch or scrape the  wire on any sharp

piece of metal.  Refer to tip 12.

[ ] 13. Before putting your computer completely back together it

might be a good idea to confirm that the system is working properly

with The Multi-kickstart Board Project installed.  Put the computer

together to the point where it will work.  Set the switch to a ROM

position and turn on the computer.  The computer should boot as normal

into that version of Kickstart.  Turn off the computer and set the

switch to the next ROM position.  Turn on the computer again; the

computer should now boot into that version of Kickstart.  Repeat this

test for all the ROMs that you have installed.  If the computer does

not operate as expected check your installation.  If the installation

seems okay and the system still does not work as expected, repeat the

tests on The Multi-Kickstart Board Project (in the section "Testing Of

The Multi-Kickstart Board Project"). If everything worked out okay and

the ROMs switch properly, you can now put your computer back together,

put your tools away and play.  Refer to tip 12.

- - - - - - - - - - - - - - - - -

Installation Instructions For Amiga 2000 Systems:

[ ] 1. With the power off, remove the cover of the 2000 by taking out

the five screws holding the cover to the body of the computer.  Take

the cover off by sliding it toward the front and up.

[ ] 2. Remove the power supply and disk drive assembly by taking out

the seven screws holding it to the body.  You should also disconnect

the power supply and drive cables from the motherboard.

[ ] 3. You must now determine the revision of the mother board and

the location of the Kickstart ROM.  The older version of the 2000

mother board has " 1986 COMMODORE","AMIGA 2000" and "MADE IN GERMANY

(BSW)" on the left side of the board.  The Kickstart ROM is located

at U2, just to the right of the 68000 with pin 1 toward the front of

the machine.

The newer version of the 2000 motherboard is marked with "B2000-CR" in

the lower left corner.  The Kickstart ROM is located at U500, which is

located just to the right of the 68000 and below Fat Agnus.  Pin 1 of

the ROM is located toward the back of the machine.

The ROM should be marked with the number 315093-01 or 315093-02.

[ ] 4. Remove the Kickstart ROM from the mother board and place it in

any one of the ROM sockets on The Multi-Kickstart Board Project.  Also

place your 2.04 Kickstart ROM in any one of the other ROM sockets.

The placement of the ROMs is up to you.  Place  them in the positions

that you feel comfortable with.  The ROM positions 1, 2, and 3 match

the switch positions 1, 2, and 3.  Refer to tips 11,and 12.

[ ] 5. For the older version motherboards you must have pins 1 and 31

of the ROMs jumpered together.  In this case you do not have to do

anything because The Multi-Kickstart Board Project already has this

done at JMP1.

[ ] 6. If you have the newer version mother board you must cut the

jumper at JMP1 on The Multi-Kickstart Board Project with an X-acto

knife or something equivalent (refer to the drawing "TMKBP31Board.iff"

to find the jumper). If the 2.04 Kickstart ROM comes with a jumper

connecting pins 1 and 31 you must also cut or remove this jumper.

[ ] 7. Place the board gently inside the Amiga.  Take the 40 pin DIP

connector and insert it into the ROM socket of the mother board.  The

red stripe on the cable should be to the back of the computer if you

have the newer version mother board, and toward the front of the

computer if you have the older version mother board. When inserting

the DIP connector make sure that you do not bend any of the pins on

the connector or misalign the DIP connector.  After inserting the

connector, examine the connection to make sure it was done properly

and that no bent pins exist.  Refer to tips 11, and 12.

[ ] 8. Now comes the time for you to mount the board.  On one of our

computers we simply glued a piece of nonconductive and nonstatic foam

underneath The Multi-Kickstart Board Project and simply let the board

sit on top of the mother board.  On another computer we glued plastic

feet to The Multi-Kickstart Board Project and let it sit on the mother

board.  It is okay to mount the board any way that is acceptable to

you as long as you do not short any lines on the mother board or The

Multi-Kickstart Board Project.  On the newer version of the mother

board, The Multi-kickstart Board Project will most likely sit with the

component side up.  Because pin 1 on the older version mother board is

facing forward, you will most likely have to flip the board over and

mount it with the component side down.  In both cases The Multi-

Kickstart Board Project will most likely sit to the right of the

68000.  Refer to tip 12.

[ ] 9. The next thing you need to do is run the cable for your switch.

It is best to run the cable through one of the holes in the back of

the computer.  For the most part, any way of running the wire should

be okay as long as you don't pinch or scrape the  wire on any sharp

piece of metal.  Refer to tip 12.

[ ] 10. Before putting your computer completely back together it might

be a good idea to confirm that the system is working properly with The

Multi-Kickstart Board Project installed.  Put the computer together to

the point where it will work.  Set the switch to a ROM position and

turn on the computer.  The computer should boot as normal into that

version of Kickstart.  Turn off the computer and set the switch to the

next ROM position.  Turn on the computer again, the computer should

now boot into that version of Kickstart.  Repeat this test for all the

ROMs that you have installed.  If the computer does not operate as

expected check your installation.  If the installation seems okay and

the system still does not work as expected, repeat the tests on The

Multi-Kickstart Board Project (in the section "Testing Of The Multi-

Kickstart Board Project").  If everything worked out okay and the ROMs

switch properly, you now can put your computer back together, put away

your tools and play. Refer to tip 12.

----------------------------------------------------------------------

Tips For Building The Multi-Kickstart Board Project:

When building projects like this for the first time, it is very easy

to make mistakes and errors.  I've built many electronic projects and

have found many different ways of doing things that save both time and

headaches.  I have already built a few of these kits so that I could

get a good feel of what needs to be done in order to make this a

smooth and easy project to put together.  Below I have put together a

few tips that should help you greatly in putting this kit together. A

few of them are repeats from what I said earlier in the text.

1. Make sure all the parts NOT marked "OPTIONAL" in the parts list are

in the kit.

2. Read all instructions before you ever start a project so that you

can get a feel of what you will need to do.

3. Do not try to finish the project in one sitting.  If you rush

through it you will probably make some mistakes.

4. Plan out how you are going to do something before you do it.

5. Allow the soldering iron to heat up prior to starting and keep the

tip of the iron clean with a damp cloth.

6. Do not put blobs of solder on the board.  Put only enough solder to

get the job done.  The solder should flow between the pad and the

component's lead.

7. When soldering, do not allow the iron to stay on the board for more

than ten seconds.

8. When cutting, peeling, and stripping, ribbon cable or any other

wire, measure and mark the cable prior to doing any cutting or

stripping.  Use a permanent fine point marker to mark the points of

cutting, peeling, and stripping.  Cut the wire at the first marked

point, peel the cable to the third marked point, and strip the wire at

the second marked point.  If you do this you will come out with a

neatly prepared cable ready for installing.

9. It is a good idea to tin a wire with solder right after its been

stripped so that the fine strands of wire don't break off or frizz

out.

10. So that you don't peel your ribbon cable down farther than you

want to go, a good idea is to place a piece of tape at the point you

would like the peeling to stop.  The tape will not allow you to go

past that point.

11. Before turning on the power, always check for bent pins and proper

alignment after installing ICs.

12. Because the inside of your computer is a static sensitive

environment, do not touch any parts inside your computer unless you

have grounded yourself. To ground yourself simply touch the grounded

case of your computer or attach yourself to a grounded strap.

----------------------------------------------------------------------

If you have any problems or questions we can be reached over the modem

at:

UUCP: {hplabs!hp-sdd ucsd nosc}!crash!pnet01!neil

ARPA: crash!pnet01!neil@nosc.mil

INET: neil@pnet01.cts.com