T h e   I O   K e y p a d / L C D   D e m o

(0) OVERVIEW

This demo is meant to be used with the io.lib software.  For me, its

primary function has been to verify and test the keypad and LCD hardware, and

the associated circuitry.

(1) COMMAND SUMMARY

To run this demo, you will either need (1) compatible hardware to use with

io.lib or (2) to write yout own emulator for the io.lib routines.  The emulator

could be, for instance, a set of routines that communicate with a client over

an RS-232 line.

You will need to locate the keys that correspond to the numeric values 0 to

15 before using this demo, since all the commands are executed from the keypad.

IO Demo Command Summary

Command                   Function

0 NN                      cputs(NN)

1                         clear

2                         home

3 N                       set_cursor(N)

4                         cursor_left

5                         cursor_right

6                         shift_left

7                         shift_right

8 RC                      locate(Row, Col)

9                         scr_init

10                        cputs("This is a test")

11                        save_cursor

12                        restore_cursor

The numerical inputs, NN, N and RC are keyed in from the keypad following

the command, one hexadecimal digit at a time.  The keys 0 to 15 are mapped

onto the hexadecimal digits 0-9, a-f respectively.  The numbers NN and RC

are two hexadecimal digits, and N is one.

Because of the constraints on the command format,  it will not be possible

to "locate" past column 15 using the command 8.  The first hex digit (R) is

the row where the cursor is to be located, and the second is the cursor's

new column, which cannot be larger than 16.

The commands listed above are all described in IO.doc.