RTC-31 Monitor Firmware

Original code (c) 1989 Ron Stubbers

Modified 3/90 by: 	Robert Gallahan

11300 Melody Dr. #307

Northglenn, CO 80234

VERSION 1.1

Changes & additions:

1.   Now  displays  menu  only  at  power-up.  Menu  may  be

displayed  anytime  by simply hitting <RETURN>  or  when  an

unrecognized command is received.

2.   Added  screen  clear  (control L) to the  menu  text.  If  a

formfeed  is  not appropriate for your terminal,  this  may

need  to  be changed or may be deleted if  you  prefer.  For

those  of  you without an assembler, the object code  is  at

locations

3.   Added  ASCII  display of characters to external  RAM  memory

dump.

4.   Added X command to display register banks and internal RAM.

5.   Provided  support for interrupts as follows:

When an interrupt occurs, control is unconditionally  passed

to  a  corresponding  location  in  RAM.  For  example,  external

interrupt  0 (INT0 pin) normally vectors to location 0003h.  This

is,  of  course,  in  the EPROM.  The  monitor  contains  a  jump

instruction  at  location  0003h which  transfers  control  to  a

corresponding location in RAM. As written, this would be location

2003h   (RAMBASE  +  hardware  vector  location.)   The   monitor

initializes  the  first 5 RAM vectors with a  RETI  (return  from

interrupt).  The first free location after the interrupt  vectors

is 2024h (RAMBASE + 23h +1.)

If  interrupts  are  utilized,  the  user's  program  should

overlay  the  RETI  instructions  with  appropriate  code  before

enabling interrupts. The monitor program does not enable, disable

or  alter  the  interrupt  system  in  any  way  except  for  the

initalization described above.

If interrupts are not enabled, all of RAM may be used in any

manner at the programmer's discretion.

6.   Added equates and modified menu's banner for revision  level

control.

BAUD RATE:	If you don't have the PseudoSam assembler & need

to change the baud rate, here's where to patch the object code:

004B 758DFD           	MOV	TH1,#h'FD 	;#0FDH=9600 BAUD

Byte at 004Dh controls baud rate, initally set for 9600

Other rates:  4800 use FA, 2400 use F4, 1200 use E8

END OF MON31-11.DOC

################################################################################