INTEL INSITE LIBRARY FILE BF10:

TB51 V2.2 (MCS-51 TINY BASIC)

FUNCTION:

TB51 is an interpreter program for a language based on the Palo Alto

Tiny Basic specifications modified for Intel's MCS-51 family of

single-chip microcontrollers.

REQUIRED HARDWARE:

An 8051 or 8751 with the interpreter program in the ROM or EPROM, CRT,

level shifters on the serial I/O pins.  External program and/or data

memory may be added to develop and execute larger programs.  The

interpreter program occupies 4K (3K if the included BASIC example

program is eliminated), and can operate in a limited manner using only

the 128 bytes of RAM available on the 8051.  Full use is possible

only with the addition of external RAM.

SOURCE LANGUAGE:

ASM-51 V 2.0

Page 1

Intel MCS-51 Tiny BASIC

-----------------------

In 1976, Dr. Dobb's Journal of Computer Calisthenics and Orthodontia

proposed a subset of the BASIC language which would be appropriate for

small microcomputer systems.  One early version of the language (dubbed

"Palo Alto Tiny BASIC") was defined and implemented by Dr. Li-Chen Wang,

and was later adapted for the 8085 and 8088 MPUs.

TB51 is an interpreter program for a language based on the Palo Alto Tiny

BASIC specification which has been implemented for the Intel MCS-51 family of

single-chip microcontrollers.  Extensions to the language facilitate hexa-

decimal arithmetic, logical operations, and bit manipulation for controller-

oriented applications.  Rudimentary system monitor capabilities are also

provided.  Some minor changes have been made to make TB51 more closely resemble

BASIC-80, PLM-80, and the ISIS-II operating system.

TB51 operates in a number of modes to support various hardware configura-

tions.  The simplest configuration requires only an 8051 or 8751 single-chip

microcomputer containing the interpreter program in on-chip ROM/EPROM, a CRT,

and level shifters on the microcomputer serial I/O pins.  External program

and/or data memory may be added by the user to develop and execute larger pro-

grams.  The following discussion defines the operation of TB51 initially

assuming there is at least one page (256 bytes) of external RAM provided,

describes the difference between the DECIMAL and HEX operating modes, explains

the additional system monitor capabilities, and lists the special considera-

tions for true single-chip operation.

TB51 has an auto-baud detection algorithm.  Following system reset, typing

either a space character or a lower-case "c" once or twice will initialize the

baud rate.  Which character works, and how many times it must be typed, is a

function of the baud rate of the connected terminal, and of the system clock

rate.  Sometimes other characters work as well.

If the RxD serial I/O pin is jumpered to ground, TB51 will automatically

begin execution of a program stored in external PROM following a reset, and the

auto-baud detection will be skipped.

Page 2

THE LANGUAGE

------------

Numbers

All numbers are signed integers and must be between -32767 and 32767.

BASIC Variables

There are 26 variables denoted by the letters A through Z.  These are

represented internally as 16-bit, two's-complement integers.

Functions

There are 2 functions:

ABS(X) gives the absolute value of X.

RND(X) gives a random number between 1 and X (inclusive).

Arithmetic, Logical, and Compare Operations

All operations are performed to 16 bits of precision.  Arithmetic operations

which overflow 15 bits of magnitude will produce an error message.  The arith-

metic operators are:

+  addition.

-  subtraction.

*  multiplication.

/  integer division  (note that 14/5 = 2).

MOD  remainder from division (14 MOD 5 = 4).

The logical operators are:

NOT  bit-wise logical complement (NOT 0 = -1).

AND  bit-wise logical AND (3 AND 6 = 2).

OR  bit-wise logical OR (3 or 6 = 7).

XOR  bit-wise logical Exclusive - OR (3 XOR 6 = 5).

The compare operators are:

>  greater than.

<  less than.

=  equal to.

<>  not equal to.

>=  greater than or equal to.

<=  less than or equal to.

Arithmetic and logical operations result in a value between -32767 and

32767.  (-32768 is also allowed in some cases.)  All compare operators

result in a 1 if true and a 0 if not true.

Page 3

Expressions

Expressions are formed with numbers, variables, and functions with

arithmetic, logical, and compare operators between them.  Operations are per-

formed with four levels of precedence.  The first step is to perform Unary

operations (+, -, NOT), then Multiplicative operations (*, /, MOD, AND),

additive operations (+, -, OR, XOR), and finally,

Relational operations (>, <, =, <>, >=, <=).

Within each precedence level, the value of an expression is computed from

left to right.  Parentheses can also be used to alter the order of evaluation.

Note that compare operators can be used in any expression.  For example:

10  LET A = (X>Y) * 123 + (X=Y) * 456 + (X<Y) *789

20  IF (U=1) AND (V<2) or (U>V) AND (U<99) PRINT "YES"

30  LET R = RND(100): A = (R>3) + (R>15) + (R>56) + (R>98)

In statement 10, A will be set to 123 if X>Y, to 456 if X=Y, and to 789 if

X<Y.  In statement 30, Y will be a random number between 0 and 4 with a pre-

scribed probability distribution of: 3% being 0, 15 - 3 = 12% being 1,

56 - 15 = 41% being 2, 98 - 56 = 42% being 3, and 100 - 98 = 2% being 4.

Page 4

Direct Commands

Commands may be entered in upper or lower case, but the letter will be con-

verted to upper case.  All the commands described later can be used as direct

commands except the following three; they should only be used as direct commands

and not as part of a statement:

RUN

Will start to execute the program starting at the lowest statement number.

LIST

Will print out all the statements in numerical order.

LIST 120

Will print out all the statements in numerical order starting at statement

120.

NEW

Will delete all statements and set variables A-Z to zero.

Abbreviation and Blanks

You may use blanks freely, except that numbers, command key words, and func-

tion names can not have embedded blanks.

You may truncate all commands keywords and function names and follow them by

a period.  "P.", "PR.", "PRI.", and "PRIN." all stand for "PRINT".  Also, the

words LET in the LET command and THEN in the IF command can be omitted.  A var-

iable name by itself will print the value of that variable on the console.

The "shortest" abbreviation of all command keywords are as follows:

C. = CALL            D. = DECIMAL     E. = END

F. = FOR             G. = GOTO      GOS. = GOSUB

H. = HEX             I. = IF         IN. = INPUT

L. = LET(Optional)  LI. = LIST        N. = NEXT

NEW = NEW             P. = PRINT     PRO. = PROM

R. = RETURN         RA. = RAM       RES. = RESET

RO. = ROM            RU. = RUN        REM = REMARK

T. = TO              T. = THEN(Optional)

Implied = LET   Variable name = PRINT variable

Page 5

The following abbreviations are the shortest allowed within a variable name

or expression:

A. = ABS         A. = AND         C. = CBYTE

D. = DBYTE       M. = MOD         N. = NOT

O. = OR          R. = RBIT       RN. = RND

X. = XBYTE       X. = XOR

Statements

A statement consists of a statement number between 1 and 32767 followed by

one or more commands.  Commands in the same statement are separated by a

colon ":".  "GOTO", "STOP", "FOR", and "RETURN" commands must be the last

command in any given statement.

Commands

Tiny Basic commands are listed below with examples.  Remember that commands

can be concatenated with colons.  In order to store the statement, you must

also have a statement number in front of the commands.  The statement number

and the concatenation are not shown in the examples.

REM or REMARK Command

REM anything goes

This line will be ignored by TB51

LET Command

LET A = 234 - 5 * 6: A = A/2: X = A - 100

Will set the variable A to the value of the expression 234 - 5*6 (i.e., 204),

set the variable A (again) to the value of the expression A/2 (i.e., 102), and

set the variable X to the value of the expression A - 100 (i.e., 2).

LET U = A <> B: V = (A>B) * X + (A<B) * Y

Will set the variable U to either 1 or 0 depending on whether A is not equal

to or is equal to B; and set the variable V to either X, Y or 0 depending on

whether A is greater than, less than, or equal to B.

Page 6

Sequential variables may be initialized by separating sequential expressions

by commas.

LET A = 1, 2, 3, A, B * C

Will set variable A to 1, B to 2, C to 3, D to 1 (the updated contents of A),

and E to 6.

PRINT Command

PRINT

Will cause a carriage-return (CR) and a line-feed (LF) on the output device.

PRINT A * 3 + 1, "ABC 123 !@#", 'CBA'

Will print the value of the expression A * 3 + 1 (i.e., 4), the string of

characters "ABC 123 !@#", and the string "CBA", and then a CR-LF.  Note that

'single' or "double" quote marks may be used to bracket a string.

PRINT A * 3 + 1, "ABC 123 !@#", 'CBA',

Will produce the same output as before, except that there is no CR-LF after

the last item is printed.  This enables the program to continue printing on the

same line with another "PRINT".

Each comma in the above examples causes one or more spaces to be printed

until the cursor has reached a tabulation column, which occur every eight

spaces.  All spaces between variables and character strings may be eliminated

by using semicolons instead of commas.  (There are no spaces inserted auto-

matically before or after numbers or strings.)

PRINT , , , "X="; :X = -1: PRINT X; "!"

Will print the five characters "X=-1!" on one line, starting in column 25.

Page 7

INPUT Command

INPUT A,B

When this command is executed, Tiny BASIC will print ":" and wait to read in

a number from the input device.  The variable A will be set to this value.  A

tab and another ":" is printed and variable B is set to the value of the next

number read from the input device.  A number being entered may be terminated

with any non-numeric character, such as a space or <CR>.

INPUT "WHAT IS THE WEIGHT", A, "AND SIZE", B

This is the same as the command above, except the prompt ":" is replaced by

"WHAT IS THE WEIGHT:" and the second ":" is replaced by "AND SIZE:".

INPUT A, 'STRING'; "ANOTHER STRING", B;

The strings and semicolons have the same effect as in "PRINT".

IF Command

IF A < B LET X = 3: PRINT 'THIS STRING'

Will test the value of the expression A < B.  If it is odd (i.e., if it is

true), the commands in the rest of this statement will be executed.  If the

value of the expression is even or zero (i.e., if it is not true), the rest of

this statement will be skipped over and execution continues at the next state-

ment.  The keyword "THEN" may optionally be used after the expression.

GOTO Command

GOTO 120

Will cause the execution to jump to statement 120.  Note that the GOTO

command cannot be followed by a colon and other commands.  It must be ended

with a CR.

Page 8

GOTO A * 10 + B

Will cause the execution to jump to a different statement number as computed

from the value of the expression.

GOSUB and RETURN Commands

GOSUB command is similar to GOTO command except that the current statement

number is remembered.

GOSUB 120

Will cause the execution to jump to statement 120.

GOSUB A * 10 + B

Will cause the execution to jump to different statements as computed from the

value of the expression 10 * A + B.

RETURN

A RETURN command must be the last command in a statement and followed by CR.

When a RETURN command is encountered, it will cause the execution to jump back

to the command line following the most recent GOSUB command.

GOSUB can be nested.  The depth of nesting is limited only by the stack

space.

FOR and NEXT Commands

FOR X = A + 1 TO 3 * B

The variable X is set to the value of the expression A + 1.  The value of the

expression (not the expression itself) 3 * B is remembered.  The name of the

variable X and the statement number are also remembered.  Execution continues in

the normal way until a NEXT command is encountered.

Page 9

NEXT X

The name of the variable (X) is checked with that of the most recent FOR com-

mand.  If they do not agree, an error message is generated.  When a match is

found, this variable will be set to its current value plus 1.  The updated value

is then compared with the value of the TO expression also saved by the FOR com-

mand.  If this is within the limit, execution will jump back to the command

following the FOR command.  If this is outside the limit, execution continues

following the NEXT command itself.

FOR can be nested.  The depth of nesting is limited only by the stack space.

END Command

END

This command stops the execution of the program and returns control to direct

commands from the input device.  It can appear many times in a program but must

be the last command in any given statement, i.e., it cannot be followed by a

colon and other commands.  END is optional as the last line of a program.

Stopping the Execution

The execution or listing of the program can be aborted by pressing the

Control-C key on the input device.  The command in progress or line being listed

will first be completed.  Output to the console may be momentarily stopped by

typing Control-S.  Output will resume when Control-Q is typed.

RESET Command

RESET

Will restart the TB51 interpreter from the beginning, as if a hardware reset

were performed.

Page 10

Error Report

There are only three error conditions in TINY BASIC.

(1) WHAT? means it does not understand you.  Example:

210  PTINT "THIS"        where PRINT is mistyped

When this line is executed, TB51 will halt and print "WHAT?".

(2) HOW? means is understands you but does not know how to do it.  Examples:

310  LET A = B * C + 2   where B * C is greater than 32767

410  GOTO 412            where 412 does not exist

When these lines are executed, TB51 will halt and print "HOW?".

(3) SORRY means it understands you and knows how to do it, but there is not

enough memory to do it.

Note: TB51 does not retype the erroneous statement or indicate where the

error occurred.

Page 11

Error Corrections

In interactive command or line insertion mode, line editing capabilities

similar to ISIS-II are allowed.  If you notice an error in typing before you

hit CR, you can delete previously typed characters with the rubout key (ASCII

127).  TB51 will echo a BKSP-SP-BKSP for each rubout.  A line may be retyped by

hitting Control-R, or cancelled by typing Control-X.  A BELL will be echoed

when the input line buffer is full (32 characters).

When numbers are being entered during the execution of an input command, the

rubout key will be echoed by a "#" and CR, the current entry will be aborted,

and a new number may be entered.

To correct a statement, you can retype the statement number and the correct

commands.  Tiny BASIC will replace the old statement with the new one.

To delete a statement, type the statement number and a CR only.

Verify the corrections by "LIST nnnn" and hit the Control-C key as soon as

the printing of the line begins.  (Note that TB51 will not honor the Control-C

until it completes the current line.)

Page 12

BASIC Program Buffer Data Structure

Each BASIC statement is stored in the program buffer using the following for-

mat.  At the beginning of the line is a two-byte field giving the 16-bit binary

value of the statement number, high-order byte first.  Thus line number 10 will

be stored as 00H, 0AH.  These numbers may range from 0000H to 7FFFH.  Then comes

an arbitrary number of bytes giving the ASCII codes for the characters compris-

ing any legal sequence of commands.  At the end of each line comes the byte 0DH,

the ASCII representation for a carriage return.  Following the CR after the last

source statement, in place of what would be the high-order byte of the following

statement number, is the data byte 0FFH.

RAM, ROM, PROM

TB51 provides for BASIC programs to be buffered in three different address

spaces.  Following a reset or the RAM command, programs entered from the key-

board will be buffered in up to 4K bytes of external RAM, beginning at location

2000H.  By typing the command ROM, the RAM buffer will be disabled and a BASIC

program included with the TB51 4K byte ROM code will be activated.  Thereafter

this program can be LISTed, RUN, terminated by Control-C, and so forth, but ob-

viously cannot be modified from the keyboard.  Similarly, a user-written BASIC

program stored in an external PROM or EPROM may be activated by typing PROM.

This program is assumed to begin at location 1080H, and may be up to 60K bytes

long.  It is also assumed that this PROM be addressed as 8051 program memory.

For example, a program to print the first 10 squares:

10  FOR A = 1 TO 10

20  PRINT A * A: NEXT A

could be translated for an external EPROM using ASM51 and the following assem-

bly language source lines:

ORG  1080H

DW  10

DB  'FOR A = 1 TO 10', 0DH

DW  20

DB  'PRINT A * A: NEXT A', 0DH

DB  0FFH

(Note -- to insert an apostrophe into a text string such as those above, ASM51

requires two adjacent apostrophes in its place.)

Page 13

A program located in external PROM will be enabled and executed automatically

following a reset if the RxD pin is jumpered to ground.  As noted earlier, the

auto-baud detection routine will be skipped, so if the program in PROM needs to

use serial communications, it will have to initialize the appropriate registers.

The above has been a discussion of the BASIC-like aspects of TB51.  In addi-

tion to these features, the following monitor-like capabilities are provided.

Note that these capabilities are fully compatible with, and may be freely inter-

changed with, the commands and statements described thus far.

DBYTE, RBIT, CBYTE, XBYTE Variable Arrays

In addition to the 16-bit BASIC variables denoted A through Z, various 8051

system resources may be read or written as 8-bit or one-bit positive variables.

Anywhere an alphabetic variable name is allowed, these variables may be accessed

by a keyword representing a memory space followed by a number, simple variable,

or parenthesized expression indicating the address of the byte or bit desired.

The addresses interpreted by the DBYTE and RBIT commands are the same as those

used by the 8051 CPU for direct byte or bit addressing.  Addresses between 0 and

127 access the on-chip RAM array; between 128 and 255 access bytes or bits in

the special function register space.  (Only the low-order eight bits of the spe-

cified address are significant.)  The decimal addresses of accessible registers

are:

SFR     DBYTE      SFR     DBYTE

---     -----      ---     -----

P0      128        SCON    152

P1      144        SBUF    153

P2      160        IE      168

P3      176        IP      184

TCON    136

TMOD    137

TL0     138

TL1     139

TH0     140

TH1     141

Attempting to read or write registers other than those above will cause an

error message.

Page 14

PRINT DBYTE 144

Will print the input data present at P1 to the console.

LET DBYTE 144 = DBYTE 8 + 1

Will read the contents of register 0 of register bank 1, increment that

value, and output it on P1.

PRINT DBYTE (DBYTE 8)

Will print the contents of the internal RAM location or SFR indirectly

addressed by the same register.

Reading a bit address will return a one or zero, depending on the state of

that bit.  The value written to a bit address is the least significant bit of

the expression specified.  Other bits of the affected register are not changed.

RBIT 144 = RBIT 145 AND NOT RBIT 146

Will set P1.0 only if P1.1 = 1 and P1.2 = 0.

XBYTE is used to access up to 64K bytes of external data memory controlled by

the 8051  RD and WR strobes.  CBYTE is used to read internal program memory at

addresses less that 4096, and external program memory controlled by the PSEN

strobe.  Writing to CBYTE addresses uses the WR strobe, the same as the XBYTE

command.

Notice that the implicit PRINT and LET commands can be used to examine reg-

isters and initialize sequential memory locations using a syntax similar to that

of Intel's In-Circuit Emulators.

DBYTE 140

Will print the current contents of the byte TH0.

XBYTE 8192 = 0, 1, 2, 3, 4, 5

Will initialize six bytes of external data RAM with values from 0 to 5.

Page 15

HEX, DECIMAL Commands

In the DECIMAL operating mode, variables are treated as two's-complement

signed decimal integers.  An error message is generated whenever the magnitude

of a number or expression exceeds 32767.  DECIMAL is the default radix follow-

ing a reset or the NEW command.

To facilitate monitor interaction, TB51 also has a hexadecimal operating mode

invoked by the command HEX.  In this mode all variables are treated as unsigned

16-bit values.  Numbers may be entered as a combination of decimal and hexadeci-

mal digits, provided the first digit is between 0 and 9.  If more than four dig-

its are entered, the higher order bits are ignored.  In the interest of compati-

bility with other Intel software conventions, a hex number may be optionally

followed by an "H", but its presence or absence will not affect the radix as-

sumed.  Numbers will be printed out in HEX mode as two to four hex digits fol-

lowed by an "H".  All arithmetic will be performed modulo 2^16 (65536), and

arithmetic overflow (other than division by zero) will be ignored.

Decimal operation may be restored by the command DECIMAL.

Caution must be exercised when changing modes while a program is being

developed.  Line numbers preceding stored commands are interpreted and saved

assuming the radix in effect as the line is entered.  Line numbers included in

a command (as in GOTO  20) are interpreted using the radix in effect as the

statement is executed.

CALL Command

CALL 8192 will call an 8051 machine language subroutine assumed to be present

starting at address 2000H.  This routine will execute until a RET instruction

(machine code 22H) is encountered, at which time execution of the BASIC program

will continue with the following command.

When the machine language program begins executing, register bank 3 will be

enabled, but all CPU registers will be undefined.  The 8051 stack pointer will

hold 57H; internal RAM locations 56H and 57H contain a return address to within

the TB51 interpreter.  RAM locations 68H - 7FH are available for the user's

stack.  In addition, register bank 2, register bank 3, and RAM locations 20H -

27H are not touched by TB51 (except in the single-chip mode described below) and

may be used by the machine language program.  Notice that the DBYTE accessing

capability allows these variables to be read, monitored, and written by the

interpreter programs.

Page 16

Single Chip Operation

As implied at the beginning of this discussion, the TB51 interpreter may be

used even in systems with no external RAM.  However, this naturally requires

certain limitations.

1.) With no program line buffering, BASIC and machine language programs can-

not be written and edited.  However, single and compound command lines may be

executed from the keyboard.

2.) Variables used by BASIC commands and expressions are automatically stored

in internal RAM, rather than external.  To conserve this RAM, only 12 such var-

iables (A-L) are supported, overlapping internal RAM locations 10H - 28H.  R0 of

register bank 1 would be the same location as the high order byte of variable E.

Conversion to internal variable memory is automatic, following a hardware reset.

Notice that these restrictions do not preclude the execution of programs

stored in internal ROM or external PROM, provided they are written to use only

variables A through L.

Page 17

Errata

TB51 release V2.2 (as identified in the sign-on message) contains the follow-

ing known bugs.  They should not seriously affect the program's usability.

1.) The random number generator seed is initialized to 0 on reset, and is

thereafter permuted by the recurrence relation

R(n) = [(25,173 * R(n-1)) + 13,849] mod 65536.

(reference "Programming in PASCAL" by Peter Grogono, pg. 117).  A characteristic

of this algorithm is that the low-order bits of the seed produce a cyclically

repeating pattern.  If the argument of the RND function is a low-order power of

two (such as 4, 8, or 16) then the values returned by the function will follow

a similar sequence.  One solution is to obtain a random number within wider

limits, then eliminate certain cases:

120  A = RND(3): IF A = 3 GOTO 120

Will produce a random number between 1 and 2.

2.) If the upper limit of a FOR-NEXT loop (the TO clause) is equal to the

maximum integer representable in 16 bits (32767 in DECIMAL mode, 0FFFFH in HEX

mode), the loop will not terminate.  TB51 increments the loop variable before

testing it, with the result that the loop variable can overflow, wrapping around

to the bottom of the number range (32767 becomes -32768, 0FFFFH becomes 0H).  It

then passes the comparison test to the upper bound of the loop.  For example:

120  FOR I = 32766 to 32767

Will loop forever, with I taking all values from -32768 to 32767.

3.) The value -32768 is legal when in DECIMAL mode, but cannot be printed or

input.  The following program will halt with a HOW? message:

10  A = -32767

20  A = A - 1

30  PRINT A

If statement 30 is changed to PRINT A + 1, the program will output -32767, in-

dicating that A actually has the value -32768 even though it cannot be dis-

played.  Furthermore, the following statements

10  A = -32768

20  INPUT A      where -32768 is typed in response to the input prompt

will both result in error messages when executed.