6-21-88

UN51.EXE is a general purpose disassembler for the INTEL 8051 series of

micro-controllers.  UN51.EXE expects its input from "standard-in" and

produces its output through "standard-out". General usage is as follows:

UN51 <hexfile.HEX >source.ASM

Input is assumed to be in standard INTEL-HEX.  The full 64k address space

is valid, and an optional offset value to be added to or subtracted from the

address specified in the INTEL-HEX records may be specified between the

INTEL- HEX records as "-0100" or "+2000" (examples).  Consider the case

where the object to be disassembled sits in two 2764 EPROMS: Using almost

any standard EPROM programmer, turn these chips into two INTEL-HEX files

each addressed from 0000h to 1FFFh inclusive.  Then delete the "end-of-file"

record from the first .HEX file and append the second to it.  Finally insert

the line "+2000" between the two.  UN51 will load this data to 8051

addresses 0000h through 03FFFh.

Once the input file has been created, invoke UN51 to produce an .ASM

file.  UN51 works by first assuming that the data at the 8051 hardware

interupt addresses is executable code, and any code referenced by this code

(i.e. jump, branch, or call instructions) is also code etc.  Any addresses

not so refereced are assumed to contain (unexecutable) data. Therefore: this

first .ASM file may not identify the destinations of indirect jumps (i.e.

jump table destinations) as executable code.  By hand analysing the .ASM

file try to identify these indirect jump destination addresses, and specify

them in the input file by adding lines after the INTEL-HEX "end-of-file"

record. Exaples:

>1A20 >07BE >001A >336F

>33D0

Addresses must be four digits hexadecimal immediately preceded by a '>'.

During hand analysis, if any subroutines are encountered which expect a

fixed length block of data following the call instruction, these may be

specifed anywhere after the INTEL-HEX "end-of-file" record as follows:

#1DE9+04

Where the subroutine's 8051 address is 01DE9h and it expects four bytes

(or two words) of data to follow the call instruction.

NOTE: variable length byte following code can be disassembled, but you

must write the code yourself and reassemble Un51.  This can produce

some very clean disassemblies though, (knowing the meaning of the

byte following data) and is a good approach.

When a jump, call, or branching instruction has a destination address

which has been disassembled as the data field of another instruction, the

destination address will be defined following that instruction as follows:

JP01DE7H:      EQU       $ - 2

Where the label JP01DE7H is the second byte of the preceding three byte

instruction.  This condition usually implies that the call, jump or branch

instruction which references JP01DE7H is not really executable code but was

specified as code by a >XXXX in the input file.

In any event, the output .ASM file should always assemble without error to

produce an exact copy of the original object.

For convenience, object which is disassembled as code will have in the

comment field of each line a reference to the address it should assemble to,

and the data bytes that should be assembled there; and object which is

disassembled as data will have in the comment field for each line a

reference to the code it would have disasembled to if it were not data, as

well as the address and data that instruction would have produced.

- Scott Jennings

#