Signetics Application Brief : 8051 Family Warm Boot Determination

-----------------------------------------------------------------

For some classes of operations, it may be desirable to know if the

application of the reset signal to a microcontroller is due to an

initial power-on sequence, or is the result of an external signal such

as an operator pressing a reset pushbutton, or the result of a watchdog

timer or similar event.

While there are perhaps numerous hardware solutions that can be

employed, a simple software solution can offer a high degree of

confidence in making this determination. The task is to determine the

differences in state of resources internal to the microcontroller that

would occur as a result of these two types of reset conditions. With

respect to the 80C51 family of microcontrollers, on-chip resources

consist of the special function registers (SFRs) and the internal data

memory (RAM). Most of the SFR locations are initialized as a result of

a reset condition and thus cannot be used for this determination. The

data memory contents are unaffected by reset. Thus, valid data loaded

into the RAM of the 80C51 while executing a program would not be

affected by the application of an external reset signal provided the

power source for the microcontroller has not been removed (as is the

case for a "warm boot").

The contents of data memory as a result of an initial application of

power, however, is indeterminate. While this effect has not been

extensively characterized, empirical observation suggests that it is

highly random in nature. If it is assumed, for the moment, that the

behavior of a given byte of data memory is such that it will power-up

with a value that is totally random, then there is a one in 256 chance

that it will power-up with a predetermined value. If the assumption is

extended to two bytes, a 16-bit number, then there is one in 2 to the

16th chance that both bytes will power-up with predetermined values.

Extending this to four bytes results in a one in 2 to the 32 chance; a

very small probability. This is the basis for the software

determination of a warm or cold boot condition.

The technique consists of evaluating the contents of four consecutive

bytes of data memory following a reset condition to determine whether

these bytes had been previously loaded with known data values. If the

contents of all four bytes match predetermined values, this is

interpreted to be a warm boot condition. If there is no match, it is

interpreted to be a cold boot condition. At this point, it is necessary

to load these four bytes with predetermined data to prepare for the

possibility of a subsequent warm boot condition.

The software example included in this application brief can be used to

perform this warm or cold boot determination.

The symbols WARM1 through WARM4 represent the predetermined values. The

symbol WARM is the address of the first of the four consecutive bytes

in data memory. It is set to 30H to avoid conflict with the four

register banks, the stack, and the bit-addressable locations in data

memory. The symbol WARMBT is a bit-addressable location used as a

status bit. It is set as the result of a warm boot and cleared as a

result of a cold boot.

The label START is the location of the first instruction to be executed

following a reset (address = 0000H). An instruction is located here to

jump into the main body of the program to bypass the interrupt vector

locations.

The main program body begins by loading register R0 with the address of

the first byte in data memory to be evaluated. The contents of this

first byte are compared with the first predetermined value. If there is

no match, the conclusion is that it is a cold boot. However, if a match

is found, this does not imply that it is a warm boot since all four

bytes must match, and therefore the remaining three bytes must also be

evaluated. Register R0 is incremented to point to the second byte and

then compared to the second predetermined value. Comparison of the

bytes proceeds until either a no match condition is found or until all

four bytes have been evaluated successfully. If all four bytes compared

favorably, then a status bit (WARMBT) is set to indicate a warm boot

and the remainder of the application program is completed.

An unsuccessful comparison results in branching to the label COLD. This

section of code clears the status bit (WARMBT) to indicate a cold boot,

and loads the four bytes of data memory with the predetermined values

preparing the system for a subsequent possible warm boot. Program flow

then continues with the remainder of the application program.