8051 forth cross assembler  documentation

This assembler assembles not to a file but to the PC's memory,

has a full set of conditionals.

One feature/bug of it, is the ability to open the editor

on the current file,  whenever an error has ocurred,

placing the cursor on the instruccion FOLLOWING the

wrong instruccion.

Every piece of 8051 assembly code must be located between a

CODE51: word and must end with a CODE51; word.  The definition of

constants can be made outside the CODE51: .... CODE51; pair.

Labels must be defined before they are used ( !!!! ), because

these are dumb labels and :

1) this is an one pass assembler.

2) smart labels can be done on one-pass assemblers

but i didn't worried about them, because this assembler

has a complete set of conditional instruccions.

List and syntax of the instruccions

On this list of instruccions the abreviations used are

these:

rn   = r0, r1,...., r7

@ri  = @r0, @r1

Normal assembler words

add a , byte

add a , # byte

add a , rn

add a , @ri

addc a , byte

addc a , # byte

addc a , rn

addc a , @ri

subb a , byte

subb a , # byte

subb a , rn

subb a , @ri

inc byte

inc a

inc rn

inc @ri

inc dptr

dec byte

dec a

dec rn

dec @ri

mul ab

div ab

da a

anl  a , byte

anl  a , # byte

anl  a , rn

anl  a , @ri

anl  byte , # byte

anl  byte , a

anl  c , bit

anl  c , / bit

orl  a , byte

orl  a , # byte

orl  a , rn

orl  a , @ri

orl  byte , # byte

orl  byte , a

orl  c , bit

orl  c , / bit

xrl  a , byte

xrl  a , # byte

xrl  a , rn

xrl  a , @ri

xrl  byte , # byte

xrl  byte , a

clr  bit

clr  a

clr  c

cpl  bit

cpl  a

cpl  c

rl   a

rlc  a

rr   a

rrc  a

swap a

mov  a , byte

mov  a , # byte

mov  a , rn

mov  a , @ri

mov  byte , byte

mov  byte , # byte

mov  byte , a

mov  byte , rn

mov  byte , @ri

mov  bit , c

mov  c , bit

mov  rn , byte

mov  rn , # byte

mov  rn , a

mov  @ri , byte

mov  @ri , # byte

mov  @ri , a

mov  dptr , # data16

movc a , @a+dptr

movc a , @a+pc

movx  @ri , a

movx  @dptr , a

movx  a , @ri

movx  a , @dptr

push  byte

pop   byte

xch  a , byte

xch  a , rn

xch  a , @ri

xchd a , @ri

setb bit

setb c

nop

ret

reti

Structured Control Flow Instruccions

( these ones will give your code an structured look and feel )

begin ... again

begin  .... until cond

if cond  ........ endif

if cond ....... else ......... endif

begin ....... while cond ....... repeat

Conditions

bit?   bit        \  bit set                   ex:  bit?  56

/bit?  bit        \  bit reset                 ex:  /bit? 33

Z?                \  zero flag set             ex:  z?

/Z?               \  zero flag reset           ex:  /z?

C?                \  carry flag set            ex:  c?

/C?               \  carry flag reset          ex:  /c?

A= byte           \  acc equal to byte         ex:  a= 22

A= # byte         \  acc equal to const        ex:  a= # 59

Rn= # byte        \  Rn equal to const         ex:  R5= # 90

@Ri= # byte       \  @Ri equal to const        ex:  @R1= # 67

A<> byte          \  acc different from byte   ex:  a<> 22

A<> # byte        \  acc different from const  ex:  a<> # 59

Rn<> # byte       \  Rn different from const   ex:  R5<> # 90

@Ri<> # byte      \  @Ri different from const  ex:  @R1<> # 67

Normal Assembler Control Flow Instruccions

jc    address

jnc   address

jz    address

jnz address

jb  bit ,  address

jnb bit ,  address

jbc bit ,  address

sjmp  address

jmp   @a+dptr

ljmp   address

lcall  address

ajmp   address

acall  address

djnz   rn , addr

djnz   byte , rel

cjne   a , byte , addr

cjne   a , # byte , addr

cjne   rn , # byte , addr

cjne   @ri , # byte , addr

Note: you should avoid at all cost the classical jump instruccions

on your code, take care with the spaces between the operands

of the instruccions, and indent your source code to show

clearly the relations between the conditionals.

Pseudo-Instruccions

org     addr

label   name

byte    value

word    value

string  " every char between the double quotes "

Take care with the labels because these cannot be used before

being defined. This may look to you as a very serious limitation

but it isn't because you have the structured control flow

instruccions on this assembler.

To define a constant one still has the word constant from Forth,

the predefined constants are included here for reference.

Constants Used for Register and Bit Naming

Register Addresses

$e0     constant        ACC

$f0     constant        B

$d0     constant        PSW

$81     constant        SP

$82     constant        DPL

$83     constant        DPH

$80     constant        P0

$90     constant        P1

$a0     constant        P2

$b8     constant        IP

$a8     constant        IE

$89     constant        TMOD

$8c     constant        TH0

$8a     constant        TL0

$8d     constant        TH1

$8b     constant        TL1

$98     constant        SCON

$99     constant        SBUF

$87     constant        PCON

\ Adresses of the Internal Bits

\  P0

$80     constant        P0.0

$81     constant        P0.1

$82     constant        P0.2

$83     constant        P0.3

$84     constant        P0.4

$85     constant        P0.5

$86     constant        P0.6

$87     constant        P0.7

\  TCON

$88     constant        IT0

$89     constant        IE0

$8A     constant        IT1

$8B     constant        IE1

$8C     constant        TR0

$8D     constant        TF0

$8E     constant        TR1

$8F     constant        TF1

\  P1

$90     constant        P1.0

$91     constant        P1.1

$92     constant        P1.2

$93     constant        P1.3

$94     constant        P1.4

$95     constant        P1.5

$96     constant        P1.6

$97     constant        P1.7

\ SCON

$98     constant        RI

$99     constant        TI

$9A     constant        RB8

$9B     constant        TB8

$9C     constant        REN

$9D     constant        SM2

$9E     constant        SM1

$9F     constant        SM0

\  P2

$A0     constant        P2.0

$A1     constant        P2.1

$A2     constant        P2.2

$A3     constant        P2.3

$A4     constant        P2.4

$A5     constant        P2.5

$A6     constant        P2.6

$A7     constant        P2.7

\ IE

$A8     constant        EX0

$A9     constant        ET0

$AA     constant        EX1

$AB     constant        ET1

$AC     constant        ES

$AD     constant        ET2

$AE     constant        IE.6

$AF     constant        EA

\  P3

$B0     constant        P3.0

$B1     constant        P3.1

$B2     constant        P3.2

$B3     constant        P3.3

$B4     constant        P3.4

$B5     constant        P3.5

$B6     constant        P3.6

$B7     constant        P3.7

\  IP

$B8     constant        PX0

$B9     constant        PT0

$BA     constant        PX1

$BB     constant        PT1

$BC     constant        PS

$BD     constant        PT2                     \ 8052 only

$BE     constant        IP.6

$BF     constant        IP.7

\ T2CON         8052 only

$C8     constant        CP/-RL2

$C9     constant        C/-T2

$CA     constant        TR2

$CB     constant        EXEN2

$CC     constant        TCLK

$CD     constant        RCLK

$CE     constant        EXF2

$CF     constant        TF2

\  PSW

$D0     constant        P

$D1     constant        PSW.1

$D2     constant        OV

$D3     constant        RS0

$D4     constant        RS1

$D5     constant        F0

$D6     constant        AC

$D7     constant        CY

\  ACC

$E0     constant        ACC.0

$E1     constant        ACC.1

$E2     constant        ACC.2

$E3     constant        ACC.3

$E4     constant        ACC.4

$E5     constant        ACC.5

$E6     constant        ACC.6

$E7     constant        ACC.7

\  B register

$F0     constant        B.0

$F1     constant        B.1

$F2     constant        B.2

$F3     constant        B.3

$F4     constant        B.4

$F5     constant        B.5

$F6     constant        B.6

$F7     constant        B.7