IDE-INFO v1.00

==============

This utility can be used to diagnose your harddisk(s) and is put

into the public domain. However, the author is not responsible

for any damage that may occur by using this program.

It can be very useful, when you do not have specifications of

your harddisk and want to find the optimum BIOS parameters. It

provides also some information about the caching functions and

the way data is stored on the disk, partition tables and BIOS

information.

Information:

------------

IDE-INFO provides several "pages" of information, which are

discussed next in further detail with a screendump of my own

system. The first 4 pages are shown for all IDE drives in the

system, but the last page is not specific for one IDE drive,

so it is only displayed once.

Page 1: IDE/ATA Interface ID Information

----------------------------------------

This page displays information, which it obtains from the IDE

controller on your harddisk. It requires direct access to some

hardware ports, which isn't supported with OS/2 or Windows v3.1

(with 32-bit harddisk access). If IDE-INFO can't get the

information it wants, then it displays an error message. An

example of the IDE/ATA Interface ID Information page is the

next screendump:

Model: Conner Peripherals 200MB - CP3204F

Serial Number: Q987BH

Drive Parameter Information

Firmware Rev: 2.31          Bytes/Index Gap: 0

Cylinders: 684                 Bytes/PLO: 12

Heads: 16              Buffer Scheme: Dual Port Multisector Cache

Bytes/Track: 25740           Sector Buffer: 128 (64Kb cache)

Bytes/Sector: 660                 Bytes/ECC: 7

Sectors/Track: 38           Sectors/Transfer: 32896 (READ command)

Bytes/ISG: 50                   Capacity: 203Mb (212926464 bytes)

Double Word: Not Capable   Defect Realloc.: Yes

General Configuration

Sectoring: Hard                 Transfer Rate: > 10Mhz

Encoding: RLL               Rotational Speed: > 0.5% Tolerance

Head Switch: > 15�s          Data Strobe Offset: Not available

Spindle Control: No                    Track Offset: Not available

Media: Fixed            Frmt Spd Tol. Gap: Not required

Recording: Magnetic

Most information is quite obvious, but I will describe them anyway.

I do not know the meaning of all fields, so if you do know them

please send me a message about what it means. The following fields

are displayed and this is their meaning:

Model:            The modelnumber of the harddisk and sometimes the

brand of the harddisk (Conner mentions it, but

Seagate doesn't).

Serial Number:    The serial-number of your harddisk.

Firmware Rev:     The firmware revision of the IDE controller, which

is located on your harddisk.

Cylinders:        The logical number of cylinders. This doesn't mean

that the physical number of cylinders is the same.

Heads:            The logical number of heads of the harddisk. This

doesn't mean that the physical number of heads is

the same.

Bytes/Track:      The number of bytes (unformatted) per logical track.

Bytes/Sector:     The number of bytes (unformatted) per sector.

Sectors/Track:    The logical number of sectors per track. This

doesn't mean that the physical number of sectors

per track is the same.

Bytes/ISG:        Bytes in the Inter Sector Gap, which is located

between sectors.

Double Word:      Specifies if the drive can do double word

transfers (sometimes called 32-bit transfers).

Bytes/Index Gap:  Sorry, I don't know what it means :-(

Bytes/PLO:        Number of bytes in the synchronization fields.

Buffer Scheme:    Number of buffer ports and its size.

Sector Buffer:    The number of sectors, which the drive can buffer

Because most vendor's mention the size in kilobytes

I also showed this value.

Bytes/ECC:        Maximum Error Checking and Correction data burst

length.

Sectors/Transfer: Sorry, I don't know what it means :-(

Capacity:         The formatted capacity of the drive in Mb as well

as in bytes.

Defect Realloc.:  Specifies if the drive can relocate defect sectors.

Sectoring:        Specifies hard or soft sectoring

Encoding:         Specifies MFM or RLL encoding

Head Switch:      Sorry, I don't know what it means :-(

Spindle Control:  Sorry, I don't know what it means :-(

Media:            Specifies if your drive is fixed or removable.

Transfer Rate:    Specifies the transfer-rate of the drive. If it's

over 10Mhz it means that it can transfer over

10 Mbit per second.

Rotational Speed: Sorry, I don't know what it means :-(

Data Strobe Off.: Sorry, I don't know what it means :-(

Track Offset:     Sorry, I don't know what it means :-(

Frmt Spd Tol Gap: Sorry, I don't know what it means :-(

Recording:        Specifies magnetic or non-magnetic recording.

Page 2: BIOS HD parameter table

-------------------------------

This page shows the parameters, which are currently in used for

the drive. I didn't experience any problems with this page when

using Windows, DESQview or OS/2. An example of the BIOS HD

parameter table is the next screendump.

Harddisk type: 47

Location of HD parameters: F000:E13D

Number of Cylinders: 683

Number of heads: 16

Starting Reduced Write Current Cylinder: 0       (XT-only)

Starting Write Precomp. Cylinder Number: 0

Maximum ECC burst length: 0       (XT-only)

Drive Option: 0       (XT-only)

More than 8 Heads: Yes     (AT-only)

Manufactorer's Defect Map on 'maxcyl+1': No      (AT-only)

Disable ECC retries: Off

Disable access retries: Off

Standard Timeout: 0       (XT-only)

Formatting Timeout: 0       (XT-only)

Timeout for Checking Drive: 0       (XT-only)

Cylinder number of Landing Zone: 0       (AT-only)

Number of Sectors per Track: 38      (AT-only)

Total Capacity (calculated by BIOS par): 202Mb (212615168 bytes)

The entries in this page are quite obvious and I won't discuss

them further. The location of the HD parameters can be used to

find the HD parameters in the BIOS. Some fields are only used

for XT-computers and are not of any interest to AT users and

visa versa.

Page 3: Diagnostics

-------------------

This page shows some diagnostics about the parameters you use

for your drive. It can warn you if you use dangerous parameters

or parameters that may cause other problems. If you do not use

the full capacity of your drive it will show a message to. When

IDE-INFO thinks that you do not use the optimal settings it will

try to find the optimal parameters for your drive.

In the following screendump IDE-INFO found out that I didn't use

the last cylinder of my drive. This is the message it generated:

You don't use the full capacity of your harddisk. Choose another

HD-type in your BIOS (or adjust user-definable type if available).

There are 304Kb unused on your HD.

The following parameters are suggested for your harddisk if you

have a user definable type:

Cylinders : 684     Heads : 16      Sectors per Track : 38

If you don't have a user definable type, choose HD-type 46. This

entry is the closest match to the proposed parameters. It leaves

you with 51876Kb of unused diskspace.

46   Cylinders : 1224    Heads : 15      Sectors per Track : 17

Because not all BIOS versions allow user-definable types, it

tries to find the best matching type in your BIOS. If you change

these parameters in your BIOS you need to repartition your

entire harddisk, which will result in a losing all data, so you

should make a backup first!!!

Page 4: Partition Table

-----------------------

This page shows the partition table of the drive and it tries to

find out, which kind of operating system or file-system is on it.

The following partition table is the one of my drive:

�������������������������������������������������������������������������ͻ

�  Operating  �      �    Start     �     End      � Relative � Number of �

�    System   � Boot � Hd  Cyl Sect � Hd  Cyl Sect �  sector  �  sectors  �

�������������������������������������������������������������������������Ķ

� DOS-extend. �  No  �  0   71   1  � 15  679  38  �    43168 �    370272 �

�    DOS-16   �  No  �  1    0   1  � 15   70  38  �       38 �     43130 �

� Coher/Bmgr. � Yes  �  0  680   1  � 15  683  38  �   413440 �      2432 �

�    empty    �  No  �  0    0   0  �  0    0   0  �        0 �         0 �

�������������������������������������������������������������������������ͼ

Because several operating systems use the same identifier for the

identification of the partition, you may see the wrong operating

system. This could be confusing, but it isn't dangerous. So don't

worry.

Page 5: BIOS HD parameter types

-------------------------------

This page shows the entire HD parameter list for your BIOS. As far

as I know a BIOS can have up to 47 types. The AMI BIOS uses type 47

as the user-definablei type, but the Phoenix BIOS uses type 48 and

49 for this purpose. Because these aren't always the same, there's

no need to print it. The following table is from my AMI BIOS:

Cyls  Hd  PrCmp  ECC  Ctl  Park  Sec �   Cyls  Hd  PrCmp  ECC  Ctl  Park  Sec

1  306   4    128    0    0   305   17 �25  925   9  65535    0    0   925   17

2  615   4    300    0    0   615   17 �26  754   7    754    0    0   754   17

3  615   6    300    0    0   615   17 �27  754  11  65535    0    0   754   17

4  940   8    512    0    0   940   17 �28  699   7    256    0    0   699   17

5  940   6    512    0    0   940   17 �29  823  10  65535    0    0   823   17

6  615   4  65535    0    0   615   17 �30  918   7    918    0    0   918   17

7  462   8    256    0    0   511   17 �31 1024  11  65535    0    0  1024   17

8  733   5  65535    0    0   733   17 �32 1024  15  65535    0    0  1024   17

9  900  15  65535    0    0   901   17 �33 1024   5   1024    0    0  1024   17

10  820   3  65535    0    0   820   17 �34  612   2    128    0    0   612   17

11  855   5  65535    0    0   855   17 �35 1024   9  65535    0    0  1024   17

12  855   7  65535    0    0   855   17 �36 1024   8    512    0    0  1024   17

13  306   8    128    0    0   319   17 �37  615   8    128    0    0   615   17

14  733   7  65535    0    0   733   17 �38  987   3    987    0    0   987   17

15    0   0      0    0    0     0    0 �39  987   7    987    0    0   987   17

16  612   4      0    0    0   663   17 �40  820   6    820    0    0   820   17

17  977   5    300    0    0   977   17 �41  977   5    977    0    0   977   17

18  977   7  65535    0    0   977   17 �42  981   5    981    0    0   981   17

19 1024   7    512    0    0  1023   17 �43  830   7    512    0    0   830   17

20  733   5    300    0    0   732   17 �44  830  10  65535    0    0   830   17

21  733   7    300    0    0   732   17 �45  917  15  65535    0    0   918   17

22  733   5    300    0    0   733   17 �46 1224  15  65535    0    0  1223   17

23  306   4      0    0    0   336   17 �47    0   0      0    0    0     0    0

24  925   7      0    0    0   925   17 �

A harddisk type, which contains only zeros is disabled in the BIOS

and can't be used. Type 47 is user-definable, but in the BIOS

it is listed as only zeros. Check 'page 2 : BIOS HD parameter

table' to check how your type 47 looks like (assuming you have an

AMI BIOS).

Because the information is read out of the BIOS it can cause

trouble when you use the Stealth function with QEMM-386. Disable

Stealth, when you want to check this page.

Sourcecode:

-----------

All source-code is provided, but please don't judge me on it

because this is definetely not the best style I've used :-) The

only reason why I distributed it is, that you adjust it for your

own needs. However read the section 'Copyrights' if you want to

adjust the source-code!!!

Problems:

---------

This is low-level software and it might give weird results if

you use it with tricky programs, such as memory managers which

relocate the BIOS. It requires direct hardware access, so it

may crash under Windows or OS/2, because they often don't allow

it.

Copyrights:

-----------

This program may be used for free and no fee may be charged for

distributing this program (except media-costs) as long as the

archive isn't modificated (although it may be repacked with another

compiler and archive-remarks may be added). The following files

should be present in the archive:

IDE-INFO.EXE        IDE-Info executable

IDE-INFO.DOC        IDE-Info documentation (this file)

IDE-INFO.C          Complete IDE-Info source-code (in C)

If you want to modificate and distribute the archive you should

contact the author first for the conditions.

Contacting the author:

----------------------

If you want some more information about this program and its

results, or if you have some improvements you can contact me

on the following address:

Ramon de Klein

Thomas de Keyserstraat 298

7545 AJ  Enschede

The Netherlands

Or by E-Mail:

2:283/7.21@fidonet.org          (prevered)

27:4331/107.21@signet.fnt

81:431/16@os2net.fnt

Have fun with it...