Chapter 15

Sound Effects

330   Fastgraph User's Guide

Overview

In the realm of the IBM PC and PS/2 family of systems, a sound is defined

by its frequency, duration, and volume. The frequency of a sound is measured

in units called Hertz. While the PC and PS/2 can produce sounds ranging from

18 to more than one million Hertz, the average human can hear sounds between

20 and about 20,000 Hertz. The length of a sound, called its duration, is

expressed in clock ticks; there are either 18.2 of 72.8 clock ticks per

second, depending on the method used to produce the sound. Finally, the volume

determines the loudness of the sound. As we'll see in this chapter, we can

control a sound's volume only on the PCjr and Tandy 1000 systems.

Fastgraph offers several different methods for producing sound effects.

These include single tones, a series of tones expressed numerically, or a

series of tones expressed as musical notes. The sound effects may be discrete,

continuous, or performed at the same time as other activity. The sound-related

routines are independent of the other parts of Fastgraph and do not require

any initialization routines be called.

Sound Sources

All members of the PC and PS/2 families can produce sounds using the

8253-5 programmable timer chip and the internal speaker. This method is

limited to producing single sounds of given frequencies and durations,

although we can combine these sounds to create interesting audio effects or

play music. When we use this technique, we have no control over the sound

volume. In fact, sound volumes often vary slightly on different systems

because the physical properties of the speaker and its housing are not always

the same.

The PCjr and Tandy 1000 systems have an additional, more powerful chip

for producing sounds. This is the Texas Instruments SN76496A sound chip,

called the TI sound chip for short. The TI sound chip has three independent

voice channels for producing pure tones, and a fourth channel for generating

periodic or white noise. Each voice channel has a separate volume control that

allows us to control the loudness of the sound it emits.

Synchronous Sound

A sound effect is said to be synchronous if it is produced while no other

activity is being performed. In other words, a program makes a synchronous

sound by starting the sound, waiting for a specified duration, and then

stopping the sound. The program must wait for the sound to complete before

doing anything else. As long as the duration is relatively short, the fact

that the sound is synchronous has little or no effect on the program's

execution speed. Fastgraph includes routines for producing synchronous sound

using either the 8253-5 programmable timer or the TI sound chip.

The fg_sound routine uses the programmable timer to produce a sound of a

given frequency and duration. The frequency, defined by the first argument, is

expressed in Hertz and must be an integer value between 18 and 32,767. The

second argument defines the duration and is expressed in clock ticks; there

Chapter 15:  Sound Effects   331

are 18.2 clock ticks per second. If the duration is zero or negative, the

sound will continue until it is stopped with fg_quiet.

Example 15-1 uses fg_sound to create different sound effects, pausing for

one second between each. It first produces three distinct sounds of 20, 100,

and 1,000 Hertz. Each of these sounds lasts for approximately 1/6 of a second

(three clock ticks). The program then makes a warbling noise by quickly

alternating sounds of similar frequencies. Finally, the program creates a

sliding tone of increasing frequencies between 100 and 500 Hertz. Each tone in

this sequence lasts for two clock ticks, so it takes about 4.5 seconds to play

the entire sequence. In all cases, example 15-1 displays an identifying

message just before each sound.

Example 15-1.

#include <fastgraf.h>

#include <stdio.h>

void main(void);

void main()

{

int freq;

fg_initpm();

printf("20 Hz tone...\n");

fg_sound(20,3);

fg_waitfor(18);

printf("100 Hz tone...\n");

fg_sound(100,3);

fg_waitfor(18);

printf("1000 Hz tone...\n");

fg_sound(1000,3);

fg_waitfor(18);

printf("warble...\n");

fg_sound(400,1);

fg_sound(410,1);

fg_sound(400,1);

fg_sound(410,1);

fg_waitfor(18);

printf("sliding tone from 100 to 500 Hz...\n");

for (freq = 100; freq <= 500; freq+=10)

fg_sound(freq,2);

}

The fg_voice routine is analogous to fg_sound, but it uses the TI sound

chip rather than the programmable timer to create sound. For this reason,

fg_voice can only be used on the PCjr or Tandy 1000 systems. The TI sound chip

allows us to control the volume of a sound, and it also offers four distinct

voice channels. Thus, fg_voice requires two additional arguments besides

frequency and duration to define the voice channel and sound volume.

332   Fastgraph User's Guide

The first argument to fg_voice defines the voice channel, as shown here:

value meaning

1   voice channel #1

2   voice channel #2

3   voice channel #3

4   voice channel #4, periodic noise

5   voice channel #4, white noise

If we use voice channels 1, 2, or 3, the second argument defines the sound

frequency in Hertz, between 18 and 32,767. If we use voice channel 4, however,

the second argument instead is a value that represents a specific frequency,

as shown in this table:

value frequency

0   512 Hertz

1  1024 Hertz

2  2048 Hertz

The third argument defines the sound volume. It must be between 0 and 15,

where 0 is silent and 15 is loudest. The fourth argument defines the sound

duration in clock ticks. As with fg_sound, there are 18.2 clock ticks per

second, and if the duration is zero or negative, the sound will continue until

stopped with fg_quiet.

Example 15-2 uses fg_voice to create different sound effects using the TI

sound chip. As in example 15-1, there is a pause of one second between each.

The program first calls fg_testmode to be sure it is running on a PCjr or

Tandy 1000 system (video mode 9 is only available on these systems). If so,

the program uses voice channel #4 to produce a 2,048 Hertz periodic noise,

followed by white noise of the same frequency. Both sounds are emitted at the

maximum volume level (15) and last for about 1/6 of a second each (three clock

ticks). After these noises, example 15-2 produces a 500 Hertz tone of

increasing volume. In all cases, the program displays an identifying message

just before each sound.

Example 15-2.

#include <fastgraf.h>

#include <stdio.h>

#include <stdlib.h>

void main(void);

void main()

{

int volume;

fg_initpm();

if (fg_testmode(9,0) == 0) {

printf("This program requires a PCjr or ");

printf("a Tandy 1000 system.\n");

exit(1);

}

Chapter 15:  Sound Effects   333

printf("2048 Hz periodic noise...\n");

fg_voice(4,2,15,3);

fg_waitfor(18);

printf("2048 Hz white noise...\n");

fg_voice(5,2,15,3);

fg_waitfor(18);

printf("500 Hz tone of increasing volume...\n");

for (volume = 1; volume <= 15; volume++) {

fg_voice(1,500,volume,0);

fg_waitfor(4);

}

fg_quiet();

}

Note how example 15-2 uses a duration of zero (continuous sound) and

fg_waitfor to specify the duration for each volume level the 500 Hertz tone

sequence. This causes the transition between changes in volume to blend better

with each other. The fg_quiet routine, which stops continuous sound started

with fg_sound or fg_voice, ends the sound after the final volume level.

Both fg_sound and fg_voice produce a single sound. We've seen how to

combine sounds to produce sound effects, but still the individual sounds are

defined numerically -- that is, by a certain frequency and duration. It is

often easier to create sounds from musical notes, and for this reason

Fastgraph includes a routine fg_music that produces such sounds. The fg_music

routine uses the programmable timer to produce synchronous sound; it does not

support the TI sound chip.

The fg_music routine has a single argument called the music string,

passed by reference as a byte array or character string. The music string is

simply a variable-length sequence of music commands, followed by a dollar-sign

($) terminator. Music commands are summarized in the following table.

command   meaning

A thru G  Play the specified note in the current octave.

#         May be appended to a note character (A through G) to make that note

sharp.

.         May be appended to a note character (A through G) or a sharp (#) to

extend that note by half its normal length. Multiple dots may be

used, and each will again extend the note by half as much as the

previous extension.

Ln        Set the length of subsequent notes and pauses. The value of n is an

integer between 1 and 64, where 1 indicates a whole note, 2 a half

note, 4 a quarter note, and so forth. If no L command is present, L4

is assumed.

On        Set the octave for subsequent notes. The value of n may be an

integer between 0 and 6 to set a specific octave. It also can be a

334   Fastgraph User's Guide

plus (+) or minus (-) character to increment or decrement the

current octave number. Octave 4 contains middle C, and if no O

command is present, O4 is assumed.

P         Pause (rest) for the duration specified by the most recent L

command.

Sn        Set the amount of silence between notes. The value of n is an

integer between 0 and 2. If n is 0, each note plays for the full

period set by the L command (music legato). If n is 1, each note

plays for 7/8 the period set by the L command (music normal). If n

is 2, each note plays for 3/4 the period set by the L command (music

staccato). If no S command is present, S1 is assumed.

Tn        Set the tempo of the music (the number of quarter notes per minute).

The value of n is an integer between 32 and 255. If no T command is

present, T120 is assumed.

The fg_music routine ignores any other characters in the music string. It also

ignores command values outside the allowable range, such as T20 or O8.

Example 15-3 illustrates some uses of fg_music. The program plays the

first few bars of "Mary Had a Little Lamb", followed by the musical scale

(including sharps) in two octaves, and finally the introduction to Beethoven's

Fifth Symphony. There is a pause of one second between each piece of music,

and the program displays the titles before playing the music. Blank characters

appear in the music strings to help make them more readable.

Example 15-3.

#include <fastgraf.h>

#include <stdio.h>

void main(void);

void main()

{

{  fg_initpm();

printf("Mary Had a Little Lamb...\n");

fg_music("T150 L8 EDCDEEE P DDD P EGG P EDCDEEE L16 P L8 EDDEDC$");

fg_waitfor(18);

printf("up the scale in two octaves...\n");

fg_music("L16 CC#DD#EFF#GG#AA#B O+ CC#DD#EFF#GG#AA#B$");

fg_waitfor(18);

printf("Beethoven's Fifth Symphony...\n");

fg_music("T180 O2 L2 P L8 P GGG L2 D# L24 P L8 P FFF L2 D$");

}

Chapter 15:  Sound Effects   335

Asynchronous Sound

Sounds made concurrently with other activity in a program are said to be

asynchronous. Fastgraph's routines that produce asynchronous sound just start

the sound and then immediately return control to the calling program. The

sounds will automatically stop when the end of the sequence is reached, and

you also can suspend or stop it on demand before that time. None of

Fastgraph's asynchronous sound routines have any effect if there is already

asynchronous sound in progress. In addition, the asynchronous sound routines

temporarily disable the synchronous sound routines (fg_sound, fg_voice, and

fg_music) while asynchronous sound is in progress.

To expand the range of sound effects and to play fast-tempo music,

Fastgraph temporarily quadruples the clock tick interrupt rate from 18.2 to

72.8 ticks per second while producing asynchronous sound. Because many disk

controllers rely on the 18.2 tick per second clock rate to synchronize disk

accesses, your programs should not perform any disk operations when

asynchronous sound is in progress.

The fg_sounds routine is the asynchronous version of fg_sound. It uses

the programmable timer to play a sequence of tones simultaneous to other

operations. This routine expects as its first argument a variable-length

integer array, passed by reference, containing pairs of frequency and duration

values. As with fg_sound, each frequency is expressed in Hertz and must be

between 18 and 32,767. The durations are also measured in clock ticks, but

because the interrupt rate is quadrupled, there are 72.8 instead of 18.2 ticks

per second.

The format of the frequency and duration array passed to fg_sounds is

shown here:

[0]    frequency of sound 1

[1]    duration  of sound 1

[2]    frequency of sound 2

[3]    duration  of sound 2

.

.

.

[2n-2]    frequency of sound n

[2n-1]    duration  of sound n

[2n]       terminator (0)

Note that a null character (that is, a zero byte) terminates the array. The

second argument passed to fg_sounds is an integer value indicating the number

of times to cycle through the frequency and duration array. If this value is

negative, the sounds will continue until stopped with fg_hush or fg_hushnext.

336   Fastgraph User's Guide

Example 15-4 uses fg_sounds to play the 100 to 500 Hertz sliding tone

sequence of example 15-1. To prove the sounds are being made concurrently with

other operations, messages are displayed while the sequence is playing. This

is controlled by the Fastgraph routine fg_playing, which returns a value of 1

if asynchronous sounds are in progress, and 0 if not. Note how the duration

must be specified as 8 clock ticks (instead of 2 as in example 15-1) to

compensate for the quadrupled clock tick interrupt rate.

Example 15-4.

#include <fastgraf.h>

#include <stdio.h>

void main(void);

void main()

{

int i;

int freq;

int sound_array[83];

i = 0;

for (freq = 100; freq <= 500; freq+=10) {

sound_array[i++] = freq;

sound_array[i++] = 8;

}

sound_array[i] = 0;

fg_initpm();

fg_sounds(sound_array,1);

while(fg_playing())

printf("Still playing...\n");

}

Just as fg_sounds is analogous to fg_sound, there is a Fastgraph routine

fg_voices that is similar to fg_voice. That is, fg_voices uses the TI sound

chip to play an asynchronous sequence of tones. Its arguments are the same as

those of fg_sounds, but the structure of the sound array is different. Its

structure is:

[0]    channel # of sound 1

[1]    frequency of sound 1

[2]    volume    of sound 1

[3]    duration  of sound 1

.

.

.

[4n-4]    channel # of sound n

Chapter 15:  Sound Effects   337

[4n-3]    frequency of sound n

[4n-2]    volume    of sound n

[4n-1]    duration  of sound n

[4n]       terminator (0)

The channel numbers, frequencies, volumes, and durations must be in the same

ranges as discussed in the description of fg_voice, except the durations are

quadrupled because of the accelerated clock tick interrupt rate. Again, note

that a null character (that is, a zero byte) terminates the array.

Example 15-5 uses fg_voices to play the 500 Hertz tone sequence of

increasing volume introduced in example 15-2. As in example 15-4, the program

displays messages while the tone sequence is playing to demonstrate the sounds

are being made concurrently with other operations. Note how the duration is

now 16 clock ticks (instead of 4 as in example 15-2) because of the quadrupled

clock tick interrupt rate.

Example 15-5.

#include <fastgraf.h>

#include <stdio.h>

void main(void);

void main()

{

int voice_array[61];

int i;

int volume;

fg_initpm();

if (fg_testmode(9,0) == 0) {

printf("This program requires a PCjr or ");

printf("a Tandy 1000 system.\n");

exit(1);

}

i = 0;

for (volume = 1; volume <= 15; volume++) {

voice_array[i++] = 1;      /* use channel 1 */

voice_array[i++] = 500;    /* 500 Hz frequency */

voice_array[i++] = volume; /* variable volume */

voice_array[i++] = 16;     /* duration */

}

voice_array[i] = 0;

fg_voices(voice_array,1);

while(fg_playing())

printf("Still playing...\n");

}

338   Fastgraph User's Guide

There is also an asynchronous version of fg_music. It is called

fg_musicb, and it uses the same format music string as fg_music does. However,

fg_musicb has a second argument that specifies the number of times to cycle

through the music string. If this value is negative, the music will play

repetitively until you stop it with fg_hush or fg_hushnext.

Example 15-6 plays the same three pieces of music as example 15-3, but it

does so concurrently with other operations. As the music plays, the program

continuously displays the title of each piece. Note how we can take advantage

of the repetition in the music string for the "up the scale" sequence by

playing the sequence twice.

Example 15-6.

#include <fastgraf.h>

#include <stdio.h>

void main(void);

void main()

{

fg_initpm();

fg_musicb("T150 L8 EDCDEEE P DDD P EGG P EDCDEEE L16 P L8 EDDEDC$",1);

while (fg_playing())

printf("Mary Had a Little Lamb...\n");

fg_waitfor(18);

fg_musicb("L16 CC#DD#EFF#GG#AA#B O+$",2);

while (fg_playing())

printf("up the scale in two octaves...\n");

fg_waitfor(18);

fg_musicb("T180 O2 L2 P L8 P GGG L2 D# L24 P L8 P FFF L2 D$",1);

while (fg_playing())

printf("Beethoven's Fifth Symphony...\n");

}

The next example demonstrates the effects of the Fastgraph routines

fg_hush and fg_hushnext, which stop sounds started with fg_sounds, fg_voices,

or fg_musicb. The fg_hush routine immediately stops asynchronous sound, while

fg_hushnext does so when the current cycle finishes. Neither routine has any

arguments, and neither routine has any effect if no asynchronous sound is in

progress. Furthermore, note that fg_hushnext has no effect unless the

asynchronous sound is continuous.

Example 15-7 runs in any text or graphics video mode. It displays

rectangles in up to 16 colors while playing continuous asynchronous music. The

program periodically checks for keystrokes with fg_intkey, and it continues to

play the music while there is no keyboard activity. If you press the Escape

key, the program uses fg_hush to stop the music immediately; this causes an

exit from the while loop. If you press any other key, the program uses

fg_hushnext to stop the music as soon as the current repetition finishes. Once

it does, the program exits the while loop because fg_playing will return a

value of zero.

Chapter 15:  Sound Effects   339

Example 15-7.

#include <fastgraf.h>

void main(void);

#define ESC 27

void main()

{

int color;

int old_mode;

unsigned char key, aux;

fg_initpm();

old_mode = fg_getmode();

fg_setmode(fg_automode());

color = 0;

fg_musicb("O4 L16 CC#DD#EFF#GG#AA#B O+ CC#DD#EFF#GG#AA#B$",-1);

while (fg_playing())

{

color = (color + 1) & 15;

fg_setcolor(color);

fg_rect(0,fg_getmaxx(),0,fg_getmaxy());

fg_waitfor(4);

fg_intkey(&key,&aux);

if (key == ESC)

fg_hush();

else if (key+aux != 0)

fg_hushnext();

}

fg_setmode(old_mode);

fg_reset();

}

Example 15-7 also demonstrates an important side-effect of fg_musicb when

playing continuous music. Any length, octave, silence, or tempo values changed

within the string are not reset to their original values at the beginning of

each repetition. If we did not include the O4 command at the beginning of the

string, the later O+ command would cause the music to play in octaves 4 and 5

during the first repetition, 5 and 6 during the second repetition, and octave

6 for all subsequent repetitions (because you cannot increase the octave

number above 6).

The final two routines relating to asynchronous sound are fg_resume and

fg_suspend. The fg_suspend routine suspends music previously started by

fg_musicb, while fg_resume restarts the music from the point where it was

suspended. Example 15-8 plays the first few bars of "Mary Had a Little Lamb".

If you press any key while the song is playing, it stops. Then, after another

keystroke, the music resumes and continues until finished.

340   Fastgraph User's Guide

Example 15-8.

#include <fastgraf.h>

#include <stdio.h>

void main(void);

void main()

{

fg_initpm();

fg_musicb("T150 L8 EDCDEEE P DDD P EGG P EDCDEEE L16 P L8 EDDEDC$",1);

fg_waitkey();

fg_suspend();

printf("Music suspended.  Press any key to resume.\n");

fg_waitkey();

fg_resume();

printf("Music resumed.\n");

while (fg_playing());

printf("Music finished.\n");

}

The fg_suspend routine has no effect if there is no asynchronous music in

progress. Similarly, fg_resume has no effect if there is no suspended music.

If you call fg_suspend and then need to cancel the music or exit to DOS

instead of restarting the music, call fg_hush instead of fg_resume.

Summary of Sound Routines

This section summarizes the functional descriptions of the Fastgraph

routines presented in this chapter. More detailed information about these

routines, including their arguments and return values, may be found in the

Fastgraph Reference Manual.

FG_HUSH immediately stops asynchronous sound started with fg_sounds,

fg_voices, or fg_musicb.

FG_HUSHNEXT is similar to fg_hush, but it does not stop the asynchronous

sound until the current repetition finishes.

FG_MUSIC uses the programmable timer to play a sequence of musical tones.

FG_MUSICB is the asynchronous version of fg_music. It uses the

programmable timer to play a sequence of musical tones, concurrent with other

activity.

FG_PLAYING determines whether or not there is any asynchronous sound in

progress.

FG_QUIET stops continuous synchronous sound started with fg_sound or

fg_voice.

FG_RESUME restarts asynchronous music previously suspended by fg_suspend.

Chapter 15:  Sound Effects   341

FG_SOUND produces a tone of a specified frequency and duration using the

programmable timer.

FG_SOUNDS is the asynchronous version of fg_sound. It can play a series

of tones of specified frequencies and durations, concurrent with other

activity.

FG_SUSPEND suspends asynchronous music previously started by fg_musicb.

FG_VOICE produces a tone of a specified frequency, duration, and volume

using one of the TI sound chip's four voice channels.

FG_VOICES is the asynchronous version of fg_voice. It can play a series

of tones of specified frequencies, durations, and volumes, concurrent with

other activity.

342   Fastgraph User's Guide