Chapter 5

The Use of Color

68   Fastgraph User's Guide

Overview

The use of color is an important part of any text or graphics

application. This chapter explains color as it applies to text and graphics

modes. It also describes palettes and video DAC registers for the graphics

video modes that offer this functionality. Finally, an explanation of

Fastgraph's virtual colors is provided.

Text Modes

The term color is not really correct in text modes because each character

cell has an associated attribute that controls the character's appearance in

that cell. The meaning of the attribute differs for color and monochrome text

modes.

Color Text Modes

In color text modes (modes 0, 1, 2, and 3), the attribute determines a

character's foreground color (the color of the character itself), its

background color (the color of that part of the character cell not covered by

the character), and whether or not it blinks. Sixteen foreground colors

(numbered 0 to 15) are available, but only eight background colors (numbered 0

to 7) are available. The colors assigned to these values are listed in the

following table.

number     color    number     color

0       black       8     dark gray

1       blue        9    light blue

2       green      10    light green

3       cyan       11    light cyan

4        red       12     light red

5      magenta     13   light magenta

6       brown      14      yellow

7       gray       15       white

At first it may seem the numbers have been arbitrarily assigned to the colors.

Upon further inspection, however, it becomes apparent this is not the case.

Each color number is a four bit quantity of the form IRGB, with I representing

the intensity, R the red component, G the green component, and B the blue

component. If the corresponding bit is 1, it means the intensity or color

component is set. For example, normal red would be represented by the IRGB bit

pattern 0100, which is 4 decimal, the color number for red.

The fg_setattr routine defines the current text attribute. Once

fg_setattr is called, Fastgraph displays all subsequent text using that

attribute. The first argument of fg_setattr defines the foreground color,

which must be an integer between 0 and 15. Its second argument defines the

background color, which must be between 0 and 7. Its third argument determines

if the foreground color blinks (1 means it blinks, 0 means it does not). For

example, the statement

fg_setattr(14,1,0);

Chapter 5:  The Use of Color   69

specifies subsequent text will be displayed with a yellow foreground (14) on a

blue background (1) and will not blink (0).

Another Fastgraph routine, fg_setcolor, also can define text attributes.

The fg_setcolor routine packs the three values passed to fg_setattr into a

single argument, as shown here:

bits         attribute

0-3     foreground color

4-6     background color

7          blinking

For example, calling fg_setcolor with an argument of 30 (1E hex) is equivalent

to calling fg_setattr with arguments of 14, 1, and 0.

The Fastgraph routine fg_getcolor returns the current text attribute, as

defined in the most recent call to fg_setattr or fg_setcolor. The fg_getcolor

routine has no arguments and returns the attribute as its function value. The

returned value is encoded using the same scheme for passing a text attribute

to fg_setcolor.

Monochrome Text Mode

In the monochrome text mode (mode 7), colors are obviously not available.

The attribute instead determines whether a character is invisible, normal,

bold, reversed, or certain combinations of these. The following table shows

the values assigned to the available display characteristics.

foreground     background   characteristic

0              0          invisible

0              7          reversed

1              0         underlined

7              0           normal

9              0       underlined bold

15              0            bold

Additionally, you can turn blinking on or off for each of these combinations.

Any combination of foreground and background values not listed in the above

table produces a normal display characteristic.

As in the color modes, the Fastgraph routines fg_setattr and fg_setcolor

define the current text attribute. For example, the statement

fg_setattr(0,7,1);

specifies subsequent text will be displayed in reverse video (0,7) and will

blink (1). The same attribute could be defined by calling fg_setcolor with an

argument of 240 (F0 hex). The fg_getcolor routine is also available and works

as it does in the color text modes.

70   Fastgraph User's Guide

Graphics Modes

In graphics modes, each pixel has an associated color value that

determines the color in which the pixel is displayed. The number of available

colors depends on the video mode and can be obtained through the fg_colors

function. Some graphics modes also have palette registers or video DAC

registers to provide additional color capabilities, such as choosing a set of

colors from a larger palette of colors. The example programs presented in this

section show the use of color in specific graphics video modes.

The following subsections will discuss the use of color in each graphics

video mode. In these discussions, there will be several references to a group

of colors called the standard color set. This is a set of 16 colors common to

many of the graphics video modes (and to the color text modes). The colors in

the standard color set are listed in the following table.

number     color    number     color

0       black       8     dark gray

1       blue        9    light blue

2       green      10    light green

3       cyan       11    light cyan

4        red       12     light red

5      magenta     13   light magenta

6       brown      14      yellow

7       gray       15       white

At this point we must understand the difference between the terms color

number and color value. Color number refers to the number that defines a color

in the standard color set (for example, green is color number 2). Color value

refers to the actual value of a pixel in video memory, which ultimately

determines the color in which that pixel is displayed. The color value is

sometimes just called the color.

In each graphics mode, video memory is cleared when the fg_setmode

routine is called. This means all pixels are initially set to color value 0,

which by default is black. For this reason, color value 0 is often called the

background color in graphics video modes.

Fastgraph's fg_setcolor routine defines the color in which subsequent

graphics operations are performed. This color is called the current color.

Depending on the video mode, the current color can reference a color value (in

CGA and Hercules graphics modes), a palette register (in Tandy, EGA, and VGA

graphics modes), or a video DAC register (in 256-color modes). The fg_setcolor

routine takes a single integer argument that specifies the color. When

fg_setmode is called, it sets the current color to 0. The Fastgraph routine

fg_getcolor returns the current color, as defined in the most recent call to

fg_setcolor. The fg_getcolor routine has no arguments and returns the current

color as its function value.

CGA Color Modes

The CGA color modes (modes 4 and 5) have six sets of available colors,

called palettes, numbered 0 to 5. Each palette consists of four colors,

Chapter 5:  The Use of Color   71

numbered 0 to 3. In each palette, the background color (color value 0) can be

selected from the standard color set, but the other 3 colors are fixed. The

following table shows the fixed colors assigned to each palette.

palette 0       palette 1       palette 2

color 1   light green     light cyan      light cyan

color 2   light red       light magenta   light red

color 3   yellow          white           white

palette 3       palette 4       palette 5

color 1   green           cyan            cyan

color 2   red             magenta         red

color 3   brown           gray            gray

Palette 1, with a black background, is the default palette when you select

mode 4. Palette 2, with a black background, is the default palette when you

select mode 5.

The CGA color modes have a border area called the overscan between the

addressable pixel space and the physical edges of the screen. The overscan

area is always displayed in the background color, regardless of which CGA

palette is used.

In CGA color modes, fg_setcolor defines the current color by referencing

one of the four color values. The fg_palette routine selects one of the six

palettes and defines the background color for that palette. The first argument

of fg_palette is an integer between 0 and 5 that specifies the palette number.

The second argument is an integer between 0 and 15 that defines the background

color, using the color numbers in the standard color set.

Example 5-1 demonstrates the use of fg_palette and fg_setcolor in mode 4.

After establishing the video mode, the program selects palette 0 and makes the

background color blue (color number 1). It then makes color 3 in palette 0

(yellow) the current color and displays the word "Hello". Finally, it restores

the original video mode and screen attributes before returning to DOS.

Example 5-1.

#include <fastgraf.h>

void main(void);

void main()

{

int mode;

fg_initpm();

mode = fg_getmode();

fg_setmode(4);

fg_palette(0,1);

fg_setcolor(3);

fg_text("Hello",5);

72   Fastgraph User's Guide

fg_waitkey();

fg_setmode(mode);

fg_reset();

}

CGA Two-Color Mode

The CGA two-color mode (mode 6) has a fixed background color (color value

0) and a user-definable foreground color (color value 1). The background color

is always black. The foreground color is white by default but can be changed

to any of the colors in the standard color set. We should mention that

changing the foreground color works on true CGA adapters, but there are very

few EGA, VGA, and SVGA adapters that correctly implement changing the

foreground color in their mode 6 emulation.

In mode 6, fg_setcolor defines the current color by referencing one of

the two color values. The fg_palette routine defines the actual foreground

color (that is, the color of pixels whose color value is 1). For consistency

with other graphics modes, fg_palette has two arguments, but the first one is

not used. The second argument is an integer between 0 and 15 that defines the

foreground color, using the color numbers in the standard color set.

Example 5-2 demonstrates the use of fg_palette and fg_setcolor in mode 6.

After establishing the video mode, the program makes the foreground color

yellow (color number 14). It then makes color 1 the current color and displays

the word "Hello". Finally, it restores the original video mode and screen

attributes before returning to DOS.

Example 5-2.

#include <fastgraf.h>

void main(void);

void main()

{

int mode;

fg_initpm();

mode = fg_getmode();

fg_setmode(6);

fg_palette(0,14);

fg_setcolor(1);

fg_text("Hello",5);

fg_waitkey();

fg_setmode(mode);

fg_reset();

}

Chapter 5:  The Use of Color   73

Tandy and PCjr Modes

The supported Tandy 1000 or PCjr graphics mode (mode 9) has 16 color

values, numbered 0 to 15. Each color value references one of 16 user-definable

palette registers, often simply called palettes, also numbered 0 to 15. The

values assigned to the palette registers determine the colors in which pixels

are displayed. For example, if you assign palette register 2 the value for

red, then pixels whose color value is 2 will be red.

Each palette can assume one of the 16 colors in the standard color set.

By default, the values assigned to the 16 palettes correspond to the

identically numbered colors in the standard color set. In other words, palette

0 is assigned the value for black, palette 1 is assigned the value for blue,

and so forth.

In mode 9, fg_setcolor defines the current color by referencing one of

the 16 palette registers. The fg_palette routine defines the actual color

assigned to a specific palette register. The first argument of fg_palette is

an integer between 0 and 15 that specifies the palette number. The second

argument is an integer between 0 and 15 that defines the palette value (the

color assigned to the palette), using the IRGB color numbers in the standard

color set.

You also can use the Fastgraph routine fg_setrgb to define the color

assigned to a specific palette register. While fg_palette does this using a

color number from the standard color set, fg_setrgb defines a palette register

using red, green, and blue color components plus an intensity component. The

first argument of fg_setrgb is an integer between 0 and 15 that specifies the

palette register number. The remaining three arguments are each integer values

between -1 and 1 that respectively specify the red, green, and blue color

components for that palette register. The meanings of the color components

are:

-1 = color bit and intensity bit are set

0 = color bit is reset

1 = color bit is set

Since there is only one intensity bit in mode 9 color values, specifying -1

for any of the RGB color components produces an intense color. For example,

the color light cyan is color number 11 in the standard color set, and it is

produced by combining green and blue and setting the intensity bit. This means

any of these four statements

fg_palette(1,11);

fg_setrgb(1,0,-1,1);

fg_setrgb(1,0,1,-1);

fg_setrgb(1,0,-1,-1);

could be used to define palette register 1 as light cyan in mode 9.

Example 5-3 demonstrates the use of fg_palette and fg_setcolor in mode 9.

After establishing the video mode, the program defines palette 0 to be blue

(1) and palette 1 to be yellow (14). Note that defining palette 0 changes the

74   Fastgraph User's Guide

background color. It then makes color 1 the current color and displays the

word "Hello". After waiting for a keystroke, the program changes the color of

"Hello" by changing palette 1 to white (15). Finally, it restores the original

video mode and screen attributes before returning to DOS.

Example 5-3.

#include <fastgraf.h>

void main(void);

void main()

{

int mode;

fg_initpm();

mode = fg_getmode();

fg_setmode(9);

fg_palette(0,1);

fg_palette(1,14);

fg_setcolor(1);

fg_text("Hello",5);

fg_waitkey();

fg_palette(1,15);

fg_waitkey();

fg_setmode(mode);

fg_reset();

}

Hercules Mode

The Hercules graphics mode (mode 11) has a fixed background color (color

value 0) and a fixed foreground color (color value 1). The background color is

always black, and the foreground color is dependent on the monochrome display

being used (typically it is green, amber, or white).

The fg_setcolor routine defines the current color value by referencing

one of the two color values. The fg_palette routine has no effect in mode 11.

Example 5-4 demonstrates the use of fg_setcolor in mode 11. After

establishing the video mode, the program makes color 1 the current color and

displays the word "Hello". It then restores the original video mode and screen

attributes before returning to DOS.

Example 5-4.

#include <fastgraf.h>

void main(void);

void main()

{

Chapter 5:  The Use of Color   75

int mode;

fg_initpm();

mode = fg_getmode();

fg_setmode(11);

fg_setcolor(1);

fg_text("Hello",5);

fg_waitkey();

fg_setmode(mode);

fg_reset();

}

Hercules Low-Resolution Mode

The Hercules low-resolution graphics mode (mode 12) has four color

values, numbered 0 to 3. The background color is always black, colors 1 and 2

are normal intensity, and color 3 is full intensity. Colors 1 and 2 both

produce normal intensity colors, but they do so with different pixel

patterns -- color 1 turns on the odd-numbered physical pixels, while color 2

turns on the even-numbered physical pixels. The appearance of colors 1 to 3 is

dependent on the monochrome display being used (typically it is green, amber,

or white).

The fg_setcolor routine defines the current color value by referencing

one of the four color values. The fg_palette routine has no effect in mode 12.

Example 5-5 demonstrates the use of fg_setcolor in mode 12. After

establishing the video mode, the program makes color 3 the current color and

displays the word "Hello". It then restores the original video mode and screen

attributes before returning to DOS.

Example 5-5.

#include <fastgraf.h>

void main(void);

void main()

{

int mode;

fg_initpm();

mode = fg_getmode();

fg_setmode(12);

fg_setcolor(3);

fg_text("Hello",5);

fg_waitkey();

fg_setmode(mode);

fg_reset();

}

76   Fastgraph User's Guide

EGA 200-Line Modes

The 200-line EGA graphics modes (modes 13 and 14) have 16 color values,

numbered 0 to 15. Each color value references one of 16 user-definable palette

registers, often simply called palettes, also numbered 0 to 15. The values

assigned to the palette registers determine the colors in which pixels are

displayed. For example, if you assign palette register 2 the value for red,

then pixels whose color value is 2 will be red.

Each palette can assume one of the 16 colors in the standard color set.

By default, the values assigned to the 16 palettes correspond to the

identically numbered colors in the standard color set. In other words, palette

0 is assigned the value for black, palette 1 is assigned the value for blue,

and so forth.

In modes 13 and 14, fg_setcolor defines the current color by referencing

one of 16 available palette registers. The fg_palette routine defines the

actual color assigned to a specific palette register. The first argument of

fg_palette is an integer between 0 and 15 that specifies the palette number.

The second argument is an integer that defines the palette value (the color

assigned to the palette). Although the actual colors are taken from the

standard color set, the binary structure of a palette value is different from

the IRGB format used in the standard color set. In modes 13 and 14, the binary

structure of a palette value is IxRGB; bit 3 is ignored. The mode 13 and mode

14 palette values that correspond to the standard color set are thus:

value     color     value     color

0       black      16     dark gray

1       blue       17    light blue

2       green      18    light green

3       cyan       19    light cyan

4        red       20     light red

5      magenta     21   light magenta

6       brown      22      yellow

7       gray       23       white

You also can use the Fastgraph routine fg_setrgb to define the color

assigned to a specific palette register. While fg_palette does this using a

color number from the standard color set, fg_setrgb defines a palette register

using red, green, and blue color components, plus an intensity component. The

first argument of fg_setrgb is an integer between 0 and 15 that specifies the

palette register number. The remaining three arguments are each integer values

between -1 and 1 that respectively specify the red, green, and blue color

components for that palette register. The meanings of the color components

are:

-1 = color bit and intensity bit are set

0 = color bit is reset

1 = color bit is set

Since there is only one intensity bit in mode 13 and 14 color values,

specifying -1 for any of the RGB color components produces an intense color.

For example, light cyan is represented by the color value 19, and it is

Chapter 5:  The Use of Color   77

produced by combining green and blue and setting the intensity bit. This means

any of these four statements

fg_palette(1,19);

fg_setrgb(1,0,-1,1);

fg_setrgb(1,0,1,-1);

fg_setrgb(1,0,-1,-1);

could be used to define palette register 1 as light cyan in modes 13 and 14.

The Fastgraph routine fg_setcolor defines the color value (that is, the

palette number) in which subsequent graphics operations are performed. The

fg_setcolor routine takes a single integer argument that specifies this color.

When fg_setmode is called, it sets the color value to 0. The Fastgraph routine

fg_getcolor returns the current color value, as defined in the most recent

call to fg_setcolor. The fg_getcolor routine has no arguments and returns the

current color as the function value.

Example 5-6 demonstrates the use of fg_palette and fg_setcolor in mode

13. After establishing the video mode, the program defines palette 0 to be

blue (1) and palette 1 to be yellow (22). Note that defining palette 0 changes

the background color. It then makes color 1 the current color and displays the

word "Hello". After waiting for a keystroke, the program changes the color of

"Hello" by changing palette 1 to white (23). Finally, it restores the original

video mode and screen attributes before returning to DOS.

Example 5-6.

#include <fastgraf.h>

void main(void);

void main()

{

int mode;

fg_initpm();

mode = fg_getmode();

fg_setmode(13);

fg_palette(0,1);

fg_palette(1,22);

fg_setcolor(1);

fg_text("Hello",5);

fg_waitkey();

fg_palette(1,23);

fg_waitkey();

fg_setmode(mode);

fg_reset();

}

78   Fastgraph User's Guide

EGA Monochrome Mode

The EGA monochrome graphics mode (mode 15) assigns display attributes to

its four color values, numbered 0 to 3. Each color value references one of

four user-definable palette registers, often simply called palettes, numbered

0, 1, 4, and 5. This strange numbering results from the disabling of two of

the four video memory bit planes in mode 15. The values assigned to the

palette registers determine the pixel display attribute. For example, if you

assign palette register 1 the value for bold, then pixels whose value is 1

will be bold.

In mode 15, fg_setcolor defines the current color (actually, a display

attribute) by referencing one of the four palette registers. The fg_palette

routine defines the actual display attribute assigned to a specific palette

register. The first argument of fg_palette is an integer that specifies the

palette number. The second argument is an integer that defines the palette

value (the display attribute assigned to the palette). For each palette

register, the following table shows the default palette value and its

associated display attribute.

palette   palette   display

number     value   attribute

0         0     invisible

1         8      normal

4        24       bold

5        24       bold

Example 5-7 demonstrates the use of fg_palette and fg_setcolor in mode

15. After establishing the video mode, the program makes color 4 (actually,

palette 4, which is bold by default) the current color and displays the word

"Hello". After waiting for a keystroke, the program changes the display

attribute of "Hello" by changing palette 4 to normal intensity (palette value

8). Finally, it restores the original video mode and screen attributes before

returning to DOS.

Example 5-7.

#include <fastgraf.h>

void main(void);

void main()

{

int mode;

fg_initpm();

mode = fg_getmode();

fg_setmode(15);

fg_setcolor(4);

fg_text("Hello",5);

fg_waitkey();

fg_palette(4,8);

fg_waitkey();

Chapter 5:  The Use of Color   79

fg_setmode(mode);

fg_reset();

}

EGA Enhanced Mode

The EGA enhanced graphics mode (mode 16) has 16 color values, numbered 0

to 15. Each color value references one of 16 user-definable palette registers,

often simply called palettes, also numbered 0 to 15. The values assigned to

the palette registers determine the colors in which pixels are displayed. For

example, if you assign palette register 2 the value for red, then pixels whose

color value is 2 will be red.

Each palette can assume one of 64 available colors. By default, the

values assigned to the 16 palettes correspond to the identically numbered

colors in the standard color set. In other words, palette 0 is assigned the

value for black, palette 1 is assigned the value for blue, and so forth. There

are a few EGA-compatible adapters that do not properly assign the default

colors to the 16 palette registers, so it is a good practice to do this

explicitly in mode 16.

In mode 16, fg_setcolor defines the current color value by referencing

one of the 16 palette registers. The fg_palette routine defines the actual

color assigned to a specific palette register. The first argument of

fg_palette is an integer between 0 and 15 that specifies the palette number.

The second argument is an integer that defines the palette value (the color

assigned to the palette). The binary structure of a palette value is different

from the IRGB format used in the standard color set. In mode 16, the binary

structure of a palette value is a 6-bit quantity of the form rgbRGB, where the

lower case letters represent the low intensity (1/3 intensity) color

components, and the upper case letters represent the normal intensity (2/3

intensity) color components. The mode 16 palette values that correspond to the

standard color set are:

value        color          value        color

0          black           56          dark gray

1          blue            57          light blue

2          green           58          light green

3          cyan            59          light cyan

4          red             60          light red

5          magenta         61          light magenta

20          brown           62          yellow

7          gray            63          white

The normal intensity components in mode 16 produce the same normal

intensity colors as in other 16-color graphics modes. Similarly, combining the

low and normal intensities in mode 16 produces the high intensity colors of

the other modes. The only exception to this is for the default brown, formed

from the bit pattern 010100 (20 decimal). This value produces a more true

brown than the value 6 decimal, which is really an olive green.

80   Fastgraph User's Guide

The palette values used in mode 16 are 6-bit quantities, which means

there are 64 different colors available in mode 16. This group of 64 colors

consists of the 16 colors in the standard color set plus 48 additional colors

that are not available in any other EGA modes. However, because the EGA

palette registers hold 4-bit quantities, only 16 of these colors can be

displayed at the same time. In other words, the EGA enhanced mode provides the

capability of displaying 16 simultaneous colors from a group of 64.

You also can use the Fastgraph routine fg_setrgb to define the color

assigned to a specific palette register. While fg_palette does this using a

value between 0 and 63, fg_setrgb defines a palette register using red, green,

and blue color components. The first argument of fg_setrgb is an integer

between 0 and 15 that specifies the palette register number. The remaining

three arguments are each integer values between 0 and 3 that respectively

specify the intensities in thirds of the red, green, and blue color components

for that palette register. For example, the color cyan is represented by the

value 3 in the above table, and it is produced by combining normal intensity

(2/3 intensity) green and blue. This means either of the statements

fg_palette(1,3);

fg_setrgb(1,0,2,2);

could be used to define palette register 1 as cyan.

Example 5-8 demonstrates the use of fg_palette and fg_setcolor in mode

16. It uses the Fastgraph routine fg_rect (discussed in the next chapter) to

draw rectangles of a specified size. After establishing the video mode, the

program uses a for loop to draw 16 equal-size rectangles, one in each of the

16 color values. In the same loop, the program uses fg_palette to change each

palette to black. The while loop that follows performs four iterations. The

first iteration changes palette 0 to 0, palette 1 to 1, and so forth. Hence,

the 16 rectangles appear in the palette values 0 to 15. The rectangles remain

in these colors until is key is pressed to begin the next iteration. The

second iteration changes palette 0 to 16, palette 1 to 17, and so forth. This

makes the 16 rectangles appear in the palette values 16 to 31. Iterations

three and four are similar, so the overall effect of the program is to display

all 64 colors, 16 at a time. Finally, the program restores the original video

mode and screen attributes before returning to DOS.

Example 5-8.

#include <fastgraf.h>

void main(void);

#define COLORS 16

#define WIDTH  40

void main()

{

int base;

int color;

int minx, maxx;

int mode;

Chapter 5:  The Use of Color   81

fg_initpm();

mode = fg_getmode();

fg_setmode(16);

base = 0;

minx = 0;

maxx = WIDTH - 1;

for (color = 0; color < COLORS; color++) {

fg_palette(color,0);

fg_setcolor(color);

fg_rect(minx,maxx,0,349);

minx = maxx + 1;

maxx = maxx + WIDTH;

}

while (base < COLORS*4) {

for (color = 0; color < COLORS; color++)

fg_palette(color,base+color);

base += COLORS;

fg_waitkey();

}

fg_setmode(mode);

fg_reset();

}

VGA and MCGA Two-Color Mode

The VGA and MCGA two-color mode (mode 17) has a background color (color

value 0) and a foreground color (color value 1). Each color value references

one of two user-definable palette registers, often simply called palettes,

also numbered 0 and 1. Each palette register in turn references one of 16

user-definable 18-bit video DAC registers, numbered 0 to 15. The values

assigned to the palette registers and video DAC registers determine the colors

in which pixels are displayed. For example, if palette register 1 contains the

value 3, and video DAC register 3 contains the color value for red, then

pixels whose color value is 1 (that is, the foreground pixels) will be red.

By default, palette register 0 references video DAC register 0, and

palette register 1 references video DAC register 1. In addition, video DAC

register 0 initially contains the color value for black, while the other 15

video DAC registers (1 through 15) contain the color value for white. This

means background pixels (color value 0) are black by default, while foreground

pixels (color value 1) are white.

The 18-bit video DAC values consist of three 6-bit red, green, and blue

color components. Hence, each color component is an integer between 0 and 63;

increasing values produce more intense colors. The default color components

for DAC register 0 are red=0, blue=0, and green=0, which produces black. The

default values for the other DAC registers are red=63, blue=63, and green=63,

which produces white. Because the video DAC registers are 18 bits long, each

DAC can specify one of 262,144 (2**18) colors. However, because the palette

registers hold 1-bit quantities, only two of these colors can be displayed at

82   Fastgraph User's Guide

the same time. In other words, mode 17 provides the capability of displaying

two simultaneous colors from a group of 262,144.

In mode 17, fg_setcolor defines the current color by referencing one of

the two palette registers. The fg_palette routine defines the value of a

palette register by referencing one of the 16 video DAC registers. That is,

fg_palette specifies the video DAC register that a palette register

references. The first argument of fg_palette is either 0 or 1 and specifies

the palette number. The second argument is an integer between 0 and 15 that

specifies the video DAC register for that palette.

The Fastgraph routine fg_setrgb defines the value of a video DAC register

in mode 17. The first argument of fg_setrgb is an integer between 0 and 15

that specifies the DAC register number. The remaining three arguments are each

integer values between 0 and 63 that respectively specify the red, green, and

blue color components for that DAC register.

Example 5-9 demonstrates the use of fg_palette, fg_setrgb, and

fg_setcolor in mode 17. After establishing the video mode, the program defines

DAC register 0 to be blue (red=0, green=0, blue=42) and DAC register 1 to be

yellow (red=63, green=63, blue=21). Note that defining DAC register 0 changes

the background color because palette 0 references DAC register 0. The program

then makes color 1 the current color (palette 1 still references DAC register

1) and displays the word "Hello" in yellow. After waiting for a keystroke, the

program changes the color of "Hello" by making palette 1 reference DAC

register 15 (which still contains its default value, white). Finally, it

restores the original video mode and screen attributes before returning to

DOS.

Example 5-9.

#include <fastgraf.h>

void main(void);

void main()

{

int mode;

fg_initpm();

mode = fg_getmode();

fg_setmode(17);

fg_setrgb(0,0,0,42);

fg_setrgb(1,63,63,21);

fg_setcolor(1);

fg_text("Hello",5);

fg_waitkey();

fg_palette(1,15);

fg_waitkey();

fg_setmode(mode);

fg_reset();

}

Chapter 5:  The Use of Color   83

VGA/SVGA 16-Color Modes

The VGA and SVGA 16-color modes (modes 18, 28, and 29) have 16 color

values, numbered 0 to 15. Each color value references one of 16 user-definable

palette registers, often simply called palettes, also numbered 0 to 15. Each

palette register in turn references one of 16 user-definable 18-bit video DAC

registers, also numbered 0 to 15. The values assigned to the palette registers

and video DAC registers determine the colors in which pixels are displayed.

For example, if palette register 1 contains the value 3, and video DAC

register 3 contains the color value for red, then pixels whose color value is

1 will be red.

By default, each of the 16 palette registers references the video DAC

register of the same number. In addition, the 16 video DAC registers

respectively contain the color values for the 16 colors in the standard color

set.

The 18-bit video DAC values consist of three 6-bit red, green, and blue

color components. Hence, each color component is an integer between 0 and 63;

increasing values produce more intense colors. The default RGB color

components for the 16 video DAC registers are:

DAC  R  G   B    color      DAC  R  G   B      color

0   0  0   0    black       8  21  21 21    dark gray

1   0  0  42    blue        9  21  21 63   light blue

2   0  42  0    green      10  21  63 21   light green

3   0  42 42    cyan       11  21  63 63   light cyan

4  42  0   0     red       12  63  21 21    light red

5  42  0  42   magenta     13  63  21 63  light magenta

6  42  21  0    brown      14  63  63 21     yellow

7  42  42 42    gray       15  63  63 63      white

Because the video DAC registers are 18 bits long, each DAC can specify one of

262,144 (2**18) colors. However, because the palette registers hold 4-bit

quantities, only 16 of these colors can be displayed at the same time. In

other words, mode 18 provides the capability of displaying 16 simultaneous

colors from a group of 262,144.

In the 16-color VGA and SVGA modes, fg_setcolor, fg_palette, and

fg_setrgb behave exactly as in mode 17 with one exception: there are 16

palette registers instead of just two. Example 5-9 demonstrates the use of

these routines in mode 17, but it also would work in mode 18, 28, or 29 if the

call to fg_setmode were changed accordingly.

256-Color Modes

The 256-color modes (modes 19 through 27) have 256 color values, numbered

0 to 255. Each color value directly references one of 256 user-definable 18-

bit video DAC registers, also numbered 0 to 255. The values assigned to the

video DAC registers determine the colors in which pixels are displayed. For

example, if video DAC register 3 contains the color value for red, then pixels

whose color value is 3 will be red.

84   Fastgraph User's Guide

By default, the first 16 video DAC registers (0 to 15) contain the color

values for the standard color set. The next 16 DAC registers (16 to 31)

contain the color values for a gray scale of gradually increasing intensity.

The next 216 DAC registers (32 to 247) contain three groups of 72 colors each,

with the first group (32 to 103) at high intensity, the second group (104 to

175) at moderate intensity, and the third group (176 to 247) at low intensity.

Each group consists of three ranges of decreasing saturation (increasing

whiteness), with each range varying in hue from blue to red to green. Finally,

the last 8 DAC registers (248 to 255) alternate between black and white. This

information is summarized in the following table.

DACs         default color values

0 to 15      standard color set

16 to 31     gray scale of gradually increasing intensity

32 to 55     high saturation, high intensity colors

56 to 79     moderate saturation, high intensity colors

80 to 103    low saturation, high intensity colors

104 to 127   high saturation, moderate intensity colors

128 to 151   moderate saturation, moderate intensity colors

152 to 175   low saturation, moderate intensity colors

176 to 199   high saturation, low intensity colors

200 to 223   moderate saturation, low intensity colors

224 to 247   low saturation, low intensity colors

248 to 255   alternate between black and white

The 18-bit video DAC values consist of three 6-bit red, green, and blue

color components. Hence, each color component is an integer between 0 and 63;

increasing values produce more intense colors. Because the video DAC registers

are 18 bits long, each DAC can specify one of 262,144 (2**18) colors. However,

because the color values are 8-bit quantities, only 256 of these colors can be

displayed at the same time. In other words, modes 19 through 27 provide the

capability of displaying 256 simultaneous colors from a group of 262,144.

In the 256-color graphics modes, fg_setcolor defines the current color by

referencing one of the 256 video DAC registers. The fg_setrgb routine defines

the actual color of a video DAC register. The first argument of fg_setrgb is

an integer between 0 and 255 that specifies the DAC register number. The

remaining three arguments are each integer values between 0 and 63 that

respectively specify the red, green, and blue color components for that DAC

register. Another Fastgraph routine, fg_getrgb, returns the color components

for a specified DAC register. Its arguments are the same as for fg_setrgb,

except the last three arguments (the return values) are passed by reference

rather than by value.

You also can use the Fastgraph routine fg_palette to define the value of

a video DAC register in modes 19 through 27. The first argument of fg_palette

is an integer between 0 and 255 that specifies the DAC register number. The

second argument is an integer between 0 and 63 that specifies the color value

for that video DAC register, using the same 64 values as in the EGA enhanced

mode (mode 16).

Example 5-10 demonstrates the use of fg_setcolor in mode 19. The program

uses the Fastgraph routine fg_rect to draw vertical lines. After establishing

the video mode, the program uses a for loop to draw 256 vertical lines, one in

Chapter 5:  The Use of Color   85

each of the 256 colors (using the default DAC values). Finally, the program

restores the original video mode and screen attributes before returning to

DOS.

Example 5-10.

#include <fastgraf.h>

void main(void);

#define COLORS 256

void main()

{

int base;

int color;

int mode;

int x;

fg_initpm();

mode = fg_getmode();

fg_setmode(19);

x = 0;

for (color = 0; color < COLORS; color++) {

fg_setcolor(color);

fg_rect(x,x,0,199);

x++;

}

fg_waitkey();

fg_setmode(mode);

fg_reset();

}

Example 5-11 shows an interesting effect available in video modes that

support DAC registers. The program uses the Fastgraph routine fg_waitfor

(discussed in Chapter 16) to delay the program's execution. After establishing

the video mode, the program displays the word "Hello" in color 103, which by

default is a pastel blue. It then uses fg_getrgb to retrieve the color

components for this color. The while loop gradually decreases the color

components until all three components are zero, which makes the word "Hello"

smoothly fade to black. Finally, the program restores the original video mode

and screen attributes before returning to DOS.

Example 5-11.

#include <fastgraf.h>

void main(void);

void main()

{

int old_mode;

int red, green, blue;

86   Fastgraph User's Guide

fg_initpm();

old_mode = fg_getmode();

fg_setmode(19);

fg_setcolor(103);

fg_text("Hello",5);

fg_waitfor(18);

fg_getrgb(103,&red,&green,&blue);

while (red+green+blue > 0) {

if (red > 0) red--;

if (green > 0) green--;

if (blue > 0) blue--;

fg_setrgb(103,red,green,blue);

fg_waitfor(1);

}

fg_setmode(old_mode);

fg_reset();

}

The fg_setrgb and fg_getrgb routines work with individual DAC registers.

If you want to define or retrieve a block of consecutive DAC registers,

fg_setdacs and fg_getdacs are more efficient. The fg_setdacs routine defines

the values of a block of consecutive DAC registers. Its first argument is the

index of the first DAC register to define (between 0 and 255), and its second

argument is the number of DAC registers to define (between 1 and 256). The

third argument is a byte array containing the RGB color components for the DAC

registers being defined. The array's first three bytes contain the red, green,

and blue components for the first DAC, the next three for the second DAC, and

so forth. The size of this array must be at least three times the value of the

second argument. The fg_getdacs arguments are the same as those for

fg_setdacs, but the RGB array instead receives the current values of the

specified DAC registers. Both routines treat the DAC register numbers in a

circular fashion (for example, defining four DACs starting with number 254

will define DACs 254, 255, 0, and 1).

Example 5-12 is similar to example 5-11, but it fades many colors

simultaneously. The program displays seven asterisks, one each in colors 9

through 15. It uses fg_getdacs to obtain the current settings of the

corresponding DAC registers; these values are stored in the array RGBvalues.

The while loop gradually fades the RGB components to zero, using fg_setdacs to

update their values, similar to the method of example 5-11. This illustrates

an attractive way of turning an image into a blank screen.

Example 5-12.

#include <fastgraf.h>

void main(void);

void main()

{

int decreasing;

int i;

Chapter 5:  The Use of Color   87

int old_mode;

char RGBvalues[21];

fg_initpm();

old_mode = fg_getmode();

fg_setmode(19);

for (i = 9; i <= 15; i++) {

fg_setcolor(i);

fg_text("*",1);

}

fg_getdacs(9,7,RGBvalues);

fg_waitfor(18);

do {

decreasing = 0;

for (i = 0; i < 21; i++)

if (RGBvalues[i] > 0) {

RGBvalues[i]--;

decreasing = 1;

}

fg_setdacs(9,7,RGBvalues);

fg_waitfor(1);

}

while (decreasing);

fg_setmode(old_mode);

fg_reset();

}

Note that examples 5-11 and 5-12 also would work in 16-color VGA and SVGA

video modes as long as you just use the first 16 video DAC registers.

Using Video DAC Registers in EGA Modes

The fg_getdacs and fg_setdacs routines also work in modes 13, 14, and 16

when used on a VGA or SVGA system. This lets you choose 16 colors from a

palette of 262,144 colors, as in mode 18. If you try to use these routines on

an EGA system, the results are unpredictable. Applications that use these

routines should therefore first insure they are running on a VGA or SVGA

system by checking if fg_testmode(18,0) returns a nonzero value.

Before trying to use fg_getdacs and fg_setdacs in modes 13, 14, and 16,

you should first be aware of the relationship between VGA palettes and DAC

registers. On the EGA, palette values directly determine the color displayed.

On the VGA and SVGA, however, there is an added level of indirection. VGA and

SVGA palette registers can be thought of as pointers to video DAC registers

whose RGB components determine the displayed color.

Each palette register in the VGA 640x480 16-color graphics mode (mode 18)

initially points to the DAC register of the same number. We can thus pretend

the indirection does not exist because changing DAC register n affects those

pixels whose color value is n (unless, of course, we've changed the value of

88   Fastgraph User's Guide

palette register n). In modes 13, 14, and 16, we can't ignore the indirection

because the palette registers contain different values. In mode 13, for

instance, palette register 8 contains the value 16 by default, not the value 8

as in mode 18.

The easiest way around this inconsistency is to set the palette and DAC

registers explicitly so they correspond to the default values of mode 18.

There are two cases to consider -- one for modes 13 and 14, and the other for

mode 16.

In modes 13 and 14, palettes 0 to 7 contain the values 0 to 7, but

palettes 8 to 15 contain the values 16 to 23. Hence, if you want to use

fg_getdacs and fg_setdacs in these modes, you should include the following

code after calling fg_setmode.

char RGBvalues[3];

int i;

for (i = 8; i < 16; i++) {

fg_getdacs(i+8,1,RGBvalues);

fg_setdacs(i,1,RGBvalues);

fg_palette(i,i);

}

This code will set the values of DACs 8 to 15 to the values of DACs 16 to 23.

It also sets palettes 8 to 15 to point to DACs 8 to 15. You can then ignore

the palette-DAC indirection because setting DAC register n affects pixels of

color n.

In mode 16, palette 6 is initially assigned the value 20, and palettes 8

to 15 are assigned the values 56 to 63. All other palettes point to the DAC of

the same number. Hence, if you want to use fg_getdacs and fg_setdacs in mode

16, you should include the following code after calling fg_setmode.

char RGBvalues[3];

int i;

fg_getdacs(20,1,RGBvalues);

fg_setdacs(6,1,RGBvalues);

fg_palette(6,6);

for (i = 8; i < 16; i++) {

fg_getdacs(i+48,1,RGBvalues);

fg_setdacs(i,1,RGBvalues);

fg_palette(i,i);

}

This code will set the values of DAC 6 to the values of DAC 20, and also DACs

8 to 15 to the values of DACs 56 to 63. It also sets palettes 6 and 8-15 to

point to the identically-numbered DACs. You can then ignore the palette-DAC

indirection because setting DAC register n affects pixels of color n.

Chapter 5:  The Use of Color   89

While this might all seem complicated at first, it really isn't once you

understand the relationship between palettes and DACs. The ability to select

colors from a palette of 256K colors instead of 16 or 64 is usually well worth

the extra effort.

RGB Color Mapping

If you're developing an application that runs in 256-color and 16-color

graphics modes, you've probably noticed the inherent differences in defining

color values. In fact, the palette register values even use different

structures within the various 16-color modes. The Fastgraph routine fg_maprgb

helps simplify these differences. It maps three RGB color components (each

between 0 and 63) into a 16-color palette value suitable for the current video

mode. Of course, the range of available colors is much more restricted in the

16-color modes than in the 256-color modes, so fg_maprgb must map the RGB

components to the closest available color.

Example 5-13 runs in any 16-color or 256-color graphics mode and

demonstrates the use of the fg_maprgb routine. In 256-color modes, the program

simply uses fg_setrgb to define DAC register 1 to a pastel blue (red=45,

green=49, blue=63). In 16-color modes, however, the program calls fg_maprgb to

convert the color components into a palette value in IRGB, IxRGB, or rgbRGB

format (depending on the current video mode). The fg_maprgb return value is

passed to fg_palette to set palette register 1 to the closest available color

defined by the specified RGB components.

Example 5-13.

#include <fastgraf.h>

#include <stdio.h>

#include <stdlib.h>

void main(void);

void main()

{

int new_mode, old_mode;

fg_initpm();

new_mode = fg_bestmode(320,200,1);

if (new_mode < 0 || new_mode == 4 || new_mode == 12) {

printf("This program requires a 320 x 200 ");

printf("16-color or 256-color graphics mode.\n");

exit(1);

}

old_mode = fg_getmode();

fg_setmode(new_mode);

fg_setcolor(1);

if (new_mode <= 16)

fg_palette(1,fg_maprgb(45,49,63));

else

fg_setrgb(1,45,49,63);

fg_text("Hello",5);

fg_waitkey();

90   Fastgraph User's Guide

fg_setmode(old_mode);

fg_reset();

}

Defining All Palette Registers

Fastgraph includes a routine fg_palettes that defines all 16 palette

registers in 16-color graphics modes. You also can use fg_palettes to define

the first 16 video DAC registers in 256-color modes. It has no effect in other

video modes.

Using fg_palettes is much faster than calling fg_palette 16 times. The

argument to fg_palettes is a 16-element integer array that contains the color

values assigned respectively to palette registers (or video DAC registers) 0

to 15. Example 5-14 demonstrates how to zero the palette registers (that is,

change them all to black) in mode 13.

Example 5-14.

#include <fastgraf.h>

void main(void);

int zeroes[] = {0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0};

void main()

{

int mode;

fg_initpm();

mode = fg_getmode();

fg_setmode(13);

fg_palettes(zeroes);

fg_setmode(mode);

fg_reset();

}

Of course, as this example is written, it appears to do nothing more than

blank the screen. Its purpose is to show an example of the fg_palettes

routine.

Virtual Colors

By this time it should be clear the use of color is rather specific to

each graphics video mode. One of the most obvious differences is the number of

available colors in each mode; it ranges from 2 to 256. By available colors,

we mean the number of colors that can be displayed simultaneously.

To simplify programming in graphics modes, Fastgraph provides 256 virtual

colors. The virtual colors are used in the graphics video modes having fewer

than 256 available colors. Virtual colors allow you to use 256 color indices

Chapter 5:  The Use of Color   91

in all graphics modes, even if a particular mode does not have 256 available

colors.

When you establish a video mode with fg_setmode, Fastgraph initializes

all the virtual color indices. It does this by replicating the video mode's

color values through the 256 virtual color indices. For example, the CGA color

modes (4 and 5) have four color values, numbered 0 through 3. In these modes,

fg_setmode initializes color indices 0, 4, 8, ... , 252 to 0; color indices 1,

5, 9, ... , 253 to 1; color indices 2, 6, 10, ... , 254 to 2; and color

indices 3, 7, 11, ... , 255 to 3. Similarly, in 16-color graphics modes the

color indices 0, 16, 32, ... , 240 are set to 0, and so forth. In the

monochrome EGA graphics video mode (mode 15), the color values are numbered 0,

1, 4, and 5, so fg_setmode replicates the color indices in groups of eight,

even though there are only four available colors. An analysis of the color

value sequences reveals an often useful feature: by default, virtual color 0

is black and virtual colors 15 and 255 are white in all graphics video modes.

It is thus possible to write a multiple-mode program using the same color

indices for each graphics mode. For example, a program that contains the

statement fg_setcolor(5) would produce subsequent graphics in color 5 (magenta

by default) when running in a 16-color graphics mode. It would produce

subsequent graphics in color 1 (light cyan by default) when running in a CGA

color mode. This is because 1 is the default value assigned to virtual color

index 5 in the CGA color modes.

The fg_setmode routine establishes default values for the 256 virtual

color indices, but it might be desirable to assign other available colors to

them. Going back to the discussion in the previous paragraph, color number 2

is light magenta in the default CGA mode 4 palette. It might make more sense

if the color value 2 were assigned to virtual color index 5, as this would

make the graphics drawn in color 5 the same color in mode 4 as in other color

modes. The Fastgraph routine fg_defcolor is provided for this purpose.

The fg_defcolor routine assigns a color value to a virtual color index.

It has two arguments: the first specifies the virtual color index (between 0

and 255), and the second specifies the color value (between 0 and one less

than the number of available colors in the current video mode). For example,

the statement

fg_defcolor(5,2);

would assign the color value 2 to the color index 5. Another Fastgraph

routine, fg_getindex, returns the current value assigned to a specified color

index. After executing the above call to fg_defcolor, the statement

color = fg_getindex(5);

would store the value 2 (the current value of color index 5) in the integer

variable color.

We must be sure to understand the difference between virtual colors and

palette registers. Modifying the value of a palette register changes the color

of all pixels already drawn using that palette. Modifying a virtual color

index does not do this; it only specifies any graphics drawn in that color

from this point on will appear in the new color.

92   Fastgraph User's Guide

Example 5-15 demonstrates virtual colors in mode 4. After establishing

the video mode, the program uses fg_defcolor to define virtual color indices 0

and 255 to be 1, which by default is light cyan in mode 4. It then draws

characters using color indices 0, 1, and 255, and in each case the characters

appear in light cyan. Finally, the program restores the original video mode

and screen attributes before returning to DOS.

Example 5-15.

#include <fastgraf.h>

void main(void);

void main()

{

int mode;

fg_initpm();

mode = fg_getmode();

fg_setmode(4);

fg_defcolor(0,1);

fg_defcolor(255,1);

fg_setcolor(0);

fg_text("0",1);

fg_setcolor(1);

fg_text(" 1",2);

fg_setcolor(255);

fg_text(" 255",4);

fg_waitkey();

fg_setmode(mode);

fg_reset();

}

A Multiple-Mode Example

Although the color capabilities differ between the supported video modes,

Fastgraph makes it easy to write a program that runs in many video modes. This

section will present an example of such a program.

Example 5-16 illustrates a program that will run in any of Fastgraph's

supported video modes. The program first asks for the video mode number,

checks if the mode number is valid, and then checks if the requested mode is

available on the user's system. After doing this, the program establishes the

video mode and performs its mode-specific code. It then displays a brief

message that includes the video mode number in which the program is running.

This information remains on the screen until a key is pressed, at which time

the program restores the original video mode and screen attributes before

returning to DOS.

Example 5-16.

#include <fastgraf.h>

Chapter 5:  The Use of Color   93

#include <stdio.h>

#include <stdlib.h>

void main(void);

void main()

{

int mode, old_mode;

char string[5];

/* Ask for the video mode number */

printf("Which video mode? ");

scanf("%d",&mode);

/* Make sure the entered value is valid */

if (mode < 0 || mode > 29) {

printf("%d is not a valid video mode number.\n",mode);

exit(1);

}

/* Make sure the requested video mode is available */

fg_initpm();

if (mode > 23) fg_svgainit(0);

if (fg_testmode(mode,1) == 0) {

printf("Mode %d is not available on this system.\n",mode);

exit(1);

}

/* Establish the video mode */

old_mode = fg_getmode();

fg_setmode(mode);

/* Perform mode-specific initializations */

if (mode <= 3 || mode == 7)   /* text modes */

fg_cursor(0);

else if (mode == 4 || mode == 5) { /* CGA color modes */

fg_palette(0,0);

fg_defcolor(14,3);

}

else if (mode == 6) {         /* CGA two-color mode */

fg_palette(0,14);

fg_defcolor(14,1);

}

else if (mode == 11)          /* Hercules mode */

fg_defcolor(14,1);

else if (mode == 12)          /* Hercules low-res mode */

fg_defcolor(14,3);

else if (mode == 17) {        /* VGA two-color mode */

fg_palette(1,14);

fg_setrgb(14,63,63,21);

fg_defcolor(14,1);

}

94   Fastgraph User's Guide

/* Display a message that includes the video mode number */

fg_setcolor(14);

fg_text("I'm running in mode ",20);

sprintf(string,"%d. ",mode);

fg_text(string,3);

/* Wait for a keystroke */

fg_waitkey();

/* Restore the original video mode and screen attributes */

fg_setmode(old_mode);

fg_reset();

}

Example 5-16 displays its message in yellow for those video modes that

offer color. In monochrome video modes, it displays the message in normal

intensity. The program uses virtual color 14, which by default is yellow in

many video modes; the mode-specific code in example 5-16 makes color 14 yellow

in other video modes. In text video modes (modes 0 to 3 and 7), the program

uses fg_cursor to make the cursor invisible. In CGA color modes (modes 4 and

5), the program uses fg_palette to select a CGA palette that contains yellow

as color 3 and then uses fg_defcolor to assign color 3 to virtual color 14. In

CGA two-color mode (mode 6), the program uses fg_palette to make color 1

yellow and then uses fg_defcolor to assign color 1 to virtual color 14. In the

Hercules modes (modes 11 and 12), the program uses fg_defcolor to assign the

value for normal intensity pixels to color 14. In VGA two-color mode (mode

17), the program uses fg_palette to assign video DAC register 14 to palette

register 1. It then defines video DAC register 14 to be yellow with fg_setrgb

and finally uses fg_defcolor to assign color 1 (that is, palette register 1)

to virtual color 14. In all other video modes, no color manipulation is needed

because color 14 is yellow by default.

Summary of Color-Related Routines

This section summarizes the functional descriptions of the Fastgraph

routines presented in this chapter. More detailed information about these

routines, including their arguments and return values, may be found in the

Fastgraph Reference Manual.

FG_COLORS returns the number of simultaneously available colors in the

current video mode. In text video modes, fg_colors returns zero.

FG_DEFCOLOR assigns a color value to a virtual color index. This routine

is only meaningful in graphics video modes that have fewer than 256 available

colors.

FG_GETCOLOR returns the current text attribute (in text modes) or color

index (in graphics modes), as specified in the most recent call to fg_setattr

or fg_setcolor.

FG_GETDACS retrieves the red, green, and blue color components for a

block of consecutively numbered video DAC registers. This routine is only

meaningful modes that use DAC registers.

Chapter 5:  The Use of Color   95

FG_GETINDEX returns the color value assigned to a specified virtual color

index. In text modes and in graphics modes that have 256 available colors,

this routine returns the value passed to it.

FG_GETRGB returns the red, green, and blue color components for a

specified video DAC register. This routine is only meaningful in modes that

use DAC registers.

FG_MAPRGB maps six-bit red, green, and blue color components into a

suitable palette value for the current video mode. You can then pass this

value to fg_palette. This routine is meaningful only in 16-color graphics

video modes.

FG_PALETTE has different functions depending on the current graphics

video mode. For the CGA four-color modes, it establishes the current palette

(of six available) and defines the background color for that palette. In the

CGA two-color mode, it defines the foreground color. For the Tandy/PCjr, EGA,

and VGA graphics modes, it defines the value of a single palette register. For

256-color graphics modes, it defines the value of a single video DAC register.

The fg_palette routine has no effect in text modes or Hercules graphics modes.

FG_PALETTES defines all 16 palette registers (in 16-color graphics

modes), or the first 16 video DAC registers (in 256-color graphics modes). The

fg_palettes routine has no effect in text modes, CGA graphics modes, or

Hercules graphics modes.

FG_SETATTR establishes the current text attribute in text video modes.

This routine has no effect in graphics modes.

FG_SETCOLOR establishes the current color index (which may be a virtual

color index in graphics modes). In text modes, fg_setcolor provides an

alternate method of establishing the current text attribute.

FG_SETDACS defines the values of a block of consecutively numbered video

DAC registers by specifying their red, green, and blue color components. This

routine is only meaningful in modes that use DAC registers.

FG_SETRGB defines the value of a single palette register (in Tandy/PCjr

and EGA graphics modes) or video DAC register (in VGA, MCGA, and SVGA modes)

by specifying its red, green, and blue color components. The fg_setrgb routine

has no effect in text modes, CGA graphics modes, or Hercules graphics modes.

96   Fastgraph User's Guide