Version: 1.04

================================================================================

... Turtle Beach Tropez (TM)

..Answers to Frequently Asked Questions (FAQ)

...Copyright 1995 by Tow Wang

....Version 1.04

..    Release date: Friday 23 June 1995

================================================================================

....Contents

(I) .Introduction

.(A) What is new in this release?

.(B) Abbreviations

(C) How to obtain the latest release of this document.

(II).Technical specifications

(III).Errata to the User's Guide

(IV).Software included with the Tropez

.(A) Utilities

.(B) Drivers

.(C) Other software

(V).Installation

.(A) Tips

.(B) Hardware checks

(VI)    Other questions

(VII).Software and firmware upgrades

.(A) Not included but distributed

.(B) FTP sites

(VIII).Programming information

.(A) Operating systems

.(B) Sound Blaster modes

(C) Windows Sound System mode

.(D) CS4231 codec chip

(IX).Unanswered questions

(X).Credits

(XI).Limitations

--------------------------------------------------------------------------------

....(I) Introduction

Tropez:

... ~ ~     ~..~

.Pronounced as "Tropa" ( 'o' as in toe, 'a' as in pay ).

.Presumably named after the french island St. Tropez, famous for its

.beautiful beaches.

In late 1994, Turtle Beach Systems of York, Pennsylvania, USA, began

distribution of the "Tropez" soundcard, which along with the "Monte Carlo"

card, marked the entry of Turtle Beach products into the entertainment

market of sound hardware for personal computers compatible with the Intel/ISA

architecture. Keeping up with a tradition of high-quality sound products, the

Tropez promises full game compatibility while delivering audio performance

comparable to that of compact discs and digital audio tapes.

The purpose of this document, referred to as "Tropez FAQ file" from hereon,

is two-fold:

1) To supplement the documentation provided with the Tropez package and aid

owners of this product in setting it up and troubleshooting it.

2) To inform the general public of the characteristics of the Tropez and

help assess the suitability of this product for the applications of

potential end-users.

..(A) What is new in this release?

The notation "(@ ?)" means that the update is to be found in (sub)section "?"

*********

Minor re-wordings throughout the document.

Tropez is capable of sampling at 48000 Hz....(@II)

"Wave SE" v1.30 now available......(@IV.C)

More ways to obtain this document.....(@I.C)

"Set Maui" v1.20 released......(@VII.A)

Tested rev. 2.0 on different motherboards....(@VI)

Acknowledged rare clipping problem.....(@IX)

SIMM's do not require parity......(@II)

Loading "tzmode.exe" first......(@V.A)

Added sub-section.......(@VIII.D)

"Patchman" .098c Beta was released.....(@VII.A)

Windows 95 drivers released......(@VIII.A.2)

*********

...(B) Abbreviations

Following is a list of abbreviations used throughout this document.

DOS.Disk Operating System (TM)

FAQ.Frequently Asked Questions

GM.General MIDI (TM)

MIDI.Music Instrument Digital Interface

SB.Sound Blaster (TM)

SIMM.Single In-line Memory Module

TB.Turtle Beach

WSS.Windows Sound System

Note:

All references to installation information, such as directory paths, are

exemplified using default settings.

(C) How to obtain the latest release of this document.

By FTP from the following InterNet sites:

ftp.wi.leidenuniv.nl./pub/audio/tropez/

ftp.hawaii.edu../outgoing/Maui/

ftp.netcom.com../pub/jo/jonin/soundcards/faqs/

ftp.otol.fi../pub/music/TB/Maui.(Finland access only)

On the World-Wide Web:

www.wi.leidenuniv.nl/audio/

www.cs.colorado.edu/~mccreary/

www.helsinki.fi/~wikgren/tropez/tropez.html

rpinfo.its.rpi.edu/~hsiaoe/soundsite/

On the Andrew File System:

/afs/engin.umich.edu/u/t/o/towwang/Public.<-Sorry for the shuffling

On UseNet:

Newsgroup "comp.sys.ibm.pc.soundcard.misc"

..(D) Additional sources of information

1) Related FAQ files:

- Turtle Beach Maui FAQ file, available from ftp.hawaii.edu. READ THIS FILE!

Almost all the information in it is applicable to the Tropez!

- Turtle Beach FAQ file by Kevin Dangoor (tazzzzz@eecs.umich.edu), now

un-maintained.

- Gravis UltraSound FAQ file:

archive.epas.utoronto.ca./pub/pc/ultrasound/info

for information on instrument patches.

2) Mailing lists:

"Turtle Beach mailing list": to subscribe, send a message to:

.listproc@lists.colorado.edu

with subject

.SUBSCRIBE MULTISOUND <your name>

3) Independent commercial reviews:

PC Magazine, 28 March 1995.

--------------------------------------------------------------------------------

...(II) Technical specifications

The indented information is distributed by Turtle Beach Systems in the

form of free promotional literature and is also printed on the Tropez package.

Some additional comments and corrections I have added for purposes of

clarity.

.Compatibility

The Tropez emulates a Sound Blaster Pro card (revision 2.0 by Creative Labs),

including, but not limited to, DAC and mixer functions and FM music

synthesis.

The Tropez is also compatible with the Windows Sound System interface.

However, only one mode (SB Pro xor WSS) is available to produce digital audio

at any time.

Software running under DOS that supports MIDI music synthesis, using the

General MIDI patch set and the Roland MPU-401 interface, will be able to

take advantage of the wavetable synthesis capabilities of the Tropez. This

capability for music synthesis is available independent of the setting for

digital audio described in the preceeding paragraph.

In simple words: in games you can hear sound effects using either SB Pro

or WSS mode, with or without simultaneous GM music in either case.

Under Windows, all compatibility is provided by a digital audio driver and

a MIDI synthesis driver, both of which are compliant with the Windows

sound Application Programming Interface.

.Digital Audio

.Sampling Rates..Selectable between 4 KHz and 48 KHz

.Resolutions..16 or 8 bits per sample

.Audio channels..Stereo or mono

.Audio converters A/D.64x oversampled sigma-delta, 16 bit

... D/A.8x interpolating filter

....64x oversampled, 16 bit

.DMA mode..Demand mode for increased performance

.Simultaneous record and play capability

A/D and D/A conversion is performed by a Crystal CS4231 coder/decoder chip.

.Stereo mixer

.Inputs -- Synthesizer (FM and Wavetable), Microphone, Line, Auxiliar

.Supported interfaces

.MIDI interface (optional).Up to 2 MPU-401 compatible external

.....interfaces using optional cables

.CD-ROM drive interface..AT-API (Enhanced IDE)

.Joystick port

Both MIDI ports are MPU-401 compatible in dumb UART mode but not smart mode.

Reported by Martin Fay <martin.fay@umist.ac.uk>:

The pin-out of the second MIDI port (the two-row internal male connector) is

the same as that of the joystick port interface found on most I/O boards and

disk drive controllers.

Someone said that the CD drive interface can also be used with the Mitsumi

FX001D drive.

The joystick port can be disabled by software to avoid an I/O conflict if

another such port is already enabled.

.Wavetable synthesizer

.General MIDI compatible..Yes

.Simultaneous voices..Maximum is 32

.Wavetable instruments..128 instruments in 2 MB ROM

.Sample Data Formats..16 bit, 8 bit and 8 bit u-law

Maximum simultaneous voices.Sampling rates (KHz)

32....32

24....44.1

.Sample Store (TM)

.RAM size..Upgradable to a maximum of 12 MB using

....standard SIMM chips

There are 3 SIMM slots on the Tropez. They all accept SIMMs with 30

contact-pads. Any slot can accept any of the following SIMM capacities at

any time: (TB recommends an access time no slower than 70ns)

Rows..Bits per row (parity not required)

256 * 1024.8 or 9

1024 * 1024.8 or 9

4096 * 1024.8 or 9

On Thu, 8 Jun 1995, John M. Meyers wrote:

> I'm currently using 3 1x8's in my Tropez, and things are fine. A friend

> is using 2 megs of the same in his Maui, and I've heard no complaints.

The Tropez package includes no read/write memory. SIMM's must be purchased

separately.

.FM Synthesizer

.Stereo FM Synthesizer (Yamaha OPL3)

.AdLib compatible

.20 independent stereo voices of FM synthesized sounds

.4 FM operators per voice

The OPL3 and the WaveFront synthesizers can play music simultaneously.

.Audio Performance

.Signal to noise ratio.89 dB (A weighted), 87 dB (unweighted)

....(these values MUST be positive)

.Total Harmonic Distortion

....< 0.01 % (A weighted), < 0.02 % (unweighted)

.Phase response..+/- 0.1 degrees

.Stereo Crosstalk.-90 dBV at 100 Hz,

....-89 dBV at 1 KHz,

....-83 dBV at 10 KHz

.Intermodulation distortion.< 0.1 %

.Frequency response.20 Hz - 22 KHz, +0/-1 dB

--------------------------------------------------------------------------------

..(III) Errata to the User's Guide

This section applies to the "User's Guide" (Manual) version 1.0,

Copyright 1994 Turtle Beach Systems, Inc.

P 14.What's in the box?

P 24

Turtle Beach currently ships FOUR 3+1/2 inch high-density floppy disks with

software and utilities.

The "Info Now" CD may have been substituted by a "Selectware System (tm)" CD.

Refer to the software section.

P 17.Adjusting the jumper settings

All jumper sketches were printed upside down. I am providing the correct

ones below; my sketches are in the same order as they appear in the manual

but I have reordered the port numbers.

'c' means closed (a jumper connects the two pins at this position)

'o' means open (no connection between the two pins at this position)

o...c...o

o.330..c.230..c.300

o...o...c

c...o...c

o.290..o.338..c.210

o...c...c

o...c

c.320..o.260

o...c

P 30.DOS diagnostics

The "Sound Blaster music test" will play notes to the right audio channel

only.

P 32-34.WaveFront Diagnostics

"wfdiag.exe" is no longer included with any of the installation diskettes.

If "setupsnd.exe" executes without reporting any errors, then your

WaveFront synthesizer is most likely configured correctly. I have executed

"mauidiag.exe" on my Tropez and it passed all tests except the one which

requires a MIDI loop-back cable (which I did not have) and the WaveSample

memory test (I had not yet obtained any SIMM's when I did this). To obtain

"mauidiag.exe", see the FTP sites listed elsewhere in this

document.

By the way, "mauidiag.exe" is a diagnostics program for the Maui sound card,

which uses the same WaveFront synthesizer as the Tropez.

P 36.Tropez DOS utilities

P 43.Translation Utility

"tzsbmix.exe" is an utility for the (old) Monte Carlo sound card.

It was included by mistake with the Tropez package.

It cannot work with the Tropez.

It will not add any more functionality because the Tropez is already

fully compatible with SB Pro.

P 50.setupsnd.exe utility

"oswfi.mot" is no longer included with the Tropez package. Just use

"oswf.mot" with "setupsnd.exe".

The parameter "/x" should be used only if you have an external synthesizer

connected to the Tropez's MIDI port.

--------------------------------------------------------------------------------

..(IV) Software included with the Tropez

If you do not have the latest versions, refer to the FTP sites for updates.

....(A) Utilities

setupsnd.exe.v1.13

Configures the WaveFront synthesizer.

Loads operating system code to the firmware memory on the Tropez. This o.s.

code is executed by the Motorola 68000 processor to control the synthesizer

and it is necessary for the WaveFront to work under DOS.

On a 80486DX 33 MHz, "setupsnd.exe" takes about 20 seconds to load "oswf.mot"

to the Tropez.

tzacmix.exe.v2.00

Command line utility to change all mixer volumes.

tzhkmix.com.v2.00

TSR program to change mixer volumes with hot keys (hold down ALT and press

the gray keys Insert, Delete, End, Page Up, Page Down).

tzmode.exe.v2.00

Initializes the CD drive interface on the Tropez, when executed as a device

driver within "config.sys".

Lets you select the digital audio mode of the Tropez under DOS.

tzsetup.exe.v2.00

Lets you reconfigure all settings for the Tropez, except the primary

(external) MIDI port which requires jumper adjustment.

tbcd.exe

Audio-CD player. If you have problems using it, Mario Koeppen

<mk2@irz.inf.tu-dresden.de> provided this tip:

> ...look into your "system.ini" file for a line containing "CDAudio" as the

> right value of an equation. If there is any number after this (ie. CDAudio_1)

> cut it off, leaving only "CDAudio".

This number is used for operating more than one CD drive. TB tools can

only handle one. Warning: this change may render certain setups unusable.

Back up your "system.ini" before modifying it.

....(B) Drivers

oswf.mot.v1.15

The operating system code that is loaded by "setupsnd.exe" under DOS and by

"tropmidi.drv" under Windows.

tropez.drv.v1.10

Digital audio driver for using the Tropez under Windows. This includes code

to control the mixer on the Tropez.

tropmidi.drv.v1.10

MIDI synthesis driver for using the Tropez under Windows.

On a 80486DX 33 MHz, "tropmidi.drv" takes 6 seconds to load "oswf.mot" to

the Tropez.

Version 1.20 Beta was not distributed. (?)

Version 1.30 distributed as file "trop130.zip".

....(C) Other software

>From Turtle Beach's literature:

.WavePatch (TM)..Patch editor for wavetable synthesizer, v1.2

On Thu, 27 Apr 1995, Jean-Luc Piolain wrote:

> WavePatch has a few features designed specifically for the professional

> sound developer. In order to keep the average user from accessing these,

> they are only available if the following line is added to the [Preferences]

> section of WFPATCH.INI.

>

> [Preferences]

> Pro Version = 1

>

> When this line is added, the File menu will have two new items:

>

>       Replace Internal Sample

>       Pack ROM Files

Latest version is 1.3 "Release", file name "waveph13.zip" at FTP sites.

.Wave SE 1.23 (TM)..WAV audio and sample editing program with

....Loop Editor

According to David and Craig (TB sales consultants, 9 June 1995): Latest

version is 1.30, incorporating minor bug fixes but no new features.

Upgrade is shipped to TB customers for $7.50 (within USA).

"Wave SE II" version 1.30 is also available as a separate purchase.

.MicroWave (TM)..OLE compatible .WAV recorder/editor for

....business applications

.Stratos (TM)..MIDI song writing software

.Sierra (TM) Audio Rack.Featuring stealth mode

.MousePlayer..MIDI controller

The "SelectWare System (TM)" CD contains dozens of demonstration programs for

commercial software, and five free products:

.Corridor 7 (TM)...by Capstone

.MegaMaze (TM)...by Axis Interactive and CapDisc

.LINKS: The Challenge of Golf(TM)by Access Software

.Darklands (TM)...by MicroProse Software

.Freakin' Funky Fuzzballs (TM).by Sir-Tech Software

--------------------------------------------------------------------------------

...    (V) Installation

....(A) Tips

This section should be most useful if you read it along with the "User's

Guide" while you install the Tropez.

.BIOS settings

From: Roeder Sascha <roeder@mi-lab.fh-furtwangen.de>

> If you use a Asus SP3G Motherboard with an Award bios, try to

> switch off the  "ISA GAT MODE" in the bios setup. I dont know, what this

> mode is for but it interfered with my Tropez during recording.

The Tropez may not install correctly if "Plug and Play" is enabled in the BIOS.

If you are experiencing problems with the digital audio performance of the

Tropez, check that your bus speed is set to 8 or 8+1/3 MHz.

P 17.Adjusting the jumper settings

Most games require that the MPU-401 interface be at port 330h. To have this

setting, you may leave JP4 at the default setting (none of the pairs of pins

should be shorted by a jumper).

P 26.Running the DOS setup

Your "autoexec.bat" file should have at least one trailing blank line.

Otherwise, the last character of your "autoexec.bat" will be overwritten by

"install.exe" if you let it make automatic modifications for you.

P 27

If you are connecting a CD drive to the enhanced IDE interface of the Tropez,

make sure that the line

.DEVICE=?:\tropez\dosapps\tzmode.exe

in your "config.sys" file precedes the line that loads the device driver for

your CD drive. This is because "tzmode.exe" must be executed first to

initialize the Tropez so that the other device driver can detect the CD

drive interfaced with the Tropez.

If you still have problems using the Tropez's CD drive interface, try

executing ("loading") "tzmode.exe" in the very first line of your

"config.sys". You must do this if you use QEMM.

>From Rob Geraghty <r.geraghty@mailbox.uq.oz.au>:

The internal audio connector has the following configuration:

--o left signal

--o left earth

--o right signal

--o right earth

P 27.(Windows setup)

The command "win.com" to start up Windows must reside in a directory included

in the "PATH" environment variable. "install.exe" will try to execute the

command

.win.com setup.exe

before installing any Windows applications. If "install.exe" does not

find "win.com", it will prompt erratically for you to insert a diskette. In

this situation, you may press 'Esc' to exit the program. Include the Windows

directory in the 'PATH' variable and rerun "install.exe".

Side note: as "setup.exe" prepares to install the Windows applications for

the Tropez, it will probe all available logical drives for the presence of

the installation diskettes. If for example you have the installation

diskettes in drive "b:", "setup.exe" may get stuck when probing an empty

disk drive "a:". In this case, simply insert any irrelevant (but formatted)

diskette in drive "a:" and let "setup.exe" move on to drive "b:"

P 28.Windows setup

You can no longer install Windows software using "setup.exe" within Windows;

you are required to run "install.exe" from a bare DOS command prompt. Note

that there are no provisions for incremental installation of Tropez software.

Each time you choose one of two the installation options in "install.exe",

this program will overwrite any Tropez files that were previously installed

in "?:\tropez\dosapps\"

Paraphrasing Rob Geraghty <robg@medeserv.com.au>:

>Hidden in a confirmation screen during the install process (in windoze) the

>Tropez defaults to using IRQ15 for OS download. This can be changed - I used

>IRQ9, the same IRQ I used for Tropez MIDI.

Forgetting to make this change may cause IRQ conflicts with certain IDE

controllers.

Within Windows, when you are presented the dialog to choose which Windows

applets to install, consider installing them all at once so as to avoid the

hassle of re-installing everything when you later want to add a Tropez

applet. It is actually easier to manually remove applets guided by the

information dumped into the "tropez.log" text file.

Note: you should definitely install the Tropez mixer at least.

Message from David Gasior <SharkBoyFL@aol.com>:

> The Mixer for Windows can have its settings

> saved when you close it by adding the line

>

> SaveSettingsOnExit=ON

>

> under the heading [DRIVER] at the top of the TROPEZ.INI file found in the

> Windows directory.

"setup.exe" only maps MIDI channels 1 through 10 to the "Tropez Sample Player"

initially; you may want to map the remaining 16 channels to it or to the OPL3

driver using "MIDI Mapper".

P 36.Hot key sound mixer utility

The "tzhkmix.com" TSR might not work with games that run in Intel protected-

mode. For instance, it has no effect within Doom 1.

P 41.Using the command line sound mixer

If you omit the optional channel selector ( [L|R] ), the volume setting will

apply to both audio channels.

P 50."setupsnd.exe" utility

You may delete the "oswf.mot" installed in "?:\tropez\dosapps\" since

another copy of it is installed in "?:\windows\system\", provided that you

tell "setupsnd.exe" to find "oswf.mot" at the latter directory.

However, do not delete the file at the latter directory as it is required

to be there by the Windows MIDI driver "tropmidi.drv".

"setupsnd.exe" always tries to access one IRQ line and one I/O address

(actually one block of I/O addresses). If either one (or both) was set up

incorrectly, the program will report that you have an "IRQ conflict".

.Additional documentation

Do not forget to read the "*.wri" files that were installed to "?:\tropez\".

They contain updated information that was not available when the User's Guide

was printed. You can read them using the "Write" word processor included with

Windows.

."tzsetup.exe" utility (not documented in manual)

The "MPU-401 MIDI" option in "MIDI Setup" refers to the second MIDI port on

the Tropez (the one that is next to the CD drive interface). Unless you

purchased an adapter cable to use this second port, you should leave it as

disabled.

....(B) Hardware checks

As should be the case whenever you purchase computer hardware, you are

advised to test all the features of your merchandise as soon as possible

after you receive it. For the Tropez in particular, you should test the

following:

1) Microphone recording: plug a microphone, with (or near) the appropriate

impedance as indicated in the User's Guide, into the corresponding

jack. You should be able to get from the output of the Tropez the

sounds that are being picked up by the microphone.

Do not forget to UN-mute the microphone input and set its level high

enough. Use an UN-biased microphone.

2) Plug SIMM's into all the memory slots on the Tropez and test its Sample

Store capabilities using Wave Patch. Also, "setupsnd.exe" should

report the correct amount of memory installed on the Tropez.

If you cannot afford memory, you would do just fine by borrowing one

SIMM and

performing this test three times, with the SIMM plugged into a different

slot each time.

If you have any questions or problems, you should post your concerns to a

user forum or newsgroup.

--------------------------------------------------------------------------------

...(VI) Other questions

Q. Can the Tropez emulate a Roland MT-32 or LAPC1 for games?

A. Partially.

You can load a MT-32 patch bank to the memory of the Tropez and

have it play music correctly under games that DO NOT ATTEMPT TO PRODUCE

SOUND EFFECTS (i.e. that do not attempt to alter the patches).

Check file "mauimt32.zip", created by Jimmy Wan <vecna@umich.edu>, at

FTP sites. You will also need the DOS bank loader "Set Maui".

The Tropez does not support MT-32 system-exclusive messages; crashes are

possible if your game tries to send such messages to the Tropez.

Q. Is the Tropez compatible with the Sound Blaster 16?

A. No, and the Tropez does not have a WaveBlaster connector.

Q. What .MOD file players support the Tropez?

A. Any player that supports WSS and most players that support SB or SB Pro.

Some examples are: DMP (Dual MOD Player) and Morgul. Refer to the section

on FTP sites.

Q. Does the Tropez feature Hurricane Architecture?

A. No. It requires at least one DMA channel for digital audio functionality.

Q. Can I use the "Quad" software with the Tropez?

A. No.

Q. Is there a processor for effects such as reverb and chorus on the Tropez?

A. No.

Q. How long does it take to load samples to the memory on the Tropez?

A. There is an overhead of about 30 seconds to let the Wave Patch program

initialize, but I loaded about 400 KB of a drum kit in 5 seconds.

Q. What operating systems are supported by the Tropez package?

A. DOS and Windows. Also, refer to the "Programming information" section.

Q. Where can I get AIL and MIDPAK drivers for the Tropez?

A. You will probably want the WSS AIL drivers. Refer to the FTP section.

The file "t7gfix3.zip" (a patch for "The 7th Guest") has the MIDPAK

drivers. The latter is available from ftp.cdrom.com

Q. Where can I buy the Tropez? Ask TB.

Q. How do I contact TB?

A. Address:.Turtle Beach Systems

..52 Grumbacher Road

..York, Pennsylvania 17402 USA

Telephone:.800 645 5640 and 717 767 0200

Fax:..717 767 6033

E-mail:..sales@tbeach.com

Q. What differences are there between the various revisions of the Tropez?

A. On 5 Apr 1995 Alexander_H._Neustadter@BOPS.VOA.GOV wrote:

> Version 1.3 has the channels reversed.  This has been my experience with

> more than one board. (I note that the WaveFront chip is also much

> thinner on this board.) Version 2.0 has the channels wired correctly,

> but makes a nasty pop on the left channel when starting & stopping the

> playing of .wav files.

I own a rev. 2.0 board and have tested it on these motherboards:

AIR 486UL.No audible pop at end of .wav playback

486 UMC.Inconclusive: so faint I cannot tell whether it is a pop

..or some external noise.

--------------------------------------------------------------------------------

..(VII) Software and firmware upgrades

..(A) Not included but distributed

All software listed by file name. If I do not indicate where to find any

file, you can get it from the FTP sites listed in the next section of this

document.

*********

"2pat15.zip"

On Fri, 5 May 1995, Markus J�nsson wrote:

> Now available by WWW from:

>         http://www.nada.kth.se/~f93-maj/fmjsoft.html'

> Also has been uploaded to:

>         ftp://archive.epas.utoronto.ca/pub/pc/ultrasound/submit/2pat15.zip

Horvath Tamas <hotta@sch.bme.hu> wrote:

> Win32s 1.20 can be found at:

ftp.microsoft.com./SoftLib/MSLFiles/PW1118.EXE

"smaui120.zip"

Load bank files to the WaveFront's memory under DOS.

On Sat, 24 Jun 1995, Cafarella, John <JCafarel@vibuscy.ccdn.otc.com.au>

will write:

> Patchman is a utility that is designed to avoid having HUGE Wavefront Bank

> files (.WFB's) on your harddisk.

Soon to be available at ftp.hawaii.edu

*********

...(B) FTP sites

ftp.hawaii.edu

/outgoing/Maui/

.       (Software and firmware for Turtle Beach products)

ftp.wi.leidenuniv.nl.(University of Leiden, The Netherlands)

/pub/audio/

.   players/

.   (MOD players)

.   programming/dmkit.zip

.   (AIL, DIGPAK and MIDPAK drivers/programming kit)

.   tropez/

.   (Tropez-related files under this hierarchy)

--------------------------------------------------------------------------------

...(VIII).Programming information

...    (A) Operating systems

1) OS/2 (TM)

To use the Maui/Tropez as a GM synthesizer under OS/2, you must first boot DOS,

initialize the WaveFront synthesizer (refer to your manual for "tzmode.exe"

and "setupsnd.exe) and then boot OS/2 using the utility "reboot.exe".

Additional comments by John Meyers <jmmeyers@winternet.com>:

> When you call REBOOT.EXE, you >MUST< be running

> with no TSRs, no interupts redirected, and in real mode, This means, no

> EMM386, QEMM, whatever. I'm using DOS 6.22, so I've got a separate

> configuration the only runs TZMODE, TZACMIX, and REBOOT - no device

> drivers or anything.

Assembly language source code for "reboot.exe":

----------------------------

.LIST

.MODEL TINY

.CODE

.ORG     0100H

Main    PROC    NEAR

.int     19H

Main    ENDP

..

END     Main

---------------------------

The executable program may be found in the FTP sites that carry this

document, under the same directory hierarchy.

On Fri, 14 Apr 1995, Shane Lord <slord@ibm.net> reported that you can get

16-bit digital audio by installing the "Business Audio AD1848" driver and

setting the Base Port, IRQ, and DMA channel to the values configured for

the WSS mode of the Tropez. This driver comes with OS/2 3.0 "Warp", but

unfortunately only enables a maximum sampling rate of 22050 Hz.

2) Windows 95

Drivers are available at:

ftp.hawaii.edu./outgoing/Maui/win95drv.zip

...(B) Sound Blaster modes

For information on programming the Sound Blaster family of cards, check out:

ftp.eng.ufl.edu../pub/msdos/demos/incoming/SBLAST09.ZIP

Oak.Oakland.Edu../SimTel/msdos/sound/sblast09.zip

x2ftp.oulu.fi../pub/msdos/programming/mxinfo/sblast09.zip

...(C) Windows Sound System mode

No information available at this time. You may want to program for the

DIGPAK driver for WSS.

...(D) CS4231 codec chip

Ref.: Tue, 31 Jan 1995, Andrew Daviel <andrew@andrew.triumf.ca>:

A block diagram and sample source code is available by FTP from:

andrew.triumf.ca./pub

--------------------------------------------------------------------------------

..(IX) Questions that remain unanswered

If you have information about the following and would like to contribute it

to this document, please contact me at my address in the credits section.

**********

How do you extract manually the applets from the "*.z" archives of the

software diskettes? (They cannot be extracted with the "compress.exe"

utility.)

What was the intended use of the file "oswfi.mot"? I do not understand the

description in the User's Guide.

Has anyone been able to record at 44.1 KHz rate, 16 bit stereo in

Enhanced Duplex mode?

How can we obtain a software development kit for Windows Sound System?

I acknowledge that the line-output voltage of certain CD players is so

high that you cannot feed it into the Tropez without causing some

clipping during recording. I am searching for a convenient solution

(short of inserting resistors).

**********

--------------------------------------------------------------------------------

....(X) Credits

If you have contributed to this document but are not mentioned, please let

me know. I am forgetful, but certainly not ungrateful :)

Names are listed in alphabetical order.

**********

Rich Heimlich <rich.heimlich@compudata.com>, for information and goodies.

Brandon Higa <bhiga@uhunix.uhcc.hawaii.edu>, for FTP site support and for the

Maui FAQ file.

Eric Hsiao <hsiaoe@rpi.edu>, for GFX News and WWW site support.

Lord Jonin <jonin@netcom.com>, for technical information.

Janne Juntunen <ok93juja@otol.fi>, for FTP site support.

Sean McCreary <mccreary@cs.colorado.edu>, for maintaining TB Mailing List.

Daniel Tauritz <dtauritz@wi.leidenuniv.nl>, for FTP+WWW site support.

Turtle Beach Systems, for technical information and for supporting this

document.

Miro Wikgren <wikgren@cc.helsinki.fi>, for WWW distribution.

**********

With the kind collaboration of the mentioned contributors, this document

was written and is maintained by yours truly, Tow Wang. I can be reached

on the InterNet by electronic mail as: towwang@umich.edu

Please note: I am not affiliated with Turtle Beach Systems. I do not provide

technical support for the Tropez or any other computing-related products.

All trademarks mentioned are registered by their respective companies.

--------------------------------------------------------------------------------

...(X) Limitations of this document

This document is distributed at no charge to the general public. No guarantee

whatsoever is provided for the accuracy and/or fitness of the information

contained herein with respect to any purpose or task.

Readers may re-distribute this document provided that the following conditions

are met:

1) No fee whatsoever is charged to the recipient(s).

2) Recipients must get his document in its entirety, without any modifications.

Distribution of this document by other parties (such as commercial

establishments), and waivers to the preceeding conditions, require the written

permission of the author.