SNES KART v1.2

(The most complete guide to a SNES cartridge)

Copyright Ó 1996 DiskDude. All rights reserved.

Copyright Ó 1996 Damaged Cybernetics. All rights reserved.

None of the information contained in this text comes from any confidential source. It was obtained from various sources on the Internet, but also the product of my own investigation. Refer to the Acknowledgements section at the end of this text.

Use this information for your own use, I will not take any responsibility for your actions. All copyrights and trademarks are owned by their respective owners, no infringements intended.

I wrote this because all of this information is scattered in small files everywhere, if existing at all, most of it outdated. This is an attempt to conveniently bring all of the information to one file, and as up-to-date as possible.

Contents

PIN LAYOUTS

What are the cartridge pinouts?

What are the ROM pinouts?

What are the DSP pinouts?

What is the MAD-1 and it's pinouts?

What are the pinouts of the 16kbit SRAM most commonly used by Nintendo?

CARTRIDGE ADDRESSING SCHEMES

8Mbit cartridge or less:

10Mbit cartridge or more:

EMBEDDED CARTRIDGE INFORMATION

Game title (21 bytes)

ROM makeup (1 byte)

ROM type (1 byte)

ROM size (1 byte)

SRAM size (1 byte)

Country (1 byte)

License (1 byte)

Game Version (1 byte)

Inverse ROM Checksum (2 bytes)

ROM Checksum (2 bytes)

Non Maskable Interrupt / VBL Vector (2 bytes)

Reset Vector (2 bytes)

How do I know if the ROM is HiROM or LoROM?

CHEAT DEVICE DECODING

Pro Action Replay (hardware)

Gold Finger (software)

Game Genie (hardware)

Converting between CPU addresses and ROM addresses

Easily converting between codes

SNES COPIERS

What are copiers?

Super Wild Card (SWC) header information

Pro Fighter (FIG) header format

Game Doctor file name format

ROM PROTECTION SCHEMES

SlowROM checks

PAL/NTSC checks

SRAM size checks

IPS PATCH FORMAT

ACKNOWLEDGEMENTS

HOW TO CONTACT ME

# Pin layouts

All active low pins are denoted by an asterisk.

## What are the cartridge pinouts?

'Lockout' refers to the Nintendo CIC chip. If the SNES doesn't detect this chip while power is on, then it will not continue to read the cartridge. Further details of this are not known to me.

01   32

02   33

03   34

04   35

GND   05   36   GND

f                   A11   06   37   A12

r                   A10   07   38   A13

o                    A9   08   39   A14

n                    A8   09   40  *A21

t                    A7   10   41   A15

A6   11   42   A16

o                    A5   12   43   A17

f                    A4   13   44   A18

A3   14   45   A19

c                    A2   15   46   A20

a                    A1   16   47   A22

r                    A0   17   48

t                  *IRQ   18   49   ROM/RAM *CS

D0   19   50   D4

D1   20   51   D5

D2   21   52   D6

D3   22   53   D7

ROM/RAM *OE   23   54   RAM *WE

lockout - Pin 1   24   55   lockout - Pin 2

lockout - Pin 7   25   56   lockout - Pin 6

RAM CS   26   57

Vcc   27   58   Vcc

28   59

29   60

30   61

31   62

## What are the ROM pinouts?

This pin layout may be inaccurate, and was taken from a Donkey Kong Country and a Super Mario World cartridge.

A19             Vcc

GND             GND

A16   01   32   Vcc

A17   02   31   /OE

A21   03   30   A18

A12   04   29   A14

A7   05   28   A13

A6   06   27   A8

A5   07   26   A9

A4   08   25   A11

A3   09   24   A15

A2   10   23   A10

A1   11   22   /CE

A0   12   21   D7

D0   13   20   D6

D1   14   19   D5

D2   16   18   D4

GND   16   17   D3

## What are the DSP pinouts?

I have no information on this, because I don't own a cartridge with DSP. This will be updated if and when I purchase Pilotwings.

## What is the MAD-1 and it's pinouts?

The MAD-1 stands for ??? Address Decoder version 1?. It is used on the Donkey Kong Country cartridge and probably other cartridges in order to address more that one ROM on a cartridge and a static RAM. i.e. 2x16mbit ROM's for a 32mbit game.

*HI   01   16   *LO

*OE   02   15   A13

03   14   A14

04   13   A20

Vcc   05   12   A21

p10   06   11   ROM *CS (p49 SNES slot)

Vcc   07   10   p06

GND   08   09   RAM CS (p26 SNES slot)

*LO - *CS on ROM1 (lower 16mbit)

*HI - *CS on ROM2 (higher 16mbit)

*OE - *OE on Static RAM

## What are the pinouts of the 16kbit SRAM most commonly used by Nintendo?

It seems that Nintendo uses this SRAM in many of their games, mainly because it is very cheap, only $A5 (retail).

A7   01   24   VCC

A6   02   23   A8

A5   03   22   A9

A4   04   21   *WE

A3   05   20   *CE

A2   06   19   A10

A1   07   18   *CS

A0   08   17   D7

D0   09   16   D6

D1   10   15   D5

D2   11   14   D4

GND   12   13   D3

# Cartridge addressing schemes

## 8Mbit cartridge or less:

| read ROM | ROM/RAM *CS, ROM/RAM *OE, RAM CS low RAM *WE high |
| --- | --- |
| read SRAM | ROM/RAM *CS, ROM/RAM *OE low RAM CS, RAM *WE high A19, A20, A21 high |
| write SRAM | ROM/RAM *CS, RAM *WE, low ROM/RAM *OE, RAM CS high A19,A20,A21 high |

## 10Mbit cartridge or more:

| read ROM | ROM/RAM *CS, ROM/RAM *OE, RAM CS low RAM *WE high |
| --- | --- |
| read SRAM | ROM/RAM *OE low RAM CS, RAM *WE, RAM/ROM *CS high A13,A14,A20 high |
| write SRAM | RAM *WE low RAM CS, RAM/ROM *OE, RAM/ROM *CS high A13,A14,A20 high |

Note: The use of A13 and A14 high when accessing the SRAM limits the size of the SRAM to 64kbits.

# EMBEDDED cartridge information

Most of the information in this section was obtained from Mindrape's SNES ROM, available from http://www.goodnet.com/~staheli/

All values are in decimal unless specified with a trailing 'h'.

The starting offset for this information is:

| LoROM | offset 32704 |
| --- | --- |
| HiROM | offset 65472 |

## Game title (21 bytes)

The title is in upper case on most games.

## ROM makeup (1 byte)

Upper nibble (4 bits):

| Value | ROM speed |
| --- | --- |
| 0 | SlowROM |
| 3 | FastROM |

Lower nibble (4 bits):

| Value | Bank size |
| --- | --- |
| 0 | LoROM |
| 1 | HiROM |

## ROM type (1 byte)

| Byte | ROM type |
| --- | --- |
| 0 | ROM only |
| 1 | ROM and RAM |
| 2 | ROM and Save RAM |
| 3 | ROM and DSP1 chip |
| 4 | ROM, RAM and DSP1 chip |
| 5 | ROM, Save RAM and DSP1 chip |
| 19 | ROM and Super FX chip |
| 227 | ROM, RAM and GameBoy data |
| 246 | ROM and DSP2 chip |

## ROM size (1 byte)

| Byte | ROM size |
| --- | --- |
| 8 | 2 MegaBits |
| 9 | 4 MegaBits |
| 10 | 8 MegaBits |
| 11 | 16 MegaBits |
| 12 | 32 MegaBits |

32 Mbits of ROM is the most the SNES can address, while the 8 Mbit cartridges are the most common. There are cartridge sizes of 10Mbit, 12Mbit and 24Mbit, which are reported as 16 Mbit, 16Mbit and 32Mbit respectively.

Another way of calculating the ROM size is:  1 shl (ROMbyte-7) MegaBits

## SRAM size (1 byte)

| Byte | SRAM size |
| --- | --- |
| 0 | none |
| 1 | 16 KiloBits |
| 2 | 32 KiloBits |
| 3 | 64 KiloBits |

64 Kilobit SRAM's are the largest Nintendo uses (except DOOM?), while most copiers have 256kiloBits on board.

Another way of calculating the SRAM size is:  1 shl (SRAMbyte+3) KiloBits

## Country (1 byte)

| Byte | Country | Video system |
| --- | --- | --- |
| 0 | Japan | NTSC |
| 1 | USA | NTSC |
| 2 | Australia, Europe, Oceania and Asia | PAL |
| 3 | Sweden | PAL |
| 4 | Finland | PAL |
| 5 | Denmark | PAL |
| 6 | Drance | PAL |
| 7 | Holland | PAL |
| 8 | Spain | PAL |
| 9 | Germany, Austria and Switzerland | PAL |
| 10 | Italy | PAL |
| 11 | Hong Kong and China | PAL |
| 12 | Indonesia | PAL |
| 13 | Korea | PAL |

## License (1 byte)

| Byte | Company |
| --- | --- |
| 1 | Nintendo |
| 3 | Imagineer-Zoom |
| 5 | Zamuse |
| 6 | Falcom |
| 8 | Capcom |
| 9 | HOT-B |
| 10 | Jaleco |
| 11 | Coconuts |
| 12 | Rage Software |
| 14 | Technos |
| 15 | Mebio Software |
| 18 | Gremlin Graphics |
| 19 | Electronic Arts |
| 21 | COBRA Team |
| 22 | Human/Field |
| 23 | KOEI |
| 24 | Hudson Soft |
| 26 | Yanoman |
| 28 | Tecmo |
| 30 | Open System |
| 31 | Virgin Games |
| 32 | KSS |
| 33 | Sunsoft |
| 34 | POW |
| 35 | Micro World |
| 38 | Enix |
| 39 | Loriciel/Electro Brain |
| 40 | Kemco |
| 41 | Seta Co.,Ltd. |
| 45 | Visit Co.,Ltd. |
| 49 | Carrozzeria |
| 50 | Dynamic |
| 51 | Nintendo |
| 52 | Magifact |
| 53 | Hect |
| 60 | Empire Software |
| 61 | Loriciel |
| 64 | Seika Corp. |
| 65 | UBI Soft |
| 70 | System 3 |
| 71 | Spectrum Holobyte |
| 73 | Irem |
| 75 | Raya Systems/Sculptured Software |
| 76 | Renovation Products |
| 77 | Malibu Games/Black Pearl |
| 79 | U.S. Gold |
| 80 | Absolute Entertainment |
| 81 | Acclaim |
| 82 | Activision |
| 83 | American Sammy |
| 84 | GameTek |
| 85 | Hi Tech Expressions |
| 86 | LJN Toys |
| 90 | Mindscape |
| 93 | Tradewest |
| 95 | American Softworks Corp. |
| 96 | Titus |
| 97 | Virgin Interactive Entertainment |
| 98 | Maxis |
| 103 | Ocean |
| 105 | Electronic Arts |
| 107 | Laser Beam |
| 110 | Elite |
| 111 | Electro Brain |
| 112 | Infogrames |
| 113 | Interplay |
| 114 | LucasArts |
| 115 | Parker Brothers |
| 117 | STORM |
| 120 | THQ Software |
| 121 | Accolade Inc. |
| 122 | Triffix Entertainment |
| 124 | Microprose |
| 127 | Kemco |
| 128 | Misawa |
| 129 | Teichio |
| 130 | Namco Ltd. |
| 131 | Lozc |
| 132 | Koei |
| 134 | Tokuma Shoten Intermedia |
| 136 | DATAM-Polystar |
| 139 | Bullet-Proof Software |
| 140 | Vic Tokai |
| 142 | Character Soft |
| 143 | I''Max |
| 144 | Takara |
| 145 | CHUN Soft |
| 146 | Video System Co., Ltd. |
| 147 | BEC |
| 149 | Varie |
| 151 | Kaneco |
| 153 | Pack in Video |
| 154 | Nichibutsu |
| 155 | TECMO |
| 156 | Imagineer Co. |
| 160 | Telenet |
| 164 | Konami |
| 165 | K.Amusement Leasing Co. |
| 167 | Takara |
| 169 | Technos Jap. |
| 170 | JVC |
| 172 | Toei Animation |
| 173 | Toho |
| 175 | Namco Ltd. |
| 177 | ASCII Co. Activison |
| 178 | BanDai America |
| 180 | Enix |
| 182 | Halken |
| 186 | Culture Brain |
| 187 | Sunsoft |
| 188 | Toshiba EMI |
| 189 | Sony Imagesoft |
| 191 | Sammy |
| 192 | Taito |
| 194 | Kemco |
| 195 | Square |
| 196 | Tokuma Soft |
| 197 | Data East |
| 198 | Tonkin House |
| 200 | KOEI |
| 202 | Konami USA |
| 203 | NTVIC |
| 205 | Meldac |
| 206 | Pony Canyon |
| 207 | Sotsu Agency/Sunrise |
| 208 | Disco/Taito |
| 209 | Sofel |
| 210 | Quest Corp. |
| 211 | Sigma |
| 214 | Naxat |
| 216 | Capcom Co., Ltd. |
| 217 | Banpresto |
| 218 | Tomy |
| 219 | Acclaim |
| 221 | NCS |
| 222 | Human Entertainment |
| 223 | Altron |
| 224 | Jaleco |
| 226 | Yutaka |
| 228 | T&ESoft |
| 229 | EPOCH Co.,Ltd. |
| 231 | Athena |
| 232 | Asmik |
| 233 | Natsume |
| 234 | King Records |
| 235 | Atlus |
| 236 | Sony Music Entertainment |
| 238 | IGS |
| 241 | Motown Software |
| 242 | Left Field Entertainment |
| 243 | Beam Software |
| 244 | Tec Magik |
| 249 | Cybersoft |
| 255 | Hudson Soft |

## Game Version (1 byte)

The version is stored as version 1.VersionByte and must be less than 128. i.e. Less than 1.128.

## Inverse ROM Checksum (2 bytes)

This is the same as XORing the two checksum bytes. i.e. The checksum bits are inversed.

## ROM Checksum (2 bytes)

The checksum is calculated by adding all of the bytes in the ROM, including the 4 checksum bytes (when you add a 32bit number and it's inverse, you get FFFF hex), and taking the lower 32 bits of the result.

## Non Maskable Interrupt / VBL Vector (2 bytes)

| LoROM | at offset 33274 |
| --- | --- |
| HiROM | at offset 66042 |

## Reset Vector (2 bytes)

Where to start the ROM code.

| LoROM | at offset 33276 |
| --- | --- |
| HiROM | at offset 66042 |

## How do I know if the ROM is HiROM or LoROM?

When you OR the checksum bytes and the inverse checksum bytes, the result should be FFFF hex. Therefore, to detect whether a ROM is HiROM or LoROM, you must read those bytes, OR them, and see if they equal FFFF hex.

The ROM's type depends at which location the OR'd bytes equal FFFF hex. If it isn't found at either location, then the other way of checking is to see at which location the title contains uppercase alphanumeric characters. (But this fails with most Japanese cartridges)

As a general rule of thumb, if you can't detect which ROM type it is, default to LoROM.

# cheat device decoding

We'll start with the easiest first then work our way down. These codes work by replacing a byte at a specific location in the ROM.

E.g. In the game F-Zero, at a particular position in the ROM, there is a number 3 indicating 3 lives to start off with. What a cheat code will do is replace this byte with, let's say, the number 9, so now when the game is run, the player starts off with 9 lives.

## Pro Action Replay (hardware)

Code format:		AAAAAADD			(8 digits)

A - Address

D - Data

These codes are in Hex, the address being a CPU address, not a direct ROM location (more about this later).

## Gold Finger (software)

Code format:		AAAAADDDDDDCCW 	(14 digits)

A - Address

D - Data

C - Checksum

W - What to change (DRAM or SRAM)

This code was designed for the copiers, and are straight Hex characters. Therefore the Address is a ROM address, not a CPU address. Data bytes are arranged in 2 characters (2 D's per byte), which allows for 3 bytes. If a byte is not being used, it is denoted by 'XX'.

The address (A's) is a base address. The first data byte (D's) is to be placed at this address. The second at address+1, the third at address+2 (if to be used, that is, if they are not 'XX').

To calculate the checksum you must take the A's and D's, add a zero (0) to the front of the shortened code, then divide into block's of 2 hex digits (bytes). Add these hex digits together (2 characters per hex digit) then minus 160 hex (352 decimal). Now OR this number by FF hex (255 decimal) to get the lower 8 bits (byte). Convert this number to hex and you have your checksum (C's).

W tells the copier whether to replace the byte in the DRAM (ROM image) or the SRAM (Saved game static RAM) of the copier.

| Value of W | Where to place byte |
| --- | --- |
| 0 | DRAM (ROM image) |
| 1 | SRAM (Saved game image) |

The rec.games.video FAQ specifies that there may be non-standard values of 2, 8, A, C, F for W, which may be converted to 0. I personally have only seen Gold Finger codes with W=0.

## Game Genie (hardware)

Code format:		DDAA-AAAA		(8 digits)

A - Address

D - Data

This is the most difficult code to decipher out of the lot. It is as follows:

First take the code in the form xxxx-xxxx and take out the dash ('-') to form xxxxxx. Convert these characters (Genie Hex) to normal hex characters using the following table:

| Genie Hex: | D | F | 4 | 7 | 0 | 9 | 1 | 5 | 6 | B | C | 8 | A | 2 | 3 | E |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal Hex: | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | A | B | C | D | E | F |

The first two characters is the data byte in Hex. Now take the other 6 following characters (encoded address) and put it into it's binary form of 24 bits.

Now take bit 10,11,16,17,18, etc of the encoded address to form the real address (most significant bit first, in other words, the bit on the right) according to the following table:

| Encoded address: | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Real address: | 15 | 14 | 13 | 12 | 7 | 6 | 5 | 4 | 9 | 8 | 23 | 22 | 21 | 20 | 3 | 2 | 1 | 0 | 19 | 18 | 17 | 16 | 11 | 10 |

Bit 23 of the encoded address (bit 15 of the real address) is always 1. The reason being that SNES CPU address must be 1 for it to access the ROM.

## Converting between CPU addresses and ROM addresses

This is very easy once you understand how it is done. To convert from a CPU address to a ROM address, all you need to do is remove bit 15. By doing this, I don't mean just setting it to 0, I mean by removing it, then moving all bits after it down one.

e.g.	ROMaddress = (CPUaddress and 7FFFh) or ((CPUaddress and FF0000h) shl 1)

Therefore, to convert from a ROM address to a CPU address, you must insert a high bit into position 15 (bit 15).

e.g.	CPUaddress = (ROMaddress and 7FFFh) or ((ROMaddress and FF0000h) shr 1) or 8000h

## Easily converting between codes

I have made available two files on my WWW pages which allow you to enter a code which will be converted automatically. The ZIP which contains the two files gg2fingr.exe and fingr2gg.exe are available freely from http://connexus.apana.org.au/~papa/

# snes Copiers

## What are copiers?

A copier is a device which sits on top of the SNES and allows you to backup your cartridges as well as play your backed up games. It does this by storing the ROM image of a cartridge to floppy disks via a 1.44Mb disk drive. Most copiers also include a parallel PC port interface, allowing your PC to control the unit and store images on your hard drive.

Copier's contain DRAM from 1 Megabyte to 4 Megabytes, 8MegaBits to 32MegaBits respectively. This is the reason why they are so expensive.

It is legal to own and use a copier for your own personal backup of cartridges which you legally own in this point in time, although it is illegal to distribute this copy (only one copy is allowed).

If you wish to make your own home brew copier for the SNES, more information can be found at http://connexus.apana.org.au/~papa/CartDisk/

## Super Wild Card (SWC) header information

The SWC (Super Wild Card) image format consists of a 512 byte header. It's layout is as follows (set unused bytes to 00h):

| Offset | Function |
| --- | --- |
| 0 | Lower 8 bits of size word |
| 1 | Upper 8 bits of size word |
| 2 | Image information byte |
| 8 | SWC header identifier (set to AAh) |
| 9 | SWC header identifier (set to BBh) |
| 10 | SWC header identifier (set to 04h) |

The size word is calculated by multiplying the image size, not game size (in MegaBits) by 16. e.g. Image is 4 Mbits, so size word would be 4*16=64.

Image information byte (in the form of 76543210):

| Bit | Description |
| --- | --- |
| 7 | 1 - Run program in Mode 0 (JMP $8000) 0 - Run program in Mode 1 (JMP RESET Vector) |
| 6 | 1 - Multi image (there is another split file to follow) 0 - Not multi image (no more split files to follow) |
| 5 | 1 - SRAM memory mapping Mode 21 (HiROM) 0 - SRAM memory mapping Mode 20 |
| 4 | 1 - DRAM memory mapping Mode 21 (HiROM) 0 - DRAM memory mapping Mode 20 |
| 3/2 | 00: 256kbit SRAM 01: 65kbit SRAM 10: 16kbit SRAM 11: no SRAM |
| 1/0 | reserved |

## Pro Fighter (FIG) header format

This format is similar to the SWC. It consists of a 512byte header who's layout is as follows (set unused bytes to 00h):

| Offset | Function |
| --- | --- |
| 0 | Lower 8 bits of size word |
| 1 | Upper 8 bits of size word |
| 2 | 40h - Multi image 00h - Last image in set (or single image) |
| 3 | 80h - if HiROM 00h - if LoROM |
| 4 | If using DSP1 microchip: FDh - If using SRAM (SRAM size>0) 47h - If no SRAM (SRAM size=0) 77h - If not using DSP1 and no SRAM (SRAM size=0) |
| 5 | If using DSP1 microchip: 82h - If using SRAM (SRAM size>0) 83h - If no SRAM (SRAM size=0) 83h - If not using DSP1 and no SRAM (SRAM size=0) |

## Game Doctor file name format

The Game Doctor does not use a 512 byte header like the SWC, instead it uses specially designed filenames to distinguish between multi files. I'm not sure about the size of the image though.

Usually, the filename is in the format of: SF00000A.078

Where SF means Super Famicon, 00000 refers to a catalogue number in Hong Kong shops identifying the game title. (0 is Super Mario World, 1 is F-Zero, etc). The letter A indicates a multi file, B would indicate the second file, etc. 078 specifies the size of the file. In this case 8 MegaBits.

The information above isn't completely accurate, I know. But this is all the information I have. If you would like to correct me, by all means e-mail me.

# rom protection schemes

This section details ways of bypassing the FastROM, PAL/NTSC and SRAM size checks implemented in many SNES games in order to stop people backing them up using copiers.

Note: You don't necessarily have to find and replace all strings to remove the check(s).

## SlowROM checks

Most cartridges these days use 70ns ROM in order to get the most out of the ageing SNES. However, there are still many copiers around which emulate ROM at speeds of 80ns meaning they cannot backup the newer cartridges correctly.

Changing the ROM code to bypass the SlowROM check, found in many, but not all FastROM games, allows many people with SlowROM copiers to backup FastROM games.

To patch a ROM and bypass the SlowROM check, you must find any of the following strings in the image and replace it with the patch string: (all codes in hex)

| Search for | Replace with |
| --- | --- |
| A9 01 8D 0D 42 | A9 00 8D 0D 42 |
| A9 01 8E 0D 42 | A9 00 8E 0D 42 |
| A2 01 8D 0D 42 | A2 00 8D 0D 42 |
| A2 01 8E 0D 42 | A2 00 8E 0D 42 |
| A9 01 00 8D 0D 42 | A9 00 00 8D 0D 42 |
| A9 01 8F 0D 42 00 | A9 00 8F 0D 42 00 |

## PAL/NTSC checks

Most SNES games have code which detects which video system the cartridge is being played on and refuses to run if not in the right mode. This is to stop people from buying games from other countries before they are released locally.

To bypass the PAL/NTSC check the following patterns must be found and replaced with the ones specified: (all codes in hex)

| Search for | Replace with |
| --- | --- |
| 3F 21 29 10 C9 10 F0 | 3F 21 29 10 C9 10 80 |
| 3F 21 89 10 C9 10 F0 | 3F 21 89 10 C9 10 80 |
| 3F 21 29 10 F0 | 3F 21 29 10 80 |
| 3F 21 00 89 10 F0 | 3F 21 00 89 10 80 |
| 3F 21 00 29 10 F0 | 3F 21 00 29 10 80 |
| 3F 21 89 10 00 F0 | 3F 21 89 10 00 80 |
| 3F 21 29 10 00 F0 | 3F 21 29 10 00 80 |
| AD 3F 21 29 10 00 D0 | AD 3F 21 29 10 00 80 |
| AF 3F 21 00 29 10 D0 | AF 3F 21 00 29 10 80 |
| AF 3F 21 00 29 10 00 D0 | AF 3F 21 00 29 10 00 EA EA |
| AD 3F 21 29 10 D0 | AD 3F 21 29 10 EA EA |
| AD 3F 21 29 10 F0 | AD 3F 21 29 10 80 |
| AD 3F 21 89 10 D0 | AD 3F 21 89 10 80 |
| AD 3F 21 29 10 C9 00 F0 | AD 3F 21 29 10 C9 00 80 |
| AF 3F 21 00 29 10 00 F0 | AF 3F 21 00 29 10 00 80 |
| AF 3F 21 00 89 10 00 F0 | AF 3F 21 00 89 10 00 80 |

## SRAM size checks

Some SNES games check to see how much SRAM is connected to the SNES as a form of copy protection. As most copiers have 256kbits standard, the game will know it's running on a backup unit and stop to prevent people copying the games. However, the newer copiers now get around this detection somehow.

To disable the SRAM size check in a ROM image, search for the following and replace as appropriate.

Note: All codes are in hex, although 'XX' means anything, while a comma means search for either of the two or more (enclosed in brackets).

Search for 	(8F, 9F) xx xx 70 (CF, DF) xx xx 70 D0

Replace with	(8F, 9F) xx xx 70 (CF, DF) xx xx 70 EA EA 	(if SRAM size of game = 64kbit)

(8F, 9F) xx xx 70 (CF, DF) xx xx 70 80 	(if SRAM size of game <> 64kbit)

Search for 	(8F, 9F) xx xx (30, 31, 32, 33) (CF, DF) xx xx (30, 31, 32, 33) D0

Replace with	(8F, 9F) xx xx (30, 31, 32, 33) (CF, DF) xx xx (30, 31, 32, 33) 80

Search for 	(8F, 9F) xx xx (30, 31, 32, 33) (CF, DF) xx xx (30, 31, 32, 33) F0

Replace with	(8F, 9F) xx xx (30, 31, 32, 33) (CF, DF) xx xx (30, 31, 32, 33) EA EA

Search for 	(8F, 9F) xx xx (30, 31, 32, 33) AF xx xx (30, 31, 32, 33) C9 xx xx D0

Replace with	(8F, 9F) xx xx (30, 31, 32, 33) AF xx xx (30, 31, 32, 33) C9 xx xx 80

Many thanks to Chp for making available his uCON 1.41 source, from which these strings came.

# IPS patch format

This patch format is used a lot for patching SNES ROM images. Therefore I have included it's format in this text. For a more detailed explanation of the IPS format, please visit the Damaged Cybernetics WWW page http://www.goodnet.com/~staheli/

The format is as follows:

| Description | Size |
| --- | --- |
| IPS file identifier | 5 bytes (characters PATCH) |
| Offset in file to place patch | 3 bytes |
| Number of bytes in patch | 2 bytes (allows 65535 patch bytes) |
| Patch byte(s) | (specified by 'No. of bytes in patch') |
| . | . |
| . | . |
| Start again, looking for new offset, unless and EOF is found. | 3 bytes (characters EOF) |

Sample IPS file contents with 2 offset points:  PATCHooonn?ooonn?EOF

o - Offset in file

n - Number of bytes in patch

? - Data byte(s) (n number of bytes)

# Acknowledgements

The following people have contributed to this text, whether they know it or not. Many thanks goes to them for their wonderful contribution(s).

Mindrape			E-Mail: mindrape@goodnet.com

Chp				E-Mail: ronaldm@netcom.com

Thomas Rolfes		E-Mail: Thomas_Rolfes@ms.maus.de

Also used for the creation of this text was the rec.games.video FAQ, a FAQ with a huge amount of information on consoles in general.

# How to contact me

If you wish to contact me, please send me e-mail or visit my WWW pages:

E-Mail:		papa@connexus.apana.org.au

WWW:	http://connexus.apana.org.au/~papa/