PIC TIPS							Feb., 1993

Q.	Erasing my PIC17C42's takes much longer than I'm used to, as compared to the PIC 16C5x parts, is 	something wrong?

A.	No. The PIC 17C42 devices use the next generation (1 micron) process technology, which as a side 	effect makes them less susceptible to UV light (which increases data retention time). UV light is 	what erases the memory cell. Presently it takes approximately 1.5 hours to erase the device. Through 	design considerations, this time may decrease. All devices that use this process technology should 	exhibit similar erase times.

Q.	Using the PRO Master programmer. When trying to program PIC 17C42 devices, we were unable to 	successfully read or program the fuses reliably. Is there a problem with the PRO MASTER?

A.	No.  The PIC 17C42 Rev A parts (see January's PIC Tips to determine Rev level), do not allow the 	fuses to be read. An attempt to read the fuses will result with the PRO MASTER displaying a 	random 	value. The program memory area should be readable. The fuses are readable in the Rev. 	B parts. When using PRO Master, the most current software version (V 1.9) should be used.

Q.	I am measuring Power Consumption (Idd) of the PIC 16C5x in my system, an the values seem to be 	higher then those show in the Characterization Data Section of the Data Sheet. What is the problem?

A.	The issue is probably due to the measurement technique differences. To measure the Idd of the PIC 	device, all I/O pins are unconnected. We assign all  I/O pins as output low, the RTCC pin is tied to 	Vdd through a 10 Kohm resistor, and the CPU (if executing) is executing NOP instructions. Floating 	I/O can really increase the Idd value measured. Program execution that changes the state of  I/O 	pins, will also increase the measured Idd value.

Q.	Using the PRO Master programmer. When trying to program a PIC devices, we were unable to 	successfully read or verify a good (programmed) part. Is there a problem with the PRO MASTER?

A.	The socket modules' contact mechanism (to the PRO MASTER) may be at fault. Early socket 	modules may display this problem, due to losing good contact to the PRO MASTER. Socket modules 	that display a sticker with Rev. 2.0 (on the PWB side) should not display this problem.

Q.	Last week my PRO Master programmer wooked fine. This week it does not seem to work (program 	devices). Is there a problem with the PRO MASTER?

A.	No. PRO MASTER needs to be recalerbrated at regular intervals. The frequency of recalerbration is 	dependent on its intended use (Developement vs Production). In a production enviornment it is 	recommended to recalerbrate on a weekly basis. For a developement enviornment, a monthly 	ecalerbration is recommended. Recalibraion is easily accomplished by the 4 button PRO MASTER  	interface. By selecting <Menu> (F4), <Cfg> (F4), and <Cal> (F1) the promaster will recalabrate 	itself in approximately a minute.