PIC TIPS							Jan., 1993

Q.	How do I know if I have the most recent version of a PIC errata sheet?

A.	Contact one of the following:

1.	Your local Microchip sales office (see last page of the Data Sheet)

2.	The Microchip Literature Center	FAX:  (602) 786-7277

3.	The Microchip's PIC Bulletin Board, via your local Compuserve number (Compuserve

membership NOT required). See Microchips Bulletin Board document (literature # DS30128B)

Please specify which PIC device, revision of silicon and Data Sheet (include literature # you

are using. As of now, only the PIC 17C42 and 16C71 have an associated errata sheet.

Q.	How can I determine which revision of silicon I have?

A.	This can be accomplished by decoding the package marking information code on the package.

See the Data Sheets Package Marking Information section. Note: the PIC17C42 Data Sheet 	presently does not contain this information. The PIC 17C42 has both Revision A and B parts, while 	the 16C71 only has Revision A parts.

PIC 17C42 Package Marking Information:

Line 1:	Part Number

Line2:	Optional Customer Specific Information

Line3:	YYWW	CDE

Line4:	Logo

YY	- Year Code

WW	- Week Code

C	- Facility Code

D	- Silicon Revision Number

E	- Internal Assembly Code

Q.	If I want to Drive PIC devices with an external clock, which oscillator type should I use?

A.	You may use all crystal type options (HS, XT, or LP), but you may not use the RC option.

Q.	I have noticed that the PIC device in my system behaves differently depending on weather the device 	is in the encapsulated plastic (OTP) or the windowed ceramic version. Are they exactly the same 	device?

A.	Yes, they are exactly the same silicon. Typically the behavior differences can be attributed to 	"unitialized" registers or SRAM locations. These locations power-up with a "random" value. This 	"random" value which may be different. To verify that uninitialized registers locations are not the 	root of your problem, check the following:

1.	Verify that ALL used registers are initialized.

2.	After erasing the windowed devices, cover the quartz window.