LCD MODULE FAQ                                     Version 35A (March 95)

==============                                     ======================

0.0  LCD MODULE FAQ introduction

General description of LCDs

References

Acknowledgements

Where to find this FAQ

Definitions

History

1.0  Sources for modules

2.0  Specs

2.1  Pin description

2.2  Character set

2.3  Instruction set

2.3.1  Write Operations

2.3.2  Read Operations

2.4  Timing

2.5  Memory map

2.5.1  Custom characters

2.5.2  Addressing display ram

2.6  Initialization

2.6.1  Initialize for 8-bit operation

2.6.2  Initialize for 4-bit operation

3.0  Interfacing

3.1  Manual test circuit

3.2  Interfacing to CPU bus

3.3  Interfacing to CPU port

3.3.1 4-Bit Port Interface

3.3.2 8-Bit Port Interface

3.4  Interfacing to Parallax BASIC STAMP

3.5  Serial Interface

4.0  FTP Sites

5.0  NEC UPD44780 LCD Controller Chip pinout

9.0  Notice

0.0 LCD MODULE FAQ (Frequently Asked Questions list) Introduction

=================================================================

LCDs are manufactured by quite a few different companies.

Units typically seen in the surplus market come from Densitron,

Epson, Hewlett Packard, Optrex, or Sharp.  Common configurations

are 16, 20, 24, 32, or 40 characters by 1, 2, or 4 lines.  This

FAQ sheet applies only to LCD modules with Hitachi 44780 or

equivalent controllers.

I've been loose with the term LCD here.  One can find LCDs

at any level of integration from what looks like a glass slide

and will need drivers and controller, to a PCB that includes the

row and column drivers, to the modules I'm actually talking about

which also include an on-board controller (usually a Hitachi

HD44780).  I'd recommend staying away from modules that do not

say they have a controller or otherwise indicate that it's

included, such as by describing the character set or noting an

ASCII interface.  The units to look for are usually called

character-type dot matrix LCD modules.

If you're interested in the basic principles behind operation

of Liquid Crystal Displays, take a look at Scott M. Bruck's LCD

FAQ, which is available from ftp.ee.ualberta.ca as the file

/pub/cookbook/faq/LCD2.doc  Since his FAQ came first, I've changed

the name of this one to the LCD MODULE FAQ.

Most of the modules I've seen use the Hitachi controller.

These all use the same interface and memory map.  The character

set is almost always the same with a mixture of English and

Japanese characters, but customized models with different

character sets also turn up in the surplus market.  Also found

are modules with extra segments--fixed numbers or words arranged

around the dot matrix display area for extra functions.  These

extra segments look just like additional 5x7 dot boxes to the

controller and can be driven by defining custom characters with

appropriate bit patterns to energize these special segments as

desired.

References used to generate this version:

OPTREX Dot Matrix LCD Module datasheet

June '94 issue of Nuts & Volts magazine

Intel Embedded Control Applications book-1988

Texas Instruments TTL Logic Data Book-1988

Microchip 1994/95 Embedded Control Handbook

_The Miniboard 2.0 Technical Reference_

by Fred G. Martin

_Mobile Robots: Inspiration to Implementation_

by Joseph Jones and Anita Flynn

My gratitude to correspondents and contributors Chris

Abbott, Magnus Andersson, John Edwards, Doug Girling, Frank

Hausmann, John Payson, Robert Rolf, Philipp Schaeufele, J. R.

Spidell, Frank Vorstenbosch, and Brian Wing.  Special thanks and

my apologies to any correspondents or contributors whom I might

have failed to mention.

The current version will be available on the ftp site

ftp.ee.ualberta.ca, as /pub/cookbook/faq/lcd.doc, and will be

occasionally posted to alt.comp.hardware.homebuilt, and

sci.electronics.  It is also available through the World Wide

Web at URL: http://www.cen.uiuc.edu/~cb5222/lcd.doc

Some definitions:

Backlit: uses lamp behind the display instead of reflected light.

Controller: the on-board dedicated microcontroller, such as the

Hitachi HD44780 or SMOS SED1130, which acts as an interface

between your CPU and the row and column drivers.  It may have

on-chip ram, or may need external ram.  The controller takes care

of generating characters, refreshing the display, and so on.  The

modules discussed in this FAQ have integral controllers.

Driver: LCD modules of all sorts have column and row, or x and y,

drivers.  To control one of these modules directly from a CPU

would take a great deal of time and software overhead, because

each bit (dot) has to be written separately, usually 4 dots at a

time.

Electroluminescent: a kind of lamp used for backlighting that

gives off a cool glow, powered by hi-voltage, high-frequency

AC from a dc-ac inverter.

Extended temperature: modules designed for use in temperatures

outside the standard range.  Standard range is 0 to 50 C,

extended range is -20 to 70 C.  Extended range units need -7VDC

as well as +5VDC.  Standard range units need only +5VDC.

LCD: liquid crystal display

LED: light emitting diode, the other popular backlight

Supertwist: a design that improves contrast

History of this FAQ

-------------------

I originally wrote this frustrated by the terse and

cryptic datasheets manufacturers supply.  I refer to it more

often than I write to it.  I hope engineers, hobbyists,

students, and experimenters find it helpful.

Version 0 released mid 1994

Version 35 released March 1995

Version 35A released March 1995

1.0 Sources for Modules

=======================

These are the sources I've seen that are most accessible to the

hobbyist.  If you know of more, please email me.

All Electronics, 818-904-0524

B. G. Micro, 214-271-5546

Cronin Electronics, 617-449-5000

Digikey Corporation, 800-DIGIKEY

Herbach & Rademan, 215-788-5583

Hosfelt Electronics, 800-524-6464

...and hundreds more, no doubt.

My favorite references are the Optrex databook for dot matrix

modules, available from Digikey for $2, and the Amateur Robotics

column in the June '94 issue of Nuts & Volts magazine, 430

Princeland Court, Corona, CA  91719.

2.0 Specifications

==================

2.1 Pin Description

-------------------

These numbers are the same no matter the physical arrangement of

the pins, (for instance, a row along the top on Optrex units, or

a 2x7 .100" center array on the side of Sharp units) on 14-pin

units.  A reader has noted modules with only 10 pins--designed to

be used with 4-bit wide data only.

Pin#  Symbol  Level  Function

1     Vss     GND    Ground

2     Vcc     +5V    Module power

3     Vee    note1   Liquid crystal drive

4     RS_     H/L    Register select, H=data, L=instruction

5     R/W     H/L    Read/Write, H=read (module->CPU), L=write

(CPU->module)

6     E       H/L    Enable

7     DB0    note2   Data bit 0  (least significant bit)

8     DB1      "

9     DB2      "

10     DB3      "

11     DB4      "

12     DB5      "

13     DB6      "

14     DB7      "

Backlight drive:  If the module has a back light, it will be

driven by a pair of pads separate from the interface pads.  Check

your datasheet for power requirements.  Electroluminescent strips

often specify around 200VAC at 400Hz from a DC-AC converter

driven off the 5V power supply.

NOTE1:  On standard modules Vee is between GND and 5V; on

temperature extended modules it is between GND and -7V.  The

potentiometer is the contrast adjustment.

Standard:

+5V ------------*----------- Vcc

|

/

10k to  \<---------- Vee

20k pot /

\

|

GND ------------*----------- Vss

Temperature extended model:

+5V ------------------------ Vcc

GND ------------*----------- Vss

|

/

10k to  \<---------- Vee

20k pot /

\

|

-7V ------------'

Extended temperature types may also employ fancy

temperature correction circuitry to provide automatic contrast

adjustment.

For operation over a narrow temperature range (such as always

indoors), a pair of resistors can be substituted for the

potentiometer.  At first, try a 10Kohm resistor between Vcc and

Vee, and a 330ohm resistor between Vee and Vss, and adjust from

there for optimum contrast.

NOTE2:  The data pins DB7-DB0 (or DB7-DB4 when using a 4-bit

interface) need to be driven by the CPU for a write, but must be

switched to hi-Z (or pulled up with pull-up resistors only) so

the module can drive the lines on a read or BUSY FLAG check.

When using a 4-bit wide interface, the most significant nybble is

written first (bit7-bit4), then the least significant nybble is

written (bit3-bit0) on the next Enable cycle.  DB3 to DB0 are

left unconnected.  The pins have internal pullups, so open-

collector drivers may be used with them.

Power consumption:  Modules use between 10 and 25mW (2 to 5 mA),

not counting the backlight, roughly proportional to number of

rows and columns.

Be careful when hooking power to the module.  Reversing +5V and

GND will destroy the unit.  Carefully examine your datasheet to

correctly identify Pin 1.

2.2 Character Set

-----------------

None of the standard ASCII control codes [chr(1)-chr(31),

chr(127)] are implemented.

Standard ASCII is used for chr(32) <space> through chr(125) '}'

(<00100000b..01111101b>), except that tilde (~) is replaced by a

yen symbol.

The lower case characters do not have decenders.  This is because

some LCD's (those with 5x7 dots or 5x8 dots) would chop off the

bottom.  Lower case characters with decenders appear near the top

of the character table for use on modules which have 5x11 dot

matrices.  You can access them readily by adding 128

(<10000000b>, 80x) if you want decenders and have a 5x11 dot unit

that will properly display them.

Eight user-defined characters are displayed by chr(0) through

chr(7), and redundantly with chr(8) through chr(15).  These are

sometimes used to implement a comma and lower case characters g,

j, p, q, and y with a decender into the cursor row on 5x8 units.

Font Table

---- -----

LSN   x0  x1  x2  x3  x4  x5  x6  x7  x8  x9  xA  xB  xC  xD  xE  xF

MSN +---------------------------------------------------------------

0x  | cg0 cg1 cg2 cg3 cg4 cg5 cg6 cg7 cg0 cg1 cg2 cg3 cg4 cg5 cg6 cg7

1x  | <------------------------- UNDEFINED ------------------------->

2x  |      !   "   #   $   %   &   '   (   )   *   +   ,   -   .   /

3x  |  0   1   2   3   4   5   6   7   8   9   :   ;   <   =   >   ?

4x  |  @   A   B   C   D   E   F   G   H   I   J   K   L   M   N   O

5x  |  P   Q   R   S   T   U   V   W   X   Y   Z   [  (*)  ]   ^   _

6x  |  `   a   b   c   d   e   f   g   h   i   j   k   l   m   n   o

7x  |  p   q   r   s   t   u   v   w   x   y   z   {   |   }  --> <--

8x  | <------------------------- UNDEFINED ------------------------->

9x  | <------------------------- UNDEFINED ------------------------->

Ax  |     (*) (*) (*) (*) (*) (*) (*) (*) (*) (*) (*) (*) (*) (*) (*)

Bx  |  -  (*) (*) (*) (*) (*) (*) (*) (*) (*) (*) (*) (*) (*) (*) (*)

Cx  | (*) (*) (*) (*) (*) (*) (*) (*) (*) (*) (*) (*) (*) (*) (*) (*)

Dx  | (*) (*) (*) (*) (*) (*) (*) (*) (*) (*) (*) (*) (*) (*) (*) (*)

Ex  | (*) (*) (*) (*) (*) (*) (*) (g) (*) (*) (j) (*) (*) (*) (*) (*)

Fx  | (p) (q) (*) (*) (*) (*) (*) (*) (*) (y) (*) (*) (*) (*) (*) (*)

Notes:

cg0-cg7  user-definable characters

(chr)    lower-case character with descenders

(*)      see list below:

Hex     Decimal Meaning

---     ---     -------

5C       92     Yen

A0      160     (blank)

A1      161     katakana

:        :         :

A5      165     katakana (looks like dot in center)

:        :         :

DF      223     katakana (looks like degree symbol)

E0      224     LC alpha

E1      225     "a" with umlaut (" a) (German)

E2      226     LC beta (or German es-szet) with

descender

E3      227     LC epsilon

E4      228     LC mu with descender

E5      229     LC sigma

E6      230     LC rho

E7      231     "g" with descender

E8      232     radical (square root sign)

E9      233     "- |" (Katakana?)

EA      234     "j" with descender

EB      235     tiny 3 by 3 'x' in upper left corner

EC      236     cent sign

ED      237     pounds sterling

EE      238     "n" with tilde (Spanish) or overbar

EF      239     "o" with umlaut (" o) (German)

F0      240     "p" with descender

F1      241     "q" with descender

F2      242     LC theta

F3      243     infinity (lazy-8)

F4      244     LC omega

F5      245     "u" with umlaut (" u) (German)

F6      246     UC sigma

F7      247     LC pi

F8      248     "x" with overbar

F9      249     "y" with descender

FA      250     katakana

FB      251     katakana

FC      252     katakana

FD      253     division symbol (: -)

FE      254     (blank)

FF      255     solid black cursor

(Font table by Doug Girling)

2.3 Instruction Set

-------------------

Binary data from bit7 to bit 0.  If using a 4-bit interface,

bit7-bit4 of data are sent first, then bit3-bit0, on sucessive

enable cycles.  No delay is required between these cycles (except

that the minimum E time (450ns) and TcycE time (1us) must be

met).

2.3.1 Write Operations

----------------------

_

RS=0 (except as noted), R/W=0, command/data from CPU to LCD on

bit7 to bit0.

* Clear display                   00000001

Clears display and returns cursor to home position

(address 0).  Execution time: 1.64ms

* Home cursor                     0000001x

Returns cursor to home position, returns a shifted display

to original position.  Display data RAM (DD RAM) is unaffected.

Execution time: 40us to 1.64ms

x=don't care

* Entry mode set                  000001is

Sets cursor move direction and specifies whether or not to

shift display.  Execution time: 40us

i=1: increment, i=0: decrement DD RAM address by 1 after

each DD RAM write or read.

s=1: display scrolls in the direction specified by the

"i" bit when the cursor is at the edge of the display

window

* On/off control                  00001dcb

Turn display on or off, turn cursor on or off, blink

character at cursor on or off.  Execution time: 40us

d=1: display on.

c=1: cursor on

b=1: blink character at cursor position

* Cursor/shift                    0001srxx

Move cursor or scroll display without changing display data

RAM.  Execution time: 40us

s=1: scroll display, s=0: move cursor.

r=1: to the right, r=0: to the left.

x= don't care

Function set                    001dnfxx

Set interface data length, mode, font.  Execution time: 40us

d=1: 8-bit interface, d=0: 4-bit interface.

n=1: 1/16 duty, n=0: 1/8 or 1/11 duty (multiplex ratio).

For 2-line displays, this can be thought of as

controlling the number of lines displayed

(n=0: 1-line, n=1: 2-line) except for 1x16

displays which are addressed as if they were

2x8 displays--two 8-character lines side by side.

f=1: 5x11 dots, f=0: 5x8 dots.

Character RAM Address Set       01aaaaaa

To read or write custom characters.  Character generator

(CG) RAM occupies a separate address space from the DD RAM.  Data

written to, or read from the LCD after this command will be

to/from the CG RAM.  Execution time: 40us

aaaaaa: 6-bit CG RAM address to point to.

Display RAM Address Set         1aaaaaaa

Reposition cursor.  Display Data (DD) RAM occupies a

separate address space from the CG RAM.  Data written to, or read

from the LCD  after this command will be to/from the DD RAM.

Execution time: 40us

aaaaaaa: 7-bit DD RAM address to point to.

Write Data to CG or DD RAM      dddddddd (RS=1)

Data is written to current cursor position and (DD/CG) RAM

address (which RAM space depends on the most recent CG_RAM_

Address_Set or DD_RAM_Address_Set command).  The (DD/CG) RAM

address is incremented/decremented by 1 as determined by the

"entry mode set" command.  Execution time: 40us for display write,

120us for character generator ram write

dddddddd: 8-bit character code

2.3.2 Read Operations

---------------------

_

RS as noted, R/W=1, bit7 to bit0 output from LCD to CPU.

Read Busy Flag                  baaaaaaa (RS=0)

Read the status of the busy flag, and the value of the RAM

address currently being pointed at.  Execution time: 1 cycle

b=1: busy, b=0: OK to send

aaaaaaa: 7-bit current (DD/CG) RAM address counter (as in

"character RAM address set" or "display RAM address

set").

Read Data from CG or DD RAM     zzzzzzzz (RS=1)

Data is read from current (DD/CG) RAM address position, and

the RAM address is automatically incremented/decremented by 1 as

determined by the "entry mode set" command.  NOTE that the

display/cursor is not shifted on data reads.  Execution time:

40us for display reads, 120us for character generator ram reads

zzzzzzzz: DB lines must be inputs (or pulled high with

pullup resistors) to be driven by the module.

An 8-bit character code will be read back from

LCD RAM.

2.4 Timing

----------

Execution times:  Clear display 1.64ms; home cursor 40 us to

1.64ms depending on how far display is scrolled; all others 40us,

except read busy flag which is complete in a single enable cycle

(or two cycles in 4-bit mode), and character generator ram reads and

writes which should be separated by 120us delays.  These execution

times mean that after an operation, the CPU must do Busy Flag checks

until the BF (bit 7) is 0, or, when the connection to the module

from the CPU is write-only, wait more than the execution time before

the next operation.  These are maximum execution times, some units

(in particular those with 5x11 dot matrices) will execute some

instructions faster.

WRITE:

______ _____________________________ ___________

RS  ______X_________valid_RS_level______X__________

|                             |

|                             |

|<--tAS-->|        tAH-->|    |<--

______|         |              |    |____________

R/W ______\_________|___R/W_low____|____/_____________

|              |

|<----PWEH---->|

|              |

|<-------------|-------TcycE----->|

|______________|                  |_________

E   ________________/              \__________________/

tR-->||<--       -->||<--tF

||

|<--tDSW-->||

|        -->|   |<--tWH

__________________|_______________|________________

D0-D7 __________________X__valid_data___X____________

READ:

______ _____________________________ ___________

RS  ______X_________valid_RS_level______X__________

|                             |

|                             |

|<--tAS-->|        tAH-->|    |<--

______|_________|__   _    ____|____|____________

R/W ______/         |  R/W high    |    \_____________

|              |

|<----PWEH---->|

|              |

|<-------------|-------TcycE----->|

|______________|                  |_________

E   ________________/              \__________________/

tR-->||<--       -->||<--tF

|              ||

tDDr-->|   |<--       ||

|        -->|   |<--tRH

___________________|_______________|________________

data ___________________X__valid_data___X____________

TcycE   Enable cycle time               1000ns min

Operation cycle time cannot be less than 1 microsecond

PWEH    Enable pulse width, high         450ns min

Enable pulse must be at least 450 nanoseconds long, no maximum

length

tRE     Enable rise time                  25ns max

Enable line must change state (L->H) in less than 25ns

tFE     Enable fall time                  25ns max

Enable line must change state (H->L) in less than 25ns

tAS     Address setup time               140ns min

Register Select and R/W lines must be valid 140ns before

enable pulse arrives

tAH     Address hold time                 10ns min

RS and R/W must be valid at least 10 ns after enable goes low

tDDR    Data delay time                  320ns max

When doing a read, the return data will be valid within 320ns

of enable going high

tRH     Data hold time, read              20ns min

When doing a read, the return data will be valid at least 20ns

after enable goes low

tDSW    Data setup time                  195ns min

When doing a write, data on lines bit7-bit0 (or bit7-bit4 in

4-bit mode) must be valid at least 195 ns before enable goes

low

tWH     Data hold time, write             10ns min

When doing a write, data on lines must be valid for at least

10ns after enable goes low

Generally, there are no max time requirements on the user except

Enable rise and fall times.  An LCD module can be driven with

just toggle switches for data, RS, and R/W, and a debounced

pushbutton on the enable line.

2.5 Memory map

==============

Character generator RAM appears to reside at locations 64-127 and

display RAM seems to be at locations 128-255.  This is an

artifact of the control scheme.  They are actually separate non-

contiguous blocks of RAM.

2.5.1 Custom Characters

-----------------------

The display has 64 bytes of CG RAM, which supports 8

user-definable characters, each defined as a 5X8 character in an

8X8 block.  The 5X8 region is right-justified.  Note that in 5X10

matrix mode, only 4 user-defined characters are supported, using

11 bytes each.

CG Address      CG Data

D7-----------D0

x+0             x x x 1 1 1 1 1         *****

x+1             x x x 1 0 0 0 0         *

x+2             x x x 1 0 0 0 1         *   *

x+3             x x x 1 1 1 1 1   =     *****

x+4             x x x 1 0 0 0 1         *   *

x+5             x x x 1 0 0 0 0         *

x+6             x x x 1 0 0 0 0         *

x+7             x x x 0 0 0 0 0         (cursor line)

Where "x" is the base address for the character: CG0=0, CG1=8,

CG2=16, CG3=24, CG4=32, CG5=40, CG6=48, CG7=56.

As you can see in the above figure, each byte of CG data

represents 1 row of pixels in the character, with D0 being the

rightmost pixel, and D4 being the leftmost (the upper 3 bits are

not displayed).  The first address for a character is the topmost

row, while the 7th is the bottom.  (The 8th is ORed with the

cursor underbar).  A '1' in any pixel position becomes a dark

pixel on the display.

CG RAM occupies a separate address space from the data display

(DD) RAM.  One must set the (first) CG address before writing any

CG data.  Each write automatically increments/decrements the CG

address pointer to the next location.  Remember to set the

display back to DD addressing mode before trying to write

characters to be displayed.

As an example, we'll define character #3 to be the above symbol

by first reading the DD address, writing the CG address to the

start of the character's RAM, writing the successive rows of

pixel data, then restoring the addressing mode back to DD

addressing (which requires the DD address we saved at the

beginning.

RS R/W D7----D0

0  1  (baaaaaaa)        Read current DD address

0  0   01011000         Set CG RAM address to char #3

(3x8=24=11000b)

1  0   00011111         Write top row of pixels

1  0   00010000                 :

1  0   00010001                 :

1  0   00011111                 :

1  0   00010001                 :

1  0   00010000                 :

1  0   00010000         Write bottom row of pixels

1  0   00000000         Write cursor row (descender)

0  0   1aaaaaaa         Restore DD mode (& address)

NB. There should be a 120 uSec delay between successive reads or

writes.

(Section 2.5.1 by Doug Girling)

2.5.2 Addressing Display RAM

----------------------------

In the list below, "line 1" is the topmost line, and "line n" is

the bottommost line.  On each line, the leftmost character has

the lowest address, and addresses increase to the right.  Also,

DD RAM addresses are shown without the 80h mode bit set.

16x1 module is arranged as two 8-character lines side by side.

"Line 1" (left)  addresses are 00h to 07h

"Line 2" (right) addresses are 40h to 47h

As you write characters to the module, the cursor will

automatically increment until you get to the 9th

character--you have to move the cursor to address 40h

before writing the 9th character on the 1x16 module.

16x2 module is two lines by 16 chars

Line 1 addresses are 00h to 0Fh

Line 2 addresses are 40h to 4Fh

20x1 module

Line 1 addresses are 00h to 13h

20x2 module

Line 1 addresses are 00h to 13h

Line 2 addresses are 40h to 53h

20x4 module

Line 1 addresses are 00h to 13h

Line 2 addresses are 40h to 53h

Line 3 addresses are 14h to 27h

Line 4 addresses are 54h to 67h

40x2 module

Line 1 addresses are 00h to 27h

Line 2 addresses are 40h to 67h

The full 128 bytes of display RAM exist no matter how many

characters appear on the display.  These extra bytes can be typed

on when display window scrolling is enabled, or they can be used

to store other information--external data RAM for the CPU, if you

like.

2.6 Initialization

==================

Modules with Hitachi controllers will properly self-initialize if

Vcc rises from 0 to 4.5v in a period between .1mS and 10mS.  I

suppose an RC circuit would be needed to keep powerup rise time

as slow as .1ms, so the manual initialization will be required in

most applications.  If you do use auto-initialization, it will

come up in this mode: 8-bit interface, 1/8 duty cycle (1 line

mode), 5x7 font, cursor increment right, no shift.  On most

displays, you want to switch to 1/16 duty cycle (2 line mode)

because for all but the 20x1, there are two logical lines as the

controller sees it. If you have an 5x11-size character dot matrix

module, you'll want to switch to the 5x10 font as well (the 11th

line is the cursor).

2.6.1 Initialization for 8-bit Operation

----------------------------------------

<POWER ON>

<Wait 15ms>

RS R/W DB7 DB6 DB5 DB4 DB3 DB2 DB1 DB0

0   0   0   0   1   1   x   x   x   x    x=don't care

<Wait 4.1ms>

RS R/W DB7 DB6 DB5 DB4 DB3 DB2 DB1 DB0

0   0   0   0   1   1   x   x   x   x

<Wait 100us>

RS R/W DB7 DB6 DB5 DB4 DB3 DB2 DB1 DB0

0   0   0   0   1   1   x   x   x   x

<Wait 4.1ms>

RS R/W DB7 DB6 DB5 DB4 DB3 DB2 DB1 DB0

0   0   0   0   1   1   1   F   x   x    8-bit operation

1/16 duty cycle

F=font,

1 for 5x11 dot matrix

0 for 5x8 dot matrix

<Wait 40us>

RS R/W DB7 DB6 DB5 DB4 DB3 DB2 DB1 DB0

0   0   0   0   0   0   1   0   0   0    Display off,

cursor off,

blink off

<Wait 40us>

RS R/W DB7 DB6 DB5 DB4 DB3 DB2 DB1 DB0

0   0   0   0   0   0   0   0   0   1    Clear screen,

cursor home

<Wait 1.64m>

RS R/W DB7 DB6 DB5 DB4 DB3 DB2 DB1 DB0

0   0   0   0   0   0   0   1   1   0    Increment cursor to

the right when writing,

don't shift screen

<Wait 40us>

<INITIALIZATION COMPLETE>

NOTE:  Remember to turn the display back on and set up the cursor

as desired with an On/Off Control command.

2.6.2 Initialization for 4-bit operation:

-----------------------------------------

The module powers up in 8-bit mode.  The initial startup

instructions are sent in 8-bit mode, with the lower four bits

(which are not connected) of each instruction as don't cares.

After the fourth instruction, which switches the module to 4-bit

operation, the control bytes are sent on consecutive enable

cycles (no delay is required between nybbles).  Most significant

is sent first, followed immediately by least significant nybble.

<POWER ON>

<Wait 15ms>

RS R/W DB7 DB6 DB5 DB4 DB3 DB2 DB1 DB0

0   0   0   0   1   1  n/c n/c n/c n/c

<Wait 4.1ms>

RS R/W DB7 DB6 DB5 DB4 DB3 DB2 DB1 DB0

0   0   0   0   1   1  n/c n/c n/c n/c

<Wait 100us>

RS R/W DB7 DB6 DB5 DB4 DB3 DB2 DB1 DB0

0   0   0   0   1   1  n/c n/c n/c n/c

<Wait 4.1ms>

RS R/W DB7 DB6 DB5 DB4 DB3 DB2 DB1 DB0

0   0   0   0   1   0  n/c n/c n/c n/c   set 4-bit operation

<Wait 40us>

RS R/W DB7 DB6 DB5 DB4

0   0   0   0   1   0

0   0   1   F   x   x   4-bit operation

1/16 duty cycle

F=font, 1 for 5x11 dot matrix

0 for 5x8 dot matrix

x=don't care

<Wait 40us>

RS R/W DB7 DB6 DB5 DB4

0   0   0   0   0   0

0   0   1   0   0   0    Display off, cursor off, blink off

<Wait 40us>

RS R/W DB7 DB6 DB5 DB4

0   0   0   0   0   0

0   0   0   0   0   1    Clear screen, cursor home

<Wait 1.64ms>

RS R/W DB7 DB6 DB5 DB4

0   0   0   0   0   0

0   0   0   1   1   0    Increment cursor to the right

when writing, don't shift screen

<Wait 40us>

<INITIALIZATION COMPLETE>

NOTE:  Remember to turn the display back on and set up the cursor

as desired with an On/Off Control command.

3.0 Interfacing

===============

3.1 Manual test circuit

-----------------------

__________

|          |

GND---*---------| 1        |

<         |          |

contrast    10K   ><--.    |          |

<    |    |          |

+5V---*---------| 2        |

|    |          |

`---| 3        |

RS                           |          |

,-----switch------------------------| 4        |

|                                   |          |

|      +5V-----.           GND------| 5        |

|              |                    |          |

|              # 3.3k pullup resis. |          |

|   Enable     |  |\                |          |

*--pushbutton--*--|  >o-------------| 6        |

|              |  |/ 74LS04 inverter|          |

|             ---                   |          |

|             --- 1uF cap           |          |

|              |                    |          |

|    DB0       GND                  |          |

*---switch--------------------------| 7        |

|                                   |          |

|    DB1                            |          |

*---switch--------------------------| 8        |

|                                   |          |

~~~                                 ~~~        ~~~

~~~                                 ~~~        ~~~

|    DB7                            |          |

*---switch--------------------------| 14       |

|                                   |__________|

|

GND

NOTE:  Extended temperature range modules need the non-grounded

end of the potentiometer tied to -7VDC instead of +5VDC.

3.2 Interfacing to a CPU Bus

----------------------------

These modules use an interface like those in Motorola (68xx) and

MOS Technology (65xx) systems.  R/W on the module can come from

the R/W line on the CPU, which is set up about the same time as

the address, while RS can be selected by the low order address

line A0.  Select Enable by ANDing the output from your address

decoding and the E clock.  Address decoding can be done with a

magnitude comparator like the 74LS688, or if you have address

space to spare, with partial address decode like running A13-A14

into a 74LS138 3-to-8 demux, or just the inverse of the high order

address line (hogging a big chunk of address space).

68xx

65xx                                   LCD

-----.                                .-----

D0  |--------------------------------| D0

:   |               :                |

D7  |--------------------------------| D7

_ |                                |   _

R/W |--------------------------------| R/W

|                        .---.   |

E   |------------------------|   |   |

|      .----------.      | & |---| E

A0  |------|          |      |   |   |

:   |  :   | (Decode) |------|   |   `-----

A15 |------|          | CS(h)`---'

-----'      `----------'

Motorola bus interface

Because Intel (e.g., 8080, 8085, 8048) and Zilog (e.g., Z80) CPUs

use separate read and write lines, and they are not set up ahead

of time, R/W as well as RS must be set via address lines.  Then

the Enable signal may be generated from (NOT(/RD AND /WR)) AND

PSEN):

80xx

Z80

_ |         .------.                                            _

RD |------*--| S  Q |----------------------------------------- R/W

_ |      |  |      |

WR |---*--+--| R    |

|   |  |  `------'

|  |  .---.                                   .---.

|  `--| & |O--*-------------------------------|   |--- E

`-----|   |   |  .---.     R         .---.    | & |

`---'   `--|   |O--v^v^v^v--*--| & |O---|   |

`---'            |  `---'    `---'

C ===

__|_

/ / /

NB. R & C must be chosen to delay the rising edge of the

enable pulse at least 140ns.

Using the data bus method limits CPU clock speed because of the

tDDR read delay and the TcycE and PWEH requirements of the

Hitachi controller.

(Section 3.2 by Doug Girling)

3.3 Interfacing to a CPU Port

-----------------------------

In the case where a direct bus interface is not desired, or is

impractical (e.g., many microcontrollers don't provide an

external data/address bus, or do so at the expense of reassigning

too many pins), the LCD module can be connected via an I/O port.

Whether you use a 4-bit or an 8-bit interface, those port pins

driving the LCD's data lines must be bidirectional if you wish to

read from the LCD.  Remember too that you'll have to write your

code to switch the port's data direction bits whenever you switch

from reading to writing or visa versa.  If you don't expect to

ever be reading back from the LCD, you can conserve resources by

grounding R/W, saving a pin, and thus using digital outputs

instead of bidirectional ports.

You may be able to eliminate the need for a software-generated

enable (E) signal if the particular port you are using

automatically generates a handshake strobe.  In practical terms,

most ports only generate a stobe on output, and expect a strobe

on input, so this approach is most likely applicable to

write-only configurations.

3.3.1 4-Bit Port Interface

--------------------------

Another interface is to drive the module in 4-bit mode using 7

I/O bits from a port.

|     .--------

|     |

P.7 |<--->| DB7

P.6 |<--->| DB6

P.5 |<--->| DB5

P.4 |<--->| DB4

P.3 |---->| R/W

P.2 |---->| RS

P.1 |---->| E

P.0 |--nc |

|     `--------

First, put the most significant nybble on P7-P4, and the

appropriate R/W and RS signals on P3 and P2, and P1 low.  Then

toggle P1 high.  Then toggle P1 low again.  Then put the least

significant nybble on P7-P4, toggle P1 high, toggle P1 low.

Forty microseconds later, you can send the next character.

Sample in-line assembly code, by Jordan Nicol, for implementation

under Dunfield's Micro-C for the Miniboard can be found on ftp

cher.media.mit.edu.  It uses port pins PA7-PA3 for 4-bit data and

PC6 and 7 for RS and E.

3.3.2 8-Bit Port Interface

--------------------------

If you have port pins to spare, then an 8-bit interface can be

done with 11 port pins (10 for write only).

|     .--------

|     |

PA0 |<--->| DB0

PA1 |<--->| DB1

PA2 |<--->| DB2

PA3 |<--->| DB3

PA4 |<--->| DB4

PA5 |<--->| DB5

PA6 |<--->| DB6

PA7 |<--->| DB7

|     |

PB0 |---->| R/W

PB1 |---->| RS

PB2 |---->| E

PB3 |--nc  `--------

PB4 |--nc

PB5 |--nc

PB6 |--nc

PB7 |--nc

|

3.4.1 Interfacing to the Parallax BASIC STAMP

---------------------------------------------

Sample program with physical hookup described in commented code

can be found on the ftp site wpi.wpi.edu in the /stamp directory.

It's a CPU port hookup as described above.  The application also

involves use of a radio control servo.

3.4.2 Interfacing to the Intel 8051 family of microcontrollers

--------------------------------------------------------------

Sample program and schematics appear in the application note

AB-39 in the Intel Embedded Control Applications book for 1988

and probably other years.

3.4.3 Interfacing to the Motorola 68HC11 microcontrollers

---------------------------------------------------------

Sample programs and circuit descriptions can be found at the

MIT cher.media.mit.edu ftp site.  There is the Miniboard hookup

mentioned above, and the use of an LCD module on the 6.270 robot

control boards.

3.4.4 Interfacing to the Microchip PIC microcontrollers

-------------------------------------------------------

Sample program and schematics appear in the application note

AN587 in the Microchip Embedded Control Handbook for 1994/95.

3.5 Serial Interface

--------------------

A way to save even more I/O space is to use a serial interface,

requiring just 3 digital output port pins.  It uses a shift

register with serial-in, parallel-out, and output latch.  This

setup allows the convenient coding of a parallel interface (just

writing the data to the USART transmit buffer) and low pin count

of a serial interface.

________________         __________

_____         |  74LS595       |       |          |

|        |              QA|-------|DB4       |

|--------|>ser. clock   QB|-------|DB5       |

CPU  |        |              QC|-------|DB6       |

OUT  |--------|>latch        QD|-------|DB7       |

PORT |        |              QE|-------|RS        |

|--------|serial data   QF|-------|E         |

_____|        |              QG|--nc   |          |

10Kohm |              QH|--nc   |          |

pullup |                |       |          |

+5V--^^^---|\Reset          |   .---|R/W       |

.-----|\OE             |   |   |__________|

|     |________________|   |

GND                        GND

The Amateur Robitics column in June '94 Nuts & Volts demonstrated

how to use this technique with the 68hc11's SPI port, using MOSI,

SCK, and /SS.  This would be especially handy with a non-

networked 68HC11-based Miniboard single-board computer, which has

MOSI, MISO, SCK, and /SS conveniently routed to the top left

corner where resistor pack 2 goes.  The experimenter could put

the contrast potentiometer and latch on a daughterboard mounted

underneath the LCD module.

An alternative with a less expensive shift register has the low

pincount advantage, but requires bit manipulation of port pins:

_____                                   __________

|                                 |          |

|---------------------------------|E         |

|         ________________        |          |

|        |  74LS164       |       |          |

|        |              QA|-------|DB4       |

|--------|>ser. clock   QB|-------|DB5       |

CPU  |        |              QC|-------|DB6       |

OUT  |        |              QD|-------|DB7       |

PORT |        |              QE|-------|RS        |

|--------|serial data   QF|--nc   |          |

_____|        |              QG|--nc   |          |

10Kohm |              QH|--nc   |          |

pullup |                |       |          |

+5V--^^^---|\Clear          |   .---|R/W       |

|                |   |   |__________|

|________________|   |

GND

Save another I/O line with this idea from Robert Rolf:

"On your SPI example, you can free up the latch line by using the

clock line with a diode, pullup, and capacitor.  I use the SPI in

normally high mode, rising clock for data bits.  10K pullup, 10nF

to GND, and the clock through diode pulls it low.  After all the

bits are shifted in, the RC times-out, and clocks [latches] the

'595."

4.0 LCD Controller chip pinout

==============================

NEC UPD44780 LCD Display Controller Pinouts:

PIN DEFINITION

1  SEG 22

2  SEG 21

3  SEG 20

4  SEG 19                    666665555555555444444444

5  SEG 18                    432109876543210987654321

6  SEG 17                  65                        40

7  SEG 16                  66                        39

8  SEG 15                  67                        38

9  SEG 14                  68                        37

10  SEG 13                  69                        36

11  SEG 12                  70                        35

12  SEG 11                  71                        34

13  SEG 10                  72                        33

14  SEG  9                  73      UPD44780 TOP      32

15  SEG  8                  74                        31

16  SEG  7                  75                        30

17  SEG  6                  76                        29

18  SEG  5                  77                        28

19  SEG  4                  78   NOTCHED CORNER       27

20  SEG  3                  79  /AND POSSIBLE DOT     26

21  SEG  2                  80 o                      25

22  SEG  1                   \         111111111122222

23  GND                       123456789012345678901234

24  OSC  1

25  OSC  2

26  V1

27  V2

28  V3

29  V4

30  V5

31  CL   1

32  CL   2

33  VCC

34  M         -

35  D         -

36  RS        - Reset, assert once

37  R/W*      - Hold low for write operation

38  E         - Clock low to latch data.

39  DB  0     - The data bus

40  DB  1     -

41  DB  2     -

42  DB  3     -

43  DB  4     -

44  DB  5     -

45  DB  6     -

46  DB  7     -

47  COM  1

48  COM  2

49  COM  3

50  COM  4

51  COM  5

52  COM  6

53  COM  7

54  COM  8

55  COM  9

56  COM 10

57  COM 11

58  COM 12

59  COM 13

60  COM 14

61  COM 15

62  COM 16

63  SEG 40

64  SEG 39

65  SEG 38

66  SEG 37

67  SEG 36

68  SEG 35

69  SEG 34

70  SEG 33

71  SEG 32

72  SEG 31

73  SEG 30

74  SEG 29

75  SEG 28

76  SEG 27

77  SEG 26

78  SEG 25

79  SEG 24

80  SEG 23

(Section 4.0 by Frank Hausman)

5.0 FTP Sites

=============

* This FAQ.

ftp.ee.ualberta.ca      /pub/cookbook/faq/lcd.doc

* LCDFAQ: Liquid Crystal Display physics & principles of operation

by Scott M. Bruck, August 1993.

ftp.ee.ualberta.ca	/pub/cookbook/faq/LCD2.doc

* 4-bit interface sample in-line assembly code, by Jordan Nicol,

for implementation under Dunfield's Micro-C for the Miniboard.

It uses port pins PA7-PA3 for 4-bit data and PC6 and PC7 for RS

and E.

ftp cher.media.mit.edu

* Parallax Basic Stamp applications.

wpi.wpi.edu             /stamp

9.0 Notice

==========

Trademarks and service marks appearing in this FAQ are the

property of their respective owners.  Use this information at

your own risk.  I can take no responsibility for damage caused by

errors or omissions in this work.  Copyright 1994, 1995 by

Christopher Burian and other contributors.  All rights reserved.

Permission is granted to distribute and reproduce this article

provided that it is intact and that it is free of charge (reasonable

copying fees allowed).

Errata, comments, and suggestions are most welcome.

=================================================================

Christopher Burian                                       03/20/95

cburian@uiuc.edu