A cheap MIDI connector box

for the Sound Blaster card

v1.2  March 22nd, 1991

by

Adam Mirowski

mir@chorus.fr

==============================

$Id: sb-midi,v 1.12 91/11/12 20:20:39 mir Exp $

Sound Blaster  has a  "built-in MIDI interface", but "all you need

is a MIDI Connector Box (optionally available) in order to connect

your SB  to MIDI  instruments or  keyboards", says  the add on the

box. The  list price  for that  gadget is  as high  as $79.95  and

anyway it  is  not  available  here  in  France.  Also,  the  MIDI

interface is  not compatible  with the  MPU-401 standard  (and  de

facto norm) and is only one-way-at-a-time. So the box is not worth

the money,  except if you try to set up it by yourself. For $5 you

can get  the basic  functionalities and  for $10  everything which

comes with the original one.

I have heard on Usenet recently  that Sound Blaster will soon come

out with a  standard MIDI interface. Chances are that this circuit

will also work with that new version.

!	Corrections (AM): SB Pro doesn't seem to be MPU compatible;

!	it is already provided with a MIDI Adaptor

!	that you can also purchase separately for $25 (probably for

!	an old SB.) I suppose that the cabling of this MIDI adaptor

!	corresponds to paragraph II of the "SUPPLEMENT" (see below.)

!	I was also told that SB v2.0 has a bidirectional MIDI port.

!	This could be also the case for SB Pro.

Parts

=====

Qty     Name               Type           Tandy ref

---------------------------------------------------

1   Optocoupler         4N25             276-9294

3   Resistors           220 ohm          271-015

1   Resistor            1 Kohm	    271-023

1   AND gate            74LS08           276-9276

2   DIN plugs           Female 5 pins    274-9110

1   Canon DB plug       Male 15 pins     Not sold

[1]  Diode               1N914 or 1N4148  276-1122/276-9014

1   Veroboard (*)       5cm by 5cm

8   Wires               Not shielded

1   Box to put all that stuff inside

[] - didn't use it myself.

(*) It is a piece of

epoxide with  a raster  of holes spaced at 2.54mm. On one side

there are  preprinted copper  tracks along  each line of holes

(in one  direction only).  You cut  off unused portions of the

tracks and  join the  other  with  straps,  so  as  to  get  a

primitive form of a printed circuit board. You then insert the

electronic parts and solder them.

Sound Blaster MIDI/Game connector REAL cabling

==============================================

+------------------------------------------+

!  Pin       Signal                        !

!------------------------------------------!

!   1     +5V                              !

!   2     X button for joystick A          !

!   3     X potentiometer for joystick A   !

!   4     Ground                           !

!   5     Ground                           !

!   6     Y potentiometer for joystick A   !

!   7     Y button for joystick A          !

!   8     +5V                              !

!   9     +5V                              !

!  10     X button for joystick B          !

!  11     X potentiometer for joystick B   !

!  12     MIDI out                         !

!  13     Y potentiometer for joystick B   !

!  14     Y button for joystick B          !

!  15     MIDI in                          !

+------------------------------------------+

The interesting pins are 12, 15, 4 (or 5) and 9 (or 8 or 1).

BTW, if your joystick Y cable assumes that pin 12 is ground (as it

is on a normal game port), it will not work with the SB.

Interface cabling

=================

The cabling  is roughly the same as in the MIDI 1.0 Specification,

except that I used a different optocoupler and "AND" gates instead

of inverters (simply because Tandy sold the former with docs :-).

Numbers between  parentheses are  pin numbers. ICs have a standard

pin numbering  scheme. Other  parts have  pin numbers  written  on

them. For  resistors, it's  simply to  make difference between the

two ends.

6    5    4

+--!----!----!--+

Usually       !               !         IC seen from the top

a hole -->    >               !

!               !

+--!----!----!--+

1    2    3

"MIDI-in", "MIDI-out"  design the  female MIDI  plugs. "Game/MIDI"

designs the 15 pin female connector on the SB.

FROM PIN                 TO PIN

MIDI input part

---------------

MIDI-in (4)             Resistor1 220ohm (1)

Resistor1 220ohm (2)    Optocoupler (1)

Optocoupler (2)         MIDI-in (5)

[Optocoupler (1)        Diode -]

[Optocoupler (2)        Diode +]

Games/MIDI (9)          Resistor4 1Kohm (1)

Resistor4 1Kohm (2)     Optocoupler (5)

Games/MIDI (15)         Optocoupler (5)

Optocoupler (4)         Game/MIDI (4)

MIDI output part

----------------

Games/MIDI (12)         AND gate (1)

AND gate (1)            AND gate (2)

AND gate (3)            Resistor2 220ohm (1)

Resistor2 220ohm (2)    MIDI-out (5)

MIDI-out (4)            Resistor3 220ohm (1)

Resistor3 220ohm (2)    Game/MIDI (9)

Games/MIDI (9)          AND gate (14)

Games/MIDI (4)          AND gate (7)

Final remarks

=============

When using  a "veroboard", don't forget to cut tracks under the IC

and the optocoupler, or they will be short-circuited.

You will probably also want to setup two MIDI cords, to attach the

interface to  an instrument.  To stay under $5, you could use male

plugs instead  of female  ones for  the interface  and  plug  them

directly into the instrument.

When preparing the cords, be careful about not swapping the wires.

The pin 4 from one plug must be wired to pin 4 from the other end,

and the  pin 5  must be  attached to pin 5. None of the other pins

must be  wired, nor the overall cable shielded. There is no ground

in MIDI cords.

If you  want more  outputs, you  have to  slightly extend the MIDI

output part.  I would rather have only one AND gate input attached

to the  SB MIDI-out  pin and  only one  MIDI-out plug per AND gate

output. If  Creative Labs  offer  5  MIDI  outs  in  the  original

Connector Box,  it is  probably because  they use simple inverters

(or buffers)  and there  are 6  per IC.  As I  previously said,  I

bought AND  gates only  because they  had  data  sheets  attached,

whereas inverters/buffers had not...

Direct your remarks to mir@chorus.fr, or to

M. Adam Mirowski

Chorus systemes

6, avenue Gustave Eiffel

F-78182 Saint-Quentin-en-Yvelines CEDEX

FRANCE

My phone number is +33 (1) 30-64-82-00

I obviously  won't take  any responsibility  for the  damages  you

could do  to your  Sound Blaster  or to  your MIDI equipment while

using this interface.

SUPPLEMENT

----------

I) You can use invertors in place of AND gates. The schematic

diagram would be the following (for 2 outputs), instead of

the old "MIDI output part" description.

!\         !\

SB      ! \ 2      ! \ 4    +--------+    5  MIDI   4   +--------+      SB

MIDI ---!  >o--+---!  >o----! R=220  !----[  out    ]---! R=220  !---- +5V

out    1! /    !  3! /      +--------+      plug 1      +--------+    (pin 9)

12     !/     !   !/

!

!

!   !\

!   ! \ 6    +--------+    5  MIDI   4   +--------+       SB

+---!  >o----! R=220  !----[  out    ]---! R=220  !----- +5V

!  5! /      +--------+      plug 2      +--------+    (pin 9)

!   !/

!

!

+--- etc.

Pin 14 of the chip must be connected to +5V, and pin 7 to ground, just

like with the previous one. Also don't forget to remove the strap between

old pins 1 and 2! It is no more necessary and would court- circuit the

first inverter.

The other inverters in the chip are cabled between pins 13 (input) and

12 (output), 11 and 10, 9 and 8 (all the inverters point to the right).

If you try to use CMOS inverters (4... series) instead of TTL ones (74..

and 74LS... series), please remark that CMOS inverters "point" to the

left.

Tandy references are different for 74.., 74LS.. and 74HC.. circuits.

II) If you really want to save money, don't put any AND gate. Connect

together the wires going to pins 1, 2 & 3. Don't short-circuit the power

supply pins; leave them unconnected.

III) For the optocoupler, you could also use a 4N26, a 4N27 or a 4N28.

From: jre@earldom.UUCP (Jim Earl)

Newsgroups: sci.electronics

Subject: Re: Sound Blaster MIDI

Date: 21 Apr 92 03:20:58 GMT

here you go:

------------

DO IT YOURSELF:  SOUND BLASTER MIDI INTERFACE

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

Written by Ronald Davis, with introduction by Brad Barclay.

Here's one for all those electronics fans out there.  For those of you

interested in playing with the MIDI function of your Sound Blaster, but can't be

bothered to either search for Creative Lab's interface or who don't with to spend

the money it costs to buy one, here's the simple step-by-step method to build

your own.  All that you need are the parts listed below, and a little knowledge

on how to put things together, and you have yourself one working MIDI interface,

with the joystick port and all!

------------------------------

The following text explains how to build a MIDI interface for the

Sound Blaster. The circuit is my own design, but works just like

the commercially available interfaces that retails for $60-$90.

Here is the list of parts necessary:

Qty.           Part

1    Sharp PC900 or HP 6N138 opto-isolator

1    DB15P (15 pin 'D' connector, male. like joystick connector)

3    220 ohm resistors (2 for input, 1 for EACH output you need)

1    3 conductor shielded cable (length is up to you)

2    Female 5 pin DIN connector (1 for input, 1 for each output you need)

1    PC board with pre-drilled holes (about 2" x 2" from Radio Shack)

A few short lengths of hook-up wire

All of these parts, except the first 2, are available from Radio Shack.

Construction techniques are completely up to you, if you want to go

ultra-cheap, you don't even need a PC board or enclosure, and you could

use a MIDI cable you already have by cutting it in the middle. I used

a PC900 out of a non-working organ and cut the end off of a joystick I

didn't need and built the whole thing for about $4.00. Buying all the

parts new, and using an enclosure with built-in MIDI jacks may cost

about $20 or so.

Here is a diagram of the DB15P connector (looking at the solder side).

1       5     8

o o o o o o o o

o o o o o o o

9     12     15

Page 6

Here is the 5 pin FEMALE DIN plug (solder side).

2

5   o   4

o   o

3  o   o  1

This is the circuit for the MIDI INPUT.

+-----> To pin 5 of 'D' connector

|

To pin 15 of 'D' connector <---+     |     +-------+-----> To pin 9 of 'D' con.

|     |     |       |

| 6   |5    |4      \

+---------------+     /  220 ohm

|     PC900     |     \  resistor

|      or       |     /

|   HP 6N138    |     |

+---------------+     +------> To pin 6 of PC 900

|1    |2    |3

|     |     |

\     |     +----->  No connection

220 ohm /     |

resistor \     |

/     +-----> To pin 5 of DIN connector

|

To pin 4 of DIN connector <----+

Don't overlook the two 220 ohm resistors.

Also, for cables longer than a few feet, connect pin 2 of the DIN to pin

5 of the 'D' connector.

The output circuit is much simpler.

220 ohm resistor

To pin 12 of 'D' connector <---------/\/\/\/\/---------> To pin 5 of DIN con.

To pin 15 of 'D' connector <---------------------------> To pin 4 of DIN con.

Also, for cables longer than a few feet, connect pin 2 of the DIN to pin

5 of the 'D' connector.

This circuit has been built and tested by me, using Voyetra's SPJr, and

a cheap-o Casio keyboard. The input and output do work as shown here, but I

don't have any other MIDI devices to use to test using more than 1 output

device. Theoretically, it should work fine, but it has not been tested.

Page 7

The chances of hurting anything by trying is slim and none.

This circuit DOES work, but I make no guarantees, nor will I be

held responsible for any damages you may cause from your use of

this information.

If you have any questions, or would like to buy one already built for you,

I can be contacted on:

The Lighthouse BBS (207) 255-3700   (username is Ronald Davis)

The Music Studio BBS (708) 351-0288 (username is Volstagg)

-------

One thing: I built it, and it didn't work for me.  But I don't know for

sure if the midi player software I have works right.  If you find some

software, let me know!  I'd like to get this thing working...

Also: you *have* to use the optoisolator specified, or it won't work.

--Jim

*******************************************************************************

From: rjf@gdwb.oz.au (Rex Foord)

Newsgroups: comp.sys.ibm.pc.hardware,comp.sys.ibm.pc.soundcard,rec.music.synth,comp.music,sci.electronics

Subject: Re: Looking for that Sound-Blaster to MIDI circuit again.

Date: 6 Dec 92 11:44:14 GMT

I have just completed a MIDI interface for a soundblaster

compatible card - Sound Galaxy BX.

It uses the same pin outs as the soundblaster and it all seems

to work.

here is a rough diagram of the circuit I used. It is based on an

article in the soundblaster newsletter, and some other MIDI

interface circuits for other systems.

15 pin D connector             220R

1 +5v ------------------------/\/\/\---------------\

\ 4

5 gnd ---------------------------------------------2      MIDI OUT

/ 5

12 tx ------------------------/\/\/\---------------/

220R

1 +5v ---------------

|      |

\      |

220R /      |

R1  \      |

/      |

|      |                  220R

15 rx --------      |        ---------/\/\/\--------\

|C     |B       |A     |                \  4

---------------------    |                       MIDI IN

|  6      8       2 |  -----              /  5

|                   |   / \ IN914      --/

|      PC900        |   ---            |

|                   |    |             |

|  5              3 |    |             |

---------------------    |             |

|E              |K     |             |

|               |      |             |

5 gnd --------               ----------------------

PC900 is a high speed optoisolator

I built it on a small piece of veroboard in a small plastic box

with the 2 MIDI 5pin DIN plugs mounted on the lid.

You may want to check R1 above. In one circuit I saw it was a

higher value (390R). I used 220R but I haven't looked at the

output on a cro to check what the signal looks like yet.

I tested it using my synth and it played ok and I was able to record

ok.

Let me know if you have problems.

Could some kind soul who has a real midi interface confirm

the value of R1 above?

--

Rex Foord  PC Support | Information Systems Division   |"Time is nature's

Email: rjf@gdwb.OZ.AU | Geelong & District Water Board | way of making sure

Phone: +61 52 262329  | 61-67 Ryrie St Geelong	       | everything doesn't

Fax:   +61 52 218236  | Victoria 3220 Australia	       | happen all at once"

|                                |       W. Allen