(Comp.sys.handhelds)

Item: 1979 by sjthomas at cup.portal.com

Author: [Stephen J Thomas]

Subj: CMT 128K RAM cards

Date: Wed Feb 06 1991

There have been a few postings here on c.s.h.  inquiring about the CMT 128K RAM

cards for the HP 48SX.  There have been rumors about the CMT cards providing a

low battery warning and the physical size of the CMT cards.  I called CMT's BBS

and posed these questions to them.

This posting has three parts:

1.  CMT's reply to rumors that their RAM cards do not provide

low battery warnings.

2.  Several questions I posed about the physical chararcteristics

and warranty of the CMT RAM cards.

3.  CMT's reply to my questions in part 2.

I've edited out some chit-chat from my comments, but not CMT's.

Stephen J Thomas        sjthomas@cup.portal.com

Part 1 ===============================================================

From:    Reed Lacy

Msg #4, 21-Jan-91 03:00pm

Subject: 128R Low Batt & Power

To: Stephen Thomas

I am going to answer you by sending you a copy of a short report that includes

enough information to more than answer your question.  Thank-you for using our

card.  Here is the report:

The SRAM cards have two power sources.  The first of these is the battery that

is installed in the card.  The currents drawn from this battery are shown in

Table 1.  The other source of power is from the HP-48SX battery when the card

is installed in the HP-48SX.  The currents from this are shown in Table 2.

Table 1:

--------

Card Battery current of the HP 128K SRAM card versus the CMT 128K SRAM card

with the card removed from the HP-48SX.  Switch position is indicated as "Read"

or "Read/Write".

Card            Read              Read/Write

Current           Current

CMT 128K SRAM   0.2876 microamps  0.3003 microamps

HP  128K SRAM   1.649  microamps  1.676  microamps

Table 2:

--------

Current draw of the memory cards with 4.4 volts applied to the card and all

address and control lines held at ground.  This is close to the current draw

that occurs when the card is installed in a HP-48SX that is turned on.

Card            Read/Write

Current

HP-128K  SRAM    845    microamps

CMT-128K SRAM      2.0  microamps

Assuming that the card is sitting on the shelf, Table 1 would be the current

draw that is applicable.  The batteries that are used in the card are 75

mA-hour batteries.  Using this the expected life of the battery in the two

different cards is:

HP  128K SRAM ->  44,749 hours ->   5  years

CMT 128K SRAM -> 249,750 hours ->  28  years

This isn't realistic because the lithium batteries decay all by themselves in

about 5 years.  It does show that the CMT card's battery will definitely last

for 2 years and will probably last for 5 years.  The extra battery life makes

data stored in a CMT card more secure than the data in a HP card.

Table 2 shows that the CMT card draws significantly less operating current than

the HP card.  This will result in a longer system battery life for the HP-48SX.

The HP-48SX low battery indicator works with the CMT cards if a battery is

installed.  Without a battery installed, the low battery indicator does not

work.  This is a side effect of designing the card for low power usage.  We

felt that the decrease in power usage was worth this trade off.

Based on these figures I recommend that the battery in the card gets changed

every 2 years.  Since the power consumption is so low I also recommend that the

battery remain inside the card during storage and shipping.

Part 2 ===============================================================

From:    Stephen Thomas

To:      Reed Lacy                     Msg #6, 31-Jan-91 08:21pm

Subject: 128R Questions

I've heard some other questions and rumors about the CMT card, and I wonder if

you would care to address these:

1.  EduCALC is claiming that CMT has extended the warranty on the RAM cards

from 90 days to 1 year.

2.  It is obvious that the CMT fits the 48 more tightly than the HP cards --

this is especially apparent in port 2.  There are _rumors_ that HP is

concerned that the CMT card will spread the electrical pin connections in

the 48, increasing the risk that other cards may not make good electrical

contact.  Is this true?  If so, is CMT taking steps to correct this?

3.  I've read one comment that the (gold?) plating on the electrical contacts

of the CMT RAM card seem to have been fairly well etched after just a few

insertion/removals.  Do you have any information about the quality of these

contacts, or the number of insertion/ removals expected from a CMT card?

4.  [I've been asked] about the ESD protection of the CMT card since it does

not have a "shutter" (contact cover).  [What is a "transorb"?] Perhaps you

could shed some light on this.

I'm not trying to harass CMT....I've held your products in high regard.  But

many of us plugging these cards into our 48's have some genuine concerns.

Thanks for any info you can share.        Stephen Thomas

Part 3  - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

From:    Eric Gakstatter

Msg #7, 01-Feb-91 02:04pm

Subject: Re: 128R Questions

Dear Mr. Thomas,

Thank you for your questions and input.  We appreciate it that you care enough

to ask.

I will respond to your questions by answering the first one then giving an

answer that groups the next three together since they are all interrelated.

1.  Yes all CMT cards for the HP-48SX bought from EDUCALC have a 1 year

warranty.

2,3,4.  Yes our cards do fit more tightly, but this will not cause damage to

the connector.  The connector in the HP-48SX is designed with plastic bars

between each connecting spring so that the springs can not be depressed far

enough to cause any damage.  If a card is too thick, it won't fit in the

connector.

We use the same connector as HP does for our card tester and it has had

hundreds of cards inserted without a single failure.  All of the HP-48's that

we have used in testing and developing the cards have had hundreds of our cards

installed and removed without a problem.  We have tested an individual card for

over 1,000 installations with no failure that wasn't cured by cleaning the

contacts with a cotton swab moistened (not soaked) with isopropyl alcohol.

However, we do recommend that users minimize the number of insertions/removals.

This applies to all connectors of this type, not just CMT's.

The transzorb is a high speed transient absorber that is used to help protect

the card's circuitry from transient voltage spikes.  The shutter is patented by

EPSON (I believe) so we chose to use tranzorbs for protection against ESD.

We believe we have done our homework on this matter.  We have produced and

tested hundreds of cards and not experienced an ESD or mechanical problem.  As

you know, we are not new to this kind of technology.  CMT became the dominant

supplier (and in some cases, the only supplier) of plug-in modules for the

HP-41C/CV/CX, HP-71B and HP-75 hand-held calculators that proved to be VERY

reliable.  They experienced no ESD or mechanical problems and there are

literally tens or hundreds of thousands (I don't know the number) of these

modules sold worldwide.

We do recommend that when the card is not in the HP-48SX that you store the

card in its ESD protection bag and that you keep your fingers away from the

gold connectors.  This is really no different then the normal precautions that

you would take with any circuit board.  We feel that the HP-48SX, the card

connectors, and the CMT memory cards are quality products that should bring you

years of use.

=====

[Personal note: I have had two 128K RAMs in my 48 for some time now,

one is HP's and the other is CMT's.  Both work great.  -jkh-]