HP-48SX Questions, Answers, & Useful Tidbits

This list contains information which has not necessarily been verified,

and is not guaranteed to be correct, or even reflecting reality.  It was

compiled from various postings in comp.sys.handhelds, as well as other

sources.  Also note that, while the maintainer is an employee of

Hewlett-Packard, this list is not officially sanctioned by Hewlett-Packard

(the maintainer is doing this on his own time, and has absolutely no

connections with the HP division that makes calculators).

Please send any comments or suggestions to:

Darryl Okahata

Internet: darrylo%hpnmd@relay.hp.com

CompuServe: 75206,3074

Summary of questions:

1* I'm a novice, and I have some questions.  Where should I start?

2* How can I tell what ROM revision I have?

3* What bugs exist in the various versions of the HP-48SX?

4* What is \->ASC or ASC\->?

5* How can I get \->ASC and \->ASC?

6* Why does my HP-48SX occasionally "freeze" for a moment?

7* Why does (1/3)*3 equal 0.999999999999?

8* I want `pi' to be a numeric value, not a symbol.  What's happening?

9* If I add two temperatures like 34 deg.F and 11 deg.F, I get 504.67 deg.F.

Why don't I get 45 deg.F?

10* Why do I get corrupted binary files with kermit?

11* My RAM card was plugged into the calculator when I changed the RAM card

battery, yet I lost all the information on the card.  What happened?

12* Why do I get an "Invalid card data" error when I merge a RAM card?

13* My HP-48SX seems to take longer to turn on and off.  What's going on?

14* I can't account for some of the RAM in the HP-48SX (I can't tell where

it's being used).  What's going on?

15* How do I get rid of the HYDE library?

16* How can I transfer programs/data from my HP-28S to my HP-48SX?

17* I could speed up my HP-28S.  How can I speed up my HP-48SX?

18* Why does the HP-48SX display flicker slightly?

19* I've heard that other manufacturer's RAM cards will work with the HP-48SX.

Is this true?  Will it work?

20* What library ID numbers have already been used?

21* What do the funny symbols \->, \GS+, etc., mean?

22* What tricks are there for manipulating matrices?

23* How do I store fields of variable length string data in a compact, rapidly

accessible manner that does not require the overhead of storing strings in

lists?

24* What is "Vectored Enter", and how do I use it?

25* How can I tell, from within a program, if the battery is low?

26* I've heard the names "RPL", "Saturn", "STAR", "Voyager", etc..  What do

they mean?

27* Appendix A: ASC\-> and \->ASC functions:

28* Appendix B: Using non-HP RAM cards in your HP-48SX:

29* Appendix C: Compact Data Storage:

30* Appendix D: Various useful functions

1* I'm a novice, and I have some questions.  Where should I start?

Start by perusing the HP-48SX manuals.

Also, look over the section "Answers to Common Questions", in Appendix A

(Volume II) of the manual.

2* How can I tell what ROM revision I have?

Do the following:

1. Turn the calculator on (press ON and then release).

2.  Press ON again, and while holding it down, hit the "D" key (the fourth

white key in the top row) and then release the ON key.  You should get

three vertical lines in the display, one down the middle.

3.  Hit the backspace key (it says DROP and CLR over it).  You should get

some meaningless hexadecimal in the first line of the display.  (Mine says

705D9:1B8D...).

4. Press and hold down the EVAL key.  It should say:

Version HP48-*

Copyright HP 1989

where the * is an uppercase letter (A, B, etc.) indicating

the ROM version.

5. Release the EVAL key.  Hold down ON and hit "C" (the third white key in

the top row.  This is a general reset operation that returns the

calculator to normal from the diagnostic stuff (which is where ON-D puts

you).

(The above was mostly taken from a posting by Mark Adler --

madler@tybalt.caltech.edu)

3* What bugs exist in the various versions of the HP-48SX?

Version(s)   Bug

----------------------------------------------------------------------

A         DEFINE rounds numbers in user-defined functions if STD

is not the current display mode.

A         KGET on a zero length file from another machine will

cause the calculator memory to be cleared.

A         NXEQ from the Graphics Environment with flag -3 set (SYM

off in Modes) can overwrite the current equation with a

number.

A         Displaying the clock in 24-hour format causes the

one-digits for minutes to be incorrect in the alarm

catalog.

A,B,C       INV returns an incorrect result if used on an 8x8 or

larger matrix.  To get around this problem, divide the

identity matrix with the matrix to invert.

A,B,C,D      Entering a symbolic complex number in polar form like

'(A,<B)' (where < is really the funny little angle

symbol), gives an expression in A and B that is wrong

(the A and B are interchanged so A is the angle and B is

the magnitude).

A,B,C,D      If the clock is displayed during an ARCHIVE via RS-232,

there is a chance (not 100%) that calculator memory will

be cleared, after the transfer.  The workaround is to

turn the clock display off before doing the transfer.

A,B,C,D      TRNC and RND allow for an array in level 2 and a

symbolic in level 1 of the stack. This allows you to put

an array in a symbolic.  Example:

[ 2 3 ]

'7/8'

RND

returns 'RND(UNKNOWN,7/8)'

(The above was mostly copied from postings by Mark Adler

(madler@tybalt.caltech.edu) or Jurjen NE Bos (jurjen@cwi.nl).)

4* What is \->ASC or ASC\->?

These functions were written by William Wickes to facilitate transferring

HP-48SX binary objects in an ASCII format (useful when transferring objects

via electronic mail).  Some HP-48SX objects, like libraries, cannot normally

be converted into ASCII, unlike program objects, and these functions make it

possible to do so.

The \->ASC function converts the object in level 1 into an ASCII string,

which can then be uploaded to a computer for mailing.

The ASC\-> function converts the string object in level 1 back into an

object.  A checksum is used to insure that the decoding is correct.

5* How can I get \->ASC and \->ASC?

A copy of these programs are given in Appendix A, near the end of this

file.

6* Why does my HP-48SX occasionally "freeze" for a moment?

The HP-48SX must occasionally do "garbage collection" to free up unused

memory, and it is this garbage collection that is causing the momentary

"freeze".  "Garbage collection" is where the HP-48SX scans through memory,

looking for objects that are no longer used.

Using less stack supposedly makes garbage collection go faster.

7* Why does (1/3)*3 equal 0.999999999999?

This is due to the way numbers are typically represented by computers and

calculators.  There are some numbers, like `1/3' and `pi', that cannot be

exactly represented internally.  As a result, any calculations that use these

numbers are bound to be off.

8* I want `pi' to be a numeric value, not a symbol.  What's happening?

You are using "Symbolic Results Mode".  To turn this mode on or off, go to

the MODES menu (press orange/left-shift MODES), and press the "SYM" softkey.

9* If I add two temperatures like 34 deg.F and 11 deg.F, I get 504.67 deg.F.

Why don't I get 45 deg.F?

If the units attached to the number consists only of a temperature and

nothing else, operations are done using an absolute temperature scale

(relative to absolute zero).  If the units consists of a temperature and

something else, such as degrees/minute, the operation will not be done using

an absolute temperature scale.

It also does not make any physical sense to add two temperatures.  You can't

add a cup of water at 20 degrees C to a cup of water at 30 degrees C and end

up with two cups of water at 50 degrees C.

10* Why do I get corrupted binary files with kermit?

Some versions of kermit distinguish between ASCII and binary files, and so

you have to worry about the transfer modes at *BOTH* ends of the connection

(the HP-48SX end and the computer end).  For example, when receiving ASCII

files, Unix kermit must translate CR/LF pairs into LF.  If the HP-48SX is

transmitting a binary file, but the Unix kermit is expecting ASCII, any CR/LF

pairs in the binary file will get translated to LF, corrupting the binary

file.  Unfortunately, you cannot "uncorrupt" the corrupted binary file by

simply reversing the transfer and expecting kermit to translate LF to CR/LF.

This is because the binary file may contain occurrences of LF that were not

originally part of a CR/LF sequence.

Kermit running on IBM PC clones do not have to translate any incoming CR/LF

pairs, which is why you do not see this problem there.

11* My RAM card was plugged into the calculator when I changed the RAM

card battery, yet I lost all the information on the card.  What

happened?

You forgot to turn ON the calculator before changing the battery.  The

HP-48SX only supplies power to the RAM card while the HP-48SX is turned on.

While the calculator is turned off, no power is supplied by the calculator,

and all power comes from the RAM card battery.  If you then take out the

battery from the RAM card while the calculator is turned OFF, the RAM card

has no source of power, and you will lose all of the contents in the RAM

card.

12* Why do I get an "Invalid card data" error when I merge a RAM card?

This message is usually seen when you plug a brand-new RAM card into an

HP-48SX.  In this case, this message is normal and is harmless.  It just

means that there was no (valid) data on the card.

If you see this message under any other conditions, it could be one of the

following:

1. You took the battery out of the RAM card while the card was out

of the calculator.

2. You took the battery out of the RAM card while the card was in

the calculator *BUT* the calculator was turned off.

3. The RAM card battery is dead.

4. There is a problem with either the calculator, the RAM card, or

both.

13* My HP-48SX seems to take longer to turn on and off.  What's going on?

The usual causes for this are RAM/ROM cards and libraries.  When you turn the

calculator on, it checks RAM (the more you have, the longer it takes), and

checks to see if any libraries need initializing.

Also, having a lot of alarms can supposedly also cause this.

14* I can't account for some of the RAM in the HP-48SX (I can't tell where

it's being used).  What's going on?

It can be any one of a number of things:

---------------------------------------------------------------------------

Enabling the use of LAST STACK, LAST ARG, LAST CMD, etc., uses up extra RAM,

as the calculator has to keep track of the last stack, arguments, command,

etc..  This is particularly important with LAST STACK if you have a

large/complex stack, as the HP-48SX has to keep enough information around to

recreate the stack after each operation.  You can clear out these variables

by pressing ON-C (but also note that this will clear out the stack, the

graphics PICT, etc.).

---------------------------------------------------------------------------

From billw@hpcvra.CV.HP.COM Fri Mar  1 17:00:00 1991

From: billw@hpcvra.CV.HP.COM (William C Wickes)

Date: Wed, 1 Aug 1990 19:51:37 GMT

Subject: DOERR: An HP48SX memory eater

Organization: Hewlett-Packard Co., Corvallis, OR, USA

Newsgroups: comp.sys.handhelds

Here's a candidate for "most obscure way to use up memory on the HP 48SX."

When a program executes DOERR with a string argument, the string and hence

the program too if the string is embedded in the program are referenced for

the sake of the ERRM command.  Thus if you purge the program, the memory used

by the program is not reclaimed until the ERRM reference is updated to a new

string.  If the string is in a port, you will get the Object In Use error if

you try to purge the library or backup object containing the string.

A system halt prior to a PURGE will not help unless the object is in port 1

or port 2 (it will reclaim memory held by a successfully purged but still

referenced object).  For port 0 objects, the only way to break the reference

is to execute DOERR again with a new string.

---------------------------------------------------------------------------

15* How do I get rid of the HYDE library?

Getting rid of the HYDE library can be fun.  Here's how to do it:

1. Go the HOME directory.

2. Press left-shift (orange) LIBRARY.  The calculator is now

displaying a menu of libraries attached to the HOME directory.

3. Press the menu key that says "STRA".  You are now accessing the

HYDE library.

4. Press the menu key that says "JEKY".  You have now disabled the

HYDE library and restored the old (standard) set of messages.

5. Make sure that a pointer to the library is not on the stack.  The

easiest way to do this is to clear the stack using CLR.

6. Detach the library using:

:0:998 DETACH

7. Purge the library using:

:0:998 PURGE

When you execute the PURGE, the screen will "jump"/"be messed up"

for a brief moment.  Don't worry about it -- it doesn't hurt.

16* How can I transfer programs/data from my HP-28S to my HP-48SX?

You use a program called "INPRT", which is available from various archives,

and is also included in the IBM-version of the HP-48SX serial cable kit and

in the Program Development Library.

INPRT sends a program to the I/R output of the HP-28S, and the HP-48SX

receives it via its I/R input.

17* I could speed up my HP-28S.  How can I speed up my HP-48SX?

You can't.  Unlike the HP-28S, the clock speed in the HP-48SX is hardwired to

2 MHz.

18* Why does the HP-48SX display flicker slightly?

Display flicker is usually caused by fluorescent lights (the rapid pulsing of

the fluorescent lights, which is normally unnoticeable, interacts with the

rapid pulsing/scanning of the HP-48SX LCD display, which is also normally

unnoticeable).  It's normal and harmless.

19* I've heard that other manufacturer's RAM cards will work with the

HP-48SX.  Is this true?  Will it work?

While some cards may work, there is a chance that you may severely damage

your HP-48SX.

For more information, see Appendix B, "Using non-HP RAM cards in your

HP-48SX", near the end of this file.

20* What library ID numbers have already been used?

766    -- HP EPSPRINT.LIB library

767    -- HP PCLPRINT.LIB library

768    -- HP Stopwatch library

777    -- "TenK" library

998    -- "HYDE" library

1092    -- MLDL library (Jan Brittenson -- bson@erice.ai.mit.edu)

(In development -- not officially released)

1644    -- SWING ("VMS" graphical directory browser)

1791    -- TOOLs library

1793    -- New IFERR library

21* What do the funny symbols \->, \GS+, etc., mean?

These are the ASCII representation of the special HP-48SX graphical

characters.  See the section "Character Translations" in Chapter 33 (Volume

II) of the manual for a table of symbols <--> ASCII representations.

22* What tricks are there for manipulating matrices?

You can build up a matrix by rows using \GS+

You can take a matrix apart by rows using \GS-

23* How do I store fields of variable length string data in a compact,

rapidly accessible manner that does not require the overhead of

storing strings in lists?

See Appendix C, "Compact Data Storage", near the end of this file.

24* What is "Vectored Enter", and how do I use it?

From billw@hpcvra.CV.HP.COM Fri Mar  1 17:00:00 1991

From: billw@hpcvra.CV.HP.COM (William C Wickes)

Date: Mon, 12 Mar 1990 21:21:55 GMT

Subject: HP-48SX Vectored Enter

Organization: Hewlett-Packard Co., Corvallis, OR, USA

Newsgroups: comp.sys.handhelds

The HP-48SX manuals do not document a very powerful feature that we call

"Vectored ENTER," that allows you in effect to redefine or bypass the

command line parser and to have a shot at the stack etc. after the command

line has been executed.

Keys that execute an automatic ENTER perform a two-step process:

1.  The command line is parsed and evaluated.

2.  The key definition is executed.

When flags -62 and -63 are both set, the system extends this process as

follows:

1.  The current path is searched for a global variable named *aENTER (here

"*a" is the Greek alpha character--character 140).  If present, the command

line is entered as a string object and *aENTER is executed.  If absent, the

command line is parsed and evaluated normally.

2.  The key definition is executed.

3.  The current path is searched for a global variable named *bENTER ("*b" is

Greek beta--character 223).  If present, then a string representing the key

definition is put on the stack, and *bENTER is executed.  The string is the

key definition object's name if it is a command, XLIB name, global or local

name, or an empty string for other object types; its primary purpose is to

implement things like the TRACE mode on other calcs, where you can print a

running record of what you do.

A simple example of the use of *aENTER is to create a more convenient binary

calculator, where *aENTER slaps a "#" on the front of the command line so you

don't have to bother when entering numbers.

25* How can I tell, from within a program, if the battery is low?

Preston Brown (prestonb@hpcvra.CV.HP.COM) posted the following:

The following 48SX program reports the status of the ALERT annunciator.

The ALERT annunciator is activated by alarms or low battery.

This program is provided free of charge "as is" and has no warranty.  No one

is liable for any consequential damages.

Preston

%%HP:T(3)A(D)F(.);

\<<

RCLF

8 STWS #FFh #0h +

#10Bh

#6595Ah SYSEVAL

#8h AND #0h >

SWAP STOF

\>>

26* I've heard the names "RPL", "Saturn", "STAR", "Voyager", etc..  What

do they mean?

ASAP

This is a simple Saturn assembler, written in the perl language (and is

therefore usable mostly on Unix machines).

Chip8

This is a machine-code program that was inspired by the chip8 video

game interpreter for the CDP1802 microprocessor several years back.

Chip8 allows you to write a simple graphics-based video game for the

HP-48SX.  Among the games written are "clones" of breakout, pacman, and

pong, to name a few (about half?).

HYDE

This is the "HYDE" library, whose only purpose is to change the error

messages in the HP-48SX to make it "user unfriendly".

ML

Machine Language.  This is usually used in reference to HP-48SX

assembly language programming.

MLDL

Machine Language Development Library.  This is a library, which is

still under development, that allows you to debug machine language

programs with only an HP-48SX.

PDL

"Program Development Link".  This is an MSDOS program, sold by

Hewlett-Packard, that allows you to write and develop HP-48SX

applications from your IBM PC clone.  Programs are written on your PC,

transferred to the HP-48SX, and tested from your *PC* (and *not* the

HP-48SX).

RPL

RPL is the name of the language used to program the HP-48SX and HP-28

series calculators.  While many people consider RPL to stand for

"Reverse Polish Lisp", an HP Journal article described RPL as

"ROM-based Procedural Language".  To muddy the waters, it's also

interesting to note that the HP-48SX development team considers RPL to

mean "Reverse Polish Lisp", and not what was mentioned in the HP

Journal.  [Note: See RPL.TXT on this disk.  -jkh-]

SASS

SASS is a simple Saturn assembler, written in C/yacc (BSD Unix &

Bison).  It uses Alonzo Gariepy's mnemonics.

Saturn

"Saturn" is the internal code name for the processor used in the

HP-48SX, HP-28 series, and the HP-75/71B calculators.

STAR

This is the "Saturn Macro Assembler" (how "STAR" comes from this, I

don't know), an assembler that mostly uses Alonzo Gariepy's mnemonics.

It's written in C and runs on many different machines (PCs, Amigas,

Unices, etc.).  STAR is available via anonymous ftp from ftp.ai.mit.edu

in "/pub/star-1.04.4.tar.Z".  It's also available from various bulletin

boards (the HP forum in CompuServe also has a copy).  The latest

version is "1.04.4".

SWING

SWING is a library program that displays a graphical tree structure of

the directory hierarchy in your HP-48SX, and allows you to move from

directory to directory using the arrow keys (it is supposedly based

upon an old PD VMS utility).

Voyager

Voyager is an "interactive disassembler" for IBM PC clones that

disassembles HP-48SX RPL and machine code.  You download a copy of the

RAM *and* ROM in your HP-48SX to your PC, and run Voyager.  Using

Voyager, you can then view the disassembled or unthreaded code.

Unfortunately (or fortunately), Voyager uses the HP mnemonics, and not

Alonzo Gariepy's mnemonics.  Voyager is available from the HP BBS (I

think), via anonymous ftp from the HP BBS (hpcvbbs.cv.hp.com), and from

various bulletin boards (the HP forum in CompuServe also has a copy).

The latest version is "1.07".

27* Appendix A: ASC\-> and \->ASC functions:

From billw@hpcvra.CV.HP.COM Fri Mar  1 17:00:00 1991

From: billw@hpcvra.CV.HP.COM (William C Wickes)

Date: Fri, 17 Aug 1990 22:27:42 GMT

Subject: HP 48 Object Encoding

Organization: Hewlett-Packard Co., Corvallis, OR, USA

ASCII Encoding HP 48SX Objects

Sending an HP 48SX object via electronic mail can be difficult if the object

does not have an ASCII form, such as is the case for library objects.  There

are various encoding schemes available on different computer systems, but these

require that the sender and receiver have similar computers, or at least

compatible encode/decode schemes.  The programs listed below perform the

encoding and decoding on the HP 48SX itself, which has the advantage of being

completely independent of any computer.

The programs are nominally called \->ASC and ASC\->.  The former takes an

object from the stack and converts it to a string, in which each nibble of the

object and its checksum is converted to a character 0-9 or A-F.  (The object

must be in RAM, otherwise a "ROM Object" error is returned.) For sake of easy

inclusion in email letters, the string is broken up by linefeed characters

after every 64 characters.

ASC\-> is the inverse of \->ASC: it takes a string created by \->ASC and

converts it back into an object.  When you transmit the encoded strings, be

sure not to change the string; ASC\-> uses the checksum encoded in the string

to verify that the decoding is correct.  An "Invalid String" error is returned

if the result object does not match the original object encoded by \->ASC.

When you upload a string to your computer, use HP 48 translate mode 3 so that

the HP 48 will convert any CR/LF's back to LF's when the string is later

downloaded.

Two versions of ASC\-> are included here.  The first (P1) is in HP 48 user

language, using SYSEVALs to execute system objects.  P2 is a string that the

setup program uses P1 to decode into an executable ASC\->--then P1 is

discarded.  The second version is more compact than the first, and also

uneditable and therefore safer (but it can't be transmitted in ASCII form,

which helps to make the point of this exercise).

Here are the programs, contained in a directory:

(start)

%%HP: T(3)A(D)F(.);

DIR

P1              @ ASC\-> Version 1.

\<<

IF DUP TYPE 2 \=/

THEN "Not A String" DOERR

END RCWS \-> ws

\<< 16 STWS

#0 NEWOB SWAP DUP SIZE

IF DUP 4 <

THEN DROP SWAP DROP "Invalid String" DOERR

END

DUP 65 / IP - 4 - # 18CEAh SYSEVAL

"" OVER # 61C1Ch SYSEVAL

SWAP # 6641F8000AF02DCCh

# 130480679BF8CC0h # 518Ah SYSEVAL

# 19610313418D7EA4h # 518Ah SYSEVAL

# 7134147114103123h # 518Ah SYSEVAL

# 5F6A971131607414h # 518Ah SYSEVAL

# 12EA1717EA3F130Ch # 518Ah SYSEVAL

# 280826B3012808F4h # 518Ah SYSEVAL

# 6B7028080BEE9091h # 518Ah SYSEVAL

# BE5DC1710610C512h # 518Ah SYSEVAL

# 705D00003431A078h # 518Ah SYSEVAL

# 3D8FA26058961431h # 518Ah SYSEVAL

# 312B0514h # 518Ah SYSEVAL

# 18F23h SYSEVAL

DUP BYTES DROP 4 ROLL

IF ==

THEN SWAP DROP

ELSE DROP "Invalid String" DOERR

END ws STWS

\>>

\>>

P2      @ ASC\->  Version 2.  To be converted by ASC\-> version 1.

"D9D20D29512BF81D0040D9D20E4A209000000007566074726636508813011920

140007FE30B9F060ED3071040CA1304EC3039916D9D2085230B9F06C2A201200

094E66716C696460235472796E676933A1B21300ED30FD5502C230C1C1632230

CCD20FA0008F14660CC8FB97608403104AE7D814313016913213014117414317

414706131179A6F5C031F3AE7171AE214F8082103B6280821909EEB0808207B6

215C0160171CD5EB870A13430000D50713416985062AF8D341508813044950B9

F06BBF06EFC36B9F0644230C2A201200094E66716C696460235472796E676933

A1B2130B21300373"

P3      @\->ASC.     To be converted by ASC\->.

"D9D20D2951881304495032230FD5502C230A752688130ADB467FE30322306AC3

0CB916E0E30CBD30F6E30C1C1632230CCD20DC0008F14660CC8FB97608403104

AE7D8143130169174147061741431311534AC6B4415141534946908D9B026155

4A6F53131F3AE731A014C161AE215F08082103A6280821939EEC08082170A621

4C161170CD56B870A18503430000D5071351796A9F8D2D02639916D9D2085230

C2A209100025F4D402F426A6563647933A1B2130A2116B213033C0"

SETUP   @Automatic setup program

\<< P2 P1 'ASC\->' STO

P3 ASC\-> '\->ASC' STO

{ P1 P2 P3 SETUP } PURGE

\>>

END

(end)

Installation instructions:

1.  Edit the above text between (start) and (end) into a text file named CONV

(for example).  Be sure that you leave the strings exactly as entered

above, with no extra spaces or other invisible characters at the beginnings

or ends of the lines.

2.  Set the HP 48SX into ASCII transfer mode.

3.  Using Kermit, download CONV text file to the 48, verify its checksum

(6C8Ah).

4.  Execute CONV to make it the current directory.

5.  Execute SETUP.

6.  The directory CONV now contains ASC\-> and \->ASC, ready to use.

To archive the decoded versions of ASC\-> and \->ASC back on your computer,

be sure to set the HP 48SX in binary transfer mode before uploading.

Disclaimers:

+ Use the programs at your own risk.  Any time you delve into the SYSEVAL

world, there are increased dangers.  Archive your 48 memory before

experimenting with these programs!  Also, verify the checksums of objects

defined above to make sure they have been downloaded correctly, before

executing ASC\->.

+ I will not answer questions about how the programs work.  This is not because

of any great secrecy, but rather because it's hard to give any answer that

doesn't lead to more questions, and more, and more...

+ 48 hackers are welcome to mine any nuggets they can from the programs, and

from the fact that \->ASC is a convenient way to decompile an object.

28* Appendix B: Using non-HP RAM cards in your HP-48SX:

If you use RAM cards that are NOT designed for the HP-48SX, it is possible to

severely damage your HP-48SX.  If you want to be safe, you should only use

RAM cards designed for the HP-48SX.

Here is an edited discussion from comp.sys.handhelds.

From steveh@hpcvra.CV.HP.COM Fri Mar  1 17:00:00 1991

From: steveh@hpcvra.CV.HP.COM (Steve Harper)

Date: Thu, 10 May 1990 22:46:09 GMT

Subject: RE: HP48SX Memory Card Pricing

Organization: Hewlett-Packard Co., Corvallis, OR, USA

There has been a substantial amount of comment regarding the memory cards for

the HP48SX and their prices.  My purpose in this response is not to attempt

to justify any particular price, but rather to present the technical reasons

why there is a substantial price difference between the memory cards and

other types of expansion memory for PC's, for example, with which users are

probably more familiar.

Some users have correctly pointed out that the memory in the cards is static

RAM rather than dynamic RAM commonly used in PC's.  Dynamic RAM uses one

transistor and a capacitor for each bit of memory whereas static RAM requires

either four transistors and two resistors, or six transistors.  The net

result is that an equivalent amount of static RAM is much larger and

therefore much more expensive than dynamic RAM.  The advantage is that static

RAM doesn't need to continually be running and drawing current (refresh

cycles) to retain the contents of memory.

In addition, the static memory used in the cards is not just any static

memory, but is specially processed and/or selected for very low standby

current.  This allows the backup battery in the card to keep memory alive for

a very long time, rather than requiring the user to replace it every few

months.  The special processing and/or special testing to select low current

parts adds to the already higher cost of the static RAM chips.

The standard molded plastic DIP package used for most integrated circuits,

including memory chips, is relatively inexpensive because of its simplicity

and the huge volumes.  Unfortunately, these packages are too large to put

into a memory card.  Therefore, the card manufacturer mounts the individual

silicon memory chips directly on a special thin PC board together with the

memory support chips.  Because multiple chips are being placed in a single

hybrid package in a special process which has lower volume, yields are lower

and this again causes the cost to be higher.  Indeed, the yield becomes

exponentially worse as the number of chips and interconnections increases in

such a packaging process.

In addition to the memory chips themselves, two more integrated circuits and

several discrete components are required for power and logic control.  A

bipolar technology chip senses the external voltage and switches the power to

the chips from the internal keep-alive battery as needed.  A CMOS gate array

chip protects the memory address and data lines from glitches/ESD when the

card is not plugged in.  This chip also generates the proper enabling signals

when there are multiple memory chips in the card, as is presently the case

with the 128 Kbyte RAM card.  These chips must be designed for extremely low

current, just as the memory chips are.

In addition to the battery and the battery holder, the other mechanical parts

are important, too.  The molded plastic frame holds the PC board and provides

the foundation for the metal overlays and the shutter-and-springs assembly

which protects the contacts from ESD and from contaminants.  The

write-protect switch is also an important feature.  It is quite expensive for

the manufacturer to make the tools necessary to fabricate each of these parts

as well as the tools to assemble and test the complete card.  While the

volume of memory cards is relatively low this tooling cost represents a

significant part of the cost of each card.

Admittedly, there are other alternatives, such as those presently used in

PC's, to provide a memory expansion capability.  To provide that kind of

expansion would require the calculator to be much larger than it is and

possibly more expensive.  This is clearly very undesirable.

Other features that were felt to be essential were the ability to distribute

software applications and to share and archive/backup user-created programs

and data.  Other expansion alternatives do not provide these important

benefits.  The IO capabilities of the calculator provide these features only

to a limited degree.

One other item bears repeating here: Memory cards for use in the calculator

will clearly indicate that they are for use with the HP48SX.  Other memory

cards exist which are mechanically compatible with the HP48S, but these cards

cannot be relied upon to work electrically in the calculator.  The HP48SX

cards are designed for a lower supply voltage range.  Use of the other cards

may cause memory loss, and under certain circumstances may even damage your

calculator electrically.

From steveh@hpcvra.CV.HP.COM Fri Mar  1 17:00:00 1991

From: steveh@hpcvra.CV.HP.COM (Steve Harper)

Date: Fri, 11 May 1990 16:52:07 GMT

Subject: Re: Memory Card: Give Us *True* Facts!

Organization: Hewlett-Packard Co., Corvallis, OR, USA

My previous statement that under certain circumstances the calculator may

even be damaged electrically is not a ploy.  If the calculator's internal

power supply voltage happens to be near the low end of the range, say 4.1 V,

and the voltage at which the card's voltage control chip shuts it down

happens to be near the high end of its range, say 4.2 V (this can and does

occasionally occur for the non-HP48SX cards), then the calculator will start

to drive the memory address lines and the card will still have these clamped

to ground (that's what it does to protect itself when there is not sufficient

system voltage to run).  This unfortunate situation may simply trash your

memory, or if the calculator tries to drive enough of the lines high at the

same time, several hundred milliamps may flow...for awhile that is, until

something gives up...  On the other hand, your calculator and a particular

non-HP48SX card may work just fine if those voltages happen to be at the

other end of their ranges.  These voltages are also slightly temperature

sensitive.  It may work in the classroom or office and not at the beach, or

vice versa.  The voltage trip point of the HP48SX cards has been set lower (a

different voltage control chip) so that this cannot occur, regardless of part

and temperature variations.

One other item was brought to my attention yesterday by Preston Brown that I

should have included in my original posting here.  While most of us recognize

that comparing ram cards to a handful of dynamic ram chips to plug into your

PC is apples and oranges, it may be more interesting to compare the HP48SX

cards with cards for other products, like the Atari Portfolio, the Poquet,

the NEC Ultralite, etc.  I believe you will find that the prices on the

HP48SX cards are not at all out of line.

Steve

"I claim all disclaimers..."

the non-HP48SX cards

From prestonb@hpcvra.CV.HP.COM Fri Mar  1 17:00:00 1991

From: prestonb@hpcvra.CV.HP.COM (Preston Brown)

Date: Thu, 17 May 1990 17:26:53 GMT

Subject: Re: Memory Card: Give Us *True* Facts!

Organization: Hewlett-Packard Co., Corvallis, OR, USA

When the RAM cards detect that voltage is too low to operate they clamp the

address lines to ground.  This clamping is done by turning on the output

drivers of a custom chip included on the card.  The clamping current is

speced at 2mA min at the Vol output level.  Since the 48 can be trying to

drive the line all the way high even more current is typical.  10mA per fight

is not uncommon with totals of several hundred mAs.

The VDD power supply is regulated at 4.1 - 4.9 with typical parts at the low

end (4.3).  The power to the cards is switched thru a transistor, creating up

to a 0.1V drop.  Standard Epson cards have a significant chance of seeing

this voltage as to low and shutting down.  We have seen cards do this in the

lab.  When it occurs the calculator locks up with VDD pulled down to about

2.5V and 250mA being drawn from the batteries.  This current drain greatly

exceeds the ratings for the power supply and can damage your calc.  The least

that will happen is a loss of memory.

Now, why didn't we regulate VDD higher?  The 48 has two power supplies VDD at

4.3 and VH at 8.5.  VH cannot be regulated higher without exceeding the spec

for our CMOS IC process.  VH is used as the + voltage for the I/O.  In order

to meet a +3V output level VH must be more then 3.6V above VDD.  (VDD is used

as I/O ground).  Our power supply system increase the battery life and

reduces the cost greatly for the wired I/O.

Preston

29* Appendix C: Compact Data Storage:

From Jim Donnelly (jimd@cv.hp.com):

A simple length-encoding technique can be put to use for a free-format, very

compact multi-field data storage system.  Two tiny programs, SUBNUM and

STRCON are here to help the process, and are listed near the end of this

note.  At the end of the note is a directory that may be downloaded into the

HP 48 that contains the examples.

The principle is to store starting indices in the beginning of a string that

point to fields stored subsequently in the string.  The indices are stored in

field order, with an additional index at the end to accommodate the last

field.  There are several small points worth mentioning:

1) Fields may be 0-length using this technique.

2) The execution time is uniform across all fields.

3) This technique saves about 4 bytes per field after the first field,

because the string prolog and length are omitted for fields 2 -> n.

EXAMPLE:

--------

Indices  |          Fields

Character               |     1 11111111 12222222222

Position :   1  2  3  4 |567890 12345678 90123456789

+--+--+--+--+------+--------+-----------+

String :    | 5|11|19|30|Field1| Field2 |  Field 3  |

+--+--+--+--+------+--------+-----------+

This is a string that contains 3 fields, and therefore 4 index entries.  The

first field begins at character 5, the second field begins at character 11,

and the third field begins at character 19.  To keep the pattern consistent,

notice that the index for field 4 is 30, which is one more than the length of

the 29 character data string.

To extract the second field, place the string on the stack, use SUBNUM on

character 2 to extract the starting position, use SUBNUM on character 3 to

extract the (ending position +1), subtract 1 from the (ending position+1),

then do a SUB to get the field data.  NOTE: The index for field 1 is stored

as character code 5, NOT "5"!  To place the field index for field 1 in the

string, you would execute "data" 1 5 CHR REPL.

PROGRAM:

--------

The following program accepts an encoded data string in level 2 and a field

number in level 1:

DECODE   "data"  field#  -->  "field"

<<  --> f

<<

DUP f SUBNUM                ; "data" start -->

OVER f 1 + SUBNUM           ; "data" start end+1 -->

1 -                         ; "data" start end -->

SUB                         ; "field" -->

>>

>>

DATA ENCODING

-------------

The following program expects a series of 'n' strings on the stack and

encodes them into a data string suitable for reading by the first example

above.

The programs SUBNUM and STRCON are used to assemble the indices.

ENCODE      field n  ...  field 1   n   -->  "data"

<< DUP 2 + DUP 1 - STRCON --> n  data

<<

1 n

FOR i

data i SUBNUM OVER SIZE   ; ... field index fieldsize

+ data SWAP               ; ... field "data" index'

i 1 + i + SWAP CHR REPL   ; ... field "data"'

SWAP + 'data' STO         ; ...

NEXT

data                        ; "data"

>>

>>

In this example, four strings are encoded:

Input:  5: "String"

4: "Str"

3: "STR"

2: "STRING"

1:         4

Output: "xxxxxSTRINGSTRStrString"      (23 character string)

(The first five characters have codes 6, 12, 15, 18, and 24)

VARIATION:

----------

The technique above has a practical limit of storing up to 254 characters of

data in a string.  To overcome this, just allocate two bytes for each field

position.  The code to extract the starting index for becomes a little more

busy.  In this case, the index is stored as two characters in hex.

Indices  |          Fields

Character               | 11111 11111222 22222223333

Position :   12 34 56 78|901234 56789012 34567890123

+--+--+--+--+------+--------+-----------+

String :    |09|0F|17|21|Field1| Field2 |  Field 3  |

+--+--+--+--+------+--------+-----------+

<<  --> f

<<

DUP f 2 * 1 -           ; "data" "data" indx1 -->

SUBNUM 16 *             ; "data" 16*start_left_byte  -->

OVER f 2 * SUBNUM +     ; "data" start

OVER f 2 * 1 + SUBNUM   ; "data" start end_left_byte -->

16 * 3PICK f 1 + 2 *

SUBNUM + 1 -            ; "data" start end -->

SUB                     ; "field"  -->

>>

>>

TWO VERY TINY HELPFUL PROGRAMS

------------------------------

SUBNUM          "string"  position  -->  code

<< DUP SUB NUM >>

STRCON          code  count  -->  "repeated string"

<< -->  code count

<< "" code CHR 'code' STO

1 count START code + NEXT

>>

>>

A DIRECTORY YOU CAN DOWNLOAD

----------------------------

This is a directory object.  Cut after the === to the end of the file and

download to your HP 48 using the ASCII transfer.

========================================================================

%%HP: T(3)A(D)F(.);

DIR

DECODE

\<< \-> f

\<< DUP f

SUBNUM OVER f 1 +

SUBNUM 1 - SUB

\>>

\>>

ENCODE

\<< DUP 2 + DUP 1

- STRCON \-> n data

\<< 1 n

FOR i data

i SUBNUM OVER SIZE

+ data SWAP i 1 +

SWAP CHR REPL SWAP

+ 'data' STO

NEXT data

\>>

\>>

STRCON

\<< \-> code count

\<< "" code CHR

'code' STO 1 count

START code

+

NEXT

\>>

\>>

SUBNUM

\<< DUP SUB NUM

\>>

END

30* Appendix D: Various useful functions

(Shamelessly lifted from the HP-41C manual:)

The LASTX function is useful in calculations where a number occurs more than

once.  By recovering a number using LASTX, you do not have to key that number

into the calculator again.

For example, calculate:

96.704 + 52.394706

--------------------

52.394706

Keystrokes:                     X:

------------------              --------------------

96.704 ENTER                    96.704

52.304706 +                     149.098706

LASTX                           52.304706

/                               2.84568265351

@

@ This is a version of LASTX for the HP-48SX

@

%%HP: T(3)A(D)F(.);

\<< DEPTH \-> n

\<< LASTARG DEPTH n - DUP \-> s

\<< ROLLD s 1 - DROPN

\>>

\>>

\>>