HACK, 6 interesting (and dangerous) system-level routines by various authors.

Documentation by Joe Horn.

Uses Rick Grevelle's HACKIT library.

Some of these programs can easily cause a Memory Clear if not used

carefully.  Follow these instructions closely, and be sure to backup

your memory before experimenting with these tools.

����������������������������������������������������������������������������Ŀ

STRIP: Strips the << and >> off the program in level 1.

By Conrad Winchester.

#  Argument: Program object with << and >> delimiters.

#  Caution: STRIP does not check, it just runs, removing the second

five and second-to-last five nibs of the object in level 1.  If you

give it something other than a program, you may get a Memory Clear.

PACK: Packs a program's SYSEVALs into "External" form.

By Joe Horn.

#  Changes all << #nnnnn SYSEVAL >> pairs in a program into the single

2.5-byte System RPL object equivalent to that SYSEVAL.  Also works on

lists.  See PACK.DOC on EduCALC Goodies Disk #2 for details.

FNXT: Find Next.  Searches ROM/RAM for any hex digits.

#  Input: #search, #start, any.

#  Level 1 is DROPped and SYSRCLed when search terminates, which allows

repeated pressing of FNXT.  Notice that the argument on level 3 is the

binary number to look for, immediately followed by the number of digits

to look for (minus 1!), which allows you to look for leading zeros.

(#1237 looks for the 8 digits 00000123.)

#  Example: Code objects begin with #02DCCh.  That's 5 hex digits, so

add the digit "4" (one less than 5) to it: #02DCC4h.  (Leading zeros do

not need to be typed, of course).  To look for all the Code objects in

the operating system (starting at address #0h), type: #2DCC4h #0h #0h

FNXT.  Pressing FNXT again finds the next one, etc.

#  Note: If you want speed and have a PC handy, don't use FNXT; use the

SEARCH command in Derek Nickel's Voyager program.

PRCL: Recalls contents of an entire port to the stack.

# Argument: Port number (real) 0, 1 or 2.

# Example: 1 PRCL yields the contents of port 1 on the stack.

# Note: The "objects" on the stack are only poiners, not yet NEWOB'd,

so you can recall the "contents" of a very large card into a very

little amount of MEM.  However, most operations on these will perform a

NEWOB (if there is sufficient MEM), such as STO.  You can find the

address of port objects, however, by using ADDR after PRCL.

->RPN: Convert algebraic object into equivalent RPN object (list).

By Joe Horn.

#  Example: '1+2*3+4' ->RPN  -->  { 1 2 3 * + 4 + }.  (Try EVAL).

RPN->: Convert RPN object (list) into equivalent algebraic object.

By Joe Horn.

#  Example: { 1 2 3 * + 4 + } RPN->  -->  '1+2*3+4'.  (Try EVAL).

#  Caution: Converting non-well-formed "RPN objects" into algebraics

will either result in an 'Invalid Expression' or 'UNKNOWN' on the

stack, which will evaluate the same as the list before conversion; or

in Memory Clear (in extreme cases).  For safety's sake, only use lists

containing ->RPN outputs, or similar.  Things like { << 5 >> } are not

meaningful and tempt Murphy's Law.

������������������������������������������������������������������������������