(Comp.sys.hp48)

Item: 2543 by sourada@iastate.edu [Steven D Ourada]

Subj: Tricorder simulator

Date: 31 Dec 1992

[Note: This goofy program isn't really a tune or sound effect, but try

it anyway.  This can be used to test the gullibility of the natives.

Make up some outrageous story about what it's doing, and I'll bet you

a nickle that most people will believe it!  -jkh-]

Copyright 1992 Steven Ourada   Freeware

TRICORDR

========

What it is

----------

This is a silly little program to simulate the 'Tricorder' devices used on

Star Trek.

How to use it

-------------

To use TRICORDR, download it into a convenient directory, take off the

plastic cover over the infra-red pair on your 48, and execute TRICORDR.

When you bring the front of the 48 close to some object, the speaker will

repeatedly beep. Wow, lot's of fun, eh?

When the excitement dulls, press a key to get out of the program.

How it works

------------

Nothing too complicated: a small assembly language code segment pulses

the IR emitter, and checks the detector to see if there is a

reflection. It reports this back to the user-RPL code that beeps if

there was a reflection.

Don't be worried that this will drain your batteries; the IR LED is

only on for about 20 microseconds for each 10 milliseconds.

I disclaim it

-------------

This is Freeware, copy it freely as long as you give credit where it's due.

There is NO WARRANTY with this software as it is provided to you with no

charge.

That's it

---------

Keep in mind that this is my first attempt at using Saturn assembly and my

first HP48 program to be posted to Usenet. :-) If you have any comments,

please mail me.

--------------- Cut here. Assembly source follows. ------------------

; Copyright 1992 Steven Ourada    Freeware

; Embedded assembly code to do "proximity check"

; Turn the IR on, check to see if it reflected, turn it off

include class.mac

rpl $02dcc              ; DOCODE

pgmbeg

rpl pgmend-pgmbeg       ; length of code

jsr save_regs

move.a #$11a,d1

move.1 (d1),a

bclr #0,a

move.1 a,(d1)          ; clear IR detector bit

move.a #$11c,d0

move.1 (d0),a

bset #3,a

move.b a,c

bclr #3,c

move.1 a,(d0)          ; turn IR emitter on

move.1 (d1),a          ; read IR detector

move.1 c,(d0)          ; turn IR emitter off (it's only on for ~20us)

bbs #0,a,reflec

noreflec

move.a #"0",d1        ; return a "0" in the string

exg.a d1,c

bra.3 stostat

reflec

move.a #"1",d1      ; return a "1" in the string

exg.a d1,c

stostat

push

jsr restore_regs

pop

move.a (d1),a         ; assuming a string on the top of the stack

move.a a,d1

add.a #$a,d1

move.b c,(d1)

exit

jsr restore_regs

move.a (d0),a

add.a #5,d0

jmp (a)

pgmend

rpl $0312b              ; SEMI

include class.sym

-------------------- Cut here. User RPL source follows. ---------------

@ Copyright 1992 Steven Ourada    Freeware

@ 'Tricorder' simulator

@ Note that the assembly code must be inserted where shown for this program

@  to work properly.

\<<

"0"

DO

@  Assembly code inserted here

IF DUP "1" SAME THEN

200 .03 BEEP

END

UNTIL KEY

END

DROP DROP

\>>

--

-------

Steven Ourada -- sourada@iastate.edu