(Comp.sys.hp48)

Item: 3867 by jamesu@anacad.fr [James Unterburger]

Subj: HP48 floating point ML suggestions

Date: 23 Feb 1994

[Note: The JAZZ library's assembler recognizes Inf (infinity) and NaN

(Not a Number) as valid objects. These were described in the HP-71

literature ten years ago, and were never mentioned again... until this

article. Thanks, Jim! -jkh-]

Here is what I think I know about HP48 (and HP71B) floating

point.  If I'm wrong in any detail, please let me know.

The entry point addresses are for the HP48SX-E, but are

supported entry points, so should be the same on any HP48

version.

A "split" floating point number

is held in two registers.  I call this a "15-form" number

(throwback to HP71B terminology).  It is obtained by calling

SPLITA (#2BC4Ah) with a "12-form" number in A.W.  Or if you

have two 12-forms, one in A.W and the other in C.W, you can

call SPLTAC (#2BCA0h), which returns two 15-forms, in A/B and

C/D (the "/" does _not_ mean "division", it is simply notation

showing the location of the 15-form split extended real).

Each of these routines returns in DEC mode.  If you need

to know how a 15-form number is stored in the two registers

it occupies, it's like this (X is don't care, M is mantissa

digit, E is exponent digit, S is sign nibble):

12-form:  SMMMMMMMMMMMMEEE

15-form:  SXXXXXXXXXXEEEEE

XMMMMMMMMMMMMMMM

Notice that you get 15 mantissa digits (hence the term 15-form)

for an extended real.  When you first split a 12-form into

a 15-form, the extra three digits will be zeros.  The SPLITA

and SPLTAC routines also check for "funny" inputs in that

Inf and NaN (not obtainable on the HP48 from user-level

operations) are handled.  For Inf and NaN inputs,

Carry will be set upon return.  Otherwise, Carry is clear.

Also, the numbers are "normalized"

meaning that if the first mantissa digit is zero, the mantissa

is shifted left (and exponent adjusted accordingly) until the

first mantissa digit is non-zero.

To add two real numbers in ML, the routine RADDF is the

one that I use.  It expects two 15-form extended precision

reals in A/B and C/D and returns the sum in A/B.  It uses

the D0 register (and ABCD obviously).  It should be called

with the mode set to DEC (SETDEC instruction).  To subtract

C/D from A/B, simply issue the C=-C-1  S instruction (in DEC

mode) before calling RADDF.  RADDF is at #2B7CAh.

To multiply A/B by C/D, call MULTF (#2B91Eh).  It does not

use the D0 register, thankfully.

To divide A/B by C/D, call DIVF (#2B977h).  It doesn't use

D0 either.  Special cases (like Inf and NaN) are trapped

partially I believe, but can't be sure.  A better

(unsupported unfortunately) entry point is ((=DIVF)-6)

which is the same as DIVF except that Inf and NaN are checked

for as inputs as well.  (NaN can be considered the "Undefined

Result")

As a matter of fact, RADDF, MULTF, and DIVF all have, 6 nibbles

before them, the old HP71B code which checks for Inf and NaN

as inputs and handles accordingly.  Generally, this type

of exceptions causes the [XM] bit of the hardware status

to be set.  P==3 for Inf and P==4 for NaN as well.  If you

want to know more about these hidden HP71B remnants, read my ML

code for the TRID (tridiagonal matrix system solver) program which

makes full use of them.  If you want a copy, just let me know.

jamesu@anacad.fr

James Unterburger