JAZZ V4.0

System RPL and Machine Language Development Library

(c) 1995 by Mika Heiskanen & Jan Brittenson

CONTENTS

========

1. Introduction

1.1 Copyrights & Ackowledgements

1.2 The Jazz Library

1.3 Installing & Deleting the Library

2. Jazz Commands

2.1 The System RPL/Machine Language Assembler

2.2 The System RPL/Machine Language Disassembler

2.3 The System RPL Debugger

2.4 The Machine Language Debugger

2.5 The System RPL Stack

2.6 The Entries Catalog

2.7 The System RPL/Machine Language Editor

2.7.1 The Viewer

2.7.2 The Small Font

2.7.3 The Medium Font

2.8 Entries Table Utilities

1. Introduction

===============

1.1 Copyrights & Acknowledgements

---------------------------------

All files of the Jazz library are copyrighted (c) by Mika Heiskanen

unless otherwise noted.

The Jazz library is distributed in the public domain in the hope that it will

be useful, but is provided 'as is' and is subject to change without notice.

No warranty of any kind is made with regard to the software or documentation.

The author shall not be liable for for any error for incidental or

consequential damages in connection with the software and the documentation.

So there.

Permission to copy the whole, unmodified Jazz package is granted provided

that the copies are not made or distributed for resale (excepting nominal

copying fees).

Extra credits & acknowledgements:

Jan Brittenson		DB program is originally from Jan's

MLDL library and is still copyrighted by him.

The mnemonics have been changed to the ones

used by HP + some minor changes has been made.

Mario Mikocevic 	The small 4x6 font + the basis for the

machine language instruction disassembler.

Will Laughlin		Backward search in ED

Rick Grevelle		The medium 6x8 font.

Detlef Mueller		Inspiration through RPL48 package.

& Raymond Hellstern

Dan Kirkland		Sorting the default tables sensibly

Jens Kerle		Bug fixing

Cary McCallister	For answering.

Beta testing & suggestions:

Seth Arnold		Bill Levenson

Douglas Cannon		Tom van Migem

Carlos Ferraro		Mario Mikocevic

Rick Grevelle		Detlef Mueller

Joe Horn		Richard Steventon

Boris Ivanovich 	Kurt Vercauteren

Jens Kerle		Vladimir Vukicevic

Dan Kirkland		Christ van Willegen

Jeoff Krontz		Stefan Wolfrum

Will Laughlin

+anyone else who I may have forgotten

1.2 The Jazz Library

--------------------

The Jazz library provides commands for assembling, disassembling and

debugging both system rpl and machine language. This document describes only

the provided commands, not the languages themselves. For information on the

languages please refer to the tools package published by HP, especially the

files RPLMAN.DOC, RPLCOMP.DOC and SASM.DOC. Familiarity in the fundamentals

is assumed from now on.

Following files available from hpcvbbs.external.hp.com are recommended:

dist/ms-dos/tools.exe				HP Tools

dist/hp48g/programming/entries/ent_srt.zip	Sorted Entries

dist/hp48s/programming/entries/entries.zip	Address sorted entries

dist/unix/sadhp105.zip				Unix Disassembler

GNU Tools are available from srcm1.zems.fer.hr (IP 161.53.64.254)

pub/hp48/tools2.0.4.zip

1.3 Installing & Deleting the Library

---------------------------------------

To install the Jazz library you need atleast:

- GX with sufficient RAM or existing RAM card in slot 1

- S/G with sufficient expanded RAM

- SX with existing RAM card in slot 1

The Jazz commands will not work from a covered port. Thus the GX banks 2-33

are not allowed storage locations for Jazz.

To install the library to port 0: (similar for port 1)

a) Remove possible older version with :0:992 PURGE

b) Download the library to your HP48

c) Recall the library to the stack and purge the variable

created by the download procedure.

d) 0 STO

e) Execute OFF-ON keys or do ON-C

[Note: Also copy HPTABS.LIB to your HP48 for speedy assembly & disassembly.

Then skip the following paragraphs & continue at "Jazz Commands". -jkh-]

Jazz enables using two different entries tables:

RPL.TAB 	- Entries sorted by their name

DIS.TAB 	- Accessory to RPL.TAB providing address sort

Neither table is absolutely necessary but atleast RPL.TAB is recommended,

DIS.TAB also if any disassembling is to be done. For speed you can store

RPL.TAB to romptr 993 0 and DIS.TAB to romptr 993 1. The tables can be stored

in covered ports, but it will slow down all commands using the tables by the

time it takes to fetch the tables, and the memory available to the commands

is lessened by the memory taken by the tables.

A default entries table is provided along with the table. It is built from

the latest published SupRomEntr.a, with some sorting & deletions done by Dan

Kirkland to produce the most sensible names in disassembly. Also Dan added

the necessary user-rpl commands to avoid problems regarding the double hashed

command names.

gentab2.c is a modified version of Detlef Mueller's gentab.c. The modified

version will build library source code given an entries table as input.

gentab2 also takes preferred entries in disassembly into account by ordering

the names for a given address based on their order in the original input.

Syntax: gentab2 < SupRomEntr.a > table.a

For users convenience the latest table is distributed along with the package

under the name hpentr.a.  [See STUFF.DOC on this disk.	-jkh-]

2. Jazz Commands

================

2.1 The System RPL/Machine Language Assembler

---------------------------------------------

As opposed to the tools provided by HP Jazz provides only one

command to assemble source code. The assembler assumes the source

code to be RPL unless a switch is made via special tokens.

Command:	ASS

Stack:		( $ --> ob )

Description:	Assemble source string

Keys:		ON key aborts assembly

User flags:

1 - Report mode on (slows down assembly considerably!)

Errors:

Special error trap simulation is used to enable using

the small font for error messages and thus get more

information of the reason for the error.

For the small font to be used the current program (possibly

the kernel) must use SysErrorTrap to trap errors. If the

trap found is different error handling is left to the found

error trap and extra information of the reasons for the

error will be lost, only the actual error number is provided

for the trap.

For programmers conveniece also error traps that start with

":: NOP" cause showing the full error message.

When possible ASS will also output the error position to the

stack so that you can immediately edit the error line/token.

Display when the error is trapped:

Top of display

+----------------------------+

|ErrorMsg	line/position|

|Token/Source line	     |

Comments in RPL mode are:

"*" at the start of the line marks the entire line to be a comment.

"(anything)" surrounded by whitespace is considered a comment.

RPL Assembly Mode Tokens

------------------------

;		--> SEMI

{		--> DOLIST

}		--> SEMI

::		--> DOCOL

SYMBOL		--> DOSYMB

UNIT		--> DOEXT

# hhhhh 		--> system binary

#hhhhh			--> system binary (from ROM if possible)

ddddd			--> system binary

PTR hhhhh		--> pointer

ACPTR hhhhh hhhhh	--> access pointer, G/GX only

ROMPTR hhh hhh		--> rom pointer object

ddd.dd			--> real number

% ddd			--> real number 		(*)

%% ddd			--> long real number		(*)

C% ddd ddd		--> complex number		(*)

C%% ddd ddd		--> long complex number 	(*)

(*) ddd can also be -Inf, Inf or NaN

HXS	<len> <hh.h>	--> hex number

GROB	<len> <hh.h>	--> grob

LIBDAT	<len> <hh.h>	--> library data

BAK	<len> <hh.h>	--> backup object

LIB	<len> <hh.h>	--> library object

EXT1	<len> <hh.h>	--> external type 1, S/SX only

EXT2	<len> <hh.h>	--> external type 2

EXT3	<len> <hh.h>	--> external type 3

EXT4	<len> <hh.h>	--> external type 4

ARRY	<len> <hh.h>	--> array

LNKARRY <len> <hh.h>	--> linked array

CODE	<len> <hh.h>	--> code object

NIBB	<len> <hh.h>	--> misc nibbles

If <len> is zero then <hh.h> the resulting object will consist

of the prolog indicated by the first token and "00005" lenght field.

(NIBB indicates no prolog so len must be non-zero)

$ "<string>"            --> string

"<string>"              --> string

ID <string>		--> identifier object

LAM <string>		--> lambda identifier object

TAG <string> <..>	--> tagged object

CHR <char>		--> character object

xROMWORD		--> pointer or ROMPTR depending on the

library number the command belongs to

( For example "xDUP" compiles to a

pointer but "xASS" to a rom pointer)

INCLOB <name>		--> Include object stored to a variable

INCLUDE <name>		--> Include source code

DEFINE <token> <string> --> Define substitute for token

Substitution will not be done inside

multipart tokens, for example

"ID <token>" is not allowed

CODE <newline>		--> Starts machine language assembly

ASSEMBLE		--> Starts machine language assembly

ML Assembly Mode Mnemonics

--------------------------

Opcodes recognized in machine language assembly are all the normal

opcodes indicated by SASM.DOC plus the next new ones as implemented

in GNU Tools:

LCSTR	\ASCII\ 	Reversed LCASC

LASTR	\ASCII\ 	Reversed LAASC

CSTRING \ASCII\ 	NIBASC with 0-byte terminator

ABASE	expr		Sets allocation counter to address

specified by <expr>.

label	ALLOC	expr		Allocates <expr> nibbles for label

at the allocation counter, then

increases allocation counter by <expr>

MAKEROM

-------

ASS can be used to assemble MAKEROM source code with the following

tokens:

xROMID #hhh		Defines hex library number.

xROMID dec		Defines decimal library number

xTITLE <title>		Defines title to be rest of the line.

If title is missing then nulltitle is used.

xCONFIG <label> 	Defines the location of the configuration

object via a label. If label is missing then

no config is taken to exist.

xMESSAGE <label>	Defines the location of the message table

via a label. If label is missing then no

message table is taken to exist.

EXTERNAL <label>	Defines label to be external. Order of

introduction determines the command number

of the commands in the library so that

visible commands will be first, then

nullnames.

xNAME <label>		Specifies location of a visible command.

Name: "xlabel"  Hash: "label"

sNAME <label> <hash>	Specifies location of a visible command.

Name: "label"   Hash: "hash"

hNAME <label>		Specifies location of a 'visible' command.

Name: "label"   Hash: null

NULLNAME <label>	Specifies location of a hidden command.

Name: "label"   Hash: none

tNAME <label> <hash>	Specifies secondary hash for command.

xROMID and xTITLE must be used at the start of the source code,

to be specific before any actual code has been output. Also xROMID

must come before any other MAKEROM token.

xCONFIG and xMESSAGE declarations can be anywhere (or absent), but a

suitable location is after the xROMID and xTITLE declarations.

EXTERNAL declaration is needed if the corresponding command is used

before its location is specified by its NAME declaration. Suitable

location is after the header declarations, and it is probably best to

declare all commands.

xNAME, sNAME, hNAME and NULLNAME specify command location, thus they

should be right in front of the object they define as a command. All

but NULLNAME also define the romid/cmd header field properly and thus

require a propfield, typically the value 8 to mark a regular command

and 000 to mark a regular function, for other values please refer to

entries.srt or other documents. Above also define a symbol '~label'

having a 6 nibble value, low 3 nibbles containing the romid and high

3 nibbles the command number.

The internal menu display routines stop showing library menus if a

command with no hash is found. Thus any command declared after a

hNAME will not be shown in the library menu, but will of course have

typable/disassemblable command names as usual if so specified by

NAME tokens.

tNAME can be used anywhere after the declaration of the romp the

secondary hash is assigned to, a suitable location is right after the

corresponding NAME location declaration. Note that tNAME can be used

to declare names for NULLNAMEs, thus providing easy access to low

level subroutines if needed. One command can have several secondary

names.

Note that INCLOB does not do any ID --> ROMP conversion work on the

included object like the common DIR --> LIB library builders do.

Example: Jazz MAKEROM source would start like this:

xROMID	992

xTITLE	Jazz v4.0 Fin'95 10.06.95 mheiskan@gamma.hut.fi

xCONFIG JazzCfg

xMESSAGE JazzMsg

EXTERNAL xFNT1		( Fonts always come first in Jazz )

EXTERNAL xFNT2

EXTERNAL xASS		( User ASS command )

EXTERNAL Assemble	( Main assembler code object )

[..]

EXTERNAL UnShowSel!	( Low level subroutines of SSTK )

EXTERNAL >SelPict!

LABEL JazzCfg		( Configuration object )

:: 992 TOSRRP ;

LABEL JazzMsg		( Message table )

ARRY hhhhh hhh..h

NIBB 1 8		( Easier than ASSEMBLE CON(1) 8 RPL )

xNAME ASS		( Note: the EXTERNAL declarations )

:: CK1 ... ;		( declared fonts to come before ASS )

[..]			( when assigning command numbers )

Missing Tokens and Mnemonics

----------------------------

The following ones for various reasons do not behave as documented in

SASM.DOC or GNU Tools:

Not implemented:

IF, ELSE, ENDIF + any other conditional assembly mnemonic

MACRO		\

ENDM			Macros not implemented

EXITM		/

CLRFLAG expr	\	Flags not implemented

SETFLAG expt	/

ABS	expr		Not Implemented

RDSYMB	file		..

CHARMAP file		..

Dn=HEX	hh.h		..

GOSHORT label		..

JUMP	label		..

INC(n)	label		..

LINK	label		..

SLINK	label		..

NIBBIN	bb..b		..	(GNU Tools Opcodes)

NIBGRB	bb..b		..

HEX(n)	hh.h		..

HEXM(n) hh.h		..

ASC(n)	\ASCII\ 	..

ASCM(n) \ASCII\ 	..

Modified Tokens and Mnemonics

-----------------------------

Behaviour changed:

TITLE	text		Text shown on line 1, line 2 cleared

STITLE	text		Text shown on line 2

MESSAGE text		Text shown on line 1, line 2 cleared

D0=D0+	expr		\

D0=D0-	expr		 Allow values between 1 - 256, and

D1=D1+	expr		 generate multiple opcodes if needed.

D1=D1-	expr		/

Ignored:

EJECT, REL, LIST, LISTM, LISTALL, UNLIST

Expressions in machine language

-------------------------------

Factors:

#hh.h		hex integer

%bb.b		binary integer

dd.d		decimal integer

=symbol 	global symbol (RPL.TAB checked too)

:symbol 	local symbol

symbol		local symbol

*		PC counter

+ - ++ --	Local labels

Operators:	Priority:

^	9

*	8

/	8

%	8	(modulo)

+	7

-	7

&	5	(and)

!	4	(or)

Note in particular that ascii factors are not implemented.

As in sasm all symbols must be surrounded with parentheses

when operators are used.

Label generation in machine language

------------------------------------

Symbols +, ++, - and -- are location dependant symbols

and refer to the next/previous defined value of the

corresponding symbol. Example:

GOSUB	+	---+

CSTRING 'Foobar'   |

+	C=RSTK		<--+

D0=C

-	A=DAT0	B	<-------+

DAT1=A	B		|

?A=0	B		|

GOYES	+	---+	|	End of string, done

D0=D0+	2	   |	|

D1=D1+	2	   |	|

GONC	-	---|----+

+	...		<--+

Note: Symbol ++ does not refer to the 2nd + coming up, but

the next ++ label coming up, and similarly for --.

INCLOB Special Features

-----------------------

INCLOB will behave differently from code than from rpl. Depending on

the type of the object being included it is either included entirely

or the leading nibbles (prolog + possible data fields) are skipped for

following object types:

Prolog	Skip	Prolog	Skip	Prolog	Skip

------	----	------	----	------	----

DOCODE	10	DOEXT0	10	DOGROB	20

DOCSTR	10	DOEXT2	10	DOARRY	5

DOHSTR	10	DOEXT3	10

DOEXT4	10

Warnings

--------

A=A+CON fs,expr and similar instructions allow single nibble fields

(WP,P,S,XS) while SASM errors. These instructions behave badly due to

a hardware bug but are supported by ASS for those who know how to

safely use the commands.

Symbol/label handling is quite secure but allows more than SASM.

For example external values have no significance, thus for example

(=GETPTR)-(=SAVPTR) is valid in Jazz. Relative values are followed

and are significant for most opcodes. For example 'D0=D0+ label'

will error. Note especially that (label1)-(label2) is absolute.

= EQU and ALLOC require their expression fields to be resolvable on

the first pass.

2.2 The System RPL/Machine Language Disassembler

------------------------------------------------

Several disassembler commands are provided for different purposes.

Using DIS.TAB speeds up the commands considerably.

Command:	DIS

Stack:		( ob --> $ )

Description:	Disassemble object. If stk1 is a pointer to a ROM

address only the pointer will be disassembled.

Command:	DISXY

Stack:		( hxs_address hxs_end_address --> $ )

Description:	Disassemble memory area.

Guesses start mode, switches mode during

operation if necessary.

Command:	DOB

Stack:		( ob | #address | hxs_address | "entry" --> $ )

Description:	Disassemble memory area.

Guess start mode and end address.

Command:	DISN

Stack:		( hxs_address %N --> $ )

Disassemble memory area as machine language only.

Common features for the disassembler commands:

ON key aborts disassembly.

User flags:

2 - disable guess mode

4 - disable machine language disassembly for DIS

5 - disable tabulator, use spaces instead

6 - force generated labels on their own rows

In guess mode the disassembler will try to guess data structures

embedded in machine language. Currently only the following types

are recognized:

GOSUB	+

REL(5)	+		Optional leading size indicator

BSS	expr		Data is all zeros

+	C=RSTK

GOSUB	+

REL(5)	+		Optional leading size indicator

NIBASC	\ASCII\ 	\ Possibly alternating and spanning

CSTRING \ASCII\ 	/ multiple lines.

+	C=RSTK

GOSUB	+

REL(5)	+		Optional leading size indicator

NIBHEX	hh.h		Miscellanoues data

+	C=RSTK

The sufficient condition for an ascii guess to be successful is that

the data area should consist mostly (75%) of common ascii characters.

The ascii lines are splitted to CSTRINGs or by newline characters

or so that the maximum lenght will be 40 chars.

Warnings:

- DOB, DISXY and DISN disassemble areas of memory instead of well

defined objects. Using these commands to disassemble memory in

the temporary object area is dangerous since a possible garbage

collection during run-time can move the memory being disassembled.

Use only DIS to disassemble objects in tempob!!!

- Composite history is tracked up to 64 levels. If that is exceeded

a ";" may be output when "}" is due.

- As opposed to the assembler the disassembler cannot handle hidden

hash tables, thus some named ROMPTRs will be disassembled to

"ROMPTR xxx yyy".

- Label values are guessed for Dn=(2) and Dn=(4) instructions if

- Dn=(2) is likely to refer to the IO page

- Dn=(4) is likely to refer to a RAM variable

This works well for disassembling ROM but doesn't does badly when

disassembling a program that uses even-page method in its data

allocation. Benefits are clearly greater though.

2.3 The System RPL Debugger

===========================

Command:	SDB

Stack:		( seco | id | lam | romp --> ? )

Description:	Start debugging program indicated by stack level 1.

If the srpl debugger is already running then SDB command will

only show the SDB menu:

->SST		- Single step next command

If right-shifted then single-steps rest of

the stream as a single unit

->IN		- If possible then enter the program referred to

by the next command, else single step command.

SNXT		- Show next commands on status area

Pressed for the second time shows return stack

SST->		- Start continuous ->SST mode, subsequent presses

toggle slow/fast mode, eg whether stack display

is updated after each command or not.

Any other key aborts continuous evaluation

IN->		- Start continuous ->IN mode.

DB		- Start DB on next code object

xKILL		- The HP48 KILL command

(xCONT can be evaluated through LS+ON keys)

SKIP		- Skip next command. If right-shifted then skips

rest of current stream, eg executes a SEMI command.

SEXEC		- Execute stk1 as the 'next' command.

SBRK		- Set breakpoint object to STK1. If right-shifted

clears breakpoint object.

LOOPS		- Browse loop environments. Up/Down to scroll, any

other key to exit.

If right-shifted dumps topmost environment to the

stack.

LAMS		- Browse lam environments. Up/Down to scroll lams,

left/right to decrease/increase environment. Any

other key to exit.

If right-shifted dumps topmost environment to the

stack.

IN?		- Toggle ->IN mode to never enter into secondaries,

only into IDs/LAMs/ROMPTRs when allowed. Prevents

the debugger from entering into ROM subroutines

during continuous debugging.

Note that SDB is meant for debugging system rpl, not user rpl.

Thus some user rpl commands will not be single stepped right when

using SDB. One example is xHALT, for which the substitute xSHALT

is provided. Note that SDB must be running before SHALT works.

Warnings:

Debugging system-rpl is very hairy and undoubtedly

SDB cannot debug some lesser known commands correctly.

If such commands are found SDB can even cause a crash

and memory loss. This is unfortunately unavoidable since

there really is too much code in ROM to worry about.

SDB should manage to debug all normal programs though.

SDB either enters commands or executes commands (by emulation

if necessary). None of the interactive commands in HP48 ROM

are emulated, most importantly PolOuterLoop. To emulate

POLOuterLoop you need to insert SHALT commands into the

display objects or whatever you want to debug, then start

SDB, then use CONT to reach the point of the SHALT command.

LOOPS and LAMS displays are pretty lame, I'll try to improve them

later.

2.4 The Machine Language Debugger

---------------------------------

Command:	DB

Stack:		( id --> )

( romp --> )

( $entry --> )

( #address --> )

( hxs_address --> )

( code --> )

Description:	Debug machine language

Screen Keys:

[A]	- Screen 1 (general registers)

[B]	- Screen 2 (registers A-D)

[C]	- Screen 3 (registers R0-R4)

[D]	- Screen 4 (RSTK)

[E]	- Screen 5 (memory dump)

[F]	- Screen 6 (machine language disassembly)

[MTH]	- Screen 7 (breakpoints)

[VAR]	- Screen 8 (watchpoints)

[ ]	- Update display

[EEX]	- View PICT (if it exists) as long as EEX is down

Arguments:

[0]	- Start inputting argument.

[0-9A-F]	add digit

[DEL] 	abort input

[BS]		delete last digit

[+/-] 	negate arg

Movement keys:

[NXT]	- Skip instruction	(or ARG instructions)

[left]	- PC=PC-1		(or -ARG)

[right] - PC=PC+1		(or +ARG)

[up]	- PC=PC-16		(or -16*ARG)

[down]	- PC=PC+16		(or +16*ARG)

[.]	- Set mark to PC	(or to ARG)

[+/-]	- Swap PC and mark

[ENTER] - If ARG then set PC = ARG

Debug keys:

[+]	- Single step (ARG) instructions

[-]	- Single step (ARG) instructions,

debug GOSUBs as a single instruction

[*]	- Single step (ARG) instructions with display update.

If no ARG then sets ARG to #FFFFF.

[/]	- Single step (ARG) instructions with display update,

debug GOSUBs as a single instruction.

If no ARG then sets ARG to #FFFFF.

[EVAL]	- Continue until end or breakpoint

[SIN]	- Save current registers

[COS]	- Save registers with the saved ones

(first save is done at startup)

ARG + [SIN]	= clear cycle counter 1

ARG + [COS]	= clear cycle counter 2

Exit keys:

[DEL]	- Restore registers & exit

[BS]	- Exit now

[1/x]	- Exit via reset, press second time to confirm

Breakpoints:

[PRG]	- Set breakpoint to ARG

[STO]	- Set breakpoint counter to ARG

Options:

[CST]	- Toggle option number (next key)

3	- Ascii/hex mode

4	- Shift memory dump by 1

5	- Automatic switch between PICT/ABUFF

during debug

6	- Disable/enable RPL.TAB and DIS.TAB

7	- Opcode/cycles display

Sample screens which can be reasonably reproduced by:

"#>HXS" DB      (or #59CCh DB)

Screen 1  - General CPU State (key [A])

Mnemonic............................ |GOSUBL  SAVPTR	    |

Opcode.............................. | 8E4CD0		    |

PC, P, Carry, Hex/Dec mode, ST...... |@:059D1 P:0 CH ST:298 |

A.A and C.A......................... |A:059CC C:BF4F8	    |

B.A, D.A, and HST................... |B:8883E D:0AF58 HST:2 |

D0 and 6 bytes @D0.................. |D0:E8EF8/CCD205700074 |

D1 and 6 bytes @D1.................. |D1:BF4FD/000000000000 |

Top 3 levels of RSTK................ |RST:00000:00000:00000 |

Screen 2  - Arithmetic registers (key [B])

Mnemonic............................ |GOSUBL  SAVPTR	    |

Opcode.............................. | 8E4CD0		    |

PC, P, Carry, Hex/Dec mode, ST...... |@:059D1 P:0 CH ST:298 |

Register A.......................... |A:6C4475C79A7059CC    |

Register B.......................... |B:000000000008883E    |

Register C.......................... |C:36000000077BF4F8    |

Register D.......................... |D:000000000000AF58    |

Top 3 levels of RSTK................ |RST:00000:00000:00000 |

Screen 3  - Data registers (key [C])

Mnemonic............................ |GOSUBL  SAVPTR	    |

Opcode.............................. | 8E4CD0		    |

PC, P, Carry, Hex/Dec mode, ST...... |@:059D1 P:0 CH ST:298 |

Register R0......................... |R0:60000000000409C1   |

Register R1......................... |R1:6C4475C79A7059D1   |

Register R2......................... |R2:6C4475C79A7DE599   |

Register R3......................... |R3:6C4475C79A700000   |

Register R4......................... |R4:10000000004BFA18   |

Top 3 levels of RSTK................ |RST:00000:00000:00000 |

Screen 4  - Return stack (key [D])

Mnemonic............................ |GOSUBL  SAVPTR	    |

Opcode.............................. | 8E4CD0		    |

PC, P, Carry, Hex/Dec mode, ST...... |@:059D1 P:0 CH ST:298 |

|			    |

RSTK levels 0 and 4................. |RST0:00000 RST4:00000 |

RSTK levels 1 and 5................. |RST1:00000 RST5:00000 |

RSTK levels 2 and 6................. |RST2:00000 RST6:00000 |

RSTK levels 3 and 7................. |RST3:00000 RST7:00000 |

Screen 5  - Memory dump (key [E])

Locations 59A0-59AF................. |059A0:56113680913420CC|

Locations 59B0-59BF................. |059B0:4E0156716FCC56FD|

Locations 59C0-59CF................. |059C0:015B38D5E0101D95|

Locations 59D0-59DF................. |059D0:08E4CD08E46C0101|

Locations 59E0-59EF................. |059E0:D230574911191443|

Locations 59F0-59FF................. |059F0:4E4A201101311456|

Locations 5A00-5A0F................. |05A00:12280A50143174E7|

Locations 5A10-5A1F................. |05A10:8E58D01311741431|

current location is indicated by an inverse digit.

Screen 6  - ML Instruction Stream (key [F])

PC, P, Carry, Hex/Dec mode, ST...... |@:059D1 P:0 CH ST:218 |

Next 7 instructions................. | D1: GOSUBL  SAVPTR   |

| D7: GOSUBL  POP#     |

| DD: R1=A 	    |

| E0: C=0	   A	    |

| E2: LC(1)   5	    |

| E5: GOSUB   MAKE$N   |

| E9: C=R1 	    |

The next instruction is the one displayed in reverse. Currently,

it will always appear at the top. (Here somewhat crudely

illustrated with the # signs.)

Screen 7  - Breakpoint Table Screen (key MTH)

Breakpoint #1....................... | 1:6100 +02	    |

Breakpoint #2....................... | 2:6104 -02	    |

Breakpoint #3....................... | 3:613A  00	    |

Breakpoints #4-#8: not used......... | 4:0000  00	    |

| 5:0000  00	    |

| 6:0000  00	    |

| 7:0000  00	    |

| 8:0000  00	    |

Any breakpoints at the current location are displayed in reverse.

Warning:

DB uses D0 for its own purposes, thus you cannot debug

code that modifies the rpl return stack nor the current

stream.

2.5 The System RPL Stack

------------------------

The SSTK command starts a new kernel which is a modified version of

the internal one. To exit SSTK just execute SSTK again.

Modifications to the internal kernel:

- Stack has 5 lines, including the interactive stack.

- Flag 3 toggles the stack decompiler:

Set:	Use internal decompiler

Clear:	Use a system rpl decompiler

- Multi-line mode is not supported.

2.6 The Entries Catalog

-----------------------

EC command is a browser for the entry tables. Since the entries are

listed in address sorted form both RPL.TAB and DIS.TAB are needed

to run EC.

Keys are:

Up Arrow	- Up one entry

Down Arrow	- Down one entry

LS + Up Arrow	- Up one page

LS + Down Arrow - Down one entry

RS + Up Arrow	- Jump to first entry

RS + Down Arrow - Jump to last entry

Right Arrow	- View the contents for selected entry

with VV

Alpha		- Input find string (entry name grep)

F		- Input find string (entry name grep)

NXT		- Find next match

LS + NXT	- Find previous match

EEX		- Toggle grep mode (show only matches)

ENTER		- Push entry to stack as :name:address

LS + ENTER	- Push entry address to stack

RS + ENTER	- Push entry name to stack

0-9		- Find entry starting with input address

Use 0-9A-F to input a more specific

address.

ON		- Exit browser

+/-		- Toggle beep on/off

[Note: Some users have reported warmstarts in EC; the keystroke

routine seems to conflict with some ROM versions, especially R.

The time and date usually get lost during these warmstarts too.

So don't leave important stuff on the stack when using EC. -jkh-]

2.7 The System RPL/Machine Language Editor

------------------------------------------

ED is an editor intended for editing rpl and machine language source

code. ED makes no duplicate of the edited string if it is in

temporary object area, thus enabling editing very large strings.

Note that this implies that no backup of the original string is kept!

Note that ED is very fast but since it supports the tabulator

it has to do special calculations whenever the display is scrolled.

Thus scrolling the display when very long lines are present can be

quite slow.

As a hopefully useful feature ED will accept an optional cursor

position argument on stack level one. Thus if ASS gives you the error

position you will be able to jump to that position immediately.

Most of the normal character keys are in their normal places, others

can be fetched via the special character browser.

ED also allows an alternate 4x6 font to exist in variable 'FONT.ED'.

No checks are made on the correct format, but obviously the just

starting ED is an easy way to see if there is a problem.  [Note: the

FONT.ED file on this disk is a good replacement font. Just copy it to

your HOME directory. Beware: ED uses it, but VV does not. -jkh-]

Special keys having different definitions are mostly in the

non-alpha plane. The NS,LS,RS planes are defined as follows:

+-------+-------+-------+-------+-------+-------+

|BSTART |BEND	|BCOPY	|BDEL	|	|FIND	|

|	|	|	|	|	|REPL?	|

|	|	|	|	|	|REPLALL|

+-------+-------+-------+-------+-------+-------+

|ARG?	|	|MEXEC	|REVERSE|UP	|NEXT	|

|ROW?	|CHR?	|MSTART |TOHEX	|PGUP	|PREV	|

|POS?	|CHRCAT |MEND	|TOASC	|TOP	|STATUS |

+-------+-------+-------+-------+-------+-------+

|''     |STK    |       |LEFT   |DOWN   |RIGHT  |

|	|	|	|	|PGDN	|	|

|	|RCLSTK1|	|LSTART |BOTTOM |LEND	|

+-------+-------+-------+-------+-------+-------+

|GOTO	|GOSUB	|GOYES	|	|  ^	|DFIND	|

|GOLONG |GOSUBL |GONC	|	|CNTRINI|	|

|GOVLNG |GOSBVL |GOC	|	|CNTR	|	|

+-------+-------+-------+-------+-------+-------+

|EXIT	|	|TOGBEEP|DEL	|BS	|

|	|ASS	|TOGCASE|DELLINE|	|

|	|DOB	|TOGOVER|DELRGHT|	|

+-------+-------+-------+-------+-------+

|alpha	|7	|8	|9	|/	|

|	|SETMK7 |SETMK8 |SETMK9 |()	|

|	|GOMK7	|GOMK8	|GOMK9	|#	|

+-------+-------+-------+-------+-------+

|lshift |4	|5	|6	|*	|

|	|SETMK4 |SETMK5 |SETMK6 |[]	|

|	|GOMK4	|GOMK5	|GOMK6	|ASS_RPL|

+-------+-------+-------+-------+-------+

|rshift |1	|2	|3	|-	|

|	|SETMK1 |SETMK2 |SETMK3 |<<>>	|

|	|GOMK1	|GOMK2	|GOMK3	|""     |

+-------+-------+-------+-------+-------+

|REDISP |0	|.	|SPC	|+	|

|	|	|,	|TAB	|{}	|

|OFF	|GOMK0	|NEWLINE|TAB	|::	|

+-------+-------+-------+-------+-------+

Explanations:

TOGBEEP - Toggle beep on/off. Default value is taken from the system flag.

TOGCASE - Toggle lower/upper case characters.

TOGOVER - Toggle insert/overwrite mode.

DEL	- Delete character under cursor

DELLINE - Delete line under cursor

DELRGHT - Delete characters to right of cursor.

BS	- Backspace

BSTART	- Set block start address

BEND	- Set block end address

BCOPY	- Copy block/cut to cursor position

BDEL	- Delete block (copied to cut)

RCLSTK1 - Pop string from stk1 into cursor position

FIND	- Incremental search. Search is case sensitive if find string

contains lower case characters.

REPL?	- Find/replace with verification

REPLALL - Replace all

NEXT	- Find next match

PREV	- Find previous match

DFIND	- Find matching delimiter for delimiter under cursor

MSTART	- Start defining macro key sequence

MEND	- End macro key sequence

MEXEC	- Execute macro key

ARG?	- Input repeat count for next key press

ROW?	- Input row to jump to

POS?	- Input position to jump to

CHR?	- Input character number to insert

CHRCAT	- Character browser, ENTER key echos chosen character to cursor

position, ON key exits.

SETMKn	- Set mark <n>

GOMKn	- Jump to mark <n>

GOMK0	- Go to previous cursor position

TOHEX	- Convert block to hex nibbles (Suitable for NIBASC -> NIBHEX)

TOASC	- Convert block to asc nibbles (Suitable for NIBHEX -> NIBASC)

REVERSE - Reverse chars in block/word

ASS	- Assemble source code, if error occurs shows the error message and

after a keypress jumps to the error position.

DOB	- Disassembles entry under cursor using DOB, spans a new editor to

view the disassembly. After exit back to the original editor the

disassembly will be in the clip (if memory allows) ready to be

inserted into the text if so desired. Special cases:

#hhhhh		--> view (like plain entry)

Lhhhhh		--> view

ROMPTR hhh hhh	--> view

PTR hhhhh	--> view

ID name 	--> visit contents	(RCL+DIS+ED+ASS+STO)

INCLOB name	--> visit contents	(RCL+DIS+ED+ASS+STO)

INCLUDE name	--> visit text contents (RCL	+ED    +STO)

GROB hhhhh hh.h --> view grob

STK	- Starts a normal SOL. Recursive EDs are allowed. Exit back to ED

with CONT key. All internal markers except cursor position will be

lost.

CNTRINI - Initialize counter variable. Number of digits used determines width

of counter, possible leading	"#" determines a hex counter.

CNTR	- Insert counter into text and increment it. At start the width is

initialized to 1 hex nibble, so for example pressing

[ARG?] 16 ENTER [CNTR]

will produce "0123456789F"

Special keys during inputline:

ENTER		- Input ok

ON		- Cancel

DEL		- Delete char

BACKSPACE	- Delete previous char

LT/RT		- Move left/right. During find input pressing RT at

the end of input will take the next input char from

the current match location, thus making it easier

to complete the match.

NXT		- Next match during find input

PREV		- Previous match during find input.

Special keys in alpha plane:

A LS -	=	::\n;		(with indent checks)

A RS -	=	$ ""

A LS +	=	{\n}		(with indent checks)

A RS +	=	CODE\nENDCODE

A ENTER =	\n + indent the same way as the previous line

Notes:

Repetition and macro key execution can be aborted with the ON

key. Repetition, macro save and macro execution are aborted

automatically if an error occurs. Max lenght of a macro key sequence

is 50 keys.

2.7.1 The Viewer

----------------

Command:	VV

Stack:		( $|grob --> $|grob )

Description:	Simple string/grov viewer.

Keys when viewing a string:

Up/Down/Left/Right	= scroll display

PRG/STO/'/EVAL          = scroll onedisplay page

F/NXT			= top/bottom

-			= slow scrolling

+			= fast scrolling (default)

ON/ENTER		= exit

Keys when viewing a grob:

Up/Down/Left/Right	= move grob

.			= center grob

ON/ENTER		= exit

A-F			= choose scroll speed 1-6

The grob viewer uses a grob! replacement with automatic

cutting. Masking grobs less than 4 bits wide is not not properly

implemented yet. Viewing is done on the text grob, thus the following

will create a weird effect: :: ABUFF xVV ;

2.7.2 The Small Font

--------------------

The small font is fixed to ROMPTR 3E0 0 under the name FNT1.

The format of the font is in assembly:

CON(5)	=DOEXT0 	 * Library Data

CON(5)	256*6+5 	 * 256 characters, 6 nibbles each

NIBHEX	......		 * Char 00

NIBHEX	......		 * Char 01

...

NIBHEX	......		 * Char FF

2.7.3 The Medium Font

---------------------

The medium font is fixed to ROMPTR 3E0 1 under the name FNT2.

The format of the font is in assembly:

CON(5)	=DOEXT0 	 * Library data

CON(5)	256*16+5	 * 256 characters, 6 nibbles each

NIBHEX	................ * Char 00

NIBHEX	................ * Char 01

...

NIBHEX	................ * Char FF

[Note: there is no way to use FNT2 with ED or VV.  It's just here for

programmers who wish to use it in other applications.  -jkh-]

2.8 Entries Table Utilities

---------------------------

Command:	EA

Stack:		( $entry   --> hxs_addr )

( hxs_addr --> $entry	)

( ob	   --> hxs_addr )

Description:

Converts between entry name and its address.

For other argument types the address of the object

is given

Command:	RTAB

Stack:		( --> $ )

Description:	Recalls RPL.TAB

Command:	DTAB

Stack:		( --> $ )

Description:	Recalls DIS.TAB

Command:	RTB->

Stack:		( --> $ )

Description:	Converts RPL.TAB into readable form

Command:	->RTB

Stack:		( $ --> $' )

Description:	Converts an entry list into RPL.TAB form

Lines accepted are:

[=]name[whitespace][EQU #]address\n

~~~		   ~~~~~~~

optional	   optional

Note that the input should be sorted, no checks for

that are done. Also the last character in the input

string should be a newline character.

Command:	->DTB

Stack:		( --> )

Description:	Creates a DIS.TAB based on RPL.TAB, stores it

to home directory.

Adding a new entry to RPL.TAB:

- Use RTB-> to get an editable string

- Use ED to add the new entry definition, must add it to

its correct position.

- Use ->RTB to get the new RPL.TAB

- Replace the old RPL.TAB with the new one

- Use ->DTB to get a new DIS.TAB

- Replace the old DIS.TAB with the new one

Deleting entries is similar.

Preferred method of editing is to edit the given hpentr.a

and use gentab2 to produce the new tables.

[Actually, I prefer to use the HACK library's L->DIR and

D->LIB to rebuild Library 993 (the RPL.TAB and DIS.TAB

library) manually.  -jkh-]