HP-28S TO HP 48SX TRANSITIONS

by Stephen J. Thomas

This article is a summary of functional differences of HP-28S

operations as implemented on the HP 48SX.  It is not intended to

present all of the many improvements and new commands in the 48, but

is primarily an aid for 28 users who wish to use their previously-

developed programs on the 48.  The comments on any given topic are

not exhaustive, but are quick summaries of operational differences.

Further reference should be made to the appropriate manual(s).

In addition, a number of non-programmable differences are

included, as well as a table showing differences in system flags.

I would appreciate receiving any corrections or additions from

the readers.  My mail and email addresses are listed at the end of

this article.

[l-shft] and [r-shft] refer to the HP 48 orange left-shift and

blue right-shift keys, respectively.  "sigma" represents the Greek

upper-case letter sigma.

GENERAL COMMENTS:

When a special display (message, plot, etc.) is created on the

HP-28S, it usually persists after program completion until a key is

pressed.  Alternatively, the command CLMF may be included in the

program to resume the normal stack display when the program is

completed.

On the HP 48SX, special displays are normally cleared and the

stack (or graphics environment) display is resumed when a program is

completed -- unless the FREEZE command is included in the program.

The 48 does not contain the 28's R->P and P->R commands.

Instead, the calculator is set to one of rectangular,

polar/cylindrical or polar/spherical modes.  This mode affects the

interpretation of complex numbers.  Conversion is done by changing

the mode.  The rectangular form is always stored internally;  only

the DISPLAY of these values are converted when in a polar mode.

The 28 commands ARRY->, C->R, LIST-> and STR-> may be replaced

by the more general 48 command OBJ->, although the 48 does recognize

the 28 forms.

The 28's ALGEBRA FORM environment has been changed and is now

incorporated in the EquationWriter Application.

It is often possible to recover from accidentally PURGEing or

STOring into a global variable on the 48 via the LASTARG command.

The HP 48 menus are completely reorganized and renumbered.  The

current custom menu list (or the name of a global variable containing

it) is stored in the variable 'CST', a distinct one of which may

exist in each subdirectory.

Custom input menus (created with STO as the first list element

on the 28) are not supported on the 48.  The [l-shft] key can be used

to store directly into global variables displayed in the menu line.

The MENU command will accept any object as its argument and

store it in 'CST'.  However, an error is generated if 'CST' does not

contain a list (or the name of a global variable containing a list)

when the custom menu is invoked.  Specific pages within a menu can be

specified by the fractional portion of the argument -- for example,

12.02 refers to page 2 of menu #12.

In a custom menu, labels other than the key actions can be

specified -- with either a string or 21x8 graphics object.  [l-shft]

and [r-shft] actions can also be specified.

The sixty-four user flags are represented by positive flag

numbers.  The sixty-four system flags are represented by negative

flag numbers.

The PLOT system has been substantially enhanced, with multiple

plot types, increased functional abilities in the interactive

Graphics Environment, equation catalog, etc.  Points are digitized by

pressing ENTER in the Graphics Environment.  See the manual.

SPECIFIC OPERATIONAL DIFFERENCES:

ABORT  --  Is not recognized by the 48.  Use  0 DOERR.

AXES  --  Can also be used to specify axis labels.

CENTR  --  Also accepts a real number x corresponding to the point

(x,0) as the plot center.

CHR  --  Some characters and character codes have been changed.

CLLCD  --  Does not clear the menu line.  In a program, must be

followed by 7 FREEZE for the blank display to persist.

CLMF  --  Is not supported by the 48.  See "General Comments" above.

CLUSR  --  Is recognized with this spelling, but has been renamed

CLVAR, which can be placed in the command line with [r-shft] DEL.

CLUSR is "translated" to CLVAR in programs and lists.

COLsigma  --  Is supported by the 48, but not listed in a menu.  It

is replaced by the separate commands XCOL and YCOL in the STAT menu.

COMMAND --  (keyboard only)  Renamed LAST CMD.

CONVERT  --  Has been redefined to function in the new Units

Management System:

2:  x_sourceunit    CONVERT    1:  z_targetunit

1:  y_targetunit    ------->           (y is ignored)

C->R  --  Is provided on the 48, operating similarly to R->C (and

[l-shft] 2D and V-> ).   See R->C below.

DGTIZ  --  Is not supported on the 48.  Instead, the interactive

Graphics Environment is entered with the GRAPH command.

DISP  --  Now accepts arguments 1 - 7.  Must be followed by FREEZE

for the display to persist after program execution ends.

DRAW  --  Does not erase previous graphics display before plotting.

DRWsigma  --  Is not supported by the 48.  It is replaced by

SCATRPLOT or SCATTER DRAW.

EVAL  --  EVALuating a list on the 28 returns it unchanged.  On the

48, each element of the list is EVALuated in turn.

EXGET  --  Not present in the 48.  It is replaced by the SUB

command in the interactive EquationWriter Application.

EXSUB  --  Not present in the 48.  It is replaced by the REPL

command in the interactive EquationWriter Application.

FACT  --  Is recognized with this spelling, but has been renamed !.

Only the ! form is in a menu.  ! requires postfix notation.

FIX  --  In FIX display mode, real numbers are displayed with digit

separators, such as:  1,234,567.89  or  1.234.567,89  (depending on

the state of flag -51).

GETI --  Wrap-around indicated by flag -64 rather than 46.

IM  --  Performs conversion to rectangular value if in a polar mode.

INDEP  --  Can also be used to specify the plotting range of the

independent variable.

Integration  --  Completely redesigned.  Output uses the | (Where)

function.  Stack usage is different:

4:  Lower limit                  1:  Expression representing

3:  Upper limit          ===>        integral (can be EVAL'd)

2:  Integrand

1:  Variable of integ            Error stored in variable IERR

Algebraic syntax:   $(lower limit, upper limit, integrand, name)

where $ represents the integral character.

KEY  --  On the 48, returns a 2-digit rc (row-column) keycode to

level 2, rather than a string representing the pressed key.

LAST  --  Is recognized with this spelling, but has been renamed

LASTARG.  It is displayed as ARG in the MODES menu to enable/disable.

LAST is translated to LASTARGS in programs and lists.

LCD->  --  Returns a graphics object (GROB), not a string.

->LCD  --  Argument is a graphics object (GROB), not a string.

MENU  --  See "General Comments" above.

NEG  --  Does not appear in a menu.  Use the +/- key.

NUM  --  Some characters and character codes have been changed.

OBGET  --  Not present in the 48.  It is replaced by the SUB

command in the interactive EquationWriter Application.

OBSUB  --  Not present in the 48.  It is replaced by the REPL

command in the interactive EquationWriter Application.

PIXEL  --  Is not recognized by the 48.  It is replaced by PIXON,

which accepts a complex number as a "user" unit coordinate, or a list

of two binary integers as a pixel coordinate.

PMAX  --  Is supported by the 48, but is not in a menu.

PMIN  --  Is supported by the 48, but is not in a menu.

PPAR  --  Contains two additional elements:  plot type & dependent

variable.

PREDV  --  Is recognized by the 48, but has been renamed PREDY.

PRMD  --  Not present in the 48.

PRUSR  --  Is not supported by the 48.  Use  VARS  PR1.

PURGE  --  The contents of the PURGEd variable can be retrieved by

the LASTARG command when PURGE is executed from the keyboard.

Recovery is not possible when PURGE is executed from a program, the

command line, or when the argument is a list.   Use PGDIR to purge a

non-empty subdirectory.

PUTI  --  Wrap-around indicated by flag -64 rather than 46.

P->R  --  Not provided.  See "General Comments" above.

RCLF  --  Returns a list containing two binary integers -- the first

representing the states of the system flags (-1 thru -64) and the

second representing the user flags (1 thru 64).

RCLsigma  --  Is supported on the 48.  Press [r-shft] STOsigma to

execute RCLsigma or place it in the command line.

RDX,  --  Is renamed FM, (Fraction Mark) in the MODES menu.

RE  --  Performs conversion to rectangular value if in a polar mode.

RES  --  Accepts a binary value to specify pixel resolution (as a

real value does on the 28) or a real value to specify resolution in

"user" units.

RND  --  On the 48, the value in level 2 is rounded as specified by

the level 1 value:

0 <= n <= 11  --->  level 2 value rounded to n decimal places

-11 <= n <= -1  --->  level 2 value rounded to n significant digits

12  --->  level 2 value rounded according to current display format

(as the level 1 value is rounded on the 28).

Also see TRNC (Truncate) in the 48 manual.

R->C  --  Is provided on the 48.  If a polar mode is set, the values

are converted to polar form for display.  Also, if flag -19 is set,

[l-shft] 2D from the keyboard and the command ->V2 create a complex

number.  In this case, however, if a polar mode is set, the real

values are interpreted as already being polar and are not converted

for display.

R->P  --  Not provided.  See "General Comments," above.

SCLsigma  --  Is supported by the 48, but not listed in a menu.

SIZE  --  Arguments which cause a "Bad Argument Type" error on the 28S

return the real value 1 to level 1 on the 48.

STO  --  The previous contents of the global variable can be

retrieved by the LASTARG command to recover from accidental over-

writing performed from the keyboard.  Recovery is not possible when

STO is executed from a program or the command line.

STOF  --  If the argument is a single binary integer, it is

interpreted as the set of system flags and the user flags are left

unchanged.  If the argument is a list containing two binary integers,

the first corresponds to the system flags and the second to the user

flags.   See RCLF and "General Comments" above.

SYSEVAL  --  To get the ROM version on the 48, do #30794h SYSEVAL.

TRAC  --  Printer Trace mode is not directly provided by the 48, but

may be implemented using Vectored ENTER.

TYPE  --  Returns values from 0 thru 31.  On the 48, built-in

functions and commands return 18 and 19, respectively, compared to 8

on the 28.

UNDO  --  (keyboard only)  Renamed LAST STK and displayed as STK in

the MODES menu to enable/disable.

VAR  --  Is supported on the 48, but not listed in a menu.

WAIT  --  On the 48, arguments 0 and -1 interrupt program execution

until a key is pressed -- and then return its 3-digit rc.p

(row-column.plane) keycode.

+  --  If either level 1 xor level 2 contains a string, the other

object is converted to a string and they are concatenated.

Stephen J. Thomas  P.O. Box 371861  El Paso  TX  79937-1861

sjthomas@cup.portal.com

FLAG TABLE

If there are no notations, the set and clear states function

similarly on the HP 48 as on the HP-28.    S = set   C = clear

Description                 HP-28S                  HP 48SX

---------------------------------------------------------------------

Last Arguments                 31                      -55

S = arguments saved      S = arguments not saved

C = arguments not saved  C = arguments saved

Trace Mode                     32                   Not supported

Auto CR Mode                   33                      -38

Solution Mode                  34                      -1

Constants Mode                 35                      -2

S = remain symbolic      S = eval to number

C = eval to number       C = remain symbolic

Results Mode                   36                      -3

S = symbolic             S = numeric

C = numeric              C = symbolic

Binary Wordsize             37 - 42                -5 - -10

Binary Base                 43 - 44                -11 - -12

Multiline Display              45                      -52

S = ML on                S = ML off

C = ML off               C = ML on

Index Wraparound               46                      -64

Double Space Print             47                      -37

Radix/Fraction Mark            48                      -51

Display Format              49 - 50                -49 - -50

Beeper                         51             -56  Error/BEEP Command

-57  Alarm Beep

Print Speed                    52             Controlled by DELAY

Number of Decimal Digits    53 - 56                -45 - -48

Underflow Action               57                      -20

Overflow Action                58                      -21

Infinite Result Action         59                      -22

S = error               S = return +/- MAXR

C = return +/- MAXR     C = error

Angle Mode                     60                 -17 & -18

S = radians            C  &  C = degrees

C = degrees            S  &  C = radians

C  &  S = grads

Negative Underflow             61                      -23

Positive Underflow             62                      -24

Overflow                       63                      -25

Infinite Result                64                      -26

Stephen J. Thomas  P.O. Box 371861  El Paso  TX  79937-1861

sjthomas@cup.portal.com