HP Handhelds of the 1980s: The HP-75, HP-71, HP-28, HP-42S & HP-48SX

By Dan Allen  6 March 1990

One year ago (almost to the day), I compared the four major portable

computational devices that Hewlett-Packard introduced during the 1980s: the

HP-75C, HP-71B, the HP-28S, and the HP-41/42S.  This list is not exhaustive

but is representative of the four major thrusts towards portable scientific

computation that were accomplished in the decade.

A new decade has dawned, and with it Hewlett-Packard introduced on March 6th a

new top-of-the-line HP calculator, the HP-48SX.  The HP-48SX stands for

Scientific Expandable.  It is a blend of the HP-28S and the HP-41CX, hence its

moniker.  HP has gone back to its original vertical-style calculator case;

indeed, the 48SX looks like an HP-42S, although a bit longer and fatter.  Gone

are the poor display and case of the HP-28!  But the great software remainsI...

Here are the basic hardware specs:

* "Saturn" CPU at 2 MHz, nibble-oriented 20-bit bus, 64-bit registers

*  256 KB of ROM

*  32 KB of RAM, expandable to 288 KB

*  2 slots for RAM/ROM expansion (main memory, ROM applications, backup RAM)

*  Infrared bidirectional IO up to 9600 baud for printing & IO to other 48s

*  Serial port for 9600 baud communication with a PC or Macintosh

*  49 keys: 4 rows of 6 keys followed by 5 rows of 5 keys (like HP-41/42)

*  131 x 64 pixel LCD display (3 font sizes)

*  3 AAA batteries

The display allows the four level stack, a row of softkeys, and a status area

all to be displayed at once.  The status area displays the current PATH, and

optionally a continuously updated display of the current date and time, as

well as other messages when appropriate.  The display contrast is better than

the 28S and 42S.

Here is a comparison of these machines by the number of features and functions

that they offer.  The machines are listed chronologically by date of

introduction.  This evaluation has merged some of the features compared to

last years comparison, so both tests are shown in the table below:

# of Functions   HP-75C*    HP-71B*   HP-28S   HP-42S   HP-48SX

----------------------------------------------------------------

March 89 Test  |  325        284       252      221

March 90 Test  |  286        268       247      200       353

* - Note: the HP-75C includes the Math and I/O ROMs, while the HP-71B includes

the Math and HP-IL ROMs for this comparison.

A most important fact is to be seen from this table: the trend prior to the

HP-48SX was declining functionality.  A prediction from linear regression

would have the fifth generation machine have 180 functions, but instead it has

over 350!

This year our review will be divided into twenty categories: binary, complex,

control, display, files, graphics, IO, keys, logical, math, matrix, memory,

number theory, printing, stack, stats, strings, symbolic, system, and time.

----------------------

BINARY NUMBERS

This category includes all bit-level operations as well as working in

different bases.  The 75C's I/O ROM provided a great set of functions for

shifting, rotating, ANDing, and ORing ASCII strings, HEX strings, and

converting from any base to any base.  The 71B provided very little

functionality in this area, and limited its BINAND type functions to 20 bit

integers, which was not too useful.  The 28S featured a variable wordsize up

to 64 bits, but the range of its operations was somewhat limited.  For

example, there are 8 different functions for rotating and shifting in various

ways when 2 would have offered the basic functionality.  Meanwhile, there is

no facility for simply testing if a bit is set.

How does the 48SX stack up?  Well, it is almost exactly the same as the HP-

28S, so this is one of the few areas that is still somewhat weak in the HP-48.

There is still no facility to do bit testing in a simple way.  Binary numbers

take precedence in a calculation, with a real and a binary always resulting in

a binary number.  Unfortunately binary numbers are not automatically converted

to reals, so all transcendental operations are not allowed on binary numbers.

You must use B->R beforehand.

The biggest shortcoming of the HP-48's binary arithmetic is that it does not

support any signed binary numbers.  Binary numbers are always unsigned, so the

typical question "what is $FFDE in decimal?" will always be 65502 rather than

-34 in 2's complement or -33 in 1's complement.  The HP-48SX is better than

the 28S in that it does allow you to use the +/- key (similar to the 41/42 CHS

key) to calculate the negative value (an invalid operation on the 28S), but

#65502d +/- with a word size of 16 yields #34d, the 2's complement of #FFDEh

but with the wrong sign.  A step in the right direction...

I guess the saddest fact is that all of the HP machines are still short of the

HP-16C in functionality, and now that the HP-16C is no longer made, there is a

real need for progress here.  None have the masking, left justify, carry flag,

and complement modes that the HP-16C had.  The good news is that now that we

can create plug-in cards, a lot of the HP-16C like functionality can be added

to a card.  Wouldn't it be neat to be able to buy a 68000 card that emulates a

68000 and its instructions?  The HP-48SX offers the ability to do something

like that.  Until someone does, hold on to your HP-16C!

WINNER: Those that have an HP-16C are the winners.  Next runner up is the

48SX.

----------------------

COMPLEX NUMBERS

Complex arithmetic as a new data type was first found on the HP-15C. The BASIC

machines then got complex arithmetic with their Math ROMs.  The 75C was the

most awkward, as complex numbers were 2x1 matrices.  The 71B, and consequently

the 28S, used a set of parenthesis to specify a complex pair, which was much

better. The 15C and 42S required the use of two of the stack registers to

input a complex number, but once it was input it then occupied just one level

of the stack.  The 75C and 15C offered all trig functions and their inverses

in complex mode, but then the 71B came along and the inverse complex trig

functions were gone!  The 28S, 42S, and 48SX have brought them back, however.

The 71B and 75C had the finite Fourier transform built in; it has been absent

on all later models.  The 71B would not take the determinant of a complex

matrix: all others will.  The 71B did, however, support IEEE math and complex

numbers; all others do not.  The 71B and 75C also supported PROOT, a full

polynomial root finder with real and complex roots.  Although the Solver of

the 28S and 48SX is nice, it will not deliver all of the roots of a polynomial

equation.  Using TAYLR and ROOT one can obtain the real roots, but PROOT is

still sadly missed in the 48SX for complex roots.

For ease of use, the 28S and 48SX are very good when dealing with complex

numbers.  The 42S offered the display of complex numbers in either rectangular

or polar coordinates, a most welcome feature that gives a real feel for 2D

vector analysis.  The HP-48SX has combined the 28S and 42S to bring these fine

features together.  In addition, this functionality has also been extended to

2D and 3D vectors, with cylindrical and spherical coordinate modes shown as

well.

Gradually complex numbers have become first class citizens.  Taking the square

root of a negative number used to be a grievous error that stopped programs in

their tracks.  Now the 48SX always allows complex results from real functions,

so SQRT(-1) always is a valid operation, returning (0,1), as it should be.

WINNER: Although I miss PROOT, FOUR, and IEEE complex math, the HP-48SX still

is the best complex machine in town.  In fact the HP-48SX quite possibly is

the finest complex arithmetic environment around, on any computer.  (The only

other environment which may approach the 48 for its good treatment of complex

math is perhaps the Common Lisp environment.)  If you need to do complex math,

get an HP-48SX.  You will not regret it.

----------------------

CONTROL STRUCTURES

The early machine offered little in the way of structured control constructs.

Although HP's BASIC has been one of the better BASICs around, the 75C and 71B

seem primitive, as does the 41/42 style of RPN programming.  The 28 and 48

style of programming is definitely a giant step forward.

RPL, as it is called, stands for Reverse Polish Lisp.  Indeed, reading the

Common Lisp language reference with its many data types does remind one of RPL

and its 20 different object types supported on the 48SX.  The SX is a step

forward from even the 28S in that it offers a CASE statement, typing aids for

inserting a whole statement template, and better debugging.  Lists provide a

very versatile way of handling information.

So is RPL ready to compete with C and Pascal?  Well, RPL handles the following

basic data types:

Booleans: 0 = FALSE, non-zero = TRUE

Numeric types: 64-bit unsigned integers, 12 digit reals

Strings (any length) of 8-bit characters (no separate char type)

Symbolic names (local and global)

Graphics object (1 bit per pixel bitmap)

Algebraic expressions

Programs

Composite data types include:

Tagged objects (any object with an associated label)

Unit objects (a real with an associated physical unit)

Complex numbers (a pair of real numbers)

Arrays of reals or complex numbers (1 and 2 dimensions only)

Lists, composed of elements of any data types, including lists

Directories, composed of a hierarchy of objects, including directories

Backup objects (like a directory but with a checksum)

Library and XLIB (two extensible code types)

Many of the RPL data types, such as lists, algebraics, and programs, are very

powerful and have no counterpart in traditional programming languages like C

or Pascal.  They are in fact what make symbolic math possible.  However, RPL

is still somewhat limited: it does not have any record, set, or enumerated

data types; arrays are limited strictly to real numbers; and there is no

signed integer type.  Pointers are absent, but this is probably no great loss,

as pointers are the data type equivalent of the GOTO statement and cause many

programming errors.  Like many LISP systems, dynamic memory allocation is

standard, with automatic garbage collection, so there is no need for malloc-

like dynamic allocation.

WINNER: The HP-48SX is clearly the most sophisticated programming environment

of any HP handheld so far.  RPL and lists provide a very powerful environment.

----------------------

DISPLAY

The HP-75C still has the widest display yet offered on an HP portable; ensuing

machines had terrible displays, with the 28S and 42S being very poor in

display quality.  The 48SX is a step forward at last, with a nice, readable

display of up to 10 lines (depending on font size).  However, the RPL machines

are very poor when it comes to formatting output.  Yes, they do have the STD,

FIX, SCI, and ENG modes which the 75C did not have, but there is no equivalent

of C's printf.  The 70 series machines had DISP USING and IMAGE statements

which allowed many formatting options (almost as many as printf); they also

supported multiple display devices, including regular monitors via HP-IL.

As far as menus go, the 70 series had none, which was not all bad.  The 42S is

an improvement on the 28S with it automatically popping back out of a menu

after you have used it, but menus still are slow to use.  The 48 is

unfortunate in its having 28-like menus rather than 42-like menus.  The on-

line listing of parameters to 28S functions is laudable, and is also missed in

the 48SX.

Having said all that, the 48SX does greatly improve on the 28S, with better

control of menus (MENU,TMENU,CST), freezing parts of the screen (FREEZE), and

prompting for input (INPUT, PROMPT, WAIT).

WINNER: Again the HP-48SX comes in number one, but the competition is not too

stiff here.  Add a printf/scanf set of routines and more fluid handling of

menus, and then you would have a real winner.

----------------------

FILES

The 75C and 71B supported a flat file system.  The 28S and 42S do not support

any files whatsoever.  This is a major weakness of the new machines.  (The 28S

hierarchical directories are only for variables.)  The 75C I/O ROM provided a

full set of operations for dealing with records and fields in a file.  And of

course only the HP-75 had a nice text editor built-in for manipulating text

files.  Its built-in FIND was particularly nice.

The HP-48SX, like the 28s, does not have a file system, per se.  Its variables

are arranged in a hierarchy, but variables do not have any date, time, access,

or privilege information maintained by the system.  The 48 does add operations

to backup, archive, and restore its directories of information, which is very

handy.  Still, there is little protection from yourself.  Variables are always

purgeable and writable, for example.  The HP-71B allowed files to be made

read-only, something that can be done on a 48SX only moving something into a

RAM card and then setting the external physical switch to read-only.  It must

be pointed out that the main use of the HP-48 is supposed to be mathematics,

not as a database, but darn it, I want to keep names and addresses with me as

well.

WINNER: The HP-75C is still the best HP portable for note taking, small

databases, writing notes and brief letters.  Its support for file, record, and

field IO was decent; all others since have been abysmal.

----------------------

GRAPHICS

The 75C had none.  The 71B offered bit-twiddling, but with a uselessly small

display.  The result was laughable.  The 42S graphics were very slow, too

small, and generally worthless as well.  The 28S had the first passable

graphics due to a higher display.  Its ability to plot an equation or a

scatter plot of data provided actually useful capabilities.  The 48SX has a

display twice the height of the 28S, and its functionality has likewise been

more than doubled.  The 48 also adds a few interactive drawing tools for

drawing lines, rectangles, and circles.

WINNER: Clearly the HP-48SX.  Although it is not a Macintosh yet, its tools

are well crafted for the plotting needs of scientists and engineers.  The HP-

48SX is a solid step forward.

----------------------

INPUT/OUTPUT

The 28S and 42S only support the O of IO, and that in a pretty dismal way.

The 75C and 71B both tried to make HP-IL a success, but to no avail.  HP-IL

effectively was a closed system due to its being hard to interface to the

outside world.

The 48SX has bi-directional IO via infrared or serial ports.  The HP-48SX

contains a built-in Kermit application, for easily moving information to the

world of personal computers today.  Its use is incredibly simple: plug a cord

into the back of your Macintosh or PC, plug it into the 48; run Kermit on the

Mac, say send or receive file, and press the opposite on the 48 IO menu, and

everything is done.  I did it the first time in about 1 minute.  And 48 to 48

communications are even easier, as you just point both of them towards each

other and presto, the data is magically exchanged.  Basic serial primitives

are also supported, so you can roll your own protocol if you desire (as long

as RS-232 can deal with it).

WINNER: The HP-48SX is a clear, significant step forward in this area.  This

is the way computers are meant to communicate: fast, easy, and transparent.  I

do, however, miss the LAN aspects of HP-IL.  It was much more sophisticated in

what it could do (remote controllers, parallel polling, etc.), but alas, HP-IL

is gone.

----------------------

KEYBOARD and KEYS

The 75C keyboard could be touch-typed upon.  Nothing since has had that

ability, including the 71B QUERTY keyboard, due to its use of calculator keys.

The folding 28 keyboard was the worst case design yet; the 42S the perfect,

slim, well built keyboard.  Key entry of strings on the 42S was a nuisance at

best.  The 48SX has an alpha lock like the original HP-41, which is actually

quite good.  The 48 case design is very good, and considering the immense

amount of functionality, it is even better.  The 28S was pretty weak in its

ability to poll the keyboard; the 48SX is better with its extended WAIT

command.

The 75C and 71B had user definable keyboards.  The 28S had no keyboard

customization.  The 42S had some, but not like the original 41.  The 48SX has

brought back the completely user definable keyboard.  Also latent in the 48SX

is the ability to define a custom version of the ENTER key via flag -63.  This

intriguing capability is nowhere further explained, so it is something that

can be explored in the coming months.

WINNER: A hard call: the 75C still has the best keyboard feel, but the 48SX

does a good job packing lots of functions onto its keyboard.

----------------------

LOGICAL OPERATIONS

The progression in functionality of logical operators has been one in which

each succeeding model has been better than the past.  The 75C had the basic

Booleans; the 71B added flags; the HP-48SX is identical in functionality to

the 28S and 42S in both its support of the full range of Boolean functions as

well as flags.

WINNER: Three-way tie for first by the 28S, 42S, and 48SX.

----------------------

MATH and ARITHMETIC

This category includes the basic mathematical operations that are built-in.

The number of operations has stayed relatively constant until the 48SX came

along.  Some machines have included a few simple operations which are not

really needed but which are handy.  The 75C alone had CSC, SEC, and COT; the

75C and 71B had LOG2 (VERY handy and missed in later machines); the 28S and

48SX have %, %CH, and %T.  Starting with the 71B machines had LN(1+X) and its

inverse EXP(X)-1.  Although these seem unneeded at first glance, they are

invaluable in offering a special degree of precision that is needed for

solving financial equations.  The trig functions are quite similar overall;

minor differences include GRAD mode and polar display mode on the 42S and

48SX.

When the 15C was introduced, it added to the repertoire of a scientific

calculator four new features that bring together much of numerical analysis:

complex arithmetic (covered above), matrices (covered below), root finding,

and numerical integration.  Root finding has progressed greatly from the

original machines to a full-blown solver that incorporates symbolics in order

to find the variables of an equation.  The Solver of the 28s was the first

solver in this symbolic league.  The 42S Solver is not as sophisticated as the

28S Solver in that it does not rearrange terms and solve the resulting

equation directly as the 28S does, but the 42S did add a nice touch: it shows

the convergence/divergence of guesses as it goes.  At first glance the 48SX

Solver omitted this handy feature, but it turns out a key press will enable

it.

Integration was fairly straightforward on the 75C; it got better with the 71B,

as double and triple integration was supported.  The 28S was pretty good; the

42S was a small step backward (awkward interface); the 48SX perhaps the

easiest, as the EquationWriter application allows a very simple entry in a

typographical manner.  Integration with the 48SX can also be symbolic, and

although its built-in pattern matching does not cover lots of cases, it is

still a nice step forward.

One last area of note is unit conversions.  The 28S was so neat in having all

of the metric conversion constants built-in.  The only problem was that using

these constants was awkward and a pain at best.  In fact, using the

conversions was so bad that no one uses them.  Well, the 48SX has done an

about face, and its unit conversions are so slick it is hard to think of ever

solving another physics problem without a 48 at hand.  Units stick to real

numbers with the underscore character and can flow through equations, plots,

etc.  Dimensional analysis is performed along the way as a double-check on the

proper use of equations.  I am sure you will see this feature appear on HP's

less expensive machines in the coming years, and by the end of this

decade/century, all scientific calculators will have unit conversions as

standard equipment.  They are just too useful to not be.

WINNER: The HP-48SX.  Even if it only had unit conversions it would be the

winner.  Add to them the mature, full featured Solver, integration facilities,

and a nice set of standard math functions, and you will find a machine that

scores very close to a perfect 10.  (Add LOG2 and I'd give it a ten.)

----------------------

MATRIX MATH

Probably the purest form of "number crunching" is doing array manipulations.

The 15C introduced some basic array handling, but input of array elements was

a pain.  The 75C and 71B offered some pretty heavy duty capabilities, but they

too had an entry problem.  The 28S allowed you to see all of say, a 3x3

matrix, but getting quickly to elements in a large matrix was slow.  The 42S

added a nice matrix editor for interactive editing of a matrix which evolved

into the 48SX MatrixWriter application for a mini-spreadsheet view of a

matrix.  (Of course the HP-75C had a Visi-Calc ROM that provided a 42S-like

view, but that is another story.)

As far as individual matrix operations is concerned, the 42S is interesting in

that it allows any math function to work on a matrix, so that taking the LN of

a matrix actually takes the LN of each element, as an example.  The 28S/48SX

instead say that taking the log of a matrix does not make sense, so the

operations are invalid.  I think I would prefer the 42S method...

A few other useful functions have been spread across various machines: the 75C

could create vectors of row and column totals (RSUM, CSUM), give an LU

decomposition (LUFACT), and either transpose or conjugate transpose a complex

matrix.  (All other machines only do conjugate transpose of a complex matrix.)

All have the DOT product operation; all but the 71B offer the CROSS product.

The 75C/71B allowed SHORT elements and INTEGER matrices so matrices did not

take up as much room.  The 15C, 28S, and 48SX offer a residual function.  The

42S has a few other nice features: it can insert or delete or swap rows of a

matrix programmatically, and it has a unit vector function.

As far as programmatically manipulating the individual elements of an array,

call me old fashion if you want, but the simple array subscripting of BASIC

still seems simpler than having to create an algebraic expression in RPL.  And

having to check a flag when using GETI and PUTI on the 28S and 48SX is not as

nice as just incrementing a pointer to array elements, like on the 42S.  RPL

still needs a bit of work with arrays.

The 48SX revision A has a bug: it can only invert arrays smaller than 10x10.

This is too bad since it is the only one with enough memory to really deal

with large arrays, not to mention the IO ability to get data into the machine

in the first place.

WINNER: The 75C, 42S, and 48SX all have nice features that the others do not.

However, there is no clear winner, as each has major shortcomings as well.

----------------------

MEMORY

The 71B wins in the realm of being able to manipulate memory in any way you

want.  It has built-in PEEK, POKE, and ADDR$.  (Actually the 28S, 42S, and

48SX probably have these capabilities built into their undocumented

debuggers.)  Variable arithmetic is provided for in the 42S, 28S, and to a

greater degree in the 48SX.  The 48SX also has a BYTES function which

checksums and gives the size of any object.  Both the 71B and 48SX provide

memory partitions.  Both the 71 and 75 allowed different sizes of variables,

including INTEGER, SHORT, and REAL.

WINNER: HP-48SX.

----------------------

NUMBER THEORY

This category considers the operations that relate to parts of numbers, such

as modulo, as well as to the IEEE standard of arithmetic with its insistence

upon the proper handling of rounding, infinity, and exceptional conditions.

To put it succinctly, the 71B was superb, being the only HP handheld to fully

conform to the IEEE standard.  Unfortunately, all machines since have thrown

out most of the work put into the 71B.

The 71B offered 3 different types of modulo operations, for example.  The 71B

offered different rounding modes, and supported NaNs (Not A Number),

infinities, and infinity arithmetic.  Exceptions were flagged, and could also

be trapped on.  Denormalized numbers were supported; relational comparisons

handled unordered relations; signed zeros were supported; several constants

were built-in to specify machine limits.

Now some of this functionality survives today in the 28S, and to a greater

degree in the 48SX.  But the loss of the 71B meant the loss of a very special

machine.  The only other source of truly conforming IEEE machines is Apple's

Macintosh, and the Macintosh Portable still has a lot of weight to lose to

become a handheld.

WINNER: Those that have HP-71Bs are the winners.  Those that do not have one

are the losers.  The tragedy is that HP had the technology and threw it out.

----------------------

PRINTING

71 and 75 were good, as they supported real printers.  The toy infrared

printer supported by the 28S, 42S, and 48SX is neat only because it does not

need a cord.  Fortunately the 48SX also supports a serial interface, so you

can use your 48SX to drive your Apple LaserWriter if you want!  The 48SX is

also great because you can transfer all your information to a Macintosh and

then use a LaserWriter there, which is what I do.  The 48SX offers a few more

flags to control various aspects of printing when compared to the 28S.

WINNER: HP-48SX.

----------------------

STACK

The 28S has a lot of stack operators, which is necessary due to the language.

The 42's operators are sufficient for RPN.  Stacks are automatically handled

internally by the 71 and 75.  The 48SX builds upon the 28S by having an

interactive "point and shoot" stack mode, which is quite handy.

WINNER: HP-48SX.

----------------------

STATISTICS

The 75 had relatively little support for stats, the 71B a bit more, adding

linear regression.  The 71B curve fitting ROM, however, offered a tremendous

amount of power.  The 28S introduced several different probability

distributions, of which the Normal distribution should be on all machines.

(It was on the original HP-27!)  The 42S has done the best job of generalized

curve fitting, with its BEST fit function.  The 48SX added the BESTFIT

function to its statistics functions, and that combined with its graphics

plotting ability makes it the machine of choice for statistics.  Many new

types of charts have been added, such as histograms, that make routine

statistics easy.  It would be nice to add a user specified custom model to the

set of models to be fitted to your data.

WINNER: For built-in stats, the HP-48SX is the best statistics machine yet.

Although the 71B Curve Fitting ROM and Statistics ROMs offered more power,

that same functionality can appear in a HP-48SX ROM someday, and with much

better graphics.  For stats, the 48 is great.

----------------------

STRINGS

By far and away, the HP-75C with I/O ROM is the string manipulation king.  All

of the other machines provide just a few very simple functions.  The 75 did

mapping, trimming, underscoring, shifting, finding, replacing, spanning,

filling, you name it.  These functions were very handy for manipulating text

and printing it.  This went along well with the 75's record and field

manipulation tools.  The 48SX could use a String/File/IO ROM.

WINNER: HP-75C. No contest.

----------------------

SYMBOLIC

Only the 28S and 48SX do symbolic manipulation.  The 48 improves upon the 28S

by doing more symbolic integration, decimal approximation, separating an

equation, and offering its EquationWriter application for entering formulas,

fractions, etc.  These capabilities are further strengthened by the ability to

break trig functions down into their logarithmic basis, and by arbitrary

expression/pattern matching.  Maple on the Macintosh is a bit faster (well,

okay; a lot faster), but that is mainly due to hardware, not software.  Still

the 48SX has grown into a fairly sophisticated symbolic tool.

WINNER: HP-48SX. No contest.

----------------------

SYSTEM

This is the miscellaneous category for debugging and beeping and other system

types of things.  The 75C and 71B were tied here, with a password lock,

programmatic ways of turning on and off the machine, and with a flexible tone

generator.  The 48SX needs the password lock.  It does not offer a continuous

ON function, although for a battery only machine this is no great loss.

WINNER: 70 Series machines.

----------------------

TIME

The HP-80 began it all with the ability to do date arithmetic.  Then the HP-45

added HMS conversions.  Then the HP-55 added lap timers.  (Actually the HP-45

had a hidden timer.)  Then the HP-41 had its Time Module, incorporating date

arithmetic, lap timers, and appointment alarms.  It was awkward to schedule

appointments, but had great lap timers down to 1/100 second.

Then the 75C came along with its superb time and appointment modes.  Simply

input 7 AM FRI and it would figure out which Friday you meant.  Schedule an

appointment for the 1st Friday of each month, no problem.  Make it repeat

every year, no problem.  Its calendar spanned 9999 years.  Yet you could not

do date and time arithmetic, or do HMS conversions.  And its 9999 programmable

timers were awkward to use.

Then there was the 60 second disappointment.  The HP-71B had a built-in clock

but no alarms, timers, or HMS conversions.  It was supposed to get a separate

Alarm ROM, but it never appeared.

Next the 28S and 42S showed up without built-in clocks, but with HMS

arithmetic and conversions.  No timers, no alarms, no date arithmetic.

How does the 48SX fare?  Well, not too bad.  It has date arithmetic; it has

time arithmetic, and HMS conversions.  It has about an 8400 year calendar.  It

has a real-time clock, even with a continually updated display.  It can return

the system time to a resolution of 1/8192 of a second.  It has an appointment

mode with repeating alarms.  It can set and clear alarms from a program, like

the 41 Time Module.  It certainly is the best overall, but it lacks three

features:

Lap timers (for splits like the 41 Time Module)

Time calibration (EXACT like the 75C/71B)

True appointment scheduling (like the 75C)

This last feature is very important.  What it means is that you cannot use the

48SX to set appointments of the style, "first Friday of the month", "the 15th

of the month", or even birthdays.  You see, the 48SX only repeats by a fixed

number of seconds.  It does not know about months.  Yes, it can schedule an

appointment every so many minutes, hours, days, or weeks, but not months, and

there lies the tragic flaw.

WINNER: Well, if you have a 75C for appointments and a 41 with Time Module for

timing events, you have the best of all worlds, except for the fact that you

have to carry two worlds with you.  The 48SX is the best in one case, but it

needs improvement.

----------------------

SUMMARY

What to take to a desert island?  For computer science, a 16C; for time and

strings and files, a 75C; for IEEE math, a 71B; for math and stats and

graphics and programming and symbolics and IO, a 48SX.  If I had to have one

machine, it would be the 48SX and I would write a custom ROM to improve its

appointment mode and matrix handling, add IEEE math and a 68000 emulator for

binary operations, and a bunch of file/string handling routines for handling

and indexing text.

Dan Allen