* Technical specification

The CHIP-8 virtual machine is byte-addressable and has an address

space of 4 kB at addresses 000-FFF hex.  However, addresses 000-1FF

are reserved (this is where the CHIP-8 interpreter used to reside).

Therefore, the CHIP-8 program itself begins at address 200.  All

instructions are 16 bits long and by convention instructions always

start at an even address.

The machine has 16 8-bit general-purpose registers called V0..VF.  The

VF register is modified by certain instructions and works as a carry

flag and sprite collision indicator.  There is also a 16-bit pointer

register I (though only the low 12 bits are generally used).

A CHIP-8 program displays graphics by drawing sprites that are 8

pixels wide and 1..15 pixels high.  The screen resolution is 32 by 64

pixels.  The origin is the upper left corner of the screen, and all

coordinates are positive.  The sprites are stored in big-endian

format, i.e., the most significant bit corresponds to the leftmost

pixel.  All drawing is done in XOR mode.  If this causes one or

more pixels to be erased, VF is set to 01, otherwise 00.

There are two timers: the delay timer and the sound timer.  Both

timers count down about 60 times per second when nonzero; the speaker

will beep whenever the sound timer is nonzero.

In the instruction table below, NNN is a 12-bit address, KK is an

8-bit constant, and X and Y are 4-bit register numbers.  Hex

characters represent themselves.  The two first characters of the

instruction code form the lower-address byte of the instruction, the

first character being the more significant nibble.

0NNN    Call 1802 machine code subroutine at NNN

00E0    Clear display

00EE    Return from subroutine

1NNN    Jump to NNN

2NNN    Call subroutine at NNN

3XKK    Skip next instruction if VX == KK

4XKK    Skip next instruction if VX != KK

5XY0    Skip next instruction if VX == VY

6XKK    VX := KK

7XKK    VX := VX + KK

8XY0    VX := VY, VF may change

8XY1    VX := VX or VY, VF may change

8XY2    VX := VX and VY, VF may change

8XY3    VX := VX xor VY, VF may change *

8XY4    VX := VX + VY, VF := carry

8XY5    VX := VX - VY, VF := not borrow

8XY6    VX := VX shr 1, VF := carry

8XY7    VX := VY - VX, VF := not borrow *

8XYE    VX := VX shl 1, VF := carry

9XY0    Skip next instruction if VX != VY

ANNN    I := NNN

BNNN    Jump to NNN+V0

CXKK    VX := pseudorandom_number and KK

DXYN    Show N-byte sprite from M(I) at coords (VX,VY), VF := collision

EX9E    Skip next instruction if key VX pressed

EXA1    Skip next instruction if key VX not pressed

FX07    VX := delay_timer

FX0A    wait for keypress, store hex value of key in VX

FX15    delay_timer := VX

FX18    sound_timer := VX

FX1E    I := I + VX

FX29    Point I to 5-byte font sprite for hex character VX

FX33    Store BCD representation of VX in M(I)..M(I+2)

FX55    Store V0..VX in memory starting at M(I)

FX65    Read V0..VX from memory starting at M(I)

The instructions marked with * may have been undocumented (but

functional) in the original CHIP-8 interpreter for the 1802.

* Notes on the HP48SX implementation

CHIP-8 programs are stored in the HP48SX as string objects containing

the bytes of the program in increasing address order, beginning with

the byte at 0200.  The interpreter itself is a machine code object

that should be run with the CHIP-8 program string on level 1.  4 kB of

free memory is needed.  If an error is detected during execution, the

address of the current CHIP-8 instruction is returned as a binary

integer on level 1.

The clock display should be turned off.  Otherwise, CHIP48 will not run

but will clear flag -40 so that the next attempt to run it may succeed.

To quit, press the backspace key.  Pressing ENTER restarts the CHIP-8

program, and the +/- key turns the sound off or on.

The 0NNN instruction is unimplemented (of course), except for

NNN==0E0 or 0EE.  Subroutine nesting is limited to 16 levels.

Most chip-8 programs are written for a 16-key hex keyboard with

following layout:

1 2 3 C                                               7 8 9 /

4 5 6 D    This keyboard is emulated on the HP48SX    4 5 6 *

7 8 9 E    using the following keys:                  1 2 3 -

A 0 B F                                               0 . _ +

This may cause some confusion with programs requiring numerical input.

The keys 2, 4, 6, and 8 are commonly used to represent directions.

The F key (+ on the HP48SX) is sometimes used as a Fire button.

* Sample game

Not particularly exciting, but I had to include something to show how

the interpreter is used.  Uuencode, download to HP48SX using Kermit,

and run with BRIX CHIP.  Play by pressing the keys 4 and 6.

* Source code availability

The source code for the interpreter is over 30 k of assembler, and to

assemble it you need my ASAP assembler which is about 25 k.  To run

the assembler, you need Perl 3.0 which is several hundred k.

Therefore, I am not posting all these to comp.sys.handhelds.  However,

the source to CHIP48 and ASAP are available by anonymous FTP from

vega.hut.fi, directory /pub/misc/hp48sx/asap.  The source to Perl 3.0

is available from numerous Unix archive sites.

Those who want to write their own machine code games may want to look

at the interpreter source for hints.  Please note that I have no

insider technical information on the HP48SX; CHIP-8 is based solely on

information posted to comp.sys.handhelds (in particular, Alonzo

Gariepy's excellent HP28S Processor Notes) and personal HP48SX machine

code hacking.

* Fine print

CHIP-48 and BRIX are (C) Copyright 1990 Andreas Gustafsson

Noncommercial distribution allowed, provided that this

copyright message is preserved, and any modified versions

are clearly marked as such.

CHIP-48 and BRIX make use of undocumented low-level features of

the HP48SX calculator, and may or may not cause loss of data,

excessive battery drainage, and/or damage to the calculator

hardware.  The Author takes no responsibility whatsoever for

any damage caused by the use of this program.

THIS SOFTWARE IS PROVIDED "AS IS" AND WITHOUT ANY EXPRESS OR

IMPLIED WARRANTIES, INCLUDING, BUT NOT LIMITED TO, THE IMPLIED

WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR

PURPOSE.

--

Andreas Gustafsson

Internet: gson@niksula.hut.fi

Voice: +358 0 563 5592