Open Channel Flow v1.0

I.    FILES IN THE OCF.ZIP FILE

Will this work on an hp48gx?  Mostly.... hp48sx? Yep.....

OCF is the binary directory to install on your HP48SX.  OCF.ASC is the ascii

encoded file that can be converted to binary or print out on the pc.  OCF.DOC

is what you are reading now.  OCF.INF is just an information file for Chris

Spell at ftp site of wuarchive.wustl.edu (hp files in systems dir.).  Look

there for some of my other files and many other contributors.

II.    EXAMPLES IN OCF (OPEN CHANNEL FLOW) - Uniform Flow

/* Inside the /**/ are comments */

The examples listed below are on page 19-16 of the

"Engineer-In-Training Reference Manual" 8th ed. by Michael R.

Lindeburg, P.E.

1.  A wooden flume with n=0.012 of rectangular cross section is 2 feet

wide.  The flume carries 3 cfs (cubic feet per second) of water on a

1 percent grade/slope.  What is the depth?

A.  Procedure on hp: (See D. for short/expert version)

Press OCF

Displayed are: OPEN CRIT gpm-> cfs-> (OPEN is OPENC)

Press OPENC

Press EXIT  /* this goes to CST MENU (custom menu) */

Displayed are: Solve Plot Channel ooo ooo Quit

Press Channel

Displayed are: Rect Trap Circ ooo EXIT

Press Rect and then EXIT after busy signal goes off

Press Solve (the A) /* this goes to the eqn solver from cst menu */

/* Now to insert data */

Press 3 then Q which is the menu button where A is.

0.012 then N /* As above this stores the value of N as 0.012   */

/* press RS (right shift) N to see 0.012 is stored */

/* in N */

1 then DEEP, 0.01 then SLOPE or SLOP as displayed on HP.

LS Y /* left shift key and then Y solves for Y */

And the answer is: Y=0.3144 feet.

CST to go back to custom menu.

B.  EXPLANATION OF A.

1.  We know that an efficient rectangular shape has a height or

DEEP (on hp) as 1/2 of the BASE or width of the channel.

2.  Q is in cfs units.

3.  I used the Chezy-Manning equation:

v=1.486/n*(rh)^(2/3)*SQRT(S) and continuity of Q=VA

where, rh is the hydraulic radius which equals A/Pw

where, Pw=wetted perimeter.

C.  UNITS on HP48SX

1.  Since I used mannings eqn. in U.S. units,

Q is flow rate in cfs (cubic feet per second)

N is Mannings  roughness coefficient, given in charts and

tables....

Y is the actual depth of the uniform flow water in feet.

DEEP is the "as-built" depth or height of the structure and

not the depth of water.

SLOPE is the rate of slope or grade of the channel no

units.

Z (used later) is unitless but is expressed as a slope.

Think of it like m=y/x where y=1 and x=z.  This is useful

in expressing as a ratio.  If a trapezoid has a slope of

3/4 then 3/4=1/z that makes z=4/3.

T is top of trapezoid width or top of inverted Triangle in

feet.

DIAM is diameter of pipe in feet.

D.  Simplified A. (short version to be used now that you know how)

1.  OCF OPENC EXIT Channel Rect EXIT Solve

3 Q

0.012 N

1 DEEP

0.01 SLOPE

LS Y        /* LS is the left shift key    */

/* Our answer is Y=0.3144 feet */

CST         /* Back to custom menu         */

2.  An 18 inch inside diameter concrete storm drain (n=0.013) flows

half-full under the influence of gravity.  What is the capacity

if the drain slope 1 foot per 1000 feet?

A.  Procedure on HP48SX

1.  OCF OPENC EXIT  Channel Circ EXIT Solve

0.013 N

1.5 DIAM

1.5 2 / DEEP

1 1000 / SLOPE

LS Q

/* Q=1.6609 cfs */

CST

3.  What diameter cast iron (n=0.017) pipe is required to flow 1000 gal/min

of water on a 2% slope?

A.  Procedure:

1.  OCF OPENC EXIT Channel Circ EXIT Solve

100 gpm-> Q      /* or 0.2228 Q */

0.017 N

'DIAM' DEEP

0.02 SLOPE

LS DIAM

/* DIAM=0.3434 feet */

0.3434 12 *

/* The diameter is therefore 4.1207 inches or nominal 5" */

0 DEEP  /* resets DEEP variable to numeric */

CST

B.  Explanation of A.

1.  The Q or flow must be in cfs units.  I used my gpm-> to

convert from gallons per minute to cubic feet per second.

2.  Alternatively, on HP try:

100 LS UNITS VOL NEXT GAL LS UNITS TIME RS MIN 1 RS S

LS UNITS VOL FT^3 RS UNITS CONVERT

3.  Notice I stored the label 'DIAM' into the variable DEEP

and then solved for DIA.  Just be sure to reset or store

a number (I used 0) into the DEEP variable.  This is good

practice to start the solver from 0 if you want the

solution to converge to the first solution above 0.

4.  What slope is needed to flow a quantity of 3 cfs water through an

ordinary earth channel 3 feet wide and 1 food deep?  (n=0.035)

A.  Procedure:

1.  OCF OPENC EXIT Channel Rect EXIT Solve

3 Q

0.035 N

1 Y

1.5 DEEP

LS SLOPE

/* SLOPE=0.0011 ft/ft or 1.1% depending on how far you

carry the decimals out... */

B.  Explanation:

1.  Don't confuse the deep in the problem with the depth of

the channel.  I know, maybe DEEP should be DEPTH but then

it could have been confused with depth of water Y.

2.  For efficiency of rectangular sections we know the DEEP is

the height of the channel which is 1/2 of the BASE of the

channel.

III.    EXAMPLES IN OCF (OPEN CHANNEL FLOW) - Critical Flow

1.  A channel of width 5 feet and unknown critical depth carries

a Q of 375 cfs.  a) What is the critical depth?

b) What is the velocity?

A.  Procedure:

1.  OCF CRIT BEGIN Rect

375 Q

5 B

5 T

LS Y

/* therefore a)=5.5916 feet */

375 5 / 5.5916 /

/* b)=13.42 feet per second */

LS ON /* or CONT, it cleans up and back to custom menu */

B:  Explanation

1.  By continuity Q=VA so, V=Q/A where Q=375 and A is Y * B

2.  T=B since this is a rect section.

NOTES:  I hope you enjoy these programs.  Dewayne Cushman wrote the bulk of the

code for OCF and I wrote this document as well as modifying and adding

to OCF.

Several programs that I use and have posted are similar to commercial

cogo type routines.  These are free for your collection.  Try AREA,

KAREA, COINTERCEPT (in dos cointerc), HCURVE, FRACDC, VC, VCRV,

TRIANGLS (/\S or #_S on the pc)or triangle.zip, and VCM.

Most can be found at the ftp site of wuarchive.wustl.edu in the

systems/hp48 and below directories.  Some are backed up from a while

back.

VCM is the latest version of a solver for multiply vertical curves.

For a short single vertical curve I recommend my VC program.  Second,

I would recommend VCRV (Dewayne Cushman wrote this one as an

enhancement for VC that he modified from VC (it gives extra data).

Lastly, and the longest is the VCM.  It works just like the VC and VCRV but

it can do any number of curves,

even single.

PLANS: For OCF v1.10 -  Make a metric version, include more sections such as

parabola, rounded cornered rectangle, and round bottomed triangular.

Make energy equations for energy flow calculations, Add more unit

conversions, Make versions with/without units.....

BIO: - Kenneth Hobson wrote this document for you.  These programs were done

by Dewayne Cushman and Kenneth Hobson.  Dewayne was a CE student at OU.

Ken graduated in Spr.'93 and is and Intern I Civile Engineer for the

Oklahoma Department of Transportation.

These programs are useful for survey and bridge work.  They would have

been nice to have had when I took SURVEY, Hydraulics, .....

Modify anything you like, as Dewayne and I wrote these for ourselves and you

can use them if you like.  Please send me a copy is you do modify these and

document it and I will upgrade this version of OCF.

Regards,  Kenneth Hobson          kenneth.hobson@torii.nova.com

kenneth.hobson@s-box.misc.uoknor.edu

work  internet address: kenneth.hobson@dsibm.okladot.state.ok.us

And some local BBS's such as protoboard bbs 405-275-6827