TRIANGLES or /\S:

These programs are for solving the unknowns of a triangle when you know 3

properties.  Right triangles and obtuse triangles are delt with.

I hope you will enjoy this program.  Feel free to use it, and please send

any compliments/(constructive comments) to my email address at the end of

this document.

Note:  Several programs that I use and have posted are similar to commercial

cogo type routines.  These are free for your collection.  Try AREA,

KAREA, COINTERCEPT (in dos cointerc), HCURVE, FRACDC VC, VCRV, MVC

(multiply veritical curves, builds in VCRV), and my latest TRIANGLES.

B                 B

Triangles are oriented as such:  right= c /|a    obtuse=  c /\  a

A/_| C         A  /_ _\C

b                  b

Where, Capital letters are angles (degrees) and lowercase are side

lengths.

Note:  Set mode to 2 FIX, DEG, and XYZ for these examples.

When extracted the dir /\S contains these programs:

1. /\S   - The main directory.  Press it.

2. /\S0  - Just Press it and enter your three knowns and 0 for

the unknows.  Angle C must be 90 degrees if it is a right

triangle. (Right Triangle e.g. below).

: Angle A:  29.17   (down arrow)

:Angle B: 0

:Angle C: 90

:Side  a: 9.6

:Side  b: 17.2

:Side  c: 0        (Enter)

3. Results:                    (arrow keys to view)

7:   Angle A:  29.17

6:   Angle B:  60.83

5:   Angle C:  90.00

4:     Side a:  9.60

3:    Side b:  17.20

2:    Side c:  19.70

1:      Area:  82.56

(use ON key after viewing)

4. /\S   - For Obtuse triangles:

: Angle A:  32

:Angle B: 78

:Angle C: 0

:Side a: 0

:Side b: 27

:Side c: 0

5.  Results:

7:   Angle A:  32.00

6:   Angle B:  78.00

5:   Angle C:  70.00

4:     Side a: 14.63

3:    Side b:  27.00

2:    Side c:  25.94

1:      Area: 185.56

6.  COT  -  The trigonometry function.

7.  RIGHT - Branch program when C=90 degrees.

8.  OBTUSE - Branch program when C is not equal to 90 degrees.

9.  AR - Area value.  Press it to see your last value for area.

10. AA - Area of angle A.

11. AB - Area of angle B.

12. AC - Can you guess?

13. SA - Length of side a.

14. SB - Length of side b.

15. SC - Length of side c.

16. L - List of above two letter variables that are intialized to 0 when

/\S starts.

17. C1 to C16AA are specific programs/equations that test for conditions.

C's without letters are branch programs mainly.

Note:  The following is the breakdown of cases for knowns and unknowns.  If

see something that I missed please email me.

'RIGHT'  where C=90 degrees:

Sequence #      Given       Required       Case #

1.              a,b           A,B,c         C1

2.              a,c           A,B,b         C2

3.              c,b           A,B,a         C3

4.              A,a           B,b,c         C4

5.              A,b           B,a,c         C5

6.              A,c           B,a,b         C6

7.              B,a           A,b,c         C7

8.              B,b           A,a,c         C8

9.              B,c           A,a,b         C9

10.        Most any 2         Area

'OBTUSE'  where C is not equal to 90 degrees:

Sequence #      Given        Required      Case #

1.              A,B,a        b,c,C         C10, C10A, C10AA

2.              A,B,b        a,c,C         C10, C10B, C10AA

3.              A,B,c        a,b,C         C10, C10C, C10A

4.              A,C,a        b,c,B         C11, C11A, C11AA

5.              A,C,b        a,c,B         C11, C11B, C11AA

6.              A,C,c        a,b,B         C11, C11C, C11A

7.              B,C,a        b,c,A         C12, C12A, C12AA

8.              B,C,b        a,c,A         C12, C12B, C12AA

9.              B,C,c        a,b,A         C12, C12C, C12A

10.             A,a,b        c,B,C         C13, C13A, C13AA, C12AA

11.             A,a,c        b,B,C         C13, C13B, C13BB, C12A

12.             A,b,c        a,B,C         C13, C13C, C13B, C12C

13.             B,a,b        c,A,C         C14, C14A, C14AA, C12AA

14.             B,a,c        b,A,C         C14, C14B, C14AA, C12A

15.             B,b,c        a,A,C         C14, C14C, C14CC, C12C

16.             C,a,b        c,A,B         C15, C15A, C15AA, C13BB

17.             C,a,c        b,A,B         C15, C15AA, C13BB, C12A

18.             C,b,c        a,A,B         C15, C15C, C15CC, C12B

19.             a,b,c        A,B,C         C16A, C16AA, C16AAA

20.        Most any 3        Area

BIO - Kenneth Hobson wrote this document for you.  These programs were done by

Kenneth Hobson. Kenneth is a graduate of The University of Central

Oklahoma and the University of Oklahoma with a BA in Education/minor

in science and BSCE respectively.

I wrote these programs in the summer (1993) just after I graduated from OU. I am

employed by the Oklahoma Department of Transportation as an Engineer Intern I.

Regards,  internet  at   umtr121@dsibm.uoknor.okladot.us.ok

kenneth.hobson@s-box.mics.uoknor.edu

kenneth.hobson@torii.starship.com