BARS v1.0 and KBARS v1.0

BARS and KBARS are used to calculate rebar and steel wire quantities in lbs.

I used both in my days as a drafting technician for ODOT Bridge Division.

I.    FILES IN THE BARS.ZIP FILE

BARS        (binary directory for hp) contains:

WGHT        Total weight (lbs) of array on stack

CBARS       See each individual weight (lbs) of array on stack

STKWT       Takes 4 inputs from stack for one quick weight

000         Place holder, means nothing

LST         Last Array used

HELP        Help screen

BARS.ASC    Ascii readable of BARS binary file

BARS.DOC    This File

II.   FILES IN THE KBARS.ZIP FILE

KBARS       (binary directory for hp) contains:

A           String that has value of last total e.g. "Total=26.0777lbs"

SDAT        SigmaDAT - leftover statistic or total in lbs. single array.

SCTR        Center Trim Utility from Hp28S book.  (I didn't write it)

W12         No. 12 wire unit weight

W20         No. 20 wire unit weight

W30         No. 30 wire unit weight

BLIST       Main program (Run from Custom Menu)

CST         Variable containing programs for custom menu

LST         Matrix array of x by 3.  Store your stack here.

BAR1        Sub program for BLIST

KBAR.ASC    Ascii readable of KBARS binary file

KBARS.DOC   This file

III.  Example (BARS) - Calculates weight of 4 No. 5 bars at 6 ft. 3 in. long

and 3 Wire size 12 at 5 ft. 11 in. long.

Wire W12 use -12, W20 use -20 and W30 use -30.

1.   in BARS Directory

CLR     (Clear the stack)

MATRIX  (enter matrix editor, RS-Right Shift ENTER key)

4 5 6 3 ENTER DWN (DWN is the down arrow key, GO-> has box)

3 -12 5 11        (Notice the W12 bar is entered as negative)

ENTER ENTER       (Press enter key twice)

[[4 5 6 3]

[3 -12 5 11]]    (on stack now)

WGHT              (Total weight in lbs)

33.317            (Result)

BS                (Press backspace key, or whatever to clear stack)

LST               (Recall Last array)

CBARS             (Displays each Bar weight a cumulative total one

at a time, press any key to continue)

Barlist Weight

Number:  3

Size:  -12

Length:  5' - 11"

Weight:  7.242

Total:  7.242

SPC               (Any key will do to continue)

Barlist Weight

Number:  4

Size:  5

Length:  6' - 3"

Weight:  26.075

Total:  33.317     (Notice the reverse order of listing bars)

SPC

33.317              (Same as if we had pressed WGHT)

BS                  (Clear stack)

4 5 6 3             (Use stack input for one set of bars for

quick result)

STKW                (STKWT actual key)

26.075              (Weight in lbs. as expected from above results)

BS

[[4 5 6 3]

[0 1 0 0]

WGHT                 (Like the STKWT but using a matrix.  Notice

the dummy set of 0 1 0 0.)

26.075               (Result as expected for third time)

BS

IV.   Example (KBARS) - a stack input format only.  Same data as in III.

Inches are expressed as decimal of the feet.  e.g. 6'=11" is 6.11 and

3.01 is 3 ft. and 1 in.  Wire Sizes are on the custom menu.  Press the

Corresponding wire, W12 and .408 is placed on stack.

1.  in KBARS directory

CST                    (Press CST- Custom Menu Key)

CLR                    (Clear Stack)

4 5 6.03 3 W12 5.11 BLIST

Total=26.0777lbs       (Displayed on top of LCD)

A                      (Press A if you missed it)

"Total=26.0777lbs"     (Contents of A)

BS

VAR                    (Press VAR key to get back to var menu)

Note:  Place all of your bars sets on the stack and press BLIST in the

CST menu.  Here is a trick.  Enter into the MATRIX editor and

enter your data as an X by 3 matrix.  Store it in LST.  Press LST

(I put the example in LST), press PRG OBJ OBJ-> BS.  Now all of

your data is on the stack and you can press BLIST from CST menu.

Press CST and then BLIST.

To make a stack of entries into a matrix:  Reverse what we did

above.  Do not Press BLIST, let's place the stack back into a

matrix.  DEPTH 3 / 3 2 ->LIST ->ARR.  You will get the LST array.

NOTES:  I hope you enjoy this program.

Several programs that I have written are listed in civil1-2.zip

in the readme.1st file.

BIO: - Kenneth Hobson wrote this document for you.

Ken graduated in Spr.'93 and is an Intern I Civil Engineer for the

Oklahoma Department of Transportation.  405-521-2677

My friend Dewayne Cushman wrote BARS.  I wrote KBARS and this DOC.

Modify anything you like except the SCTR.  Please send me a copy if you do

and I will document it and upgrade this version of BARS or KBARS.

khobson@telepath.com

Regards,  Kenneth Hobson          kenneth.hobson@torii.nova.com

kenneth.hobson@s-box.misc.uoknor.edu

work  internet address: kenneth.hobson@dsibm.okladot.state.ok.us