(Comp.sys.hp48)

Item: 154 by dougc at bert.cs.byu.edu

Author: [Douglas R. Cannon]

Subj: HP48 currents for various operations

Date: Fri Nov 08 1991

I was playing around the other day and decieded that I wanted to know just

how much current the HP48 uses with different operations.  Everyone is always

saying, "IR transmissions cause a great drain on the batteries..." or,

"Lots of number crunching will cause significant battery drain..." So, I

wanted to know how much.

In the hopes that this is interesting to others, I am posting a partial

list that I have begun to compile.  These are simple operations that I was

able to think of in 10 minutes or less.  If anyone has ideas for other

operations that should be tested, please email me.  I would like to make this

list as complete as possible.  Also, any comments, or questions are readily

accepted.

This is the current drawn from the batteries, given the operation:

Current |       Operation                       |  Other

( mA ) |                                       |

------------------------------------------------------------------------

0.016   | HP OFF                                | probably not reliable

2.22    | idle                                  |

7.0     | "normal" program execution            | ( most programs )

7.0     | Stack manipulations                   |

7.0     | yat 1.0 ( Tetris... )                 |

6.62    | BEEP                                  | 600 Hz

8.84    | BEEP                                  | 8000 Hz

8.85    | BEEP                                  | 10000 and above

7.0     | XMITRM ( IR remote controller )       | 16.0 mA briefly

< 4.6   | Clock on in status line               | fluctuates (2.22 - 4.6 mA)

3.6     | Command line input                    | idle

2.27    | Stack depth of 500                    | idle

6.8     | Plotter                               | sin(x)

< 3.9   | Edit mode                             | ( 3.6 - 3.9 )

7.2     | Edit mode when checking syntax        | or moving cursor

--------| WIRE TRANSMISSIONS -------------------|-------------------

2.22    | idle, port closed                     |

3.22    | idle, port open! (connected)          | connected to a Mac

2.52    | idle, port open  ( not connected )    |

8.15    | transferring to a mac                 | maximum

--------| IR TRANSMISSIONS ---------------------|-------------------

2.22    | idle, port closed                     |

3.22    | idle, port open (wire connected)      | figure this out!

2.52    | idle, port open ( wire not connected )| this one too.

7.2     | Transmitting, trying to connect       | couldn't test a real trans.

15.7    | Printing ( no printer near by )       | All time high!

--------------------------------------------------------------------

A couple of notes:

Pressing ON does not close the I/O port, but turning the HP OFF, then ON,

the port is closed.  ( I guess the manual wasn't kidding when it said to

always close the port! )

Notice the clock.  The ammeter showed a lot of fluctuation, but I am guessing

that HP draws 2.22 mA as normal, then each time an interrupt is received for

the clock, it momentarily draws around 4.6 mA.

--------------------------------------------------------------------

So that you can try this at home...

How it was done:

It was rather simple, of course.  I just took out one of the batteries,

connected it back in with wires, placing my ammeter in series with the

batteries.  Then, nervously, I turned it on.  Since it didn't explode, I was

able to make the measurements shown.  I feel that the measurements are good,

since the ammeter I used has only a 250 mV maximum burden at fullscale

of 200 mA.  The only measurement I need to do more accurately is when the

HP is off.

---------------------

Douglas R. Cannon

dougc@bert.cs.byu.edu

---------------------