comp.sys.hp48 #20288 (0 + 11 more)                                         [1]

From: fredkan@ix.netcom.com (Fred K. Anderson)

[1] free HP 48 EE programs chk it out !!!

Date: Wed Jul 27 04:29:45 EDT 1994

Organization: Netcom

Lines: 814

Distribution: world

NNTP-Posting-Host: ix-la1-21.ix.netcom.com

Hi everyone, here is the Electrical Engineering Apps I promised.

This file has NOT been coded by UCODE, please do not attempt to

run uudecode to retrieve this file. Instead, cut off the headers

of this message up to and including the lines that say CUT here

which I have placed at the TOP and Bottom of the file. Choose a

file name that is typical of an hp variable name ie. EEAPPS. The

first line of the ASCII text file should read %%HP: T(1)A(D)F(.);

and the last line should read END. To transfer these programs

from your PC to your HP you must have access to a PC link program

such as KSRV48.EXE as well as a serial port cable which connects

from the HP 48 GX to a PC.

These programs will only be found useful for EE ET and EET majors

for your circuit analysis 1&2 and solid state courses. Some apps

will also be found useful for more advanced courses and also for

working engineering professionals. If your are not an EE major,

you probably wont have much use for any of the apps in this

collection but if you are interested in programming you can see

some programming techniques I have used. The author of these

programs (Fred Anderson) does NOT claim to be an GENIUS or does

he guarantee the accuracy of these APPS. The programs widely very

in complexity, some APPS containing two lines of code while some

are well over 1k long. Many of these APPS have been widely used by

a number of other students who also proclaimed these APPS to be quite

useful. All of the source code may be viewed and modified on an HP as

seen fit. The author (me) Fred Anderson encourages you to freely

distribute these programs to anyone who may find them useful.

DOCS -

Sorry I haven't gotten around to writting any documentation for these

apps as of July 94. I the future I plan to write complete doccumentation

but in the meanwhile, anyone having trouble may post on the HP48 usenet

group or EMAIL to fredkan@ix.netcom.com .While I have implemented a user

friendly input interface into some programs, other apps may require the

user to open the program and see how the variables are entered onto the

STACK. for example the IDST program (current divider rule) you must first

enter the value of the resistors (2 or more) onto the stack and then enter

the amount of current, then hit IDTST. So for example if you had a

350 Ohm a 100 Ohm and a 470 Ohm branch and 25mA going into the branch

you enter 350 100 470 25E-3 then IDST. your results should be 4.77E-3,

16.7E-3, & 3.55E-3 (AMPS) (the current accross each branch) . Kirchoffs

current law say that the current into a branch should equal the current

exiting the branch. Your values added should equal 25E-3. Thats just one

app, theres about 40 apps for EE in this collection. I will post the DOCS

for individual APPS as people make inquiries.  Happy HP hunting !

7:^) fred out

comp.sys.hp48 #20358 (0 + 1 more)                                          [1]

From: fredkan@ix.netcom.com (Fred K. Anderson)

[1] ee apps qwik docs (save this)

Date: Sat Jul 30 06:16:03 EDT 1994

Organization: Netcom

Lines: 89

Distribution: world

NNTP-Posting-Host: ix-la2-21.ix.netcom.com

Heres some quick docs that might help you get started with the software I

wrote.

First Directory contains a few small progams that will be used in later

programs so they are on the first level.

EET -- a subdir with more EE programs

CAP -- enter the Xc (reactance) on the stack and CAP will put your

reactance into appropriate phasor notation. This is useful

when your adding a series of caps and resistors together

CST -- a CUSTOM menu that can be accessed by your CST button containing

all of the MOST essential programs.

IND -- same as CAP but does the appropriate phase notation for an inductor

also very useful for addition.

KILO -- simple multiplier of 1E3

MICRO -- MULTIPLIES BY 1E-6

MILI -- MULTIPLIES BY 1E-3

SRT -- nice program for sorting all your directors and programs in your

current directory. Very useful

STAK -- you can display up to 10 levels of the stack on the graphics

screen. Also very useful

under the EET directory you will see the programs

IDST -- current divider rule, first enter in however many resistor branches

you have and then enter the total number of current that you will

be dividing. This will return the amount of current going through

each branch. No limit on number of branches.

VDST -- voltage divider rule. Resistors in a series will each have a voltage

drop according to the size of the resistor. Enter in each resistor

value then enter the supply voltage and VDST will calculate the

voltage drop accross each resistor.

VZ<IZ

IZ>VZ   OHMS LAW CALULATION

Y>DELTA

DELTA>Y   ENTER IN THREE RESISTOR VALUES AND THIS WILL PERFORM THE LENGTHY

EQUATION OF CONVERTING BETWEEN THE TWO.

ZADDP   ENTER IN A SERIES OF RESISTORS AND REACTANCE VALUES THAT WITH TO

ADD IN PARALLEL, REPEATE THE PROGRAM UNTIL YOU HAVE JUST ONE VALUE

1K OHMS RES 10K OHMS 740 OHMS 540 OHMS ZADDP ZADDP ZADDP ZADDP

RETURNS A VALUE OF 232.4 (OHMS). This also works for addition of

capacitor in series.(make sure you have the caps in their proper

phasor notation.)

ZADDS   ADDS STUFF IN SERIES.

UNDER THE ac directory there are many useful programs including :

BRIDGE -- THIS NIFTY PROGRAM CALCULATES A BALANCED BRIDGE PROBLEM. PUSH

THE BUTTON AND ENTER IN EACH Z1 Z2 & Z3 (THEY ALL MUST BE IN

COMPLEX FORM (X,Y) FOR THE PROGRAM TO WORK. THIS PROGRAM WILL

CALCULATE WHAT VALUE OF Z4 YOU NEED TO MAKE Z1 AND Z3 IN SERIES

BALANCED WITH Z2 AND Z4.

IMPSP --  INPEDANCE CONVERSION FROM SERIAL TO PARALLEL (ENTER VALUES IN

COMPLEX FORM) (X,y)

MESH  --- MESH ANALYSIS (WORKS WITH ONLY TWO LOOPS)

NODAL --- NODAL ANALYSIS

PWEQ -- POWER EQUATIONS, ENTER IN A VOLTAGE AND AN IMPEDANCE AND IT WILL

RETURN SOME USEFUL INFO.

VP>R

VR>P    CONVERSION BETWEEN VP (VOLTAGE PEAK) AND VOLTAGE ROOT MEAN SQUARE RMS

XC   -- CAPACITOR REACTANCE, ENTER IN FREQUENCY ON STACK THEN ENTER THE CAP

VALUE AND IT WILL RETURN THE REACTANCE VALUE

XL  -- SAME AS ABOVE BUT WORKS FOR INDUCTORS

XC -1 -- ANTI-REACTANCE CONVERSION, ENTER FREQUENCY AND REACTANCE VALUE ON

STACK AND WILL RETURN FACE VALUE OF THE CAP.

XL- 1  -- SAME AS ABOVE BUT WORKS FOR INDUCTORS.

w>F

F>w    -- conversion between Rad/Sec and frequency

microF  -- great little program. hit the button, enter in the those confusing

three digit code on ceramic disk capacitors and this will return

the actual value of the Cap in microFarads

ztzl  -- small program returns the complex conjugate of a complex number

I will discuss the AC filters, power correction, resonance, and thevinin

program directories in a later post.

have fun