HP 48SX GROB Format

Many people seem interested in working with HP-48SX graphics objects (grobs)

on their PC's.  To save you some trouble, here is the binary format:

<prolog><length><height><width><data....>

where the first four are 5-nibble fields:

prolog = grob object identifier = 02B1E

length = distance in nibbles from start of length field to (just past) the

object end = F + # of data nibbles

height = height in pixels

width = width in pixels

To make life more interesting, the fields are written backwards (this is

how the CPU reads memory), so e.g. the prolog is written E1B20.

The data starts at the upper left corner of the grob and proceeds

left-to-right, top-to-bottom.  But--each pixel row must be a integral

number of bytes (even number of pixels).  The data may therefore be

padded with garbage bits to satisy this.  And--each nibble is written

backwards compared to the LCD order, so the leftmost pixel in a group

of four is the least-significant bit of the stored nibble.

A blank screen-sized grob looks like this:

E1B20|F8800|04000|38000|000000.... (2176 0'S),

where the |'s are for legibility.  The width of 83h = 131d pixels needs

17 bytes or 34 nibbles, so 40h = 64d rows gives 34x64 = 2176 nibbles of

data. 2176 + 15 = 2191d = 88Fh, which is the length field content.

If the upper-left-most pixel was turned on, the first data nibble would

be a 1.

William C Wicker