. HP48 feature list

Alonzo Gariepy has pointed out to me that people might like to see

a list of HP 48SX features, sent out by the firmware design team

leader, Bill Wickes, to the reviewers before we got the calculators

themselves. Here is a copy of that list. Many thanks, Alonzo and Bill.

Wlodek Mier-Jedrzejowicz,

Space Physics, Imperial College, London

BITNET: MIER @ SPVA.PH.IC.AC.UK

Disclaimer: This information is presented as is. NO CLAIMS about

its sutitability for any purpose are made by HP, my employers or

myself. It is simply a copy of a private communication whose author

has allowed me to make it available to a wider audience.

----------------------------------------------------------------------

Here's a slightly edited, although still not exhaustive list of features

(in no particular order) that I made recently:

...   HP-48SX Feature Summary

Hardware

.7.1"x3.2" footprint with tapered .7" - 1.1" thickness

.single vertical format 49-key keyboard

.64 x 131 pixel STN LCD display

.256K ROM, 32K RAM built in

.Saturn CPU @ 2MHz (note--throughput ~1.5MHz compared to 28S @1MHz)

.2 memory card ports

..--battery backed RAM (32K or 128K)

..--ROM or OTP (OneTime Programmable)

.Bidirectional IR I/O at 2400 baud

.Wired serial port at 1200, 2400, 4800, 9600 baud

..cables available for IBM or Apple computers (~$35)

.3 AAA cells

.List price $350

.Two part-manual (800p) plus quick reference guide

.Brushed nylon case with velcro fastening, pocket for cards and guide

Firmware

(Most HP 28 features not explicitly mentioned are included; "new" means "in

addition to that provided by the HP 28S.")

.HP-28-style multi-object-type RPN calculator, superset of 28 language

.Interactive stack environment, for point-and-shoot stack operations

.Prompts and menus to help beginners

.Coexistent graphics and stack/edit display memory

..Graphics display can be larger than physical display,

..with hardware scrolling

.2-dimensional graphical entry and display of algebraic expressions

.Spreadsheet-like matrix editor

.Redefinable keyboard

.Real-time clock & date display

.ECMA94 character set, 3 display font sizes

.Appointment and control alarms

.Plotting features:

..Eight types of automatic plots:.function, polar,

conic sections, parametric, "truth", scatter,

bar charts, histograms

..Autoscaling option

..Interactive plot environment

...zoom conrol, recentering, cursor readouts, labelling,

...etch-a-sketch, line & circle drawing,

copying and erasing

...integration, differentiation, root-finding

.Graphics features:

..Dot and line drawing

..Boxes and Circles

..Graphics objects of any size

..OR or XOR superposition

..Sub-region extraction

..Pixel or logical addressing

.Unit Management

.."Unit objects"--real numbers with physical dimensions

...allowed in algebraic expressions

..Automatic unit conversion in unit object operations

...arithmetic, integration, plotting, solving

..Interactive unit menus for simple conversions

.I/O

..Kermit protocol built in for calc-to-calc (IR) or

calc-to-calc (serial)

..Server mode

..Low level serial commands

..Print to serial or IR

.Plugin Management

..Merged or independent RAM (max RAM 288K)

..Automatic menu of plugin applications and commands

..Archiving and restoring (also via I/O)

.Mathematics

..Common transcendentals on hard keyboard

..New:

...Symbolic integration (pattern matching)

...Summation function

...Programmable pattern match/replace

..."Where" function for sticky local variable bindings

...Generation of user-defined functions from

algebraic assignments

...decimal to rational fraction conversion

.New supported object types:

..Graphics (replaces HP28 graphic strings)

..Tagged (for labeling objects)

..Unit

..XLIB Name (plug-in command)

..Directory

..Library (plug-in application/commands)

..Backup.(Independent RAM variable)

..Library Data (for use by plug-in applications)

.New programming features:

..CASE structure

..Programmable OFF and CONT

..Autostart for single step; step through subroutines

..Memory size and checksum for objects

..INPUT command (programmable command line)

..PROMPT (HALT with display string)

..Wait-for-key with timeout

..Custom error generation

..One-key typing aids for entire structures

..Extended custom and solver menu control

..Increment and decrement (variable) commands

..Storage arithmetic for all object types

..Global variable commands extended to local variables

..Decompile stack objects into command line

Wlodek Mier-Jedrzejowicz