Subject: v01INF4:  Hp48sx Hardware Registers Document v1.0

BEGIN_DOC hw_reg.doc

.This is my list of hardware registers on the HP48, and their uses. It

is not by any means complete, but is the best I can manage, compiled from

various documents and source codes obtained from seq.uncwil.edu... If anybody

else has amore complete list, please mail me. However, I will not be reachable

at this address after 30th August, so any response after that will be no use.

#00100h: bits 0-1 control the 4 pixel offset of the display.  Useful for

smooth scrolling.  Bit 2 is also involved here but messes up the

scan length.  Bit 3 seems to put the machine into a coma

#00101h: LSBs of the 5 bit contrast control word.

#00102h: bit 0 is the MSB of the contrast.  Bits 1 and 3 control the

voltage difference across the LCD; this is what you modified.

Can't see what bit 2 does.

#00103h: bit 3 is also involved in the LCD voltage control.

#0010Bh: bit 0,1,2,3: \<-, \->, alpha, and alarm indicators

#0010Ch: bit 0,1: busy and I/O indicators; bit 3 must be set for the

indicators to be on.

#0010Dh, #0010Eh, #0010Fh: 11bit register for baud rate:

.#600.9600

.#400.4800

.#200.2400

.#000.1200

#00110h:.R/W    UINTR - UART Interrupt Register:

.Bit 3: Recv Interrupt flag

.Bit 2: Recv Interrupt Enable

.Bit 1: XMit interrupt flag

.Bit 0: XMit Interrupt Enable

#00112h: USTAT: Flag register for uart: Bit 1 RECV Ready, 0 = XMit ready

#00114h, #00115h: RECV - UART reciever. Reading gets most recent IO

..character recieved

#00116h, #00117h: XMit - Uart transmitter register - Writing causes serial

..sending to active IO Port

#0011Ah: Bit 3 is trigger bit of I.R. Diode - Latched, so

.   you have to reset it all the time if sampling from IR. Also contains

.   IR Control Bits, but I dunno what they do exactly

#0011Ch: Bit 3 = I.R. Led Control

#00120h - #00124h: display address base register.  Ususally set to #F097Ch

(or #F09BC with the equation card installed) but you can change it,

to look at any memory you want.  Note that you can't read this

register, just write to it.  Remember to type addresses in

backwards.

#00125h - #00127: amount of overscan per pixel line

#00128h : offset to menu area bitmap, normally 7.

#00129h : bits 0-1 control the location of the menu area, normally 3.  Don't

mess with bit 2, which can totally remap your calculator's display!

ON-C does not fix what happens, but turning it off does, strangely

enough.

----------------------------------------------------------------------------

Julian Taplin....|Home of <<The Computer Graveyard>>!

35 Shelly Garden, CAMBRIDGE, ENGLAND    |Retirement home for all old, tired,

(0223) 324815....|and generally dead hardware!

----------------------------------------------------------------------------

EMail: JS12@UK.AC.CAM.PHX     <<<--- No, it's not strictly my MBox, but it

.....will have to do for now!

//*

END_DOC