HP48SX Keyboard Input

A Guide for the Machine Language Programmer

Copyright 1991 Joe Ervin

1  INTRODUCTION

In this document, we will examine the workings of the HP48 keyboard

hardware and how to do your own keyboard input on the HP48 from

machine language.  Additionally, we will describe how the HP48

keyboard input scheme works during normal operation, and how you can

disable the normal operation and take over direct control of the

keyboard.

2  OVERVIEW OF HP48 KEYBOARD INPUT

During normal operation, the CPU scans the keyboard for key presses

every 1ms.  This 1ms keyboard scan is performed directly by the CPU

hardware with no involvement from software, and therefore has a

negligible effect on CPU performance.  During this automatic keyboard

scan, the entire keyboard is scanned in a single operation.  If no

keys are currently being pressed, then nothing else happens and the

CPU continues executing instructions normally.  If, however, this

automatic keyboard scan indicates that one or more keys are being

pressed, then the hardware interrupts the CPU.  The HP48's interrupt

handler then determines the exact key or keys which are being pressed

and updates the keyboard data structures in memory to reflect the new

state of the keyboard.  The keyboard data structures are described

later in this document.

In addition to determining which specific keys are pressed, the HP48's

interrupt handler also scans all other possible sources of interrupts

and services them appropriately.  In the case of pressed keys, the

interrupt handler additionally schedules a timer interrupt for 1/16 of

a second into the future before returning to the interrupted program.

When the timer expires, another interrupt is generated, and the whole

operation repeats.  As a result, the CPU is interrupted 16 times per

second, for as long as a key is held down.

The use of the timer interrupt is necessary because the keyboard

interrupt hardware only generates interrupts when keys are pressed,

Page 2

i.e.  no interrupts are generated directly as a result of keys being

released.  Therefore, by continually scheduling the timer interrupt to

retrigger an interrupt in the near future, the interrupt handler can

effectively "poll" the keyboard 16 times per second, checking for keys

being released.  This is done so that the "key-released" event can be

recorded in the keyboard data structures, described below.  Machine

language applications can then check these data structures to

determine the exact state of each key on the keyboard.

3  SCANNING THE KEYBOARD FROM MACHINE LANGUAGE

Unlike the automatic 1ms keyboard scan performed by the CPU's

hardware, the interrupt handler must force keyboard scans from

software to determine the exact state of the keyboard.  This is

necessary for the interrupt handler because any interrupts generated

by the 1ms keyboard scan indicate only that at least one key has been

pressed.  The interrupt handler then must scan the keyboard row by row

to determine exactly what keys are being pressed.  This same approach

can be used from machine language programs to scan the keyboard.

Scanning the keyboard from software happens in two general phases; an

output phase and an input phase.  During the output phase, software

causes the HP48 to output signals to the keyboard which indicate the

exact row or rows which are being scanned.  Then, during the input

phase, the software reads the keyboard to see which of the scanned

rows have keys pressed.

The keyboard is wired in a matrix as shown in Figure 1.

Page 3

IN  #20 #10 #08 #04 #02 #01

OUT   (bit)  5   4   3   2   1   0

#100    8         B   C   D   E   F

#080    7       PRG CST VAR  up NXT

#040    6       STO EVL <<< dwn >>>

#020    5       COS TAN sqt pwr inv

#010    4   ON* ENT +/- EEX DEL <==

#008    3   alp SIN  7   8   9   /

#004    2   yel MTH  4   5   6   x

#002    1   blu  A   1   2   3   -

#001    0        '   0   .  SPC  +

(*) The ON key is actually in a column of its own.

The ON key is represented in bit 15 of the data returned

from an IN.4 instruction.  #xxx refers to the keyboard scan

bits driven with the OUT instruction.

Figure 1

3.1  The OUT Instruction.

During the first phase of the keyboard scan, software executes the

OUT.X C instruction, which loads the HP48's output register with the

contents of C.x, and then drives those contents into the key matrix.

The bits in OUT<8:0> are connected to keyboard ROWS <8:0>

respectively.  Each "1" bit in OUT<8:0> therefore drives a HIGH

voltage level out to the corresponding keyboard row.  If any key in

that row is currently pressed, then the HIGH level propagates to the

bit of the input register which corresponds to the COLUMN of the key

being pressed.  Normally a HIGH voltage level on the input port causes

an interrupt, but we will show later how to disable this interrupt so

that the machine language program will be unhindered by the HP48's

interrupt handler.

Page 4

3.2  The IN.4 Instruction

During the second phase of the keyboard scan, software executes the

"IN.4 C" instruction to see if any keys in the scanned rows were

pressed.  In this way, a machine language program can scan the

keyboard to test whether specific keys are being pressed.

With the exception of the ON key, the keys which are "visible" to the

input port is a function of the bit pattern driven during the OUT

instruction.  For example, if the output register contains 1FFh, then

all keyboard rows are visible to the IN register.  If, say, the [<-]

key is pressed (or any other key or keys in that column), then after

the "IN.4 C" instruction executes, bit 0 of the C register will be

set, indicating that at least one key in the rightmost column of the

scanned rows was pressed.  Note that if more than one keyboard row is

being scanned, then the data in IN<5:0> does not indicate a specific

key, but rather a specific column of any of the scanned keyboard rows,

as depicted by Figure 1.  The state of the ON key is always reflected

in bit 15 of the data returned from the IN.4 instruction.

Because of the row/column wiring of the HP48 keyboard, in order to

determine exactly which keys are pressed it is necessary to perform

multiple OUT/IN pairs in which each "OUT.x C" scans only a single

keyboard row.

For example, "OUT.x C" with C.x=001h scans only the bottom row of the

keyboard; C=002h scans the second row from the bottom; C=004h scans

the third row from the bottom, and so on.  In this way, any set bits

in the data returned by the IN.4 instruction indicate that a specific

key is being pressed.  This is what the HP48's interrupt handler does

to determine the exact status of the keyboard after the 1ms keyboard

scan has generated an interrupt due to one or more keys being pressed.

During normal operation, the interrupt handler then updates the

"KeyBuf" and the "KeyState" in memory (described below) to indicate

the key status to the rest of the RPL system.

3.3  Interrupt Woes

In the process of servicing other I/O devices, checking battery

voltage, etc., the interrupt handler executes many hundreds of

instructions which have nothing to do with the keyboard, making the

handling of keyboard input through this mechanism very inefficient in

terms of CPU utilization.  More significantly, however, the interrupt

handler debounces the keyboard by capturing the state of the entire

keyboard repeatedly, waiting 2ms between samples, until it sees

exactly the same keyboard state for 5 consecutive samples.  Because of

this, the interrupt handler is guaranteed to require over 10ms to

detect a single keystroke.

Furthermore, the keyboard service routine in the interrupt handler is

implemented as a loop which synchronizes itself to the 16Hz timer

(TIMER1).  If the keyboard service routine detects any keys being held

down, then after updating the keyboard data structures (see below) it

Page 5

checks to see whether there is enough time before the next 16Hz tick

to do another full pass through the keyboard service routine and if

so, the code loops back to the top of the keyboard service routine.

If the next 16Hz clock tick is less than approximately 17ms into the

future, then the keyboard service routine exits, and the interrupt

handler completes.  However, since the interrupt handler scheduled

TIMER1 to interrupt on the next 16Hz tick, the CPU will bounce back

into the interrupt service routine in just a few milliseconds.  Thus,

holding down a key while the HP48's normal keyboard interrupt service

is in operation causes the keyboard interrupt service routine to hog

approximately 75% of the CPU, leaving the remaining 25% for your

application.

When writing machine language programs that do not require much CPU

power, this may not be a concern and it may be desirable to allow the

interrupt system to handle keyboard input normally.  In this case, the

machine language program can retrieve keyboard status from the two

keyboard data structures described below.  However, for CPU-intensive

applications it is generally desirable to disable the HP48's normal

keyboard interrupts and perform keyboard I/O directly in the

application.

3.4  HP48 Keyboard Data Structures

There are two main keyboard-related data structures that the HP48

keeps in memory.  The first is called the "KeyBuf", and occupies 34

nibbles starting at #704EA.  The second is called the "KeyState" and

occupies 13 nibbles starting at #704DD.  Additionally, there are three

other datum kept in memory which the interrupt handler uses in

conjunction with the keyboard.  These are the "ORshadow" (Output

Register shadow), which occupies 3 nibbles at #704C3, "KBdisable",

which is a single nibble at #704DC, and a two nibble value at #706C3

representing the state of the display annunciator flags at 10B/10C.

These data structures are described below.

3.4.1  The KeyBuf

The first two nibbles of the KeyBuf are the "get" and "put" pointers,

respectively, and the remaining 32 nibbles comprise a 16-entry key

buffer, with each entry occupying one byte.  The "get" pointer

provides an index into the KeyBuf for the next available key code.

Similarly, the "put" pointer provides a index to the next entry in the

KeyBuf to be written.  If the "get" and "put" pointers are equal, then

the buffer is empty.  The "get" and "put" pointers point to byte

locations within the buffer.

You may have noticed how your calculator beeps at you defiantly when

you have exceeded this 16 entry type-ahead buffer.  This is the

interrupt handler telling you that there is no more room in the

KeyBuf.

Page 6

The key codes which are used to represent keys in the KeyBuf are

different than the scan patterns read in during the OUT/IN procedure.

Each key is given a unique 1-byte key code as follows:  key codes are

numbered sequentially from the upper left of the keyboard counting

across and down.  Thus a key code of 1 represents the [A] key, 2 for

[B], 3 for [C]..., #19h for [ENTER],...  #1Fh for [7],...  #31h for

[+].  Note that four keys do not obey this ordering:  [alpha] is #80h,

[leftshift] is #40h, and [rightshift] is #C0h, and [ON] has no key

code.  The ON key is handled as a special case by the interrupt

handler and does not appear in KeyState or in the KeyBuf.

If the keyboard service routine detects the presence of [alpha],

[leftshift], or [rightshift] at the same time as another key, then the

key code for the "shift" key is ORed into the keycode for any

non-"shift" keys which may also be pressed, and the resulting keycodes

are inserted into the KeyBuf.  For example, the "right-shifted"

keycode for the [A] key is [rightshift]![A] = C0h!01h = C1h.

Similarly, the keycode for a "alpha-shifted" [ENTER] is

[alpha]![ENTER] = 80h!19h = 99h.  If one of the shift keys is detected

alone, then it's keycode is simply inserted into the KeyBuf.

The code example in Appendix A shows a simple way to remove keys from

the key buffer.  In addition, this code example shows how to put the

calculator into "light sleep" awaiting a key press.

3.4.2  The KeyState

The 13 nibbles at location #704DD provide a bit pattern which reflects

the status of the keyboard, with each bit representing the state of a

specific key.  This bit pattern is updated whenever a key is pressed,

and is also updated when keys are released via a timer interrupt as

described above.  When a key is pressed, its corresponding bit is set

to 1.  There are 13 nibbles, 4 bits each, making 52 bits.  The [ON]

key is not represented in KeyState.  There are 48 keys remaining, so

four bits are unused.  The low bit is unused, the next bit corresponds

to the bottom rightmost key [+], the one following corresponds to SPC,

then comes period [.], [0], ['], after which immediately follows the

next row:  [-], [3], and so forth, up to the upper leftmost key, which

is [B].  (See Figure 1 above for the physical layout of the keyboard).

Below is a table of all key bit codes.  More than one bit is set if

more than one key is being held down simultaneously.

Page 7

Key    704E7 704E2 704DD            Key    704E7 704E2 704DD

B    |  1  |     |     |           <==   |     |   1 |     |

C    |   8 |     |     |           alp   |     |    8|     |

D    |   4 |     |     |           SIN   |     |    4|     |

E    |   2 |     |     |            7    |     |    2|     |

F    |   1 |     |     |            8    |     |    1|     |

PRG   |    8|     |     |            9    |     |     |8    |

CST   |    4|     |     |            /    |     |     |4    |

VAR   |    2|     |     |           yel   |     |     |2    |

up   |    1|     |     |           MTH   |     |     |1    |

NXT   |     |8    |     |            4    |     |     | 8   |

STO   |     |4    |     |            5    |     |     | 4   |

EVL   |     |2    |     |            6    |     |     | 2   |

<<<   |     |1    |     |            x    |     |     | 1   |

dwn   |     | 8   |     |           blu   |     |     |  8  |

>>>   |     | 4   |     |            A    |     |     |  4  |

COS   |     | 2   |     |            1    |     |     |  2  |

TAN   |     | 1   |     |            2    |     |     |  1  |

sqt   |     |  8  |     |            3    |     |     |   8 |

pwr   |     |  4  |     |            -    |     |     |   4 |

inv   |     |  2  |     |            '    |     |     |   2 |

ENT   |     |  1  |     |            0    |     |     |   1 |

+/-   |     |   8 |     |            .    |     |     |    8|

EEX   |     |   4 |     |           SPC   |     |     |    4|

DEL   |     |   2 |     |            +    |     |     |    2|

Figure 2  KeyState

3.4.3  KBdisable And ORshadow

As with any computer system that utilizes interrupts, the servicing of

interrupts must be transparent to the currently running application,

aside from the time delay associated with servicing the interrupt.  In

order to do this, the entire state of the CPU is saved at the

beginning of the interrupt handler, and restored again at the end.

Unfortunately, the contents of the OUT register is not readable, so it

is not possible to directly save its contents.  Since the interrupt

handler invariably modifies the contents of the OUT register, the HP48

interrupt system requires applications to maintain a copy of the OUT

register in RAM.  The 3 nibbles stored at #704C3, designated as the

"ORshadow", is used for this purpose.

The ORshadow is used by applications to shadow the contents of the OUT

register.  It is the responsibility of any application which intends

to use the OUT register for its own purposes to modify the ORshadow

along with the OUT register.  By doing this, the interrupt handler is

provided with a readable copy of the contents of the OUT register so

that if an interrupt should occur while the application is using the

OUT register, the interrupt handler will restore the OUT register to

Page 8

its correct contents before returning control to the application.

The KBdisable flag is a single nibble which applications can write

with a nonzero value to indicate to the interrupt handler that the

keyboard is currently being scanned by an application.  When this

nibble is nonzero, the interrupt handler will not run the keyboard

service routine.  This is very useful for applications which have

their own keyboard input routines as it allows the programmer to

prevent the high CPU utilization by the interrupt handler as discussed

above.  The programmer should be aware, however, that the annunciator

flags will still be updated when the corresponding keys are pressed,

unless interrupts are disabled altogether as described below.

3.4.4  Utilizing The Built-in Data Structures

Using the KeyBuf and KeyState data structures for your keyboard input

is very straightforward.  The routine provided in Appendix A shows how

to access the KeyBuf, and accessing the KeyState is trivial.  An

application need only examine the appropriate bits in KeyState to

determine if any given keys are being held down.  For applications

which can tolerate the additional CPU load of the interrupt handler,

it probably makes sense to just allow the normal interrupt mechanism

to service the keyboard, and then for the application to utilize the

KeyState and KeyBuf data structures.  It is generally only for

applications which perform CPU intensive or time-critical operations

that manual keyboard scanning techniques are required.  The following

sections will discuss this issue further, as well as how to write your

own keyboard input routines.

4  CUSTOM KEYBOARD I/O

The main motivation for writing your own keyboard input routines is to

steal back valuable CPU time from the interrupt system for

applications that need it.  In order to recover the CPU time that the

HP48 normally uses up in the keyboard interrupt service routine, you

will need to write your own keyboard I/O routines, or you can just

cut/paste the routines given later in this document.

4.1  Controlling The HP48 Interrupt System

Writing your own keyboard I/O routine does little good unless one also

disables the normal keyboard interrupt mechanism.  Fortunately, there

are a few options open to the machine language programmer in this

regard.  Described below are some general approaches to disabling

normal keyboard servicing.

Page 9

4.1.1  The Big Hammer

The first method, which we will call "The Big Hammer", is to clear bit

15 of the status register.  This bit is checked at the top of the

interrupt handler, and if it is clear then the interrupt handler

disables further interrupts and returns to the interrupted program.

Thus, clearing ST<15> effectively shuts off all I/O on the calculator,

namely the keyboard.  The interrupt system also sets ST<14> to

indicate that an interrupt request has been posted but was not

serviced.

One disadvantage to the "big hammer" approach are that once you clear

ST<15>, if a code bug causes your program to "hang" with ST<15>=0, you

have no control over the calculator.  You can't turn it off; you can't

do [ON]-[C]; you can't do [ON]-[A][F].  The only thing you _can_ do is

to pull off the rubber foot hiding the reset button and jam a paper

clip in the hole.  Not much of a way to quit your application.

Another problem is that preventing certain interrupts from being

serviced for extended periods of time can lead to problems in the

calculator.  Since the interrupt handler does nothing if ST<15> is

clear, none of the possible interrupt sources in the HP48 will be

serviced.  This includes, among other things, the low battery detect

circuitry.  Hence if a machine language program which has cleared

ST<15> is left running, it can drain the batteries completely,

resulting in total loss of memory.  Normally the low-battery detect

circuitry would interrupt the CPU, allowing the interrupt handler to

safely shut the system down into a very low power consumption state,

thus preserving RAM.  With ST<15> clear, however, this safety net is

removed.  For programs which run for durations of less than several

hours, however, this should not be a problem.

Other sources of interrupts such as timer rollover and serial I/O

activity will be totally ignored as long as ST<15> is clear.  However,

the lack of interrupt-driven serial I/O capabilities may not be an

issue for many applications, and interrupt requests due to rollover of

the 32-bit hardware timer (TIMER2) can be ignored for 72 hours without

effecting the calculator's sense of time.

An example of when clearing ST<15> can be particularly useful is when

the keyboard is actually being scanned.  In this way, software can

avoid the need to shadow the OUT register in ORshadow.  This works

because no interrupts are possible while ST<15>=0.  The application

should then set ST<15>=1 after completing the keyboard scan to allow

other interrupts, if desired.  See the ENABLE_INTR routine in Appendix

B for more information on how to re-enable interrupts after disabling

them via ST<15>.

4.1.2  The Little Hammer

Another approach to disabling the keyboard interrupts is to shut off

keyboard scanning at the source by executing an INTOFF instruction.

Page 10

This disables the automatic 1ms keyboard keyboard scan described

above.  As a result, interrupts no longer occur due to key presses,

with the exception of [ON] which always causes an interrupt.

Furthermore, since we have not disabled I/O altogether such as is the

case when ST<15> is cleared, we can still abort our ML application by

doing [ON]-[C] if the need arises.

The only minor pitfall to this technique is that the INTOFF

instruction prevents _only_ the keyboard interrupts.  If one of the

other devices in the system causes an interrupt, then the interrupt

handler will still execute.  As it turns out, the vast majority of the

time spent in the interrupt handler is due to the keyboard service

routine.  Therefore, aside from the keyboard service routine in the

interrupt handler, allowing interrupts does not cost very much in

terms of CPU time.

Fortunately, the keyboard service routine can be completely disabled

by writing the KBdisable nibble to a nonzero value, so that if an

interrupt should occur, the keyboard service routine will not be

executed.  The programmer should note that the [alpha], [leftshift],

and [rightshift] keys are polled outside the keyboard service routine

and the corresponding display annunciators updated.  Unfortunately,

there is no way to prevent this from happening aside from turning off

interrupts altogether via ST<15>.  This is generally not necessary,

however, since in applications which do not use serial I/O, interrupts

should not occur.

The programmer should be aware that the interrupt service routines for

the serial I/O contain INTON instructions, so if serial I/O is used,

then INTOFF instruction will need to be repeatedly executed to prevent

keyboard interrupts from occurring.

4.1.3  Recommendations

Because of the different features built into the HP48 interrupt

service routine, there are several approaches that a ML programmer can

take to disable normal keyboard operations.  While each technique has

it's own strengths and weaknesses, there are some general programming

practices that can lead to "cleaner" solutions.  Below are a few hints

to keep in mind with respect to programming for custom keyboard input.

1.  Use INTOFF as a general technique to disabling keyboard

interrupts, with critical "uninterruptable" sections of code

protected by clearing ST<15>.  This generally works well,

although there are a few things to keep in mind.

-  After re-enabling interrupts by setting ST<15>=1, you

need to check ST<14> to determine whether an interrupt

was requested while interrupts were disabled.  ST<14>=1

means an interrupt was requested but has not yet been

serviced.  Basically what happened is that an interrupt

occurred, but since ST<15> was clear the interrupt

handler immediately terminated via a RET instruction

Page 11

rather than servicing the interrupt and terminating with

an RETI instruction, as it does normally when ST<15>=1.

The interrupt handler sets ST<14>=1 to inform the

application that an interrupt is "pending".

The implication of this is that the HP48 believes that it

is still executing in the interrupt handler (since no

RETI has been executed since the last interrupt) and will

not allow further interrupts until an RETI instruction is

executed.  See the ENABLE_INTR routine in Appendix B for

an example of how to handle this.

-  The Ticking Clock display or any user alarms which come

due will be serviced whenever interrupts are enabled.

This can lead to the INTON instruction being executed in

the interrupt handler.  Once this occurs, then it is

possible for keystrokes to cause interrupts.

-  If the Keyboard interrupt service routine has been

disabled via the KBdisable flag, then keypresses should

not make it into the KeyBuf, even if interrupts occur.

If the application does not modify the KBdisable flag,

then the KeyBuf may need to be periodically flushed to

remove any keypresses that sneak into the KeyBuf.

-  Because any interrupts may lead to the execution of the

INTON instruction, the application should periodically

execute an INTOFF instruction.

2.  Use "ST<15>=0" sparingly.  Try to keep sections of code which

are protected from interrupts via the clearing of ST<15> as

small as possible, and try to keep the "CLRB 15, ST" and

"SETB 15, ST" instructions as local to each other as

possible.  Remember, if you make a mistake and leave

ST<15>=0, all keyboard control is lost.  Try to keep

"uninterruptable" sections of code as small as possible, such

as in the code example of Appendix B when the OUT register is

being modified.

4.2  Example Keyboard Input Routines

The code example in Appendix B shows how custom keyboard I/O routines

can be written.  The code shown in this example is actually the

keyboard scanner process which runs in the game program "Vaders".

Vaders was written using MPE, a machine language multiprogramming

environment for the HP48.  The keyboard scanner process in Appendix B

runs concurrently with the other processes which make up the Vaders

game, making the keyboard scanning transparent to the other processes

which comprise the game.

The example code maintains two data structures which reflect the

Page 12

status of the keyboard in a manner similar to the "KeyBuf" and

"KeyState" data structures maintained by the HP48's interrupt system.

APPENDIX A

KEYBUF CODE EXAMPLE

;;+

;;

;; Keyboard Interface.

;;

;; Keyboard scan codes are numbered 1 for [A], 2 for [B], 3 [C]...

;; #19h [ENTER]... #1Fh [7]... #31h [+]. [alpha] is #80h, [yellow] is

;; #40h, and [blue] is #C0h. ON has no scan code.

;;

;; This program reads the keyboard buffer. If a key is present, it

;; returns it in A.A. If no key is present, it enters light sleep and

;; waits for one.

;;

;; kb_poll polls the keyboard buffer, carry set if non-empty, key in A.A.

;; kb_get does the same, but waits until a key is pressed.

;;

;; Jan Brittenson, April 1991

;; This program is in the Public Domain

;;

;;-

radix   ^d16

event_mask = 10e

;; Poll keyboard buffer

kb_poll:

move.5  keybuf+1, d0    ; KB Put ptr

move.s  @d0, a          ; A.S = put ctr

dec     d0

move.s  @d0, c          ; C.S = get ctr

breq.s  c, a, $100      ; Ctrs are equal - buffer empty

move    c.15, p         ; P = get ctr

inc.s   c               ; Remove key

move.s  c, @d0

swap    c, d0

add     p+1, c

add     p+1, c          ; C += get ctr, in bytes

KEYBUF CODE EXAMPLE                                           Page A-2

clr     p

move    c, d0           ; D0 = &next key

clr.a   a

move.b  @d0, a          ; A.A = key

retsetc

$100:

clr.a   a

retclrc

;; Wait for a key to become pressed, then return scan code in

;; A.B. Uses C.A and B.B.

kb_get:

call    kb_poll         ; Get key, if any

retcs                   ; Return if there was a key in the buffer

; No keys are down - enter light sleep

move.5  event_mask, d0

move.p1 8, c

move.1  c, @d0

rsi

shutdn                  ; Go asleep

move.p1 0xc, c          ; Restore event mask

move.1  c, @d0

jump    kb_get          ; Check buffer again

APPENDIX B

CUSTOM KEYBOARD I/O ROUTINES.

;*********************************************************************

;*********************************************************************

; This process is responsible for scanning the keyboard and updating

; the KEY_SCAN and NEW_KEYS data structures.  The routines GET_KEYS

; and GET_NEW_KEYS are used to check these two data structures,

; respectively, for key presses.

keybuf          =       ^x704EA ; keyboard buffer.

DO_IN_4         =       ^x1160  ; Does IN.4 C.

KEY_SCAN:       DATA.w  0       ; This data structure holds a shadow

; of the status of the keyboard.  Each

; bit represents a key, although some

; bits are unused.

NEW_KEYS:       DATA.w  0       ; This word indicates whether any keys

; have been newly pressed.  Each bit

; represents one key, although some

; bits are unused.

GET_KEYS:       ; This routine just looks in the key_scan data for

; the presense of any of the keys specified in the

; key mask in C.w.

addr    key_scan, d0    ; Address of key_scan data in D0.

move.w  @d0, a          ; Get key status.

and.w   a, c            ; Allow only the selected keys.

ret

GET_NEW_KEYS:   ; This routine checks the new_key data for new key

; presses. The calling procedure must supply a key

; mask in C.w.  Any new keys selected by the mask

; are cleared from the new_key data.  A nonzero value

; is returned in C.w representing which of the selected

; keys were pressed.

addr    new_keys, d0    ; Trashes A.

CUSTOM KEYBOARD I/O ROUTINES.                                 Page B-2

move.w  @d0, a          ; Get new_keys.

and.w   a, c            ; Allow only selected keys.

not.w   c               ; Invert result and apply to A

and.w   c, a            ; to clear selected keys from new_keys.

move.w  a, @d0          ; Write updated new_keys data.

not.w   c               ; Invert result and apply to A

ret

ENABLE_INTR:                    ; This routine turns interrupt servicing back

; on after an interrupts were disabled by

; clearing ST<15>.  This routine checks for

; pending interrupts and ensures that they are

; serviced.

setb    ^d15, st        ; Reenable IO interrupt service.  We want to

; do this so that any important interrupts in

; the system will still be recognized.

brbc    ^d14, st, $1    ; If ST<14> is set, then we have missed an

; interrupt while ST<15> was clear, so we

; need to re-enable interrupts.

clrb    ^d14, st        ; Clear the pending interrupt flag.

RSI                     ; Reset the keyboard interrupt state machine.

$1:     RETI                    ; Re-enable interrupt servicing.  If there is

; an pending interrupt, it will be serviced

; now.

PROCESS8_INIT:

; R0:   Holds current key status.

; R1:   Holds the old key status.

; R2:

; R3:

; R4:

cur_process_start process8_code

addr    new_keys, d1    ; D1 holds pointer to new_keys data.

addr    key_scan, d0    ; Get address of the key scan bit pattern.

clr.w   a

move.w  a, r0

move.w  a, r1           ; Initialize current and old key status.

move.w  a, @d0          ; Clear the keyboard scan pattern.

move.w  a, @d1          ; Clear new_keys.

PROCESS8_CODE:

intoff                  ; Shut off the system 1ms keyboard scan, again.

; This is necessary because

; if other interrupts have occurred, then the

; INTON instruction may have been executed in

; the interrupt handler, re-enabling keyboard

; interrupts.

CUSTOM KEYBOARD I/O ROUTINES.                                 Page B-3

move.5  keybuf+1, d0    ; Point at "put" pointer.

move.1  @d0, c          ; Get the "put" pointer.

dec     d0              ; Point at "get" pointer.

move.1  c, @d0          ; Flush the keybuf just in case any characters

; have snuck into the keybuffer.

addr    key_scan, d0    ; Get address of the key scan bit pattern.

; First we just want to check to see if any keys are being pressed.

clrb    ^d15, st        ; Shut off IO interrupt service for now.  This

; needs to be done here so that any interrupts

; generated in the process of scanning the

; keyboard will be ignored by the system.

; Keyboard interrupts are possible because

; if other interrupts have occurred, then the

; INTON instruction may have been executed in

; the interrupt handler, re-enabling keyboard

; interrupts.

clr.w   c

move.p3 ^x1FF, c        ; OUT value for entire keyboard.

out.x   c

call.a  do_in_4

clr     p

brnz.w  c, $1           ; If no keys are pressed we just fall through.

clr.w   c

move.w  c, @d0          ; Zero out the key_scan scoreboard.

move.w  r0, a           ; Old "current" key status.

move.w  a, r1           ; R1 holds the old key status.

move.w  c, r0           ; R0 holds the current key status.

jump.4  process8_sched  ; Go reschedule the process.

$1:     ; Now we need to scan the keyboard and build up the key scoreboard.

addr    new_keys, d1    ; D1 holds pointer to new_keys data.

clr.w   a               ; Temp storage for scanned keys.

clr.w   c

move.p3 ^x100, c

move.w  c, d            ; Leave a copy in D, where it will be shifted.

$3:     move.w  d, c            ; Put scan pattern into C.

out.x   c               ; scan the next row.

call.a  do_in_4         ; Read the keys.

add.w   a, a            ; Shift the data in A 6 bits to the left.

add.w   a, a

add.w   a, a

add.w   a, a

add.w   a, a

add.w   a, a

OR.w    C, A            ; Add in the new keys to the data in A.

srb.w   d               ; Shift to look at next keyboard row.

brnz.w  d, $3           ; Do another row if not done yet.

move.w  a, @d0          ; Write out the scanned keyboard data.

move.w  r0, c           ; Old "current" key status.

move.w  c, r1           ; R1 holds the old key status.

move.w  a, r0           ; R0 holds the current key status.

CUSTOM KEYBOARD I/O ROUTINES.                                 Page B-4

not.w   c               ; Invert the old key status...

and.w   a, c            ; and AND in the new keys.  The resulting word

; in C indicates the "new" key presses.

move.w  @d1, a          ; Get existing value of NEW_KEYS data.

or.w    a, c            ; Add in the new keys without destroying any

; "new" keys that haven't been serviced yet.

move.w  c, @d1          ; D1 points to New_keys data.

clr.w   c

move.p3 ^x1FF, c        ; OUT value for entire keyboard.

out.x   c               ; Just to reset things so when we exit to

; the RPL environment the system can still

; read the keyboard.  This is needed because

; the 1ms keyboard does not reload the OUT

; register.

PROCESS8_SCHED:

call.4  save_context    ; Save the process context.

call.4  enable_intr     ; Re-enables I/O by setting ST<15>=1 and

; executes an RETI instruction to allow

; interrupt servicing.

CLR.W   c

MOVE.P3 ^xFF, c         ; Reschedule this process to look at the

; keyboard roughly 30 times per second.

call.4  resch_cur       ; Reschedule the current process.

PROCESS8_EXIT:

JUMP.4  TO_SCHEDULER    ; Return control to the scheduler.

; Thus ends process #8.