Sattrack Documentation File.

Version 1.1  April 1994

Sattrack is intended to produce a table of time and position

data when a desired satellite will be found above an observer.

This information is useful to Amateur Radio operators who

can communicate with satellites only when they are above the

horizon.  It may also be useful to the curious to find out when

the Space Shuttle STS-59 or the Russian space station MIR are

overhead.  If you have a scanner, you can listen to MIR and

STS-59 (scheduled to launch on April 8, 1994) in the 2 meter

Amateur Radio band at 145.550 Mhz.  Both stations will usually

be running packet (you'll hear short bursts of tones that sound

like noise), but sometimes they will switch over to voice.  For

those that don't have radios, these two objects are actually

quite visible when they pass overhead at dawn or dusk when they

are illuminated by sunshine but it is still dark on the ground.

I have no claim to the algorithm, I only converted it to

run on the HP48S/SX and the HP48G/GX.  I have not tested

it on the HP48G/GX calculator, but since it is all user

RPL, I don't forsee any problems.  I have compared the

HP48 calculations with the C program and another program

called "STSPLUS", and found the HP48 to usually be within

a minute of the the other two programs.

To make the program accurate for your location, you will

have to change the location data if you don't live in

Colorado Springs, CO.  However, if you want to test the

program to make sure all is well, see the note at the

end of this file.

Before running the program, you will need to change a few

things.  You can either edit the source file and then

download it to your calculator, or download the file and

then edit on your calculator.

The first thing to change is your location data.  Find the

variable "LOCATION.DAT".  The first value in the string

is your latitude in decimal degrees.  Degrees north is a

positive number.  The second value is your longitude also

in decimal degrees.  Degrees west is a positive number.

The last value is your elevation above sea level in meters.

If you don't know these values for your location, and

you have telnet access, you can try:

telnet martini.eecs.umich.edu 3000

and enter your town name or zipcode.  If found, it will

come back with all three values you need.  You only need

to convert them to the proper units.

You will at some point also want to change "ELEMENTS.DAT"

to update the satellite elements or add more satellites.

The program will handle any number of satellites, but is

limited by your patience and your calculator's memory.  The

element data must be in the form of one string ("....").

(If you load this file on a workstation, edit out all the

carriage returns (^M) before loading the file.  Otherwise

all the satellite names will have a square dot after them.)

The format of the data is the "Two-Line Orbital Element

Set Format" and is also referred to as the NASA or NORAD

format.  Updates of the elements are posted to

rec.radio.amateur.space.  I've taken some effort to parse

out the data properly, but things will work better if you

keep the element data in the standard 2-line element format

and don't insert or delete lines.  Also, there should be no

leading spaces at the beginning of each line.

After you've made these changes, press the CST to bring

up the custom menu.  To start the program, press "GO".

The program will then read your location, and then prompt

you for which satellite you want to track.  Only the last

7 characters of a satellite's name is displayed.  You can

press the "NEXT" or "PREV" keys to roll forward or backward

through the list of satellites.  When you see the number

of the satellite you want, enter it and then press "CONT"

to go on.  If you just press enter, the data is "entered",

but the program will won't continue until "CONT" is

pressed.

The program will then read in the orbital element data

for that satellite.  You must then enter the time period

of interest.  I've provided default answers to all the

questions.  The starting date is the present date.  The

stop date is the same which will allow calculation for

the present day only.  The starting time is defaulted to

the present time adjusted for UTC.  There is a simple

calculation to determine the correct time zone and

daylight saving flag to arrive at the current UTC time

value.  Time zones don't follow longitude exactly, so

there may be an hour of error here.  You can still just

change the time to what you want.  The stop time is

assumed to be at the end of the day.  Note that the

time format is in 24 hour form, with the hours before

the decimal point and the minutes after.  Also note that

the program uses UTC time, and displays the results in

UTC.  (UTC is universal coordinated time, which used

to be called Grenwich Mean Time or Zulu time I believe.)

The last question is the time increment.  I've found 1

minute works fairly well for MIR and space shuttle

orbits.  Satellites in higher orbits move slower, and

you may want to increase the time increment to help

speed up calculations.  Any entry can be edited by

simply using the back-arrow or left-arrow keys and then

entering the new data.

When all time data is entered, the program begins to

calculate.  I have set a short beep to occur on each

iteration.  Since the program is not very fast, I change

the time increment value by 10X when the satellite is

below the horizon.  This cuts down on the number of

calculations, but sometimes results in an "overshoot"

where you will see the satellite above the horizon a

minute or two late.  The beep has a high pitch when 10X

multiplier is in effect, and a low pitch the rest of

the time.

When the satellite is above the horizon, you will see 5

columns of data displayed.  The first is the current time

in UTC.  To get local time, you will have to adjust this

value.  For mountain daylight time (MDT), I subtract 6

hours.  The second column is the azimuth value in degrees.

This is the direction you would have to face to "see" the

satellite.  0 degrees is north, 90 degrees is east, 180

degrees is south, and 270 degrees is west.  The third

column is altitude in degrees.  This is the angle above

the horizon you would have to look up to see the satellite.

0 degrees is at the horizon, and 90 degrees would be

straight above you.  The fourth column is the range.  This

is the distance between you and the satellite in kilometers.

The last column is the phase, and I don't know what it is

useful for.

When the screen is filled up, the program pauses.  At this

point you can examine the data.  To continue, press any

key except the "ON" key.  If you press the "ON" key the

program will terminate and the display will return to

normal.

The SHOW key in the custom menu just recalls the satellite

data you previously selected to the stack.  You can view

this to see how old your element data for the satellite

is.  The time is usually in the fourth field of line 1.

It will start with 94nnn.mmmmmmmm which indicates the

year 1994.  The "nnn" is day of the year numbering from

1 for January 1 to 365 for December 31.  The "mmmmmmmm"

are the hours and minutes expressed as a fraction of a

day.  For the data I've included, MIR is 94090 which is

March 31.  STS-59 is 94098 which is April 8, the date which

is scheduled for launch.  If the launch is delayed again,

this element data won't be valid.

Program Testing Example

Download the sattrack.rpl as shipped and start the program

by pressing the CST key, and then the "GO" softkey.  Select

MIR as the desired satellite.  Enter the starting date as

"4.081994" (Apr 8, 1994).  The ending date is not different.

Enter the starting time as "6.30" and enter the default

time as the ending time.  Leave the time increment at one

minute.  The first display should read:

POSITION LOCATION FOR: MIR

TIME    AZ     ALT    RANGE  PHASE

4/8/94

6:30   167.8   0.3   2243   188

6:31   158.2   2.8   1992   191

and so on.  This indicates that MIR will rise in the south-

southeast at 12:30 AM since I live in Colorado and have to

subtract 6 hours from the displayed UTC time to get mountain

daylight time.  The rest of the lines indicate it will climb

to a maximum of 6 degrees elevation in the southeast at

12:33 AM, and then go out of sight at 12:37 AM in the east

(hit the ENTER key to see the next screen of data after the

pause from the first screen).

Happy satellite hunting!

Dave Kurth  N0UVR

dfk@col.hp.com   n0uvr @ w0lkd.#seco.co.usa.noam