Statistical Process Control

for The G\GX

version 2.4

By Othniel Rawlins

ostcroix@aol.com

This program is freeware; it is copyright (c) 1995

by Othniel Rawlins. This program my be  Redistributed freely as long

there is no exchange of cash. Although I spent hours debugging this

program, use it at your own risk; I am not responsible for any

damages or memory lost to your calc as a result of using this program.

REVISION HISTORY FIXES

=====================

Version 2.4                     ==

ADDED                 The ATTN has been error trap to pop

back to the previous display during

execution of STAT HIST X&R and PRINT.

ADDED                 SPC can now tell difference between

printing via wire and printing via Infrared.

That change was influence by

Jarno Peschier (jpeschie@cs.ruu.nl)

HP82240B infrared printer emulator

ADDED                 Depressing LS with uparrow or

downarrow repeats a page

FIXED                 fix insufficient memory problems after

control charts are printed

FIXED                 Backarrow key to abort better

CHANGED               Rewrote routines of browsers for faster

scrolling and updating of display

CHANGED               TDS variable is no longer used. I switched

to vardat to avoid conflicts with future

versions.

CHANGED               Got rid of a lot of nagging unsupported

entries; however, a few still lurks

======

Version 2.3a         The ENTER key crashed the calc

when COPY or MOVE are suspended. FIX!

I also forgot to mention that x&r chart scrolls

horizontally using left arrow & right arrow

========

Version 2.3          First release to the public

Acknowledgements

================

Jacklevy     - Thanks to the author of Qtools for posting this

SPC program to the ftp sites for me. Thanks again

Robert Tiismus  -  Thanks for the fast grob rotation program

that rotate my control chart grobs so I can

PRINT

INTRODUCTION

============

This program is a real world program on the subject of

Statistical Process Control. I am assuming the user have a

basic knowledge of SPC. I do not go into lengthy details on

how to interpret SPC charts, histogram or any Statistical info;

they are many books on that subject. My focus is getting the user

familiar with the functions of this program so SPC task can be

accomplish on the HP48.

Setting up SPC

==============

Download SPCV24.LIB library and DATA  to your calculator.

If your are not familiar with using a library, refer to owners

manual under library. Data is a HP48 directory of

example data that I have collected from the book: Statistical

Quality Control by Eugene L. Grant & Richard S. Leavenworth,

the six edition. It is a good idea to use that directory to get

familiar with the functions of SPC

Execute SPC to get into the directory/array browser.

The directory/array shows the current directory path along

with two types of objects: directories and arrays. Arrays will

always be displayed in the following format:

XXXXXX:[SSxSG]

XXXXXX - data name

SS     - Sample size

SG     - Subgroup Size

Directory/array browser menus

=============================

[NEW] -  Collects new data for current directory.

New prompts for data name. Enters HP's

Matrix Manager for data collection. New then

enters you in the database collector so important

info about your array can be stored in a database.

NOTE: When collecting data, remember column are SS and rows are SG.

Here is the database input definitions.

NAME  - Person who collected the data.

UTL   - Upper tolerance limit

LTL   - Lower tolerance limit

MACH# - Machine number

TYPE  - Machine type

PART# - Part number

LOT#  - Lot number

NOTE  - Any notes about the data you may want to keep track of.

NOTE: Each directory have a variable vardat. vardat stores database

information for the current directory. Altering the contents of

vardat may cause a MEMORY LOST.

=

[X&R] - Plots X&R charts for the current highlighted data

control charts can be horizontally by using

Left and right arrow keys

NOTE: maximum sample size 10. maximum subgroup size 52

RS X&R - calculates control limits and estimated population

[X&R]    Key assignments

DIVIDE   -  Is also assigned to [HIST] and [STAT]

this is the print key for printing to HP's

infrared printer or Jarno's infrared printer emulator.

DIVIDE prompts if you would like to print the header.

selecting YES prints header then continue to print info. NO

just skips printing header then continue print info.

ON pops back to previous display.

MULTIPLY - From left to right: upper control limit,

center line, and lower control limit

SUBTRACT - Connect pixel; disconnect pixel

PLUS     - Toggles Rchart and Xchart

ON       - Pops back to main environment

RS X&R   -  Allows you to enter Xbar, Rbar and sample size

so control limits can be calculated. It also

calculated the estimate of the population based

on those parameters.

=

[STAT]  - displays statistics. Remember that %above UTL and

%below LTL calculated based on the normal curve

which assume that your process is normally distributed

and in a state of statistical control

PCI is seen in some text as CPK which is called a Process

capability Index. A minimum value of 1.33 is desired for

a process to be satisfactory

ON    - Pops back to previous display

DIVIDE - Print key

=

[HIST] -  Plots histogram along with normal curve and tolerance

limits. Minimum bin size 6 maximum 23

DIVIDE -  Print key

RS HIST - Enter your bins size then histogram is plotted

=

[CRDIR] - Creates a directory in the current path

=

[COPY]  - Copies data to name user specifies on input.

you can still navigate through directories

all key assignment remains except ENTER.

Copy menu keys

[HERE]- Copies data to new name

[EXIT]- Returns to original directory

=

[MOVE]  - Move data to a new directory. you can still

navigate through directories

=

[EDIT]  - Edit data at highlight using matrix manager

=

[PURGE] - Deletes data at highlight

=

[INFO]  - Where to find me

=

Keys assignments for all browsers

================================

LS UP   - Move directory up

LS HOME - Goes to the home directory

LS DOWN - Page display down; Will also repeat

if depressed

LS UP   - page display up; Will also repeat if depressed

RS DOWN - Down to the end of display

RS UP   - Up to the beginning of display

ALPHA   - Search Key

RS ON   - off

ON      - Aborts program

BACKARROW  - Aborts program

ENTER - If the highlight Bar is pointed to a directory, that

directory is selected. If the highlight Bar is pointed to

a array, you will enter database environment.

Database Menus

==============

[EDIT]  - Edit or allows entry of header info

[VIEW]  - View data too long for display the view

command have no scrolling capabilities.

The entire screen is viewed only

[INDX]  - Display entire field. ENTER selects current

data to the database. ON pops back to the

database environment

[TRACE] - Display all data names of highlighted field.

ENTER selects current data to the database.

ON pops back to the database environment

[EXIT]  - Pops back to previous browser with name of

highlighted

ON     - Pops back to previous browser

RS HOME - the key works with INDX and TRACE also. this key

pops back to the array directory browser.

LEFT ARROW - scroll left

RIGHT ARROW - Scroll right