=================================

Installing {COMMO} on the HP 95LX

=================================

If you do not have the Connectivity Pack, you will need the accessory serial

cable, HP PN 82222A.  Connect your PC to the HP with the cable and follow

these steps:

----------

ON YOUR PC

----------

IMPORTANT:  This procedure assumes that you have read READ95.ME and that you

have installed the standard version of {COMMO} on your PC.  If you have not

done so, refer to COMMO.DOC for instructions.

It also assumes that the {COMMO} "home" directory is "C:\COMMO\".  If this

is not the case, you will be given an opportunity to change it while the

install macro is running.

1) Create a subdirectory below your {COMMO} "home" directory called "HP95".

The path for this directory would then be "C:\COMMO\HP95\".

2) Copy the files from the {COMMO} HP 95LX archive, COMMO9.ZIP or

CMOHPxx.ZIP, to this directory.  "xx" is the {COMMO} version number.

3) Copy the the files INSTAL95.BAT and INSTAL95.MAC to your {COMMO} "home"

directory.

4) Change to the {COMMO} "home" directory and type "instal95" to run the

installation batch file.

5) Enter the number of the serial port that is connected to the 95LX, "1"

for COM1, "2" for COM2, etc.

6) If the directory shown is correct, just press Enter.  Otherwise edit the

path and then press Enter.  Be sure there is a "\" at the end.

7) The first filename will blink at the top of the screen.  This is the

filename that you should enter on the 95LX.  After each file is

transferred, a new filename will appear.  Note that some of the names

have "95" in them on the PC, but are named without the "95" on the 95LX.

8) When you see the message "Finished!" you should press Alt-X to exit

{COMMO} on your PC.  If you need to cancel the process at any time, just

press Esc to stop and leave {COMMO}.

------------

ON YOUR 95LX

------------

1) Open FILER.

2) Open DATACOMM.

3) Press MENU, T, X, R.

4) Enter the 95LX path and the filename that the PC is sending.

5) The transfer will start.

6) Continue until all files are copied.

- end -