Subject: HP 95LX Graphics Programming

Here's the information that I promised in an earlier note regarding programming

graphics on the HP 95LX.  Caveat Programmer.

Everett Kaser                   Hewlett-Packard Company

...hplabs!hp-pcd!everett        work: (503) 750-3569   Corvallis, Oregon

everett%hpcvra@hplabs.hp.com    home: (503) 928-5259   Albany, Oregon

----------------------------- cut here --------------------------------------

.__________________________________

.HP 95LX Low-level Graphics Support

.__________________________________

.Copyright 1991 Hewlett-Packard Company

.All rights reserved

.(distributed with permission)

.____________

.INTRODUCTION

.The package of low-level routines provided in the HP 95LX that are

.available for use by applications programs support these functions:

.. 1) setting the display mode to alpha or graphics.

.. 2) setting a fill mask.

.. 3) getting information about the current graphics settings.

.. 4) setting a logical origin to which all other graphics

...operations are relative.

.. 5) setting a rectangular clip region.

.. 6) drawing an outlined, solid-filled, or pattern-filled

...rectangle.

.. 7) drawing a line.

.. 8) plotting a point.

.. 9) moving the current pen location.

..10) setting the current pen color.

..11) setting the current replacement rule.

..12) setting the current line-type.

..13) reading the current color value of a point.

..14) reading a rectangular image.

..15) writing a rectangular image.

..16) labeling text using the internal 6x8 font.

.All of the routines which plot to the display always obey the current

.logical origin, clip region, pen color, replacement rule, and (where

.appropriate) line-type and fill-mask.

.The pen color can be 0 or 1.

.The replacement rule can be one of FORCE, AND, OR, or XOR.  Writing

.a rectangular image can optionally invert the image before applying

.the specified replacement rule.  Writing an image is different from

.all other plotting in that it uses an argument as the replacement

.rule rather than the current replacement rule.

.The line-type is a 16-bit value whose bits are used repeatedly when

.drawing a line or an outlined rectangle.

.The fill-mask is an 8-byte value which specifies an 8-bit by 8-bit

.rectangular mask which is used repeatedly when drawing a pattern-filled

.rectangle.

.

.The general process to do graphics is:

..1) set the display mode to graphics

..2) set the desired pen color, replacement rule, linetype,

...fillmask, logical origin, and clip region, if different

...than the default values set by the set-mode function.

..3) perform the desired drawing using the attributes setup by

...step 2).

..4) repeat steps 2) and 3) until done.

..5) set the display mode back to alpha.

.The graphics routines are accessed through software interrupt 5F (hex).

.The required arguments are loaded into specific CPU registers, the

.requested function number is loaded into the AH register, and then an

.INT 5Fh instruction is executed.  Unless otherwise stated, all

.functions preserve ALL registers except for AX.

.X-coordinates always get larger (more positive) when moving to the

.right on the display.  Y-coordinates always get larger when moving

.down (towards the bottom) on the display.  The default origin is in

.the top-left corner of the display.

.Although the interface is designed primarily as an assembly language

.interface, it is simple to write an assembly language module that can

.provide a library of corresponding functions to a C program.  A sample

.is provided in Appendix A.

..______________

.SET VIDEO MODE

.Forces the current video mode to alpha or graphics and clears the

.display.

.Entry conditions:

..AH = 0

..AL = requested mode.

...07h = alpha (system manager compliant)

...20h = graphics (system manager compliant)

...87h = alpha (non-system manager)

...A0h = graphics (non-system manager)

.Sub-functions 07h and 20h call the System Manager routine to change

.video modes, thus letting the System Manager know that the display has

.changed modes and the display contents destroyed.  This is important

.for any applications which are intended to be system manager compliant.

.Sub-functions 87h and A0h call the BIOS directly in order to change

.display modes, thus by-passing the System Manager.  These should be

.used by programs that are NOT System Manager compliant, so that they

.will function correctly whether the system was booted into the System

.Manager or if it was booted straight into DOS (by placing a SHELL=

.command in a CONFIG.SYS file).

.After a SET_MODE call to change to graphics, the defaults are:

..Logical origin..(0, 0)

..Clip region..(0, 0) thru (239, 127)

..Pen location..(0, 0)

..Pen color..1

..Replacement rule.FORCE

..Line-type..0FFFFh

..Fill mask..0FFh,0FFh,0FFh,0FFh,0FFh,0FFh,0FFh,0FFh

.Sample assembly code:

..mov.ax,0020h.; set mode to GRAPHICS

..int.5fh

....

....

....

..mov.ax,0007h.; set mode to ALPHA

..int.5fh

.._____________

.SET FILL MASK

.Sets the eight-byte fill mask used by DRAW_RECTANGLE when pattern-

.filling.

.Entry conditions:

..AH = 1

..ES:DI = address of 8 bytes of fillmask.

.The default fillmask after a mode set to graphics is eight bytes of

.0ffh (which would result in a solid-fill).

.The fillmask is always aligned with the byte boundaries of the display

.memory, and it is then clipped by the rectangle being drawn.  This

.means that as the rectangle is shifted bit-by-bit, the pattern appears

.to exist on a plane behind the rectangle, and that the rectangle is

.a moving window onto that plane.  It's tough to describe, and a little

.experimentation should make it plain.

.Sample assembly code:

..fmask1.db.055h, 0aah, 055h, 0aah, 055h, 0aah, 055h, 0aah

....

....

....

..assume.es:dgroup

..lea.di,fmask1

..mov.ah,1

..int.5fh..; set fillmask pattern to FMASK1

..________________________________

.GET CURRENT GRAPHICS INFORMATION

.Returns current information about the state of the graphics functions.

.At entry:

..AH = 2

..ES:DI = address of a 36-byte long buffer into which the

...graphics information will be placed.  At exit from

...this function, the buffer contents will be:

..      offset  size

... 0   1-byte CURRENT VIDEO MODE

... 1   1-byte DEFAULT VIDEO MODE

... 2   1-word WIDTH OF DISPLAY IN PIXELS

... 4   1-word HEIGHT OF DISPLAY IN PIXELS

... 6   1-word CURRENT X-LOCATION OF PEN

... 8   1-word CURRENT Y-LOCATION OF PEN

...10   1-word CURRENT LINE-TYPE

...12   1-word CURRENT REPLACEMENT RULE

...14   1-word CURRENT PEN COLOR

...16   1-word CURRENT X-MINIMUM OF CLIP REGION

...18   1-word CURRENT X-MAXIMUM OF CLIP REGION

...20   1-word CURRENT Y-MINIMUM OF CLIP REGION

...22   1-word CURRENT Y-MAXIMUM OF CLIP REGION

...24   1-word CURRENT X-LOCATION OF LOGICAL ORIGIN

...26   1-word CURRENT Y-LOCATION OF LOGICAL ORIGIN

..      28-35  8-bytes CURRENT FILL MASK (for rectangle fill)

.At exit:

..DX:AX = address of the 36-byte long buffer (for return to C).

.Sample assembly code:

..infobuf..label.byte

..curmode..db.?

..defmode..db.?

..dspwidth.dw.?

..dspheight.dw.?

..curpenx..dw.?

..curpeny..dw.?

..curlinetype.dw.?

..curreprule.dw.?

..curpen..dw.?

..curclipminx.dw.?

..curclipmaxx.dw.?

..curclipminy.dw.?

..curclipmaxy.dw.?

..curlogorgx.dw.?

..curlogorgy.dw.?

..curfmask.db.8 dup (?)

.....

.....

.....

..assume.es:dgroup

..lea.di,infobuf

..mov.ah,2..; read current video info into infobuf

..int.5fh

..__________________

.SET LOGICAL ORIGIN

.Sets the logical origin in terms of absolute screen pixels, regardless

.of previous settings of the logical origin or clip region.  The (X,Y)

.of the logical origin may be specified off of the actual physical

.screen (ie, negative values or greater than (239, 127).  All other

.coordinate arguments in this graphics system are relative to the

.logical origin, including those used to specify the clip region.

.SET LOGICAL ORIGIN resets the CLIP REGION to the entire physical

.display (0,0) to (239,127).  So, if clip_region is used, it must be

.set AFTER the set_logical_origin.

.The default logical origin after a mode set is (0,0).

.At entry:

..AH = 3

..CX = x coordinate

..DX = y coordinate

.Sample assembly code:

..mov.ah,3

..mov.cx,120..; move log org to approximately the

..mov.dx,64..;   center of the display

..int.5fh

._______________

.SET CLIP REGION

.This function sets the coordinates of the upper-left and lower-right

.corners of the clip rectangle.

.All reading/writing of the display in this graphics system is limited

.(clipped) by the current CLIP REGION.

.Default after mode set is (0,0) and (239,127).

.At entry:

..AH = 4

..CX = x-minimum coordinate

..DX = y-minimum coordinate

..SI = x-maximum coordinate

..DI = y-maximum coordinate

.Sample assembly code:

..mov.ah,4..; clip (limit) all drawing to the

..mov.cx,120..;    right half of the display

..mov.dx,0

..mov.si,239

..mov.dx,127

..int.5fh

..______________

.DRAW RECTANGLE

.This draws a rectangle which has two diagonally opposite corners at

.the current pen location and (CX,DX).  Hence, you will usually first

.do a MOVE_PEN, then a DRAW_RECTANGLE.  ALL rectangle draws obey the

.current replacement rule.  The pen location is left at the starting

.location (it is not changed).

.If the rectangle drawn is just an outline, it is drawn using the

.current line-type.  If the rectangle drawn is pattern-filled, it uses

.the current fill-mask.  In all cases the current pen color and replace-

.ment rule are used.

.At entry:

..AH = 5

..AL = fill flag.0==outline, current linetype and pen color

....1==solid fill, current pen color

....2==pattern fill, current fillmask and pen color

..CX = x-coordinate of second corner of rectangle

..DX = y-coordinate of second corner of rectangle

.Sample assembly code:

..mov.ah,8..; move pen to (50,74)

..mov.cx,50

..mov.dx,74

..int.5fh

..mov.ah,5

..mov.al,1..; solidfill rectangle to (101, 99)

..mov.cx,101

..mov.dx,99

..int.5fh

._________

.DRAW LINE

.This draws a line from the current pen location to (CX,DX) using the

.current pen color, linetype, and replacement rule.  The pen location

.is left at the end point.  If another DRAW_LINE is executed after the

.first without an intervening MOVE_PEN, the starting point is not

.plotted.  This is to avoid the problem of drawing connecting lines

.with a replacement rule of XOR.  Since the starting point of the second

.line is the same as the ending point of the first line, it would get

.plotted twice, which in XOR mode is the same as not plotting it at all.

.At entry:

..AH = 6

..CX = x-coordinate of end point

..DX = y-coordinate of end point

.Sample assembly code:

..mov.ah,6

..mov.cx,21

..mov.dx,10..; draw from current pen location

..int.5fh..;   to (21, 10)

..__________

.PLOT POINT

.This moves the current pen location to CX,DX and plots a single

.point there using the current pen color and replacement rule.

.At entry:

..AH = 7

..CX = x-coordinate of point

..DX = y-coordinate of point

.Sample assembly code:

..mov.ah,7

..mov.cx,239..; plot point at (239, 127)

..mov.dx,127

..int.5fh

.________

.MOVE PEN

.This moves the current pen location to (CX,DX).  The default location

.after a mode set is (0,0).

.At entry:

..AH = 8

..CX = x-coordinate

..DX = y-coordinate

.Sample assembly code:

..mov.ah,8

..mov.cx,22..; move pen to (22, 44)

..mov.dx,44

..int.5fh

._____________

.SET PEN COLOR

.Sets the current pen color to 0 or 1.  The default after a mode

.set is 1 (black).

.At entry:

..AH = 9

..AL = new pen color (0 for white or 1 for black)

.Sample assembly code:

..mov.ah,9

..mov.al,0..; set pen color to white (0)

..int.5fh

..____________________

.SET REPLACEMENT RULE

.The replacement rule controls how the current pen color is combined

.with the existing color of a pixel on the display when performing

.any plotting function (except WRITE_AREA, which has its own replacement

.rule specified with each call).  If the current replacement rule is

.FORCE then the resulting color of a pixel is equal to the current pen

.color at the time of the plotting action.  For the other three replace-

.ment rules (AND, OR, and XOR), the resulting color is the logical

.operation between the current screen pixel color and the current pen

.color.

.The default replacement rule after a mode set is 0 (FORCE).

.At entry:

..AH = 10 (0Ah)

..AL = new replacement rule.0==FORCE

......1==AND

......2==OR

......3==XOR

.Sample assembly code:

..mov.ah,10

..mov.al,3..; set replacement rule to XOR

..int.5fh

._____________

.SET LINE TYPE

.This is a 16-bit value that is repeated over and over as each pixel

.of a line or an outlined rectangle is drawn.  0xFFFF will cause solid

.lines to be drawn. The default after a mode set is 0xFFFF (solid line).

.At entry:

..AH = 11 (0Bh)

..CX = new line type

.Sample assembly code:

..mov.ah,11

..mov.cx,0c440h.; set line type to XX000X000X000000

..int.5fh

..__________

.READ POINT

.This returns AX==the color (0 for white, 1 for black) of the requested

.point.  The current pen location is not modified.

.At entry:

..AH = 12 (0Ch)

..CX = x-location of point to read

..DX = y-location of point to read

.Sample assembly code:

..mov.ah,12

..mov.cx,49..; read point (49,57)

..mov.dx,57

..int.5fh

._____________________

.READ AREA (get image)

.This reads a rectangular area of the display into the specified

.buffer.  There is an 8-byte header at the beginning (specifying

.number of planes, number of bits/pixel, width of image, and

.height of image.  The first two are always equal to 1 on the HP 95LX.

.The size needed for the buffer is:

...8 + ((x2-x1+8)/8) * (y2-y1+1) bytes

.The diagonally opposite corner points (x1, y1) and (x2, y2) are

.included in the read area.  Bits with a value of 0 are added to

.the right end of each row of pixels (if necessary) to fill out an

.integral number of bytes of data for that row.  The image is always

.left justified within the buffer regardless of its byte-alignment on

.the display.

.At entry:

..AH = 13 (0Dh)

..CX = x-coordinate of corner 1

..DX = y-coordinate of corner 1

..SI = x-coordinate of corner 2

..BP = y-coordinate of corner 2

..ES:DI = address of buffer for image

.Sample assembly code:

..tmpbuf.db.56 dup (?)

...

...

...

..assume.es:dgroup

..mov.ah,13

..mov.cx,24

..mov.dx,55

..mov.si,41..; read a rectangular area of the screen

..mov.bp,70..;   from (24,55) thru (41,70) into

..lea.di,tmpbuf.;   'tmpbuf'.

..int.5fh

..______________________

.WRITE AREA (put image)

.This writes to a rectangular area of the display from a specified

.buffer.  The buffer should have the same eight-byte header described in

.READ_AREA (above).  This call is different from all other "write"-type

.calls in that it specifies its own replacement rule rather than using

.the "current" replacement rule.  It expands upon the replacement

.rule types by allowing the image to be inverted before being combined

.in the usual fashion (according to FORCE, AND, OR, or XOR) with the

.display contents.  This does not modify the contents of the buffer.

.If the entire image doesn't fit on the display, none of it is drawn.

.At entry:

..AH = 14 (0Eh)

..AL = replacement rule.0 FORCE

.....1 AND

.....2 OR

.....3 XOR

.....4 invert image and then FORCE

.....5 invert image and then AND

.....6 invert image and then OR

.....7 invert image and then XOR

..CX = x-location of top-left corner of image destination

..DX = y-location of top-left corner of image destination

..ES:DI = address of image

.Sample assembly code:

..tmpbuf.db.56 dup (?)

...

...

...

..assume.es:dgroup

..mov.ah,14

..mov.al,7..; invert, then XOR

..mov.cx,133

..mov.dx,66

..lea.di,tmpbuf.; put image 'tmpbuf' at (133,66)

..__________

.WRITE TEXT

.Writes the specified text (all 256 chars are legal EXCEPT 0)

.to the specified location, horizontally or rotated 90 degrees

.counter-clockwise from horizontal, using the BIOS 6x8 font, the

.current pen color and replacement rule. The specified location (CX, DX)

.is the top-left corner of the text string, or if the rotate flag

.is non-zero, the string is rotated 90 degrees counterclockwise about

.the point (CX, DX) (such that it is now the bottom-left corner of the

.text).

.At entry:

..AH = 15 (0Fh)

..AL = rotate flag.

..CX = x-coordinate of top-left corner of first character

..DX = y-coordinate of top-left corner of first character

..ES:DI = address of null-terminated string

.Sample assembly code:

..txtstr.db."This is a test",0

...

...

...

..assume.es:dgroup

..mov.ah,15

..mov.al,0..; plot text horizontally

..mov.cx,124

..mov.dx,37..; at (124, 37)

..lea.di,txtstr

..int.5fh

..___________________________________________________________________

.APPENDIX A: Sample assembly language module of C-callable functions

;[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[

;[ Graphics interface module for calling Jaguar graphics from C programs [

;[ Copyright 1990 Hewlett Packard Company.  All rights reserved.. [

;[ Author: Everett Kaser  August 14, 1990..... [

;[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[

..MODEL.LARGE,C

..CODE

.assume.ds:nothing

;[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[

; G_Mode(int BiosVideoMode);

;[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[

G_Mode..PROC.BiosVideoMode:word

.mov.ax,BiosVideoMode

.xor.ah,ah

.int.5fh

.ret

G_Mode..endp

;[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[

; G_FillMask(maskptr);

;[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[

G_FillMask.PROC.uses ES DI, maskptr:dword

.les.di,maskptr

.mov.ah,1

.int.5fh

.ret

G_FillMask.endp

;[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[

; G_GetInfo(G_INFO *gp);

;[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[

G_GetInfo.PROC.uses ES DI, gp:dword

.les.di,gp

.mov.ah,2

.int.5fh

.ret

G_GetInfo.endp

;[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[

; G_LorgA(int x, int y);

;[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[

G_LorgA..PROC.x:word, y:word

.mov.cx,x

.mov.dx,y

.mov.ah,3

.int.5fh

.ret

G_LorgA..endp

.;[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[

; G_ClipL(int xmin, int ymin, int xmax, int ymax);

;[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[

G_ClipL..PROC.uses SI DI, xmin:word, ymin:word, xmax:word, ymax:word

.mov.cx,xmin

.mov.dx,ymin

.mov.si,xmax

.mov.di,ymax

.mov.ah,4

.int.5fh

.ret

G_ClipL..endp

;[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[

; G_Rect(int x, int y, int fillflag);

;[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[

G_Rect..PROC.x:word, y:word, fill:word

.mov.cx,x

.mov.dx,y

.mov.ax,fill

.mov.ah,5

.int.5fh

.ret

G_Rect..endp

;[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[

; G_Draw(int x, int y);

;[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[

G_Draw..PROC.x:word, y:word

.mov.cx,x

.mov.dx,y

.mov.ah,6

.int.5fh

.ret

G_Draw..endp

;[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[

; G_Point(int x, int y);

;[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[

G_Point..PROC.x:word, y:word

.mov.cx,x

.mov.dx,y

.mov.ah,7

.int.5fh

.ret

G_Point..endp

.;[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[

; G_Move(int x, int y);

;[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[

G_Move..PROC.x:word, y:word

.mov.cx,x

.mov.dx,y

.mov.ah,8

.int.5fh

.ret

G_Move..endp

;[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[

; G_ColorSel(int color);

;[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[

G_ColorSel.PROC.color:word

.mov.ax,color

.mov.ah,9

.int.5fh

.ret

G_ColorSel.endp

;[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[

; G_RepRule(int rrule);

;[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[

G_RepRule.PROC.rrule:word

.mov.ax,rrule

.mov.ah,0ah

.int.5fh

.ret

G_RepRule.endp

;[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[

; G_LineType(int ltype);

;[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[

G_LineType.PROC.ltype:word

.mov.cx,ltype

.mov.ah,0bh

.int.5fh

.ret

G_LineType.endp

;[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[

; G_PointRead(int x, int y);

;[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[

G_PointRead.PROC.x:word, y:word

.mov.cx,x

.mov.dx,y

.mov.ah,0ch

.int.5fh

.ret

G_PointRead.endp

.;[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[

; G_ImageGet(int x1, int y1, int x2, int y2, char far *image);

;[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[

G_ImageGet.PROC.uses ES SI DI, x1:word, y1:word, x2:word, y2:word, image:dword

.mov.cx,x1

.mov.dx,y1

.mov.si,x2

.les.di,image

.mov.bp,y2

.mov.ah,0dh

.int.5fh

.ret

G_ImageGet.endp

;[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[

; G_ImagePut(int x, int y, char far *image, int replacerule);

;[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[

G_ImagePut.PROC.uses ES DI, x:word, y:word, image:dword, reprule:word

.mov.cx,x

.mov.dx,y

.les.di,image

.mov.ax,reprule

.mov.ah,0eh

.int.5fh

.ret

G_ImagePut.endp

;[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[

; G_Text(int x, int y, char far *string, rotflag);

;[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[

G_Text..PROC.uses DS ES SI DI, x:word, y:word, string:dword, rotflag:word

.mov.ax,rotflag

.mov.cx,x

.mov.dx,y

.les.di,string

.mov.ah,0fh

.int.5fh

.ret

G_Text..endp

@curseg.ends

.end

..______________________________________________________________________

.APPENDIX B: Sample header file for use with C programs and APPENDIX A.

/*[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[

[ Definitions........[

[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[*/

#define.G_ALPHA..0x07

#define.G_GRAPHICS.0x20

#define.G_FORCE..0

#define.G_AND..1

#define.G_OR..2

#define.G_XOR..3

#define.G_NOTFORCE.4

#define.G_NOTAND.5

#define.G_NOTOR..6

#define.G_NOTXOR.7

#define.G_OUTLINE.0

#define.G_SOLIDFILL.1

#define.G_PATTERNFILL.2

#define.MINCOLOR.0

#define.MAXCOLOR.1

/*[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[

[ Structures.........[

[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[*/

typedef.struct g_info {

.unsigned char.vidmode;

.unsigned char.defmode;

.unsigned int.xpixels;

.unsigned int.ypixels;

.int..xloc;

.int..yloc;

.unsigned int.linetype;

.int..rrule;

.unsigned int.color;

.int..xclipmin;

.int..yclipmin;

.int..xclipmax;

.int..yclipmax;

.int..xlorg;

.int..ylorg;

.unsigned char.fillmask[8];

.} G_INFO;

./*[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[

[ Graphics library function definitions.  All x,y locations are rel-.[

[   ative to the current logical origin unless specified otherwise..[

[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[[*/

void   far cdecl.G_Mode(int);

G_INFO far * cdecl.G_GetInfo(G_INFO far *);

void   far cdecl.G_ColorSel(int);

void   far cdecl.G_RepRule(unsigned int);

void   far cdecl.G_LineType(unsigned int);

void   far cdecl.G_FillMask(unsigned char far *);

void   far cdecl.G_LorgA(int, int);

void   far cdecl.G_ClipL(int, int, int, int);

void   far cdecl.G_Move(int, int);

void   far cdecl.G_Point(int, int);

int    far cdecl.G_PointRead(int, int);

void   far cdecl.G_Draw(int, int);

void   far cdecl.G_Rect(int, int, int);

void   far cdecl.G_ImageGet(int, int, int, int, char far *);

void   far cdecl.G_ImagePut(int, int, char far *, int);

void   far cdecl.G_Text(int, int, char far *, int);

/*\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\*/

/**** G_MODE(mode): mode = {G_TEXT | G_GRAPHICS}

.changes the display mode to text or graphics..*/

/**** G_GETINFO(GraphInfoPtr): GraphInfoPtr is a far pointer to a buffer of

.the programmers choosing where the graphics information will be copied.

.See the typedef for the G_INFO structure in this file for the contents

.of the buffer..*/

/*\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\*/

/**** G_COLORSEL(color): color = 0 or 1

.sets the current pen to "color"....*/

/**** G_REPRULE(reprule): reprule = {G_FORCE | G_AND | G_OR | G_XOR}

.sets the current replacement rule for all other drawing (except for

.G_ImagePut, which specifies its own replacement rule). */

/**** G_LINETYPE(linetype): linetype = a 16-bit image that is repeated while

.drawing lines and G_OUTLINE'd rectangles.  Bits that are 1 cause the

.current pen color to be plotted using the current replacement rule.

.Bits that are 0 are not plotted and leave the display un-modified. */

/**** G_FILLMASK(buffer): buffer is a far pointer to an 8-byte array which

.specifies the fillmask to use when doing drawings of rectangles

.with a fillflag of G_PATTERNFILL.  Every bit that is a 1 will

.cause a point of the current color to be plotted.  Every bit that is

.a 0 will cause that "point" of the display to be undisturbed. */

/*\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\*/

/**** G_LORGA(x,y): x,y = -32768 to +32767

.sets the logical origin (0,0) to be located at the absolute screen

.coordinate specified by x,y. */

/**** G_CLIPL(x1, y1, x2, y2): x1,y1,x2,y2 = -32768 to +32767

.sets the current clip boundary to the rectangle whose diagonally

.opposite corners are specified by the absolute screen coordinates

.equal to the x1,y1 and x2,y2 offsets from the current logical origin.*/

./*\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\*/

/**** G_MOVE(x, y): x,y = -32768 to +32767

.causes the logical pen to be moved to "x,y"..*/

/**** G_POINT(x,y): x,y = -32768 to +32767

.plots a point of the current pen color with the current replacement

.rule at "x,y" unless "x,y" is outside the current clip limits.*/

/**** G_POINTREAD(x,y): x,y = -32768 to +32767

.reads the color of the point located at x,y and returns that as the

.value of the function. */

/**** G_DRAW(x,y): x,y = -32768 to +32767

.draws a line of the current pen color with the current replacement rule

.and the current linetype from the current pen location to "x,y";

.only those points lying within the clip limits are actually plotted. */

/**** G_RECT(x,y,fillflag): x,y = -32768 to +32767

.fillflag = {G_OUTLINE | G_SOLIDFILL | G_PATTERNFILL}

.draws a rectangle with diagonally opposite corners at the current pen

.location and at "x,y", using the current pen color and replacement

.rule.  The "type" of rectangle drawn is determined by fillflag.

.if fillflag==G_OUTLINE, the outline of a rectangle is drawn, using the

..current linetype.

.if fillflag==G_SOLIDFILL, a solid, filled rectangle of the current

..color is drawn.

.if fillflag==G_PATTERNFILL, a pattern filled rectangle of the current

..color is drawn, using the current fill mask..*/

/*\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\*/

/**** G_IMAGEGET(x1, y1, x2, y2, buffer):  x1,y1,x2,y2 = -32768 to +32767

.If both points x1,y1 and x2,y2 are within the current clip boundary,

.the display image bounded by the rectangle whose diagonally opposite

.corners are x1,y1 and x2,y2 is read into the bytes pointed to by

."buffer". */

NOTE: for G_ImageGet(), the required size of 'buffer' is (on Jaguar):

.8 + ((x2-x1+8)/8) * (y2-y1+1) bytes          */

/**** G_IMAGEPUT(x, y, buffer, reprule): x,y = -32768 to +32767

.reprule = {G_FORCE | G_AND | G_OR | G_XOR | G_NOTFORCE | G_NOTAND |

...G_NOTOR | G_NOTXOR}

.if x,y and the un-specified bottom-right corner of the image are

.within the clip boundary, the image from "buffer" is drawn on the

.display using reprule as the replacement rule.  (For the "NOT" replace-

.ment rules, the image is color-inverted first, then placed on the

.display using the "rest" of the replacement rule.) */

/*\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\*/

/**** G_TEXT(x,y,buffer,rotflag): x,y = -32768 to +32767, rotflag = 0 or 1

.draws the string pointed to by "buffer" on the display at the specified

.location x,y using the current font. If rotflag = 0, it's drawn

.horizontally, else it's rotated 90 degrees counterclockwise. */

/*\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\*/

------------------------------ the end ----------------------------------------