Serial & IR hardware on the HP 95LX

Copyright 1991 Hewlett-Packard Co.

All rights reserved.

The information contained in this file, although copyrighted, is

being released for distribution free of charge.  Hewlett-Packard

Co. makes no warantees as to its accuracy, completeness, or use-

fulness.  The user accepts all risk involved in the use of this

information, and Hewlett-Packard Co. offers no support and pro-

vides no support of this information.

This document describes the hardware involved in the HP 95LX serial

and Infra Red communications.  Minimal programming information is

supplied.  Throughout this document the term "Hopper" refers to the

ASIC chip in the HP 95LX which contains much of the HP 95LX's

functionality.

WARNING!!!! It is possible to damage an HP 95LX by improperly

programming the IR port (particularly by leaving the IR LED on for

too long).  Proceed at your own risk!

INTRODUCTION

The serial communication portion of the Hopper IC contains both

wired RS232 and IR transmit and receive capabilities.  The RS232

UART ports use an 8250 cell to control transmit and receive.  The

IRO output port can be used for two types of communication, IR

transmit and REDEYE.  REDEYE is used to transmit data to an

infrared REDEYE printer port.  The IR communications portion uses

both the IRO output to trnsmit and the IRI input pin to receive

data.  These ports are used for wireless communication using

infrared light.  The 8250 and the IR ports can be used

simultaneously to implement a wireless UART mode.

UART

The serial UART block is implemented using an 8250 compatible

macro cell.  A 1.8 MHz clock will be supplied for operation of

this block.  The UART is addressed from 3F8h to 3FFh.  RS232 drive

and receive circuits are provided off chip.

The 1.8MHz UART clock is available in operating and light sleep

modes.  The UART clock should be disabled to save power when the

UART is not in use.  This is done by setting the 8250 baud rate

divisor to zero.

IR COMMUNICATION

The IR communication block enables the Hopper IC to have wireless

communication using an external infrared LED and IR receive

circuit.  The IR communication block supports 5 separate

communication formats.  These formats are REDEYE, Software

controlled communication, Modulated communication, IR UART with

both single pulse and multiple pulse communication.  To control

the IR transmission of these formats, the hardware uses 2 control

registers, the IRCNT register and the IRFMAT register.  The IRFMAT

register at I/O register address E30Ah contains control bits that

choose which format is chosen for IR communication.  These bits

are as follows:

bit  name	description

____________________________________________________________________

0    RED	This bit when set activates REDEYE transmit mode.

It turns on the REDEYE transmit hardware and sets it

to a state where it is waiting for input from software.

1    MDLTE	This bit when set activates Modulated communication

mode.  It turns on the modulation source and allows the

MDLD bit in the IRCNT register to control the output

of a modulated waveform.

2    IRURT	This bit when set activates IR UART communication mode.

This bit disconnects the 8250 from the RS232 pins and

connects it to the IR communication block.  Software,

after setting the PMOD and MDSEL bits in this register,

just transmits and receives using 8250 as though it

were connected to the RS232 ports.

3    MDSEL	This bit is used to select the modulation source for

both IR UART mode and Modulation communication mode.

If this bit is 0, the 32kHz low frequency clock

is chosen as the modulation source.  If it is set to 1,

the 8250 baud rate generator 16x clock is used for the

modulation source to be set at 38kHz to be remote

control compatible.

4    PMOD	This bit is used in IR UART mode to select between

single pulse transmission and multiple pulse transmiss-

ion.  If it is set to 0, a single pulse of duration

equal to a half cycle time of the modulation source

will be transmitted for a 0 output bit.  If it is set

to 1, a pulse train of the modulation frequency will be

used to transmit a 0.

5    LBF	LED Buffer Full.  This bit is used in REDEYE mode to

indicate that the contents of the LBR bit have not yet

been transmitted and should not be written at this

time.  Writing to the LBR automatically sets this bit.

This bit is cleared when the LBR is transferred to the

REDEYE formatter.

6    ELBE	Enable Interrupt on LBR bit Empty (LBF clear).  If this

bit is set and LBF is clear, an IR interrupt will

occur.

7    UNREG	This bit is used to test the HPIRO pad.  Setting it

high disables regulation of the pad current.

The IRCNT register at I/O register address E30Bh contains bits that are

used to transmit a bit or waveform out on the IRO pad.  The IRCNT

register also allows software to receive IR data in any of the formats

that can be transmitted.  The contents are as follows:

bit  name	description

_____________________________________________________________________

0    LED	This bit is used to turn on the IR LED connected to the

HPIRO pad.  It is used for software controlled IR

transmission.  When it is set to a 1, the IR LED is

turned on.

1    LBR	This bit contains the half-bit to be transmitted in

REDEYE format.  Write a 1 to send an "on" half-bit or

write a 0 to send an "off" half-bit.

2    MDLD	This bit is used for software to output a serial wave-

form to be modulated by the chosen modulation source.

This allows compatibility with remote control format.

5    IRE	IR Event.  This bit is set by a logic low voltage on

the IRI pin.  It is set to indicate that an IR event

has occurred.  Once set, software must reset this bit.

6    EIRI	Enable IR interrupt.  An IR interrupt will occur if

this bit and the IRE bit are both set.

7    IRI	IR Input pin.  This bit allows software to monitor the

state of the IRI pin.  It is a read only bit.

Using the last three bits, software can receive each of the transmiss-

ion formats described later.  Also, if IR UART mode is set, software

can receive data using the 8250 the same as it would in RS232 mode.

REDEYE FORMAT

The REDEYE portion consists of the RED, LBF, and ELBE bits in the

IRFMAT register, the LBR bit in the IRCNT register, the REDEYE

formatter, and the IRO LED pin.  The LED pin has an open drain device

and thus may be driven low or tristated only.  When driven low the

drain current is somewhat regulated by a feedback circuit.  The LBF

and LBR bits form a double buffered handshake mechanism that allow

automatic REDEYE half-bit formatting and pacing.  An interrupt

mechanism is provided to indicate completion of each half-bit.

The REDEYE printer requires 15-bit frames of a precise format.  Each

bit of the frame consists of two half-bits.  The duration of each

half-bit is 14 cycles of the 32768 kHz crystal oscillator.  The half-

bit is considerecd to be "on" if the LED is pulsed 6-8 times (out of

the 14 possible) at the 32768 kHz rate.  Hopper's REDEYE port uses

8 pulses.  The format of a complete REDEYE frame is shown below:

Start-bits	Three half-bits "on-on-on"

Hamming-bits	Four pairs of half-bits.

Data-bits	Eight pairs of half-bits.  Each of the four

hamming and eight data bits are encoded with

two half-bits.  A "one" data or hamming bit is

encoded by "on-off" and a zero is encoded by

"off-on".

Stop-bits	Three half-bits "off-off-off".  This is the

minimum idle time required between frames.

The ELBE, RED, LBF, and LBR bits are cleared at reset.  The REDEYE port

also uses a formatter which is turned off whenver RED is cleared.

Software initiates a half-bit transmission by writing a bit to LSR.

This automatically sets the LBF flags in IRFMAT register and starts

the state machine.  The state machine transfers the bit from LBR into

the formatter and clears LBF.  If ELBE is set, this will cause an IR

interrupt indicating that it is safe to write the next half-bit to LBR.

The state machine then times the half-bit for 14 counts of the 32768 Hz

crystal oscillator.  If the bit in the formatter is a 1, the LED is

left off.  If after the 14 counts LBF is clear, the state machine will

return to its idle state of waiting for LBF.  Otherwise, it will

immediately transfer the next half-bit and start timing it.

When LBF is clear and ELBE is set, an IR interrupt will occur.  When

the state machine clears LBF, software has 13 counts of the oscillator

to write the next bit to LBR.  Otherwise the length of the half-bits

will not be correct.

Through-put:

32768 / 14 = 2340.6 baud (half-bits/sec)

32768 / 28 = 1170.3 bps (bits/sec)

REDEYE Frame Length

1.5 start + 4 Hamming + 8 data + 1.5 stop = 15 bits

REDEYE Through-put:

1170.3 / 15 = 78.02 cps

SOFTWARE CONTROLLED MODE

The LED bit in IRCNT register is provided for software generated IR

formats.  This bit is OR-ed with the output of the REDEYE formatter,

and the other IR format outputs.  Therefore, two IR formats may not be

used simultaneously.

Due to LED current limitations, the LED output driver duty cycle must

be limited to a time average of 29%.  The duty cycle is automatically

limited to 1/2 x 8/14 or 28.6% by the REDEYE formatter.  The format of

a full REDEYE frame yields a duty-cycle of only 14.3%.  If a different

format is used (by using the LED bit) software must limit the duty-

cycle.

MODULATED MODE

The MDLD bit in the IRCNT register can be used by software to output

any custom modulated waveform desired.  To output a waveform, software

must first set the MDLTE bit in the IRFMAT register and choose the

modulation source using the MDSEL bit.  If the 8250 baud rate 16x

clock is chosen, its frequency must be set to the desired modulation

frequency.  Once this has been accomplished, software can set and

clear the MDLD bit as desired to emulate the envelope of the output

waveform.  Whenever MDLD is 1, pulses will be output of a 50% duty

cycle for the given modulation source.  As before, care must be taken

not to exceed the 29% communication duty cycle.

IR UART MODES

The 8250 may be used for half duplex IR communication of limited baud

rate.  To use this mode the IRURT bit must be set in the IRFMAT

register.  When this bit is set, the 8250 is disconnected from the

RS232 ports and connected to the IR communication block.  When using

this mode, software must first choose the transmission format.  The

two possible formats are single pulse mode and multiple pulse mode.  If

the PMOD bit is 0, single pulse mode is chosen.  In this mode, a single

pulse of one half cycle of the modulation source is transmitted for a

0.  In multiple pulse mode, a train of pulses of the modulation source

is transmitted for a 0.  In both modes, a 1 is transmitted as no

pulses.  As in Modulated mode, the modulation source again must be

chosen.  If the baud rate of 2400 baud is chosen, choosing the 8250

16x clock will give you a modulation rate of 38kHz.

After this is set up, software can use the 8250 to communicate as

though it were still connected to the RS232 ports.

MISCELLANEOUS

The IR input interrupt is tied to IRQ5 of the 8259 PIC.

Bit 5 (of 0-7) of I/O address E301h is the IR circuit interrupt/wakeup

enable.  This is a R/W register.  Modify bit 5 to 1 to enable the

IR to interrupt.  When IR is used in UART mode, IR interrupt is not

used, as everything appears to be coming in through the UART.

E30Ah  R/W	IR Format Register (IRFMAT)

bit 7	=1 disables IR pad current regulation

bit 6	=1 enable LED buffer empty interrupt

bit 5	=1 LED buffer full (read only)

bit 4	=0 single pulse transmission mode

=1 multiple pulse transmission mode

bit 3	=0 modulate using 32.768KHz

=1 modulate using baud rate generator

bit 2	=1 IR UART communication mode

bit 1	=1 modulated communication mode

bit 0	=1 REDEYE transmit mode

E30Bh  R/W	IR Transmit / Receive Register

bit 7	   state of the IRI pin (read only)

bit 6	=1 enable IR interrupt on IRI = 1

bit 5	=1 IR event has occurred, must be cleared by software

bit 4	unused

bit 3	unused

bit 2	=1 gate output when in modulated communication mode

bit 1	=0 transmit "off" half-bit in REDEYE format

=1 transmit "on" half-bit in REDEYE format

bit 0	=1 turn on LED driver, used for software controlled

transmissions.