I13FIX.COM

The initial HP 95LX's have a bug in the INT 13 (BIOS DISK) code, which causes

the machine to crash or hang if you perform disk operations with VERIFY ON

(or a COPY with the /V option).  The culprit is the INT 13 function AH=4,

VERIFY SECTORS.  The I13FIX TSR effectively disables INT 13 function 4, making

it a NOP, while passing all other calls through to the real BIOS code.  This

prevents the crash from happening.

To install I13FIX, it may be run directly from the Filer, although this will

prevent execution of any and all RAM-based System Manager compliant applicat-

ions (such as TigerFox, Hearts & Bones, etc).  The proper way to install I13FIX

is to create a C:\CONFIG.SYS file containing the line:

shell=command /p

and a C:\AUTOEXEC.BAT file containing the lines:

i13fix

$sysmgr

Once these have been created, the machine must be re-booted by pressing and

holding the CTRL and ALT keys, then pressing and releasing the DEL key.  This

causes the I13FIX TSR to be loaded before the System Manager.

Everett Kaser

10-1-91