Memory Map (MM) v4.2S Documentation (Shareware Version)

Copyright (c) 1992-1993 Andrew Powell

Features

��������

� Displays Location of Master Environment

� Help Screen

� Auto Configuration from Environment    ** Registerd Version Only **

� 'Easy' Mode Available                  ** Registerd Version Only **

� Automatically Pauses Between Pages

� Reports Total and Free Memory

� Will Run Under MS-Windows(r)

� Fast

� Optional Display of DOS v5.0 UMB Usage ** Registerd Version Only **

� Optional Display of Upper Memory Blocks

� Optional Display of Device Driver Chain

� Optional Display of Current Environment

� Optional Display of Master Environment ** Registerd Version Only **

The following documentation is based on this shareware version!!

Command-Line

������������

If you run MM without any options on the command line you will

get the default display. The default display with no command

line options and no environment variable is a fairly technical

display of the contents of memory. Below is a list of all the

command line options. You can also get this list by running MM

with the /? as follows:

MM /?

Here is a list of all the available options in this shareware

version:

Option          What it does

/d              List Device Drivers

/ec             Display Current Environment

/b              Show DOS v5.0+ Upper Memory Blocks

/?              Help List

Examples:

- To see a list of installed device drivers type:

MM /d

- To see a the current environment type:

MM /ec

The Display

�����������

When you run MM without any parameters you get a list of all the

Memory Control Blocks (MCBs) and relevant information. You also get

the amount free memory and whether UMBs are used (even if not

displayed). The top of the display looks like this:

Total DOS Memory  xxxK                  DOS version: m.mm

Free Memory       yyyK                  DOS 5+ UMBs: [Yes/No]

EBDA Allocated    [No/Yes sizeK]        Processor: [pro. type]

Total DOS Memory

����������������

The total amount of Low (below 640K) memory. This field is

usually 640K but can be lower on some IBM PS/2 models because

of the Extended BIOS Data Area (EBDA). This is an area of

memory used to store some configuration infomation for the

BIOS. This area can be as little as 1K and usually sits at

the top of memory (This will not show up on the memory list.)

Free Memory

�����������

The largest available memory block, i.e. The largest

executable program size. This is the maximum amount of

memory a program can use. It is usually about 550K, but

with modern memory managers (like 386MAX, QEMM) the

amount of free memory can be about 618K.

DOS Version

�����������

The DOS version currently running. This is cannot be

changed by the DOS 5+ SETVER program.

DOS 5+ UMBs

�����������

Reports whether UMBs are available for use regardless

wether the are displayed.

EBDA Allocated

��������������

If this field shows Yes the Extended BIOS Data Area (EBDA)

is allocated the size size in kilobytes is also shown. The

field is No, if the EBDA is not allocated. For more

infomation see the section Total DOS Memory.

Processor Type

��������������

This field is the processor type. The processor is the

main microchip in the computer. It controls the entire

computer and carries out all program execution and

arithmetic. The types this program can detect are as

follows:

Intel 8088

Intel 8086

NEC V20

NEC V30

Intel 80188

Intel 80186

Intel 80286

Intel 80386SX

Intel 80386DX

Intel 80486

The memory status display shows infomation an the Memory

Control Blocks (MCBs). Memory is split into blocks, each

block in owned by a program. The MCB holds infomation

about the block and the name of the program which ownes

it. Each field of the memory status display is outlined below.

MCB

���

The segment in memory of the MCB. PC Memory is split

into 64K segments. The first segment starts at

absolute address 0. This is written in Segment:Offset

form as 0:0. From then a new segment begins at each

16 byte interval. There are A000h (40960) segments

in the 640K low memory. An MCB begins at Seg:0.

PSP

���

The Program Segment Prefix (PSP) that owns the MCB.

A PSP is a 256 byte block that contains infomation

about a program in memory.

Parent

������

The Parent PSP of the PSP that owns the MCB. The parent

PSP is usually the PSP of the program that executed the

owner of the current MCB. The DOS shell, i.e. COMMAND.COM

is its own parent.

Size

����

The size in bytes of the area that the MCB controls.

Env.

����

If this field is blank, this is not the MCB of an

environment.

If this field has one box '�' in it, it is an MCB

that controls an environment.

If this field has two boxes '��' in it, it is the MCB

that controls the master environment. The master environment

is the environment of the current primary command

interpreter. This routine will fail if you load another

copy of COMMNAD with the /P (primary) switch. The Master

Environment is not shown in Easy mode.

Owner

�����

The name of the program that owns the MCB with the

following exceptions:

<DOS Shell> is the command interpreter, usually COMMAND.COM

<DOS Kernel> is MSDOS.SYS (or IBMDOS.COM)

<Free> is a free block

<N/A> means the name cannot be determined

Hooked Interrupts

�����������������

This is a list of all the interrupts that point into

the area controlled by the MCB.

Command Line  ** Registed Version Only **

������������

The command line of the program as it was run.

This is taken from the PSP. The command line might not

be available because DOS also puts the default DTA

(Disk Transfer Area) in the same place.

** Easy Mode and UMB Usage are available in the Registerd Version Only **

If MM is run in 'Easy' mode the display will be slightly different.

It will show the PSP, Size, Env., Owner, Command Line, and some

Hooked Interrupts. (See Hooked Interrupts section) If MM is run with

the /u switch to show UMB Usage, the UMB Usage will follow on from

the normal MCBs display in the same format.

Device Driver List

������������������

This is shown using the /d switch

A device driver is a program which provides standerdised access

to any perhiphial attached to the computer.

Device Name

�����������

If it is a character device driver it will display

the name of the device driver. Otherwise, if it is

a block device driver it will display the number of

block devices (disk drives, CD-ROMs, etc.) it controls.

High

����

If the field is blank, the device driver is loaded in

low memory. (below 640K)

If the field one box '�' in it, the device driver is

loaded in an UMB.

Upper Memory Blocks

�������������������

This shows the actual UMBs and their addresses in memory but

does not show their usage. You can only see their usage in

the registered version.

Start

�����

The Segment in memory where the UMB begins. For more

infomation see section on MCBs

Size

����

The Size in bytes of the UMB.

Type

����

This can be either 'UMB' which means it is

an Upper Memory Block, or 'SM' which means

a gap between UMBs or a name, eg. COMPAQ

for COMPAQ CEMM Memory Manager.

Environment [Current / Master] ** Master in Registerd Version Only **

������������������������������

This shows the current environment, which is the

environment used by the current program being executed

[This Program], or the master environment, which is

the environment of the <Dos Shell>.

Env. Variable

�������������

The name of the environment variable.

Contents

��������

The contents of the environment variable.

Return Codes

������������

These are numbers which can be detected by the DOS ERRORLEVEL

command. They return infomation about how the program ran.

In MM the return codes are as follows:

0 - The program run without any problems

255 - Wrong DOS Version

254 - A bad command line option

253 - MM was loaded high (with DOS 5+ LOADHIGH command)

252 - No Upper Memory Blocks (when using /b switch)

To use these codes you make a batch file with a line as follows:

IF ERRORLEVEL == 252 THEN GOTO ERROR

This means that if a number of 252 OR GREATER has been returned

goto the label ERROR.

A sample batch file has been included with this program called

MMRUN.BAT

Notes

�����

This program is compressed with LZEXE v0.91

<END>