COMLINEAR CORPORATION MACRO-MODELS DISKETTE

4800 Wheaton Drive

Fort Collins, CO. 80525

303-226-0500

These files are available from Comlinear Corporation,

located in Fort Collins, Colorado.  Comlinear is a

manufacturer and supplier of high-performance analog

signal processing components.  Comlinear's signal

conditioning product line includes high-speed hybrid and

monolithic  operational amplifiers, buffers,

multiplexers, automatic gain control integrated circuits,

track/hold amplifiers, digital-to-analog and analog-to-

digital converters.  Comlinear is focused on developing

products that make a significant contribution to solving

customer high speed problems.

Introduction

This disk is a collection of Pspice models for Comlinear

amplifiers.  Additional macro-models for existing

products, and new products will be available for

Comlinear customers.   Contact Comlinear Corporation

(303-226-0500) for release dates of the desired models

not found on this diskette.  There is one level of macro-

model, intended to provide the customer with the ability

to model Comlinear parts on Spice platforms.  Comlinear

provides models for IBM compatible computers using

analysis programs that accept Spice formats.  Comlinear

does not assume responsibility for designs created from these

macro-models.  These files give typical modeled

performance.  Before designs are released to production,

Comlinear suggest that topologies be verified through

prototyping the circuit. The part to part performance of

Comlinear op amps are specified in the most current

data sheets (Ref. 1).  For more detailed information

concerning the use of these models refer to application note

OA-18.

Files on this disk are labeled using the following format:

TABLE I,  MACROMODEL SUBCIRCUIT FILES ON THIS DISKETTE

file name          amplifier description

CLC400.CIR         low gain wide bandwidth monolithic

CLC401.CIR         high gain wide bandwidth monolithic

CLC402.CIR         low gain with fast 14-bit settling

CLC406.CIR         low cost low power monolithic

CLC410.CIR         wide bandwidth monolithic w/disable

CLC430.CIR         +/-15V low gain video monolithic

CLC404.CIR         wideband, high-slew rate

CLC409.CIR         very wideband, low distortion

CLC414.CIR         quad, low power monolithic

CLC415.CIR         quad, wideband monolithic

CLC420.CIR         unity gain stable voltage feedback

CLC425.CIR         low noise voltage feedback monolithic

CLC501.CIR         high gain, output limiting

CLC502.CIR         low gain, output limiting 14bit settling

CLC505.CIR         adjustable supply current

CLC520.CIR         amplifier with voltage controlled gain

The macro-models are found in the root directory of the

data disk supplied.

START UP INSTRUCTIONS

Comlinear suggests that the user make back up copies of spice

files to another floppy and copy all models of interest to a

library on the hard disk.  If the library directory is

not in the path of the spice program, the user should set

the path in the autoexec.bat for easier excess. The

include statement in Pspice .INC should be used in the

simulation file to include the macro-models subcircuit.

example      " .INC CLC400.CIR"

OP AMP MACRO-MODELS

The op amp macro-models were created for simulation

software that take Pspice compatible files written in

ASCII format on IBM-compatible PC's.  For detailed

information on using PSpice contact MicroSim (see ref.

2).   The op amp macro-models are designed as subcircuits

to allow the engineer simplified incorporation of these

models into larger circuits.  A listing of any op amp

subcircuit may be obtained by printing the file to a

local printer.

The subcircuit node assignments match the device pin-outs as

shown in the individual device data sheets.  An example is an

8 pin op amp.

* Connections:    NON-INVERTING INPUT PIN

*                 | INVERTING INPUT PIN

*                 | | OUTPUT

*                 | | | +Vcc

*                 | | | | -Vcc

*                 | | | | |

.SUBCKT (NAME)    3 2 6 7 4

Several schematic capture software require a different connection

order for inputs, outputs, supplies, etc., that might be different

than what is used by Comlinear.  If the existing pin connection order

creates a problem, then changing the order in the subckt (name) to

allow captibility will not effect modeling of the part.

PERFORMANCE RESULTS

When subsitutions of current feedback op amps are made for voltage

feedback op amps the results might not be acceptable.  Refer to OA-13

for a tutorial on current feedback op amp design.

PARAMETERS MODELED

The following typical performance parameters are modeled by

the subcircuits.

DC Effects

* VIO, IBI, IBN

* Supply current vs. Supply voltages

* Common Mode Input/Output Voltage range

* Load current from supplies

AC Effects

* Frequency response vs. gain & load.

* Open loop gain & phase

* Noise

* Small signal Input/Output Impedance

* CMRR

Time Domain

* Rise / fall times

* Slew Rates

Special Features (where applicable)

* Output Clamping

* Supply Current Adjustment

* Offset voltage adjust

* Disable / Enable times

PARAMETERS NOT MODELED

* Part to Part performance variation

* Variation in performance vs.temperature.

* Fine scale settling performance

* Harmonic distortion

* Differential gain

* Differential phase

* Thermal tail

* Overdrive recovery time

* PSRR

GLOSSARY OF SPECIFICATIONS SYMBOLS IN DEVICE DATA SHEETS

SSBW small signal bandwidth

LSBW large signal bandwidth

GFPL gain flatness peaking low

GFPH gain flatness peaking high

GFR gain flatness rolloff

TRS rise time for small signal

TRL rise time for large signal

TS settling time for step response

OS percent of overshoot

SR slew rate

HD2 2nd harmonic distortion

HD3 3rd harmonic distortion

SNF noise floor in dbm(1Hz) (input referred)

INV integrated noise in micro-volts (input referred)

VIO input offset voltage

DVIO average temperature coefficient of VIO

IBN input bias current non-inverting

DIBN average temperature coefficient of IBN

IBI input bias current inverting

DIBI average temperature coefficient IBI

PSRR power supply rejection ratio

CMRR common mode rejection ration

ICC supply current   no load

RIN non-inverting input resistance

CIN non-inverting input capacitance

RO output impedance closed loop at DC

VO output voltage range no load

CMIR common mode input range

IO output current

Vcc power supply voltages

TOFF disable time

TON enable time

VDIS voltage to disable output

VEN voltage to enable output

IDIS current to disable output

IEN current to enable output

OSD off isolation (part disabled)

ROD output impedance, disabled

COD output impedance, disabled

OVC overshoot voltage clamp in percent

TSO overload recovery time from clamp

CDR Vio drift after recovery from clamp

VOC voltage clamp accuracy

ICL input bias current on clamp inputs

CBW -3dB bandwidth of clamp

CMC useful clamping voltage range

NOTICE

The information provided within these file is believed to

be reliable.  Comlinear assumes no responsibility

for alternations, omissions or inaccuracies.  Comlinear

assumes no responsibility for the use of this

information, and all use of such information shall be

entirely at the user's own risk.  Comlinear does not

grant licenses or patent rights to any of the circuits

described within this document.

REFERENCES

1) Comlinear Data Book of Standard Products.

2) MicroSim Corporation, 20 Fairbanks, Irvine, CA 92718

USA, (714) 770-3022, (800) 245-3022.