Michael,

DAK's number is 800-788-7808, and the item number for

Microsoft Works Ultralite is 5826, currently sellnig for $29.90,

which includes the manuals, as you predicted.

Macronix's number is 800-432-1621, and they sell the serial cable

as part numbe MXA-000010, the MAXLITE/PC RS-232 cable, for $12.00.

The instructions for how to change the cluster size on your silicon

drive are as follows:

1.  get all your files safely off the ultralite

2.  pull out the silicon drive and wait 5 minutes to be sure it

has forgotten everything it knows

3.  re-insert silicon drive, boot ultralite, perform the standard

disk init sequence "Initialize, Format, Setup Manager". In the event

you have problems with "NON-SYSTEM DISK" messages and the machine won't

boot, try using FILE MANAGER on D: to initialize C: then break out of the

FILE MANAGER and format C: without the /S switch that makes it a system

disk.

4.  Now use laplink to get a disk sector editor onto the machine.

I am using "YASE.EXE", which I am sending to you with this

message.

5.  Run YASE, tell it to use drive C, make sure you are

looking at sector 0.

6.  Using the L key (press it 13 times), move to offset 00d, that's 13 in

decimal.  It should currently say 08 as its value.  This means 8

sectors-per-cluster, or 4K.

7.  Use E (Edit cursor position) to change this value to 02, 2

sectors-per-cluster, which is 1K.

8.  Using the L key (press it 9 times) move to offset 016, that's 22

in decimal.  If you have a 1Meg drive, the value should currently

read 01; if you have a 2Meg drive it should currently read 02.

This value is the number of sectors in a FAT.

9.  If you have a 1Meg drive, set the value to 03.

If you have a 2Meg drive, set the value to 08.

[Warning: I don't understand why this value matters, but I tested

my ultralite with a 2Meg drive but setting this value to 03 and

when I created lots of files the disk got corrupted.  However, with

08 my machine seems to work fine.  The parameters given above were

supplied to me by brian@theory3.caltech.edu.]

10.  Write out sector 0.

11.  Reboot the ultralite using control-alt-FUNCTION-delete to use drive D.

12.  Perform the standard "Initialize, Format, Setup manager" sequence.

[Note: It seems odd to me, but this uses the values from sector 0; I

would have expected this procedure to start from scratch.]

13.  Now try out drive C:, it should be using 1K blocks.  You can tell this

by creating a short file and running DIR and looking at how much the

available space went down.

To use YASE.EXE for editing sector 0, hit C when it starts up to pick

drive C, then hit ? for help on using the keyboard commands.

Hope this helps,

--Jim O'Toole

Phone: 617-253-6018

Internet: otoole@mit.edu