14.4Kbps MODEM

Using the 14.4Kbps Modem Documentation File.

This file provides installation and operating instructions for the

14.4Kbps MODEM, including internal and external versions. The

information assumes the user has basic computer skills but will

attempt to explain modem operations and command structure. All

sections should be read carefully before beginning any installation

procedures. In addition to following the instructions provided

here, you will also need to consult the documentation supplied with

your communications software.

IMPORTANT NOTICE

FCC Requirements

The Federal Communications Commission (FCC) restricts the way you

can use modems. Read the FCC compliance statement found in Appendix

C of this file.

Telephone Company Requirements

Some telephone companies require notification when any electronic

communications equipment is connected to the telephone lines.

Contact the phone company before connecting your modem to the

telephone line and provide them with the following information:

* The telephone number of the line that you are attaching the

modem.

*  The modem's FCC registration number and ringer equivalence

number. These numbers are printed on the FCC label which appears on

the main chip in the center of the internal modem board, or on the

underside of the external modem.

You can connect the modem to various types of telephone jacks. The

acceptable phone jack types are RJ-11, RJ-12, RJ-13, RJ-41S, and

RJ-45S. Most homes and businesses use one of these jacks. If your

phone system does not have a modular jack, you can purchase an

adapter to convert your jack into an RJ-11C jack. Adapters may be

purchased from any local phone or electronics store.

***************************************************************

QUICKSTART

If you are familiar with personal computers and modems, you may

follow the accompanying diagram to quickly install either the

INTERNAL or EXTERNAL version of the 14.4Kbps modem in your system.

Make all necessary connections as shown in those diagrams.

To verify line connection, lift your telephone receiver to check

for a dial tone. If you are using a dedicated line, plug the line

into any phone. After hearing the tone, power on your computer,

load your communication software, and put the program into command

mode. If you do not hear a dial tone, see Section Eight:

Troubleshooting.

****************************************************************

Introduction

The 14.4Kbps modem (internal or external version) combines

high-speed data, modem, and fax capability on a single device. It

features V.32bis, which provides high-speed data transmission plus

support for V.42bis (error correction) and MNP5 (data compression).

Features

*  Data transmission rates of 14.4K, 12K, 9600, 7200, 4800, 2400,

1200, or 300bps.

*  100% data compatibility with the Hayes 'AT' command set.

*  Programmable speaker volume control

*  Automatic voice and data switching.

*  Auto-speed sensing.

*  V.32bis, V.42bis, and MNP5 are supported as well as V.42 and MNP

2-4 error correction, offering throughput speed of 57,600 bps.

*  Group III send and receive fax capability with support for Class

1 and Class 2 fax commands.

*  Supports 100 x 196 high resolution and 100 x 98 low resolution.

*  Supports high-level data-link control framing.

*  Communication ports selectable as COM1, 2, 3, or 4 using

interrupts 3, 4, 5, or 7 (INTERNAL ONLY).

To get started, go to:

*  SECTION ONE IF YOU HAVE THE INTERNAL MODEL.

*  SECTION TWO IF YOU HAVE THE EXTERNAL MODEL.

NOTE ON TERMINOLOGY: When you encounter the following acronyms,

keep in mind their meanings:

DTE. Data Terminal Equipment. The computer or terminal, either

local (yours), or the remote (the one you're communicating with).

(computer-to-modem link)

DCE. Data Communications Equipment. The local and/or remote modem.

(modem-to-modem link)

DTR. Data Terminal Ready. Computer provides this signal to the

attached modem indicating that it is ready to receive data.

Suggested Additional Technical Resource:

* The Complete Modem Reference, Gilbert Held, John Wiley & Sons,

Inc.

Installation Hints

Your MODEM has been initialized during factory final test for

optimum performance with included fax communications software. The

following initialization (INIT) string has been found to work with

many popular communication packages:

AT&F~~~E1V1X4&C1&D2%E1

Some bulletin boards and information services have problems with

data compression. If so, add %C0 (0=zero) to the init string.

If you experience difficulties sending or receiving faxes, please

disable all TSR (terminate and stay resident) programs. A TSR

program is a program which remains in memory after it loads. The

easiest way to test the fax send/receive function by disabling the

TSRs is to boot your computer with a plain DOS boot diskette (no

config.sys or autoexec.bat files on the boot diskette). Change to

the directory containing your fax software. Load the software and

send a fax.

If you experience a data connection difficulty with a U.S. Robotics

HST Dual Standard Modem on a BBS, the following should allow you to

connect. First, set the modem software to a DTE speed of 9600. Then

enter:

ATN0      Follows the S37 Register

ATS37=9   Forces modem to connect at 9600 bps.

HOW THIS FILE IS ORGANIZED

Quick Start. If you are familiar with modem operations, this

abbreviated procedure provides a short cut for installing either an

internal or external version of the 14.4Kbps modem. Otherwise, read

Section One before continuing.

Section One and Two : Getting Started. These sections provide

communication settings (internal only), physical installation

instructions (including how to connect the phone line), and

procedures for testing the connection). Refer to your

Communications Software Manual for fax command operation.

Section Three: Data Modem Operation. A discussion of data modem

functions and AT commands and syntax. Also included are definitions

of the modem's S-Registers. These registers are used to change

certain operating characteristics and obtain information about the

modem's state of readiness, as well as test operation.

Section Four: MNP Operation. This section explains how to make use

of the Microcom Networking Protocols. MNP is an error correction

protocol developed by Microcom and is widely supported by modem

manufacturers.

Section Five: Error Correction: V.42/V.42bis. V.42 and V.42bis are

CCITT (Consultative Committee on International Telegraph and

Telephone) standards for error correction and data compression,

respectively. This section describes the AT commands and

S-registers which activate and control these functions.

Section Six: V.32bis/V.32 Operation: A brief discussion of these

CCITT standards for high-speed data transmission.

Section Seven: Diagnostics. Tests may be used to evaluate the

modem's state of operation, its connection, and the communications

link between the 14.4K Modem and the remote modem.

Section Eight: Troubleshooting. This provides a description of the

most common problems which may be encountered during installation

and operation along with possible solutions.

Appendices. These include technical specifications and FCC

compliance information as well as a comprehensive glossary.

Contents

Introduction

Section One: Getting Started (Internal)

1.1 14.4Kbps modem Package

1.2 Communication Settings

1.3 Installing the 14.4Kbps modem

1.4 Connecting the Phone Line

1.4.1 Connecting the Line to the 14.4Kbps Modem

1.4.2 Testing the Connection

Section Two: Getting Started (External)

2.1 14.4Kbps modem Package

2.2 Operational Requirements

2.3 Connecting the Phone Line

2.4 Preparing the System

2.5 Testing the Modem

2.6 Testing the Connection

Section Three: Data Modem Operation

3.1 Making a Call

3.2 Touch-Tone and Pulse Dialing

3.3 Pause While Dialing

3.4 Using the Hook Flash

3.5 Wait for a Subsequent Dial Tone

3.6 Wait for Silence

3.7 Resume Command State after Dialing

3.8 Switch Hook Control

3.9 Manual Dialing

3.10 Manual Answer

3.11 Controlling the Speaker

3.12 Dial Stored Number

3.13 The AT Command Set

3.13.1 Basic Command Set

3.13.2 Extended Command Set

3.14 S-Registers

Section Four: MNP Operation

4.1 Introduction

4.2 MNP Data Mode

4.3 AT Commands for MNP Operation

4.4 S-Register Values for MNP Operation

Section Five: Error Correction (V.42bis)

5.1 Introduction

5.2 S-Register Values for Error Correction

5.2.1 Negotiation Fallback

5.2.2 Delay Before Forced Connect

5.2.3 Protocol Selection

5.2.4 Negotiation Action

5.2.5 Bit-Mapped Registers

5.2.7 Connect Failure Cause

5.2.8 Extended Results Code

5.3 AT Commands for Error Correction

Section Six:V.32bis/V.32 Operation

Section Seven: Diagnostics

7.1 Local Analog Loopback Test

7.2 Local Analog Loopback With Self-Test

7.3 Remote Digital Loopback Test

7.4 Remote Digital Loopback With Self Test

Section Eight: Troubleshooting

Appendix A: How to Set Bit-Mapped Registers

Appendix B: Technical Specifications

Appendix C: Compliance Information

Glossary

Section One: Getting Started: Internal Model

This section explains how to install the 14.4Kbps internal Modem

into your computer.

1.1 14.4Kbps Modem Package

Verify that your modem package contains the following items:

* 14.4Kbps modem Board

* RJ-11 Telephone Cable

* Communication Software and manual

* Software diskette containing this file, and sheet with diagrams.

1.2 Communication Settings

The 14.4Kbps Modem can be configured as COM1, COM2, COM3, or COM4.

The default setting for the modem is COM1 utilizing IRQ4. The

following shows standard COM port and IRQ settings:

COM1/IRQ4      COM2/IRQ3      COM3/IRQ4      COM4/IRQ3

Because two COM ports will share a single interrupt, you must be

careful in choosing which device will share the same interrupt with

the modem. Generally, the modem cannot share the same interrupt

with any device that is driven by a TSR (terminate and stay

resident) program. For example, if you have a mouse on COM1, the

modem cannot be set to COM3; if the mouse is on COM2, the modem

cannot be set to COM4.

One way to avoid interrupt sharing is to select a non-standard

interrupt (i.e., IRQ) if it is available, such as IRQ5. However,

your communications software must be able to select this

non-standard interrupt as well.

1. Remove the 14.4Kbps Modem from its anti-static bag handling it

by its edges and metal retaining bracket. Be careful not to touch

the edge connector or any of the components on the modem.

2. Determine how many serial ports are already installed in your

system. Set the jumpers on the 14.4Kbps Modem for the next

available port. See the accompanying illustration for assistance in

jumper settings.

1.3 Installing the 14.4Kbps modem

This section provides step by step instructions for installing your

14.4Kbps Internal Modem.

1. Power off the computer and disconnect any attached devices and

power cords.

2. Remove the computer's cover and locate an available 8- or 16-bit

expansion slot. Remove that expansion slot cover and save the

screw.

3. Carefully insert the 14.4Kbps modem into the expansion slot

applying pressure to the upper metal edge until it snaps into

place.

4. Secure the 14.4Kbps modem card into place by aligning the

14.4Kbps modem's metal retaining bracket with the hole in the top

edge of the system's unit rear panel. Fasten the metal bracket with

the screw removed earlier from the expansion slot cover.

5. Replace the computer cover.

1.4 Connecting the Phone Line

An RJ-11 cable is provided with the purchase of your 14.4Kbps

modem. This section contains information you will need before you

connect the telephone line to the 14.4Kbps modem and explains how

to make the proper connections from the 14.4Kbps modem to your

telephone line.

Before you connect the telephone line to the 14.4Kbps modem, you

should familiarize yourself, if you haven't done so already, with

the information provided at the beginning of this file concerning

FCC and telephone company requirements.

1.4.1 Connecting the Phone Line to the 14.4K Modem

1. Make sure that your computer is still powered down. Plug either

end of the RJ-11 cable provided with the 14.4Kbps modem into the

"LINE" jack on the modem board.

2. If a phone is connected to the telephone wall jack, unplug the

phone cable. Then plug the other end of the 14.4Kbps modem cable

into the telephone wall jack.

3. To continue using the phone, connect the phone cable into the

"PHONE" jack on the 14.4Kbps modem. See Quick Start! illustration

on page 2.

1.4.2 Testing the Connection

1. Check for a dial tone. If a dial tone is heard, proceed with the

next step. If you do not hear a dial tone, check the connections

you made in the previous subsection (1.4.1)

2. Power on the computer.

3. Load your communications software and put the program into

command mode.

4. Type in the command ATZ followed by ENTER and the 14.4Kbps modem

will respond with OK after about three seconds. If the 14.4Kbps

modem does not respond with OK, refer to the Troubleshooting

section of this file.

5. Type ATH1 followed by ENTER and you should hear a dial tone from

the speaker on the modem. To adjust speaker volume, see the section

on AT Commands.

6. Type ATH followed by ENTER to put the modem on hook. This

confirms that the 14.4Kbps modem has been successfully installed

into the computer.

7. Your 14.4Kbps internal Modem is now ready for use. Proceed to

Section Three for Data Modem Operation.

Section Two: Getting Started: External Model

This section explains how to connect the 14.4Kbps external Modem to

your computer.

2.1 14.4Kbps modem Package

Verify that your modem package contains the following items:

*  14.4Kbps modem

*  RJ-11 Telephone Cable

*  25-pin to 9-pin RS-232 cable

*  9-pin to 25-pin converter

*  AC adapter with cord

*  Communication Software and Manual

*  Software diskette containing this file, and sheet with diagrams.

2.2 Operational Requirements

The 14.4Kbps Modem has few requirements and is easy to use. Be sure

to read the information on page 1 about connecting to the phone

company. In addition, note the following:

*  The communication settings are controlled by the RS-232

interface present in your system. The external modem is connected

to that interface.

*  Operating the 14.4Kbps modem with a computer requires

communication software, either as a stand-alone product as included

here, or as part of an application program.

*  The 14.4Kbps modem takes a DB-25P (25-pin male connector), but

computer equipment varies. Check the serial ports at the rear of

your machine.

*  The serial port requires a socket (female connector) or plug

(male connector) with either 25 or 9 pins. For example, the IBM PC,

XT, and some compatibles require a DB-25S socket connector, while

the IBM AT and some compatibles require a DB-9S socket connector.

2.3 Connecting the Phone Line

An RJ-11 cable is provided with the purchase of your 14.4Kbps

modem. This section contains information you will need before you

connect the telephones line to the 14.4Kbps modem and explains how

to make the proper connections from the 14.4Kbps modem to your

telephone line.

Before you connect the telephone line to the 14.4Kbps modem, you

should familiarize yourself, if you haven't done so already, with

the information provided at the beginning of this file concerning

FCC and telephone company requirements.

2.4 Preparing the Modem

Follow these simple steps to connect the modem to your computer:

1. Turn off your computer and all peripheral devices.

2. Examine the back of the modem and review the attached interfaces

as shown on the accompanying illustration.

3. Check to make sure that power switch is OFF. Then plug the small

end of the power adapter into the power jack on the back of the

modem, and plug the power adapter into a standard 115V AC wall

socket.

Disconnect your present phone cord from the wall jack. Plug the end

of the phone cord that came with the modem into the wall jack, and

the other end into the phone jack at the rear of the modem marked

LINE.

If you want to keep your telephone connected for conventional

calls, plug its cord into the other jack at the rear of the modem

labeled PHONE.

4. Lastly, connect the modem to the computer's serial port with the

RS-232 cable. Attach the appropriate connectors to the modem and to

the serial port. The modem is now ready to be tested and operated.

2.5 Testing the Modem

To verify that your modem is working properly, follow these steps:

Turn on your computer and clear the screen. Then turn the modem's

power switch ON. The following status indicators will flash

accordingly as explained on below:

Indicator      Definition     Status

TM             Test Mode      FLASHES when modem is in

test mode and if any errors are

detected.

AA             Auto Answer/   ON when in Auto Answer Mode

Answer Mode    and when online with the host

modem.

HS             High speed     ON when the 14.4Kbps modem is

powered on or communicating with

another modem at 4800 bps or

faster.

OH             Off Hook       ON when the 14.4Kbps modem takes

control of the telephone line

to establish a data link.

SD             Send Data      FLASHES when a data bit is sent to

the 14.4Kbps modem by the computer.

RD             Received Data  FLASHES when a data bit is

received from the phone line, or when

the modem sends result codes to the

computer.

TR             Terminal       ON when the modem receives a

Ready          Data Terminal Ready signal from

the computer via the RS-232

cable.

CD             Carrier Detect ON when the 14.4Kbps modem

receives a valid data signal

(carrier) from a remote modem. Ready

for data transmission.

2.6 Testing the Connection

1. Turn on the modem (PWR toggle switch) and check for a dial tone.

If a dial tone is heard, continue with the next step. If you do not

hear a dial tone, check the connections you made in 2.4 Preparing

the Modem. Also, refer to Section Eight: Troubleshooting.

2. Power on the computer.

3. Load your communications software and put the program into

command mode.

4. Type in the command ATZ followed by ENTER and the 14.4Kbps modem

will respond with OK. If the 14.4Kbps modem does not respond with

OK, refer to the Troubleshooting section of this file.

5. Type ATH1 followed by ENTER and you should hear a dial tone from

the speaker on the modem. To adjust speaker volume, see the section

on AT Commands.

6. Type ATH followed by ENTER to put the modem on hook. This

confirms that the 14.4Kbps modem has been successfully installed

into the computer.

7. Your 14.4Kbps external Modem is now ready for use. Proceed to

Section Three for Data Modem Operation.

Section Three: Data Modem Operation

Introduction

The purpose of this section is to define the most commonly used

Hayes-compatible AT commands for use with your modem. In addition,

we've also provided information on extended AT command sets,

S-Registers. Subsequent sections discuss commands and registers for

high-level protocols such as MNP, V.42/V.42bis, and V.32/V.32bis.

Hayes-compatible commands consist of a basic command set and an

extended command set. The basic set involves functions such as

dialing a number, or putting the modem off- or on-hook (i.e.,

lifting or replacing the telephone handset).

Extended commands allow more sophisticated control of the modem

such as transmission speed, or initiating high-level functions like

data compression or error correction. These functions are defined

and controlled by the available protocols mentioned above. A

protocol is a set of standards by which data communications

operate. Every AT command includes an AT prefix, followed

immediately by the command and, in many cases, additional

parameters. Multiple commands can be entered at the same time from

the DOS prompt.

AT Command[parameter]AT Command[parameter] ... PRESS ENTER

Example:   ATH or ATH0 tells the modem to disconnect

ATH1 tells the modem to connect, or go off-hook.

Extended commands were developed to provide greater functionality

and control over modem operations. Their format is the same as the

basic command except that an additional parameter is required

following the AT prefix and before the numerical parameter.

This additional parameter comes in three different forms:

the ampersand (&) character:    extended AT command

the backslash (\) character:    extended AT command for MNP

operation

the percent (%) character:      extended AT command for

V.42/V.42bis

Examples: AT&V tells the modem to display (view) its current

configuration and user profile (providing the modem has

non-volatile RAM, meaning it can store limited amounts of

information when power is off.

AT\J1 tells the computer to adjust its speed to match line speed

when a connection is established.

AT%E1 tells the modem to monitor line quality and request a

retrain (adjustment) to the connection.

S-Registers

Modem command language also employs a set of indicators or

registers, which are various numerical values all with a standard

S  prefix, hence S-Registers. To a large extent, the values defined

in the S-Registers regulate the operation of the modem and the

function of some commands in the AT command set.

Example: S-Register 6, or S6=n, defines the length of time the

modem will wait for a dial tone. In this case the acceptable range

is 2-255 (in seconds) with a default value of 2. With S6=2, the

modem will wait two seconds for a dial tone when going off-hook

before dialing the first digit of its telephone number.

Data Communication Protocols

This can be more than a little confusing. These protocols represent

various domestic and international standards which enhance modem

performance and reliability. The protocols are activated and

controlled by a variety of extended AT commands and S-Registers.

MNP stands for Microcom Networking Protocol and is a protocol

developed by Microcom for full-duplex, error-free communications.

This protocol detects and corrects errors which can result from

telephone line noise and other signal distortions. There are

several classes of MNP operation also referred to as service

classes. Class 5 maximizes data transfer rate and provides

compression which can significantly increase data throughput.

The V-Dot standards are more numerous, but have a single origin:

the Consulting Committee for International and Telephone and

Telegraph (or CCITT). Some of the lower-level standards such as

V.21 and V.22 have domestic equivalents as developed by the former

Bell System, also referred to as Bell standards. The V.Dot

standards may be summarized as follows:

V.21  The CCITT standard for 300bps communications. Domestic modems

follow the Bell 103 standard, but V.21 can accept international

calls at 300bps.

V.22  The CCITT standard for 1200bps communications. The domestic

equivalent is the Bell 212A standard.

V.22bis  The CCITT standard for 2400bps.

V.23  CCITT for 1200bps with a 75bps back channel. This is mostly

used in Europe and South America.

V.32  CCITT standard for 9600bps and 4800bps communications.

V.32bis  CCITT standard for an extensive class of high-speed modems

operating at 14,400bps, 12Kbps, 9600bps, 7200bps, and 4800bps.

V.42  CCITT standard for detection and negotiation for LAPM (Link

Access Procedure for Modems) error control. V.42 will also support

MNP levels 2-4.

V.42bis  An extension of V.42 specifying the data compression

protocol for use with V.42.

Basic Modem Operation

Use the D command to dial a telephone number with your modem from

the command line, i.e., DOS prompt. The format of the command is

ATD {string}. The string parameter can contain up to 40 characters,

the phone number, and dial modifiers. The dial modifier instructs

the modem how to place the call.

3.1 Making a Call

To dial, issue the command ATD followed by the phone number.

ATD 1234567 [ENTER]

The modem dials the telephone number 1234567 and then waits for a

carrier from a distant, or remote modem. If no carrier is detected

within a given time (default time is 50 seconds), then the modem

automatically releases the line and sends a NO CARRIER result code.

If a carrier is detected, the modem gives a CONNECT result code and

goes on-line, allowing communication with the remote modem. The

connection between the two modems end when any of the following

occurs:

* The local modem loses the carrier signal from the remote modem.

* The Hang Up command (ATH) is sent.

* The DTR interface signal is dropped between the local DTE and

modem with the &D2 or &D3 command in effect.

Any of the above actions will cause the modem to hang up, return to

command mode, and send the NO CARRIER response. Dial modifiers are

summarized as follows:

L Re-dial the last dialed string

P Pulse dialing (default)

T Tone dialing

, Pause before sending the next character

! Hook flash

W Wait for a subsequent dial tone

@ Wait for quiet before proceeding

; Place modem in command state after dialing

^ Disable calling tone.

S=n Dial stored number.

3.2 Touch-Tone and Pulse Dialing

The modem can be used on telephone lines that require either

touch-tone (DTMF) dialing or pulse (rotary) dialing. Pulse dialing

is the default method when you turn on the modem but you can switch

between pulse and tone dialing by using the P or T dial modifiers.

To tell the modem which dialing method to use, type P or T in front

of the numbers to be dialed. Examples:

Touch-Tone Dialing:     AT D T 555-1212 [ENTER]

Pulse Dialing:          AT D P 555-1212 [ENTER]

Touch-tone and pulse dialing can also be combined in a dial command

line when both dialing methods are required. The following command

line is used to dial through a PBX that uses touch-tone dialing and

the outside lines require pulse dialing:

AT D T 9 W P  555-1212 [ENTER]

This command line tells the modem to touch-tone dial 9 to get an

outside line, wait (W) for a secondary dial tone (see Section Four

on S-Registers), and then pulse dial the number 555-1212.

3.3 Pause While Dialing

The , (comma) dial modifier causes the modem to pause while

dialing. The modem will pause the number of seconds specified  in

the Register S8 and then continue dialing. The default value of

Register S8 is two seconds. If a pause time longer than 2 seconds

is necessary, it can be increased by either inserting more than one

, in the dial command line or changing the value of Register S8. An

example of a typical dial command line follows:

AT D 9, 555-1212 [ENTER]

This command accesses the outside (public) telephone line with the

9 parameter. The modem then waits 2 seconds with the , and then

dials the telephone number 555-1212.

3.4 Using the Hook Flash

The ! dial modifier causes the modem to go on-hook (hang up) for

one-half second and is equivalent to holding down the switch-hook

on your telephone for one-half second. This feature is useful when

transferring calls.

3.5 Wait for a Subsequent Dial Tone

The W  dial modifier causes the modem to wait for an additional

dial tone before dialing the numbers that follow the W. The length

of time the modem waits is up to the value in Register S7 (default

is 50 seconds). The modem can be instructed to dial through PBXs

(Private Branch Exchanges) or long-distance calling services that

require delays during dialing. This can be done with the W command

to wait for a secondary dial tone or with a , command to pause for

a fixed time and then dial. An example of a Dial command line when

dialing through a PBX follows: AT D 9 W 555-1212 [ENTER].

The 9 accesses the outside (public) telephone line. The W causes

the modem to wait 50 seconds for a secondary dial tone (unless the

default value of Register S7 has been changed). The 555-1212 is the

telephone number to be dialed.

3.6 Wait for Silence

The @ command causes the modem to listen for 5 seconds of silence.

If a 5-second silence has not been detected within the period

specified in the S7 register (default is 50 seconds), the modem

disconnects and returns the NO ANSWER result code. If it detects a

busy signal, it returns a BUSY result code. If the 5 seconds of

silence is detected, then the modem continues dialing the dial

string. Example:

AT DT 9 @ 1 (215) 555-1212 [ENTER]

3.7 Resume Command State after Dialing

The ; (semicolon) dial modifier returns the modem to the command

state after dialing the telephone number. The ; must come at the

end of the command line. It does not connect to the remote

modem, but holds the line for additional commands.

When the modem has finished dialing the number, it returns the OK

message. The semicolon dial modifier is useful when the number to

be entered exceeds 40 characters. The semicolon dial modifier,

breaks the number into two commands. Example:

AT DT 1 123 456; [ENTER]

OK

AT DT 7890 [ENTER]

These two commands dial a single phone number with the ; in the

first command causing the modem to return to command mode without

releasing the line. The second command dials the remaining numbers

and makes the connection to the modem.

3.8 Switch Hook Control

The H command is used to control the telephone line relay. This

command is most commonly used to hang up the phone from command

mode. The format of the command is ATHn where the n parameter

(optional) governs how the line relay is controlled. Either ATH or

ATH0 causes the phone line relay to go on-hook (disconnect). ATH1

causes the phone line relay to go off-hook (connect).

3.9 Manual Dialing

To manually dial the phone, you should be in your communications

software's command mode. Lift the receiver of the telephone and

dial the number you wish to call. Type ATH1 [ENTER] to connect the

modem and then hang up the receiver. Type ATO [ENTER] to tell the

modem to go on-line.

3.10 Manual Answer

When the automatic answer feature (Register S1) is not being used,

incoming calls can be answered manually by typing ATA [ENTER] when

an incoming call is received. The modem will answer the incoming

call and enter on-line mode.

3.11 Controlling the Speaker

The audio speaker is controlled by the M command. It can be

adjusted with the following commands. Default settings are

highlighted:

AT M0    Turns the speaker off

ATM1     Turns the speaker on until carrier is detected (DEFAULT)

AT M2    Leaves the speaker on at all times

AT M3    Turns the speaker on until carrier is detected except

during dialing

The L  command is used to adjust the speaker volume. The volume

levels available are:

AT L0, 1 Selects low volume

AT L2    Selects medium volume (DEFAULT)

AT L3    Selects high volume

3.12 Dial Stored Number

This number follows the dial command string, causing the modem to

dial a telephone number previously stored using the command AT&Z.

Up to four entries may be stored and used (0-3).

3.13 The AT Command Set

The 14.4Kbps MODEM supports the standard and extended Hayes AT

command set. These commands are the industry-standard language used

to communicate with the modem. To use these commands, load your

communications software package and enter into command mode.

NOTE: AT commands pertaining to MNP, V.32bis/V.32, and V.42bis/V.42

operation are discussed in their own sections.

The modem is always in one of three modes: the command mode, fax

mode, or data mode. The modem enters command mode when it is turned

on or reset, when it loses a connection with a remote modem, or

when it is in on-line mode and you type certain escape characters.

When the modem is in command mode, it accepts instructions in the

form of commands and return responses. This mode allows the modem

to be configured for a particular application or instructed to

perform a function. These settings will remain active until the

computer has been turned off except for configuration profile

stored in non-volatile RAM. The modem then returns to its default

settings. The modem enters data mode when it has made a connection

with a remote modem and sends the CONNECT response confirming the

connection has been established. Data can then be exchanged.

Every modem command must begin with the characters AT, with the

exception of the A/ command which is described later in this

section. The AT characters are called the attention code. They

signal the modem that one or more modem commands will follow. There

are specific command line guidelines that must be followed:

* Command lines must begin with the AT prefix. Characters must be

in all uppercase or all lowercase.

* Command lines may contain up to 40 characters.

* Space characters in the command line are ignored.

* Command lines must end by pressing the ENTER key

* Line feed character after carriage return is optional and ignored

by the modem.

* Multiple commands may appear on a single line.

* An error in the command line may be corrected by using the

backspace key, with the exception of the AT prefix. Control-H can

be used if the keyboard does not have a backspace key.

Note: Use the command AT&V to display the active configuration and

stored user profiles.

On-Line State

To transmit or receive data, the modem must be in the on-line

state. When placing a call, the modem is put on-line with the dial

command. At the remote end, the modem goes on-line when it is

instructed to answer a call automatically by setting the Register

S0 equal to any number greater than 0 and less than 255. You can

also perform this function with the ATA command.

Escape Sequence

After a connection has been established with a remote modem, you

can return to command mode without breaking that connection by

typing the escape sequence. The escape sequence consists of three

plus characters (+++). It is not necessary to begin this sequence

with the AT attention code, nor do you need to press ENTER after

typing the escape code.

Return to On-Line State

The modem can be returned to the on-line state after the escape

sequence has been issued. To return the modem to the on-line state,

issue the ATO command. To return the modem on-line and then

initiate and equalized retrain sequence, use the command ATO1.

Re-executing the Last Command Line

Each command line sent to the modem remains in the command buffer

until the next AT command is sent or until power to the modem is

turned off. To repeat the last command issued, type in the command

A/. The A/ command is neither preceded by the AT characters nor

followed by ENTER.

Omitting a Parameter

Some commands require a parameter to completely define them. If a

parameter is omitted from a command that requires one, the command

will respond with the default value.

Result Codes

A response is sent by the modem after executing, or trying to

execute a command. The response displays the modems status or the

progress of a call, and can take the form of either words or

digits. The default consists of word responses which are defined

with the V1 command. To receive digit responses rather than word

responses, use the V0 command. To disable the responses entirely,

use the Q1 command.  The tables on the next four pages list the

word responses the modem can return, their numeric equivalent, and

a short description of each.

Result Codes

Result    Numeric

Code      Value     Description

OK             0    Modem successfully executed a command line.

CONNECT        1    Connection made at 300 bps.

RING           2    Modem detected an incoming call.

NO CARRIER     3    Modem lost or could not detect a remote

carrier signal within the Register S7 time.

ERROR          4    Modem found an error in the command line.

CONNECT 1200   5    Modem established a connection 1200 at 1200

bps.

NO DIALTONE    6    Modem did not detect a dial tone within 5

seconds after going off-hook.

BUSY           7    Modem detected a busy signal.

NO ANSWER      8    Five seconds of silence was not detected

when using the @ command in the dial command

line.

CONNECT 0600   9    Modem established a connection at 600 bps.

CONNECT 2400   10   Modem established a connection at 2400 bps.

CONNECT 4800   11   Modem established a connection at 4800 bps.

CONNECT 9600   12   Connection made at 9600 bps.

+FCERROR       +F4  Fax carrier error.

CONNECT 7200   13   Connected as data modem during an answer.

CONNECT 12000  14   Connection made at 12000 bps.

CONNECT 14400  15   Connection made at 14400 bps.

CONNECT 19200  16   Connection made at 19200 bps.

CONNECT 38400  17   Connection made at 38400 bps.

CONNECT 57600  18   Connection made at 57600 bps.

CONNECT 75TX/  22

1200RX              Modem returns this result code when upon

establishing a V.23 originate connection

when the modem has been instructed to report

the DTE speed to the DTE upon connecting.

CONNECT        23

1200RX/75RX         Modem returns this result code when upon

establishing a V.23 answer connection when

the modem has been instructed to report the

DTE speed to the DTE upon connecting.

CARRIER 300    40   Carrier rate of 300 bps.

CARRIER 1200/  44   V.23 backward channel has been detected.

75

CARRIER 75/    45   V.23 forward channel has been detected

1200

CARRIER 1200   46   Carrier rate of 1200 bps.

CARRIER 2400   47   Carrier rate of 2400  bps.

CARRIER 4800   48   Carrier rate of 4800 bps.

CARRIER 7200   49   Carrier rate of 7200 bps.

CARRIER 9600   50   Carrier rate of 9600 bps.

CARRIER 12000  51   Carrier rate of 12000 bps.

CARRIER 14400  52   Carrier rate of 14400 bps.

COMPRESSION:

CLASS 5        66   The modem has connected in MNP class 5 and

COMPRESSION message reporting has been

enabled.

COMPRESSION:

V.42bis        67   The modem has connected in V.42bis and

COMPRESSION message reporting has been

enabled.

COMPRESSION:

NONE           69   The modem has connected without data

compression and COMPRESSION message reporting

has been enabled.

3.13.1 Basic Command Set

The following tables list the AT commands and their functions.

Default settings are indicated.

Command   Options   Description

A         none      Answer command. The modem will go off-hook,

transmit the answer tone, and wait for a

carrier from the remote modem.

A/        none      Re-execute last command. The last command

executed by the modem will execute again. This

command does not require the AT prefix or

ENTER.

AT        none      Attention characters. The AT command must

appear at the beginning of each command line or

the modem will not execute the command line.

Bn                  Select communications standard.

n=0       Selects V.22 2100 Hz answer tone at 1200 bps.

(DEFAULT)

n=1       Selects Bell 212A 2225 Hz answer tone at

1200bps.

Cn                  Turn modem transmit carrier signal on or off.

n=0       Transmit carrier signal off. (DEFAULT)

n=1       Transmit carrier signal on.

D         none      Dial command. Puts the modem into  originate

mode, allowing it to work as an auto-dialer for

connecting to another modem. See Section Two

for additional information.

En                  Echo back command characters.

n=0       Disables echoing of commands to the screen.

n=1       Enables echoing of commands to the screen.

(DEFAULT)

Fn                  Select Line Modulation. Selects the line

modulation according to the parameter supplied.

n=0       Auto-detect mode.

n=1       Selects V.21 or Bell 103 according to the AT B

command setting.

n=2       Not supported.

n=3       Selects V.23.

n=4       Selects V.22 or Bell 212A 1200.

n=5       Selects V.22bis 2400.

n=6       Selects V.32bis 4800 or V.32 4800.

n=7       Selects V.32bis 7200.

n=8       Selects V.32bis 9600 or v.32 9600.

n=9       Selects V.32bis 12000.

n=10      Selects V.32bis 14400.

Hn                  Switch hook control.

n=0       Instructs the modem to go on-hook (hang-up or

disconnect).

n=1       Instructs the modem to go off-hook and enter

command mode.

In                  Identification. Requests the modem to return

its product identification information.

n=0       Displays the product identification code.

n=1       Displays the ROM checksum.

n=2       Displays ROM checksum as OK or ERROR.

n=3       Displays the firmware revision level.

n=4       Displays OEM-defined identifier string.

n=5       Displays country code number.

n=6       Displays modem data pump model.

Ln                  Speaker volume.

n=0       Off or low volume.

n=1       Low volume. (DEFAULT)

n=2       Medium volume.

n=3       High volume.

Mn        Speaker Control.

n=0       Disables the modem speaker.

n=1       Turns on the modem speaker until carrier has

been detected.(DEFAULT)

n=2       Instructs the modem speaker to stay on at all

time.

n=3       Enables speaker after dialing until connection

is established.

Nn                  Automode Enable.

n=0       Requires the speed of the connection be that

specified by the value of S37.

n=1       Permits handshaking at any speed supported by

both modems. (DEFAULT)

On                  Return to on-line state mode.

n=0       Switches the modem from command mode to on-line

mode without dialing.

n=1       Switches from command mode to on-line mode and

initiates an equalizer retrain sequence during

2400 bps operation.

Qn                  Modem responses enabled or disabled.

n=0       Enables result codes to be issued to the

screen. (DEFAULT)

n=1       Disables result codes to be issued to the

screen.

Sn                  Change the value of an S-Register. n is the

number of the register whose value is to be

changed (0-95) and n is the new value (0-255).

Sn?                 Read the value of an S-register. n is the

number of the register whose value is to be

read (0-95).

Vn                  Result code format.

n=0       Numeric format.

n=1       Verbal format. (DEFAULT).

Wn                  Error Correction Message Control

n=0       Report DTE speed in error correction mode.

(DEFAULT)

n=1       Report line speed, error correction protocol,

and DTE speed.

n=2       Report DCE speed in error correction mode.

Xn                  Extended Result Codes. Determines which set of

responses and calling characteristics are used.

X3 allows blind dialing to be enabled by

country parameters. X4 sends all messages.

Response Code       n=0       n=1       n=2       n=3       n=4

0 (OK)              X         X         X         X         X

1 (CONNECT)         X         X         X         X         X

2 (RING)            X         X         X         X         X

3 (NO CARRIER)      X         X         X         X         X

4 (ERROR)           X         X         X         X         X

8 (NO ANSWER)       X         X         X         X         X

* (CONNECT XXXX)    X         X         X         X         X

6 (NO DIAL TONE)    O         O         X         X         X

7 (BUSY)            O         O         O         X         X

* Numeric response codes for "CONNECT" are 5, 9-12, 13-18, 22-23,

40, and 44-52. 1 (CONNECT) is a 300bps connection.

Yn                  Control Long Space Disconnect. Determines

whether or not modem disconnects when it

receives a continuous break from a remote

modem.

n=0       Disabled. (DEFAULT)

n=1       Enabled.

Zn                  Soft Reset and Restore Profile.

n=0       Restores stored profile 0.

n=1       Restores stored profile 1.

+++       none      Escape characters. The escape characters are

known as +++. They will switch from on-line

mode to command mode while preserving the

connection to the remote modem.

This command allows a parameter to be checked

or changed after connecting with a remote

asynchronous modem. Escape characters involve

a timed pause of 1 second (specified in

Register S12), three keystrokes of the + escape

character defined by Register S2) and another

pause (again specified in Register S12).

?                   S-register contents. Displays the contents of

the last addressed S-register.

3.13.2 Extended Command Set

Commands  Options   Description

&Cn                 Data Carrier Detect (DCD) signal.

n=0       Forces DCD signal to be on at all times.

(DEFAULT)

n=1       DCD on indicates presence of data carrier.

&Dn                 Data Terminal Ready (DTR) signal. Interprets

the ON to OFF transition of the DTR signal from

the DTE according to the & setting as

follows:

n=0       &Q 0, 5, 6. DTR ignored

&Q 1, 4. Modem hangs up; auto-answer not

affected.

&Q 2,3. Modem hangs up; auto-answer inhibited.

(DEFAULT)

n=1       &Q 0, 1 4-6. Asynchronous escape sequence.

&Q 2,3. Modem hangs up; auto-answer inhibited.

n=2       &Q 0-6. Modem hangs up; auto-answer inhibited.

n=3       &Q 0, 1 4-6.  Modem does a soft reset as if the

Z command were received; &Y determines which

profile is loaded.

&Q 2,3. Modem hangs up; auto-answer inhibited.

&F                  Recall factory defaults. Instructs the modem to

use the factory set parameters.

n=0       Recall (restore) factory profile 0.

n=1       Recall (restore) factory profile 1.

&Gn                 Guard tone select in CCITT mode. Guard tones

are always disabled in the U.S.A.

n=0       Do not use guard tones. (DEFAULT)

n=1       Disables guard tone.

n=2       Selects a guard tone 1800 Hz.

&Jn                 Telephone Jack Control. Included only for

compatibility and performs no function other

than to load the value of S Register 21

(n= 0 or 1).

&Kn                 DTE/Modem Flow Control. Determines how the

modem controls the flow of data between the

local DTE and the modem. When the modem

terminal buffer is nearly full, the modem will

either send an XOFF or drop CTS to stop the

data flow; when the buffer is nearly empty, the

modem will either send an XON or raise CTS to

resume the data flow.

n=0       disable flow control

n=3       enable RTS/CTS (DEFAULT for data modem)

n=4       enable XON/XOFF

n=5       enable transparent XON/XOFF

n=6       enable both RTS/CTS and XON/XOFF (DEFAULT for

fax modem)

Note on Flow Control. XON/XOFF is a software-based flow control

method, using standard ASCII control characters to pause or resume

data transmission. RTS/CTS pacing, a hardware-based method, uses an

electrical signal. Signals are exchanged as follows:

RECEIVER       TRANSMITTER

CTS ON ------> START SENDING

CTS ON <------ RTS ON (ready to send)

CTS OFF ------>RTS OFF (stop sending)

&Ln                 Line selection. Informs the modem of the type

of telephone line to which it will be

connected. (NOT SUPPORTED)

n=0       Selects a dialup telephone line. (DEFAULT)

n=1       Selects a leased telephone line. Each modem on

a leased line has its transmitters turned on

continuously, allowing transmission to occur

in both directions at the same time.

&Mn                 Asynchronous/Synchronous Mode Selection.

Determines the DTR operating mode. The modem

treats the &M command as a subset of the &Q

command.

n=0       Selects direct asynchronous mode. (DEFAULT)

n=1       Selects synchronous connect mode with

asynchronous off-line command mode.

n=2       Same as &M1 except modem will disconnect if DTR

is OFF for more than the period in S25.

n=3       Selects synchronous connect mode with

asynchronous off-line command mode. This mode

allows the DTR to act as a talk/data switch.

The call is initiated while the DTR is

inactive.

&Pn                 Select Pulse Dial Make/Break ratio. Determines

the ratio between the off-hook (make) and the

on-hook (break) pulse dialing intervals.

n=0       Selects a make/break ratio of 39/61 at 10pps.

Used in the United States and Canada. (DEFAULT)

n=1       Selects a make/break ratio of 33/67 at 10pps.

Used in the United Kingdom and Hong Kong.

n=2       Same as 0 except at 20pps.

n=3       Same as 1 except at 20pps.

&Qn                 Sync/Async Mode. An extension of the &M command

and is used to control the connection modes

permitted. Used in conjunction with S36 and S48

(see also \N).

n=0       Direct asynchronous mode.

n=1       Synchronous connection with asynchronous off-

line command mode.

n=2       Same as 1 plus enable DTR dialing of directory

zero.

n=3       Same as 1 plus enable DTR to act as talk/data

switch.

n=4       Hayes auto-synchronous mode.

n=5       Modem negotiates an error-corrected link.

(DEFAULT)

n=6       Selects asynchronous operation in normal mode

(i.e., speed buffering).

&Rn                 RTS/CTS Option. Controls the state of the CTS

signal. CTS is always on in the asynchronous

mode unless AT&K3 has been selected.

n=0       CTS tracks RTS.

n=1       CTS is always active (DEFAULT).

&Sn                 Data Set Ready (DSR) signal.

n=0       Causes DSR signal to be active at all times.

(DEFAULT)

n=1       Causes DSR signal to be active according to the

V.25 protocol.

&Tn                 Test and Diagnostics. Refer to the Diagnostics

section for more information.

n=0       Terminates any test in progress.

n=1       Executes the local analog loopback test.

n=3       Executes the local digital loopback test.

n=4       Enables the modem to accept a request from a

remote modem for a digital loopback test.

(DEFAULT)

n=5       Instructs the modem to deny a request from a

remote modem for a digital loopback test.

n=6       Executes the remote digital loopback test.

n=7       Executes the remote digital loopback test with

a self test.

n=8       Executes the local analog loopback test with a

self test.

&V        none      View current configuration and user profile.

Displays the active configuration and stored

user profiles.

&Wn                 Store user profile. Saves the current

configuration into non-volatile RAM as one of

two user profiles. Also use this command to

write initialization strings.

n=0       saves as user profile 0.

n=1       saves as user profile 1.

&Xn                 Select Synchronous Clock Source. Selects the

source of the transmit clock for the

synchronous mode of operation. The parameter

value, if valid, is written to S27 (bits 4 and

5). In asynchronous mode, the transmit and

receive clocks are turned OFF. (EXTERNAL modem

only)

n=0       Selects internal timing.

n=1       Selects external timing.

n=2       Selects slave receive timing. The transmit

clock signal is derived from the incoming

carrier.

&Yn                 Designate Default User Profile. Specifies which

of two user profiles will be loaded into the

active configuration at power-up.

n=0       Selects user profile 0.

n=1       Selects user profile 1.

&Zn=x     none      Store Phone Number. Stores a 36-digit dial

string (x) in the specified entry (0-3) for

later dialing. See the ATDS=n command (3.12

Dial Stored Number). An individual entry may be

made up to 40 digits, but the total number of

digits in all entries may not exceed 114.

S=n       none      Dial Stored Number. Follows the dial command

string, causing the modem to dial a telephone

number previously stored using the AT&Zn

command.

3.14 The S-Registers

This section defines the purpose of the modem registers, explains

how to read and change register values, how to access a register

repeatedly, and sequentially lists the registers and describes

their functions. These registers affect various operating

characteristics and allow you to obtain information about the

modem, as well as test the modem. Each register has a factory-set

value, which you can read or change to fit your needs. To learn how

to set bit-mapped registers and how their values are calculated,

refer to Appendix A.

Reading a Register Value

To read the current value of a register, type: AT Sn? [ENTER],

where n is a register number. The modem will respond with the

decimal value of the register in three digit form. This will be

followed by OK or 0.

To read values from more than one register, type: AT Sn? Sn?

[ENTER] from the command mode.

To read the register values of S0 and S1, type AT S0? S1? [ENTER]

The modem will display the first register value, a carriage return,

the next register value, a carriage return, and OK or 0.

Changing a Register Value

To change a register value, use the Sn command (ATSn=v), where n is

a register number and v is the new value you want to assign to the

register. Type: AT S0=3 [ENTER] to have the modem automatically

answer incoming calls after the third ring. The changed values

remain in effect until the modem is turned off and on or reset.

The modem then returns to its default settings.

Accessing a Register Repeatedly

The modem automatically places a pointer at the last register whose

value was read or changed. The pointer provides a fast way of

displaying or changing register values. Type: AT S0? [ENTER] to

read the value of Register S0. The modem responds with the value of

Register S0 and places a pointer at that register automatically.

Type: AT=0 [ENTER] to change the value in Register S0 to 0.

(Neither the S command nor the register number has to be specified;

the pointer handles this task.)

Type: AT? [ENTER] to read the S0 value. NOTE: The pointer remains

set at the last register accessed until the S command is used to

read or change the value of a different register. The pointer is

then set to the register whose value is displayed or changed.

S-Register Descriptions

The following table lists the modems registers and their functions.

Reg. Range     Units     Default   Definition

S0   0-255     rings     2         Auto-answer

Description: Holds the number of rings the modem will wait before

answering the phone. If this register is set to zero, the modem

will not answer the phone. In terminal mode, make sure this is ON

(S0 equal to anything but zero) with %C1 and %D1 in the active

profile.

S1   0-255     rings     0         Count incoming rings

Description: Counts the number of rings detected on the line. The

register is cleared if a ring is not detected over an eight second

time period. If the register value equal the value contained in S0,

the modem will answer the phone.

S2   0-255     rings     43        Escape character value.

Description: Contains the ASCII value of the character used as the

Escape Code. This enables the modem to switch from on-line mode to

command mode without losing the connection to the remote modem.

S3   0-127     ASCII     13        Carriage return character.

Description: Contains the ASCII value of the character used to send

a carriage return. This character terminates both command lines and

result codes.

S4   0-127     ASCII     10        Line feed character.

Description: Contains the ASCII value of the character used to send

a line feed. The modem sends the line feed character after a

carriage return only when word responses are sent.

S5   0-32      ASCII     8         Backspace character.

Description: Contains the ASCII value of the character used to

backspace. This character is both the character created by pressing

the Backspace key and the character echoed to move the cursor to

the left.

S6   2-255     seconds   2         Wait time for Blind Dialing.

Description: Sets the length of time to pause after the modem goes

off-hook before the modem dials the first digit of the telephone

number.

S7   1-255     seconds   50        Wait for carrier after dial.

Description: Defines two delay times: (1) time that the local modem

waits for the carrier from the remote modem before hanging up; (2)

time that the modem waits when the Wait for Dial Tone call progress

feature (W in dial string) is in effect.

S8   0-255     seconds   2         Pause time for dial delay.

Description: Determines how long the modem should pause when it

sees a comma in the dialing string.

S9   1-255     seconds   6/10      Carrier detect.

Description: Determines the amount of time in tenths of a second

from when the modem recognizes a loss of carrier to when it will

hang up.

S10  1-255     seconds   14 (1.4)  Lost Carrier to Hang Up Delay.

Description: Sets the length of time tenths of a second, that the

modem waits before hanging up after the loss of a carrier. This

allows for a temporary carrier loss without causing the local modem

to disconnect. When register S10 is set to 255, the modem functions

as if a carrier is always present. The actual interval the modem

waits before disconnecting is the value in S10 minus the value in

S9. If this value is set smaller than the value contained in

S9, the modem disconnects before it recognizes its carrier.

S11                                Reserved.

S12  0-255     seconds   50(1)     Escape code guard time*.

Description: Controls the delay time before and after typing escape

sequence codes.

*in one-fiftieth second increments

S13                                Reserved.

S14            Bit Mapped  AA,hex  Bit mapped registers.

Provides the following functions (defaults are highlighted):

BIT       Definition and Options

Bit 0     IGNORED

Bit 1     ATE (command echo)

0  Do not echo commands

1  Echo commands (DEFAULT)

Bit 2     ATQ (result code display)

0  Send responses (DEFAULT)

1  Do not send responses

Bit 3     ATV (word or number responses)

0  Send number responses

1 Send word responses (DEFAULT)

Bit 4     RESERVED.

Bit 5     Dial Method (ATP or ATT)

0  Touch-Tone (T) (DEFAULT)

1  Pulse (P)

Bit 6     RESERVED.

Bit 7     Answer/originate operation

0  Answer (A and R)

1  Originate (D) (DEFAULT)

S15                                Reserved

S16       Bit Mapped     80,hex    Modem test options.

Provides the following functions (defaults are highlighted):

Bit       Definition and Options

Bit 0     Local analog loopback test

0  Disabled (&T0) (DEFAULT)

1  Enabled (&T)

Bit 1     NOT USED

Bit 2     Local digital loopback test

0  Disabled (&T0) (DEFAULT)

1  Enabled (&T3)

Bit 3     Remote digital loopback test

0  Disabled (&T0) (DEFAULT)

1  Enabled (&T6)

Bit 4     Initiate remote digital loopback

0  Loopback not in progress (&T0)

(DEFAULT)

1  Loopback in progress (&T7)

Bit 5     Remote digital loopback with error count

0  Disabled (&T0) (DEFAULT)

1  Enabled (&T7)

Bit 6     Local analog loopback with self test

0  Disabled (&T0) (DEFAULT)

1  Enabled (&T8)

Bit 7     NOT USED

S17                                Reserved

S18       0-255          seconds   0         Test timer.

Description: Determines how long a diagnostic test will be allowed

to run before being returned to the command mode. The test timer

is disabled with the value set to zero.

S19                                Reserved

S20                                Reserved

S21       Bit Mapped               Bit mapped registers.

00,hex                   Provides the following

functions:

Bit            Definition and Options

Bit 0          Telephone jack type

0  RJ-11, RJ-41S, RJ-45S (&J0)

(DEFAULT)

1  RJ-12, RJ-13 (&J1)

Bit 1          RESERVED

Bit 2          RTS (Request to Send)/CTS(Clear to Send)

(&Rn)

0  CTS tracks RTS (DEFAULT)

1  Modem ignores RTS

Bit 3,4        Data Terminal Ready (DTR)

(&Dn)

0 = &D0 selected (DEFAULT)

1 = &D1 selected

2 = &D2 selected

3 = &D3 selected

Bit 5          Data Carrier Detect (DCD)

0  DCD is always active (&C0) (DEFAULT)

1  DCD is active when remote

carrier is detected (&C1)

Bit 6          Data Set Ready (DSR)

0  DSR is always active (&S0) (DEFAULT)

1  DSR is active when handshaking begins

(&S1)

Bit 7          Long space disconnect

0  Disabled (Y0) (DEFAULT)

1  Enabled (Y1)

S22       Bit Mapped    75,hex      Bit mapped registers.

Provides the following functions (asterisk indicates default):

Bit            Definition and Options

Bit 0, 1       Speaker volume

0 = OFF (L0)

1 = Low volume (L1) (DEFAULT)

2 = Medium volume (L2)

3 = High volume (L3)

Bit 2, 3       Speaker Control

0 = Speaker turned off (M0)

1 = Speaker on until carrier

detected (M1) (DEFAULT)

2 = Speaker always on (M2)

3 = Speaker on until carrier

detected, off when dialing (M3)

Bit 4, 5, 6    Limit Result Codes

0 = X0

4 = X1

5 = X2

6 = X3

7 = X4 (DEFAULT)

Bit 7          Reserved.

S23       Bit Mapped     07, hex        Bit mapped registers.

Provides the following functions. Defaults are highlighted.

Bit            Definition and Options

Bit 0          Request from remote modem

for digital loopback test.

0 Ignore request (&T5)

1 Grant request (&T4) (DEFAULT)

Bit 1, 2, 3    Communications Rate

0  0-300 bps

1  600 bps

2  1200 bps

3  2400 bps (DEFAULT)

4  4800 bps

5  9600 bps

6  19200 bps

Bit 4,5        Parity Setting

0  even

1  not used

2  odd

3  none

Bit 6, 7       Guard Tones

0 = none (&G0) (DEFAULT)

1 = none (&G1)

2 = 1800 Hz (&G2)

S24  0-255     seconds   0    Sleep Inactivity Timer.

Description. Sets the length of time that the modem operates in

normal mode with no line activity before entering low power sleep

mode.

S25  0-255     seconds   5    Asynchronous DTR Delay

Description: Determines the time interval that the modem will

ignore a change in the DTR status.

S26  0-255     seconds   1    Delay Interval.

Description: RTS to CTS delay interval (in hundredths of a

second).

S27       Bit Mapped          Bit mapped registers.

9, hex

Bit 0, 1, and 3 are reserved.

Bit            Definition and Options

Bit 2          Line Type (NOT SUPPORTED)

0 Dialup line (&L0)

1 Leased line (&L1)

Bit 4,5        Clock Source Selection

0  Internal clock (&X0) (DEFAULT)

1  External clock (&X1)

2  Slave clock (&X2)

Bit 6

0  CCITT V.22/V.22bis standard

(B0)(DEFAULT)

1  Bell 212A standard (B1)

Bit 7          Reserved.

S28       Bit Mapped     Bit mapped registers

Bit            Definition and Options

Bit 0          V.23 split speed (\Wn) (DEFAULT)

0 = Disabled (\W0) (DEFAULT)

1 = Enabled (\W1)

Bit 1          V.23 split speed direction

0 = 75bps transmit (%F0) (DEFAULT)

1 = 1200bps transmit (%F1)

Bit 2          V.23 half-duplex

0 = disabled (DEFAULT)

1 = enabled (%F3)

Bit 3, 4       Pulse dialing (&Pn)

0 = 512 (&P0) (DEFAULT)

1 = 1024 (&P1)

2 = 2048 (&P2)

3 = 4096 (&P3)

Bit 5, 6, 7    Reserved

S29                      Reserved.

S30  0-255     seconds   0         Inactivity Timer

Description: Determines the length of time (in tens of a

second) that the modem will wait before disconnecting when no data

is sent or received.

S31                      2         Bit Mapped Options

Bit            Definition and Options

Bit 0          RESERVED

Bit 1          Controls auto line speed detection.

0 = disabled (N0)

1 = enabled (N1) (DEFAULT)

Bit 2,3        Controls error correction progress

messages (Wn)

0 = DTE speed only (W0) (DEFAULT)

1 = full reporting (W1)

2 = DCE speed only (W2)

Bit 4-7       RESERVED

S32  0-255     ASCII     17   XON Character

Description: Sets the value of the XON character.

S33  0-255     ASCII     19   XOFF Character

Description: Sets the value of the XOFF character.

S34-35                        Reserved.

S36                      7    Bit Mapped Options

Bit            Definition and Options

Bit 0-2        Indicates what happens during a LAPM

failure. These fallback options take

effect immediately upon connection if

S48=128. If an invalid number is entered,

the number is accepted into the register,

but S36 acts as if the default value has

been entered.

0 = modem disconnects

1 = modem stays online; establishes a

direct mode connection.

2 = Reserved.

3 = modem stays online; establishes a

normal mode connection.

4 = attempts MNP connection; if it fails,

modem disconnects.

5 = attempts MNP connection; if it fails,

establishes a direct mode connection.

6 = Reserved.

7 = attempts MNP connection; if it fails,

establishes a normal mode connection.

(DEFAULT)

Bit 3-7        Reserved.

S37  0-12                0    Telephone Line Speed

Description: Desired Line Connection Speed. This is interlinked

with the Fn command. If an invalid number is entered, the number is

accepted into the register, but S37 will act as if the default

value has been entered.

Bit            Definition and Options

Bit 0-3        0 = Attempt auto mode connection (F0)

(DEFAULT)

1-3 = Attempt to connect at 300 bps. (F1)

4 = Reserved.

5 = Attempt to connect at 1200 bps. (F4)

6 = Attempt to connect at 2400 bps. (F5)

7 = Attempt to connect at V.23 at 600/1200

bps. (F3)

8 = Attempt to connect at 4800 bps. (F6)

9 = Attempt to connect at 9600 bps. (F8)

10 = Attempt to connect at 12K bps. (F9)

11 = Attempt to connect at 14.4Kbps (F10)

12 = Attempt to connect at 7200 bps. (F7)

Bit 4-7        RESERVED.

S38  0-255     seconds   20        Delay Before Forced Hangup

Description: Specifies the delay between the modems receipt of the

H command to disconnect. Applies to error correction only. This

register can be used to ensure that data in the buffer is sent

before the modem disconnects. If S38=0-254, the modem will wait

that number of seconds before disconnecting. If S38=255, the modem

does not time out and continues to deliver data in the buffer until

the connection is lost or the data is delivered.

S39  Bit Mapped          3         Bit Mapped Registers

Bit         Definition and Options

Bit 0-2   Status of Command Options

0 = no flow control

3 = RTS/CTS (&K3) (DEFAULT)

4 = XON/XOFF (&K4)

5 = Transparent XON/XOFF (&K5)

6 = Both methods (&K6)

Bits 3-7  Reserved

S40  Bit Mapped          105       Bit Mapped Registers

Bit 0     MNP Extended Services

0 = disabled (-K0)

1 = enabled (-K1) (DEFAULT)

Bit 1     Power Level Adjustment for Cellular Use ()Mn)

0 = auto adjustment ()M0) (DEFAULT)

1 = force adjustment ()M1)

Bit 2     MNP Link Negotiation Speed (*H1)

0 = highest speed (*H0) (DEFAULT)

1 = 1200 bps (*H1)

Bits 3-5  Break Handling (\Kn)

0 = \K0           1 = \K1

2 = \K2           3 = \K3

4 = \K4           5 = \K5 (DEFAULT)

Bits 6-7  MNP Block Size

0 = 64 chars. (\A0)

1 = 128 chars. (\A1)

2 = 192 chars. (\A2) (DEFAULT)

3 = 256 chars. (\A3)

S41  Bit Mapped          3    Bit Mapped Registers (for MNP)

Bits 0-1  Compression selection (%Cn)

0 = disabled (%C0)

1 = MNP5 (%C1)

2 = V.42bis (%C2)

3 = MNP5 and V.42bis (%C3)(DEFAULT)

Bit 2     Auto retrain

0 = retrains disabled (%E0) (DEFAULT)

1 = retrains enabled (%E1)

Bit 3     Modem to Modem Flow Control

0 = disabled (\G0) (DEFAULT)

1 = enabled (\G1)

Bit 4     Block Mode Control

0 = stream mode (\L0) (DEFAULT)

1 = block mode (\L1)

Bits 5-7  Reserved

S42-45    Reserved

S46  136 or 138          138       Data Compression Control

136 = error correction protocol with no

compression

138 = error correction protocol

with compression (DEFAULT)

S48  0, 7, 128           7         V.42 Negotiation Action

0 = Disable negotiation and proceed with LAPM.

7 = Enable negotiation. (DEFAULT)

128 = Disable negotiation and proceed with fallback action

specified in S36.

S49-81    Reserved

S82  3, 7, 128           128       Break Handling Option

Description: Used to get the attention of the remote modem. Three

types of break signals are provided:

3 = Expedited. Modem sends a break immediately; data integrity

is maintained.

7 = Destructive. Break is sent immediately and data being

processed at time of break is destroyed.

128 = In Sequence. Modem sends a break in sequence; data

integrity is maintained (DEFAULT).

S86  N/A                           Connection Failure Cause Code

Description: When the modem issues a NO CARRIER result code, a

value is written to the S register to help determine the

cause for the failed connection. The values are as follows:

0 = normal disconnect; no error occurred.

4 = loss of carrier

5 = V.42 negotiation failed to detect an error correction

modem at the other end.

9 = the modems could not find a common protocol.

12 = normal disconnect initiated by remote modem.

13 = remote modem does not respond after 10 re-transmissions

of the same message.

14 = protocol violation.

S91  0 to -15  dBm  10        PSTN Transmit Level

Description: Sets the transmit level, in dBm, for the PSTN mode. In

some countries, this level cannot be changed and there are checks

to prevent transmit level change.

S92  0 to -15  dBm  10        Fax Transmit Level

Description: Sets the transmit level, in dBm, for the fax mode. In

some countries, this level cannot be changed and there are checks

to prevent transmit level change.

S95  SEE SECTION 4.4: S-Registers for MNP Operation

Section Four: MNP Operation

4.1 Introduction

MNP (Microcom Networking Protocol) is a data communications

protocol developed by Microcom for full duplex, error-free

communications over telephone lines. This protocol detects and

corrects errors which can result from telephone line noise and

other signal distortions.

NOTE: MNP can only operate at speeds of 2400bps or higher.

There are several levels of MNP operation available also referred

to as service classes. The service class refers to the following

techniques the modem uses to transfer data. The 14.4Kbps Modem

supports classes 2, 3, 4, and 5 and these classes are summarized as

follows:

CLASS 2. Uses an asynchronous, byte-oriented format in which

standard byte-framing techniques (start and stop bits) and

full-duplex transmission are used.

CLASS 3. Uses a synchronous, bit-oriented format in which standard

byte-framing techniques are removed. This increases throughput by

approximately 20%.

CLASS 4. Optimizes performance by using more efficient framing

techniques and allowing for transmission of large data blocks. Data

block size is adjusted based on the quality of the telephone line

(smaller for poor lines; larger for higher quality lines). This

technique reduces the number of retransmissions and maximizes

transfer rate based on the quality of the phone line.

CLASS 5. Includes all the features of CLASS 4 and also features

data compression. Both downloaded and interactive real-time data

are compressed. A significant increase on most data throughput can

be realized. The AT%C1 command enables this class. If MNP 5 is

disabled, the Modem defaults to Class 4. MNP5 and MNP 4 are not

compatible.

4.2 MNP Data Mode

After establishing the MNP connection, the modem will perform all

functions performed during normal data mode EXCEPT:

(1) When retrains are performed. At 2400 bps operation or higher,

the Modem may lose synchronization with the incoming line signal.

Retraining is an adjustment process performed when one of the

modems detects possible line problems which threaten data

integrity.

(2) When in the command mode following an escape sequence. The

Modem will continue to acknowledge and accept data. This data is

stored until the Modem is returned to data mode and then the data

is delivered. In non-MNP mode, any data received while the Modem is

in command mode is lost.

Class 5 Data Compression. Class 5 may be enabled or disabled  by

using the AT%Cn command. During an MNP 5 connection, the Modem

compresses data into tokens  before transmitting to the remote

modem, and de-compresses the data before it is received.

Programmable Inactivity Timer. You can specify the length of time

the modem will wait before disconnecting when no data is sent or

received. In MNP mode, this timer is reset whenever data is sent or

received. Use the AT\Tn command to set this value where N is a

number between 0 and 42 minutes.

Data Transfer. You may operate MNP in either block or stream mode.

In stream mode, MNP sends data frames in varying lengths depending

on the amount of time between characters being received. Stream

mode is selected with the AT\Ln command.

In block mode, MNP sends fixed data frames of 64, 128, 192, or 256

characters in length. Your communications software must support the

use of block mode.

4.3 AT Commands for MNP Operation

The 14.4K Modem supports all of the preceding classes. The

following AT Commands apply to the MNP protocol:

AT\An          Maximum MNP Block Size

AT\Bn          Transmit Break

AT\Gn          Modem to Modem Flow Control

A\Kn           Break Control

AT\Ln          MNP Block Transfer Control

AT\Nn          Operation Mode Control

AT\Wn          Split Speed Operation

AT\Z           Switch to Normal Mode

AT*H           MNP Link Negotiation Speed

Default selections are highlighted.

Command:       AT\An

Maximum MNP Block Size. Use this command to transmit smaller blocks

of data in a reliable data link connection.

0 = set maximum block size to 64 characters.

1 = set maximum block size to 128 characters. (DEFAULT)

2 = set maximum block size to 192 characters.

3 = set maximum block size to 256 characters.

Command:       AT\Bn

Transmit Break. When this command is entered during a non-MNP

connection, a break signal is sent to the remote modem.

The length of the break is 100 times the n parameter (1-9) in

milliseconds. The default is 3.

Command:       AT\Gn

Modem to Modem Flow Control. Enables or disables modem to modem

flow control during a normal mode connection. This command is

ignored when error correction is selected.

0 = Disables flow control (XON/XOFF) (DEFAULT)

1 = Enables flow control (XON/XOFF)

Command:       AT\Kn

Break Control. Determines the modem response when a BREAK is

received from the DTE or the remote modem. The values of the

parameters vary based on the three following conditions:

(1) When a break is received from the DTE during NORMAL or

MNP mode:

0, 2, 4   =    modem enters command mode without sending a

break to  the remote modem.

1         =    modem clears the terminal and modem buffers and

sends a break to the remote modem.

3         =    modem does not clear the buffers, but sends a break

to the remote modem.

5         =    modem sends a break to the remote modem in

sequence with any transmitted data. (DEFAULT)

(2) When a break is received from the remote modem during NORMAL

mode:

0, 1      =    modem clears the terminal and modem buffers and

sends a break to the local DTE.

2, 3      =    modem does not clear the buffers but sends a break

to the local DTE.

4, 5      =    modem sends a break in sequence with any data being

buffered. (DEFAULT)

(3) When a break is received from the DTE during DIRECT MODE:

0, 1, 3   =    modem sends a break to the remote modem and enters

command mode.

2, 4, 5   =    modem sends a break to the remote modem.

(DEFAULT)

Command:       AT\Ln

MNP Block Transfer Control. Determines whether or not the modem

will use block or stream mode for MNP link.

0 = uses stream mode for MNP link connections (DEFAULT)

1 = uses block mode for MNP connections

Command:       AT\Nn

Error Correction Operating Mode. Selects the operating mode the

modem uses while connected.

0 =       NORMAL (speed buffering) mode.

1 =       DIRECT (pass-through) mode.

2 =       RELIABLE LINK mode. Specifies error correction for

the modem-to-modem connection

3 =       AUTO-RELIABLE LINK mode. Attempts error-correction

connection but will fall back to normal mode if unable to

establish an MNP link.

4 =       LAPM error correction mode.

5 =       MNP error correction mode

Command:       AT\Wn

Split Speed Operation. Supports a split-speed DCE/DTE interface for

applications which require a transmit speed of 75bps and receive

speed of 1200bps at the DTE interface.

0 = disables (DEFAULT)

1 = enables

Command:       AT*H

MNP Link Negotiation Speed. Controls the connection speed for link

negotiations.

0 = link negotiation occurs at the highest supported speed.

(DEFAULT)

1 = link negotiation  occurs at 1200bps.

4.4 S-Register Values for MNP Operation

The following S-Registers apply to the MNP protocol.

S36  LAPM Failure Control

S38  Delay Before Forced Connect

S40  Bit-Mapped

S46  Data Compression Control

S95  Extended Result Codes

S36       0-7                      7         LAPM Failure Control

Description: Indicates what happens during a LAPM failure. These

fallback options take effect immediately upon connection if

S28=148. If an invalid number is entered, the number is accepted

into the register, but S36 acts as if  the default value has been

entered.

0 =       modem disconnects

1 =       modem stays online; establishes a direct mode connection.

2 =       Reserved.

3 =       modem stays online; establishes a normal mode connection.

4 =       attempts MNP connection; if it fails, modem disconnects.

5 =       attempts MNP connection; if it fails, establishes a

direct mode connection.

6 =       Reserved.

7 =       attempts MNP connection; if it fails, establishes a

normal mode connection. (DEFAULT)

S38       0-255                    20        Delay Before

Forced Disconnect

Description: This register specifies the delay between the modems

receipt of the ATH command to disconnect.

S40       Bit Mapped.                        Bit Mapped

Registers (for MNP)

Description: Indicates the status of command options.

Bit 0          MNP Extended Services

0 = disabled (-K0)

1 = enabled (-K1) (DEFAULT)

Bit 1     Power Level Adjustment for Cellular Use

()Mn)

0 = auto adjustment ()M0) (DEFAULT)

1 = force adjustment ()M1)

Bit 2     MNP Link Negotiation Speed (*H1)

0 = highest speed (*H0) (DEFAULT)

1 = 1200 bps (*H1)

Bits 3-5  Break Handling (\Kn)

0 = \K0        1 = \K1

2 = \K2        3 = \K3

4 = \K4        5 = \K5 (DEFAULT)

Bits 6-7  MNP Block Size

0 = 64 chars. (\A0) 1 = 128 chars. (\A1)

2 = 192 chars. (\A2) (DEFAULT)

3 = 256 chars. (\A3)

S46       136, 138                      Data Compression

Control

136 = error correction protocol with no compression

138 = error correction protocol with compression

(DEFAULT)

Description: Controls selection of compression.

S95            none                     Extended Result Codes

Bit 0 =   CONNECT CODE indicates DCE speed

instead of DTE speed.

Bit 1 =   Append /ARQ (automatic repeat request)

to verbose CONNECT XXXX result code if

protocol is other than none.

Bit 2 =   Enable CARRIER XXXX result code.

Bit 3 =   Enable PROTOCOL XXXX result code.

Bit 5 =   Enable COMPRESSION result code.

Bits 4, 6, and 7 are reserved.

Description: The bits in this register can be set to override some

of the ATWn command options.

Section Five: Error Correction: V.42/V.42bis

5.1 Introduction

V.42 and V.42bis are two different communication protocol standards

set by CCITT (Consultative Committee on International Telegraph and

Telephone).

V.42 is the CCITT standard for error correction in modems and is

normally found in modems that are 2400bps or faster. V.42 supports

LAPM and MNP 4 and is therefore compatible with modems using MNP 4.

The V.42 specification is a standard that supports one of the

Microcom MNP 2-4 error correction protocols or the LAPM error

correction protocol.

The V.42bis specification is a data compression standard that

requires the LAPM error correction protocol, as well as providing

data compression up to twice as fast as the MNP 5 data compression

standard.

MNP 5 is another data compression standard that requires the MNP 4

error correction protocol. It is not compatible with V.42bis and is

slower than the V.42bis data compression standard.

Detection. Immediately after the initial handshake , the

communicating modems exchange a sequence of detection patterns to

verify that both sides support the error correction technique.

Negotiation. The communicating modems exchange configuration

information.

If the detection phase fails, or negotiation does not achieve the

desired connection type, the modem can do one of the following:

* hang up

* fall back to MNP

* fall back to a standard asynchronous mode

In this case, you would use register S36 for Negotiation Fallback

and/or S48 for Negotiation Action. Because MNP does not support a

detection phase, you must select MNP 4 as a forced fallback  with

S36 and S48. The following S registers are used for V.42 LAPM

operation:

Register            Description

S36                 V.42 Negotiation Fallback

S38                 Delay Before Forced Connect

S41                 Bit-Mapped Registers

S46                 Protocol Selection

S48                 V.42 Negotiation Action

S82                 Break Handling

S86                 Connection Failure Cause Code

S95                 Extended Results Code

These registers are described in the balance of this section.

5.2 S Registers

5.2.1 V.42 Negotiation Fallback (S36)

Register S36 specifies allows you to indicate what action to take

when a desired connection cannot be made. The options are:

S36=0  Modem hangs up.

S36=1  Modem stays on-line and a direct mode connection is

established. When running in direct mode, no flow control

characters are recognized or transmitted. Flow control is when a

receiving buffer tells the transmitting source to start and stop

transmission. Direct mode is often used for compatibility with

older style modems.

S36=2  Reserved.

S36=3  Modem stays on-line and a normal mode connection is

established. Normal mode (or speed buffering) allows the modem to

send and receive at a speed different from the speed of the

physical, or modem to modem connection.

S36=4  If S Register 48 is 128, an MNP connection is attempted; if

it fails, the modem disconnects.

S36=5  If S Register 48 is 128, an MNP connection is attempted; if

it fails, a direct mode connection is established.

S36=6  Reserved.

S36=7  If S Register 48 is 128, an MNP connection is attempted; if

it fails, a normal mode connection is established. (DEFAULT)

5.2.2 Delay Before Forced Connect (S38)

S38 allows the following:

S38 = 0-255   Specifies the delay between the modems receipt of

the ATH command to disconnect (20 is the default).

5.2.3 Protocol Selection (S46)

S46 allows the following:

S46=136  Activates LAPM protocol with NO V.42bis compression.

(DEFAULT)

S46=138  Activates LAPM protocol WITH V.42bis compression.

NOTE: In addition to V.42bis, the modem also implements MNP 5 data

compression. V.42bis is used only with LAPM; MNP 5 is used only

with MNP 4.

5.2.4 V.42 Negotiation Action (S48)

Negotiation determines the capability of the remote modem. If the

capabilities of the remote modem are known and negotiation is

unnecessary, you can bypass this function.

S48=7  Enables negotiation. (DEFAULT)

S48=0  Disables negotiation. This bypasses the negotiation and

detection phases and proceeds with LAPM.

S48=128 Disables negotiation. This bypasses the negotiation and

detection phases and proceeds at once with the fallback options

specified in S Register 36. This option can be used to force MNP.

5.2.5 Bit-Mapped Registers

These registers control compression selection, retraining, flow

control, and block mode control.

Bits 0,1  =  Compression selection

Bit 2     =  Auto retrain

Bit 3     =  Modem to Modem Flow Control

Bit 4     =  Block Mode Control

Bits 5-7 are reserved

5.2.6 Break Handling (S82)

LAPM specifies three methods of break signal handling:

* in sequence, expedited, or destructive

S82=128   In sequence. Modem sends a break in sequence with any

transmitted data; data integrity is maintained both ahead of and

after the break. (DEFAULT)

S82=3     Expedited. Modem sends a break immediately; data

integrity is maintained both ahead of and after the break.

S82=7     Destructive. Modem sends a break immediately; data being

processed by each modem at the time of the break is destroyed.

5.2.7 Connection Failure Cause Code  (S86)

When the modem issues a NO CARRIER result code, a failure code is

written to register S86. S86 records the first event that

contributes to a NO CARRIER message. The result code definitions

are as follows:

S86=0     Normal disconnect; no error occurred.

S86=4     Loss of carrier.

S86=5     V.42 negotiation failed to detect an error correction

modem at the other end.

S86=9     The modems could not find a common protocol.

S86=12    Normal disconnect initiated by remote modem.

S86=13    Remote modem does not respond after 10 re-transmissions

of the same message.

S86=14    Protocol violation.

5.2.8 Extended Results Code (S95)

You can set the bits in this register to override some of the ATWn

command options. A bit to set to 1 in this register enables the

corresponding result code regardless of the ATWn setting.

Bit 0 =   CONNECT result code indicates DCE speed instead of DTE

speed.

Bit 1 =   Append/ARQ to verbose CONNECT XXXX result code if

protocol is not NONE.

Bit 2 =   Enables CARRIER XXXX result code.

Bit 3 =   Enables PROTOCOL XXXX result code.

Bit 5 =   Enables COMPRESSION result code.

Bits 4, 6, 7 are reserved.

5.3 AT Commands for Error Correction (V.42/V.42bis)

The following AT commands apply to the V.42/V.42bis protocol:

AT%Cn          Compression Control

AT%En          Auto Retrain

AT%Fn          Split-Speed Direction Select

AT%L           Report Received Signal Level

AT%Q           Report Line Signal Quality

Command:       AT%Cn

Compression Control. Determines whether or not modem will use data

compression.

0 = disables data compression

1 = enables MNP5 data compression negotiation

2 = enables V42bis data compression

3 = enables both V.42bis and MNP5 data compression (DEFAULT)

Command:       AT%En

Auto-retrain. Determines whether or not the modem automatically

monitors the line quality and requests a retrain when necessary.

0 = disables line quality monitor auto-retrain (DEFAULT)

1 = enables line quality monitor auto-retrain

2 = enables line quality monitor auto retrain and auto fall

back/forward.

Command:       AT%Fn

Split-Speed Direction Select. Determines which direction (transmit

or receive) has the 75bps channel and which has the 7500bps

channel. Valid only if the \W1 command has been entered.

1 = selects 75Tx/1200Rx

2 = selects 1200Tx/75Rx (DEFAULT)

Command:       AT%L

Report Received Signal Level. Possible values are:

009 = -9 dBm, 010 = -10dBm, etc. all the way to 043 (-43 dBm)

Line signal and noise are determined by the unit of measurement dBm

(decibel referenced to one milliwatt). To arrive at a signal/noise

ratio, the noise level is subtracted from signal level in dBm.

Command:       AT%Q

Report Line Signal Quality. Returns a high-order byte of the

calculated EQM (eye quality monitor). This can range from 0 to 255.

When the value is 8 or greater, the modem will automatically

retrain if enabled by the AT%E1 command. The value for a normal

connection ranges from 0 to 2 and approaches 8 for a progressively

poorer connection. Returns an OK result code.

000 to 007 = no retrain

008 to 255 = retrain performed if enabled by %E1.

Section Six: V.32/V.32bis Operation

These CCITT protocols allow for high-speed data transmission and

reception up to a maximum throughput speed of 57,600 bps (with

V.42bis data compression). The 14.4Kbps modem is upwardly

compatible to both of these high-speed modulation protocols.

The V.32 CCITT standard enables full-duplex 9600 bps asynchronous

transmission to occur synchronously over the switched telephone

network and can attain throughput rates of 38,400 bps when combined

with V.42 error control. The V.32bis modulation protocol extends

the 9600 bps capability of V.32 to 14,400 bps while remaining

compatible with V.32 modems.

Possible modem to modem connection speeds are as follows:

V.32      4800 bps

9600 bps

V.32bis   14.4Kbps

12K bps

7200 bps

The ATNn command (Modulation Handshake) determines whether or not

the modem allows only particular connections, or automatically

detects a remote modems desired connection.

ATN1 (default) allows communicating at any speeds supported by both

modems.

Entering the command ATN0 requires that the speed of the connection

be that specified by the value of S-Register 37 (see p. 53). If

that value were not one supported by V.32, communication would not

be possible.

Section Seven: Diagnostics

The 14.4Kbps modem includes several tests which evaluate the

operation of the modem, its connection to the local DTE, and the

communications link between the local and remote modems. Use of

these tests requires setup of internal registers and may also

require the use of two modems and a central office line

simulator. A thorough knowledge of modem operation and registers is

needed for their use.

7.1 Local Analog Loopback Test

This test verifies the path between the local modem and the PC.

1. Set the DTE speed from 1200 to 14,400 bits per second. If the

speed is less than 1200 or greater than 14,400 the test WILL NOT

operate and a result code of ERROR will be generated if any tests

are attempted.

2. Make sure the modem is in command mode (no connection made).

3. Issue an AT&Q0 command to the modem. This sets the modem to the

direct mode.

4. Set the timer register (S18) for desired test time (in seconds).

Example:  ATS18=10  provides for a 10 second test.

5. To begin the test, type AT&T1 [ENTER]. The modem should respond

with a CONNECT message.

6. After the number of seconds specified in S18, the modem will

terminate the test and respond with OK.

7.2 Local Analog Loopback Test with Self-Test

This test verifies the integrity of the local modems transmit and

receive circuits.

1. Set the DTE speed from 1200 to 14,400 bits per second. If the

speed is less than 1200 or greater than 14,400 the test WILL NOT

operate and a result code of ERROR will be generated if any of the

tests are attempted.

2. Make sure the modem is in command mode (no connection made).

3. Issue an AT&Q0 command to the modem. This sets the modem to the

direct mode.

4. Set timer register (S18) for desired test time (in seconds).

Example: ATS18=10  provides for a 10 second test.

5. To begin, type AT&T8 [ENTER]. The modem responds with OK.

6. After the number of seconds specified in S18, the modem should

respond with 000 (meaning 0 errors were found during the self

test).

7.3 Remote Digital Loopback Test

1. Set the DTE speed from 1200 to 14,400 Bits per second. If the

speed is less than 1200 or greater than 14,400 the test WILL NOT

operate and a result code of ERROR will be generated if the test is

attempted.

The baud rate is also limited to the highest baud rate supported by

the telephone line (14,400 is possible if the telephone line is

without distortion and noise free). If the telephone line has

problems, bring the baud rate down to 9600 or 2400. If the line

quality is too poor, use a line simulator to run this test.

2. Issue an AT&Q0 command to the local modem. This sets the modem

to the direct mode.

3. Issue an AT&Q0 command to the remote modem, also setting it to

the direct mode.

4. Issue an AT&T4 to the remote modem. This instructs the remote

modem to grant a request for a remote digital loop back test.

5. Issue an ATS0=1 to the remote modem which allows it to

auto-answer the incoming call.

6. Set the timer register (S18) of the local modem for desired test

time (in seconds). Example:  ATS18=10  provides for a 10 second

test.

7. Issue a dial command to the local modem to instruct it to call

the remote modem. Wait until the two modems have established a

connection.

8. Type the escape sequence +++ to revert to command mode. The

modem will respond with OK.

9. To start the test, type AT&T6 [ENTER]. The modem will return a

CONNECT response if the loopback data link has been successfully

completed, and an ERROR response if the link has failed. If a

CONNECT response was received, key in a text message and it will

echo on the screen.

7.4 Remote Digital Loopback Test with Self-test

1. Set the DTE speed from 1200 to 14,400 bits per second. If the

speed is less than 1200 or greater than 14,400 the test WILL NOT

operate and a result code of ERROR will be generated if the test is

attempted.

The baud rate is also limited to the highest baud rate supported by

the telephone line (14,400 is possible if the telephone line is

without distortion and noise free). If the telephone line has

problems, bring the baud rate down to 9600 or 2400. If the line

quality is too poor, use a line simulator to run this test.

2. Issue an AT&Q0 command to the local modem. This sets the modem

to the direct mode.

3. Issue an AT&Q0 command to the remote modem, also setting it to

the direct mode.

4. Issue an AT&T4 to the remote modem. This instructs the remote

modem to grant a request for a remote digital loop-back test with

self test.

5. Issue an ATS0=1 to allow the remote modem to auto-answer the

incoming call.

6. Set timer register (S18) of the local modem for desired test

time (in seconds). Example: ATS18=10  provides for a 10 second

test.

7. Issue a dial command to the local modem to instruct it to call

the remote modem. Wait until the two modems have established a

connection.

8. Type the escape sequence (+++) to revert to the command mode.

The modem will respond with OK.

9. Type AT&T7 [ENTER] to initiate the remote digital loopback test.

The local modem will send a test pattern to the remote modem. This

pattern will not be visible on the screen.

10. The modem will then send a three-digit value, indicating the

number of errors detected during testing. If the result is 000, no

errors were found.

Section Eight: Troubleshooting

This section lists the most common problems that may be encountered

and their possible solutions.

SYMPTOM:                 POSSIBLE REMEDY:

No dial tone.            Check the following:

Verify that you have the cables plugged in

correctly as instructed in Section One.

Connect a telephone set directly to the

wall jack and check for dial tone. If no

dial tone is heard, the telephone line is

not working. Contact the telephone

company.

No response

when you type in

AT commands.             Check the following:

There may be a conflicting port address.

Re-configure the modem COM address.

(INTERNAL ONLY)

Verify that the communication software is

set for the same COM address.

Your RS-232 cable connection (EXTERNAL

ONLY).

AT commands are

not visible.             Check the following:

Make sure the echo command is set to off.

Change to echo with the ATE1 command.

After data connection

is established, data is

displayed as garbled

characters.              Check the following:

Make sure the local and remote modem

configurations are compatible.

Verify that both modems are operating with

the same settings, speed, data, parity,

and stop bits.

The software may not be set for correct

terminal emulation. Configure software to

correct type. ANSI terminal emulation is

most commonly used.

Turn off your modem (EXTERNAL), exit, and

re-run your communications software.

Power down your system (INTERNAL) and

re-run your communications software.

A slowdown is noticed

when running multi-

tasking software.

When multi-tasking with fast machines

(e.g., 386DXs, 486/DXs), the CPU will

easily handle a high number of interrupts.

With slower CPUs such as 286 and 386SX

machines, a slowdown can occur.

Appendix A: How to Set Bit-Mapped Registers

To set or change bit-mapped registers requires a sound working

knowledge of binary notation and should not be undertaken casually.

Bit-mapped S-Register values are calculated as follows. Each bit

(0-7) has a corresponding decimal value as shown below. Of the bit

is ON, it has that corresponding value; if the bit is OFF, its

value is zero.

Bit  Decimal

0    1

1    2

2    4

3    8

4    16

5    32

6    64

7    128

Some bit-mapped registers combine bits which make calculating

values even more complex. Example: if bits 3 and 4 were combined to

offer four options ((0-3), the value would be calculated this way:

BIT 4     (16)      BIT 3     (8)       OPTION    VALUE

0         (0)       0         (0)       0         0

0         (16)      1         (8)       1         24 (16+8)

1         (32)      0         (16)      2         48 (32+16)

1         (48)      1         (24)      3         72 (48+24)

Now, let's try a real example. Register S14 contains a set of

bit-mapped registers for echoing commands, result code display,

word/number responses, dial method, and answer/originate operation.

On the next page are the default settings and corresponding values

along with default settings.

Value(Bit)

128(7) 64(6) 32(5) 16(4) 8(3) 4(2) 2(1) 1(0)                value

|    |    |    |    |    |    |    |____ ignored       0

|    |    |    |    |    |    |_________ 0= no echo

|    |    |    |    |    |               1= echo       2

|    |    |    |    |    |______________ 0= send resp.

|    |    |    |    |                    1= no send    0

|    |    |    |    |___________________ 0= send #

|    |    |    |                         1= send word  8

|    |    |    |________________________ reserved      0

|    |    |_____________________________ 0= touch tone 0

|    |                                   1= pulse

|    |__________________________________ reserved      0

|_______________________________________ 0= answer

1= originate  128

The S-Register bit-mapped value would be 138 (2+8+128)

Appendix B: Technical Specifications

Modem Data Rate:    14.4K, 12K, 9600, 7200, 4800,

2400, 1200, or 300 bps

Fax Data Rate:      14.4K, 9600, 7200, 4800, 2400

bps.

Compatibility:      Modem Modulation

Protocols CCITT:

Bell 103, 212A

V.32bis, V.32, V.42bis, V.42, V.22,

and V.21.

Fax Modulation Protocols

CCITT: V.17, V.29, V.27 ter, and

V.21

Operation:          Asynchronous operation (MNP 3:

synchronous)

Automatic format and/or speed sensing.

Tone or Pulse dialing

Programmable speaker volume control--recognizes

busy signal, ringing, no answer, and no

dial-tone status.

Diagnostics:        Local/remote digital and analog loopback.

Automatic power-on self-test.

Physical and Electrical Characteristics

Size: 8" x 4 3/8" (INTERNAL); 5 1/2" x 5" x 2" (EXTERNAL)

Power: 500 mA@ 5V(INTERNAL); 120V AC@ 100mA (EXTERNAL)

8- or 16-bit interface (INTERNAL)

2 RJ-11 modular phone connectors.

RS-232 serial cable (EXTERNAL)

Compatible with IBM PC systems (ISA and EISA).

Appendix C: Compliance Information

FCC Statement

"This device complies with part 15 of the FCC rules. Operation is

subject to the following two conditions:

(1) This device may not cause harmful interference.

(2) This device must accept any interference received including

interference that may cause undesired operation.

THIS UNIT COMPLIES WITH FCC PART 68 AS OF DATE OF MANUFACTURE.

This equipment has been tested and found to comply with the limits

for a Class B digital device, pursuant to Part 15 of FCC rules.

These limits are designed to provide reasonable protection against

harmful interference in a residential installation. This equipment

generates, uses, and can radiate radio frequency energy and, if not

installed in accordance with the instructions, may cause harmful

interference to radio communications. However, there is no

guarantee that interference will not occur in a particular

installation. If this equipment does cause harmful interference to

radio or television reception, which can be determined by turning

the equipment off and on, the user is encouraged to try to correct

the interference by  one or more of the following measures:

* Re-orient or relocate the receiving antennae.

* Increase the separation between the equipment and the receiver.

* Connect the equipment into an outlet on a circuit different from

that to which the receiver is connected.

* Consult the dealer or an experienced radio/TV technician for

help.

Note: This unit was tested with shielded cables on the peripheral

devices. Shielded cables must be used with the unit to insure

compliance.

Note: The manufacturer is not responsible for any radio or TV

interference caused by unauthorized modifications to this

equipment. Such modifications could void the user's authority to

operate the equipment."

Notification to the Telephone Company

Notification to the telephone company is no longer required prior

to connecting the registered equipment but upon request from the

telephone company the user shall tell the telephone company which

line the equipment is connected to as well as the registration

number and the ringer equivalence of the registered protective

circuitry. In most, but not all areas, the sum of all RENs should

be 5.0 or less. The FCC Registration number and Ringer Equivalence

number are printed on the main chip in the center of the internal

modem board, or on the underside of the external modem.

Malfunction of the Equipment

In the event that the MODEM should fail to operate properly, the

customer shall disconnect the equipment from the telephone line to

determine if it is the customer's equipment which is not working

properly, or if the problem is with the MODEM, the user shall

discontinue use until it is repaired. In the event service is

needed the user  should contact the vendor from whom you purchased

the MODEM.

Telephone Connection Requirements

Except for telephone company-provided ringers, all connections to

the telephone network shall be made through standard plugs and

standard telephone company-provided jacks, or equivalent, in such

a manner as to allow for easy and immediate disconnection of the

terminal equipment. Standard jacks shall also be arranged that, if

the plug connected thereto is withdrawn, no interference to the

operation of the equipment at the customer's premises which remains

connected to the telephone network, shall occur by reason of such

withdrawal.

Incidence of Harm

Should terminal equipment or protective circuitry cause harm to the

telephone network, the telephone company shall, where practical,

notify the customer that temporary discontinuance of service may be

required; however, where prior notices are not practical, the

telephone company may temporarily discontinue service if such

action is deemed reasonable in the circumstances. In the case of

such temporary discontinuance, the telephone company shall promptly

notify customers and will be given the right to bring a complaint

to the FCC if they feel the disconnection is not warranted.

Changes in Telephone Company Equipment or Facilities

The telephone company may make changes in its communications

facilities, equipment, operations, or procedures, where such action

is reasonably required and proper in its business. Should any such

changes render the customer's terminal equipment incompatible with

the telephone company facilities, the customer shall be given

adequate notice to make modifications to maintain uninterrupted

service.

General

The FCC prohibits customer-provided terminal equipment be connected

to party lines or to be used in conjunction with coin telephone

service.

Installation

The MODEM is equipped with a USOC RJ-11 standard miniature modular

jack and is designed to plug directly into a modular jack.

DOC Compliance Statement (Canada)

The Canadian Department of Communications label identifies

certified equipment. This certification means that the equipment

meets certain telecommunications network protective operational and

safety requirements. The Department does not guarantee the

equipment will operate to the user's satisfaction.

Before installing this equipment, users should ensure that it is

permissible to be connected to the facilities of the local

telecommunications company. The equipment must also be installed

using an acceptable method of connection. In some cases, the

company's inside wiring associated with a single line individual

service may be extended by means of a certified connector assembly

(telephone extension cord). The customer should be aware that

compliance with the above conditions may not prevent degradation of

service in some situations.

Repairs to certified equipment should be made by an authorized

Canadian maintenance facility designated by the supplier. Any

repairs or alterations made by the user to this equipment, or

equipment malfunction, may give the telecommunications company

cause to request the user to disconnect the equipment.

Users should ensure, for their own protection, that the electrical

ground connections of the power utility, telephone lines, and

internal metallic water pipe system, if present, are connected

together. This precaution may be particularly important in rural

areas.

CAUTION Users should not attempt to make such connections

themselves, but should contact the appropriate electric inspection

authority or electrician, as appropriate.

The Load Number (LN) assigned to each terminal device denotes the

percentage of the total load to be connected to a telephone loop

which is used by the device to prevent overloading. The termination

on a loop may consist of any combination of devices subject only to

the requirement that the total of the load numbers of all the

devices does not exceed 100. The Load number appears on either the

lower inside portion of the modem's metal bracket (internal) or on

the underside of the external modem.

Glossary of Modem Terminology

A

ALGORITHM. A formula or procedure which employs various methods

defining how data is to be used to give a prescribed result.

ANALOG SIGNAL. Signals which can vary over a continuous range

(e.g., the human voice over conventional telephone lines). Analog

circuitry is more subject to distortion and noise, but it is more

capable of handling complex signals than are digital signals which

can have only discrete values.

ARQ. Automatic ReQuest for retransmission. A type of communications

link where the receiver asks the transmitter to re-send a block of

data when errors are detected.

ANSI. American National Standards Institute. A non-profit, private

industry association which governs most USA-standards setting

agencies.

ASCII. Acronym for American Standard Code for Information

Interchange. ASCII is an ANSI character set. The standard ASCII

character set consists of 128 decimal numbers (0-127) for letters

of the alphabet, numerals, punctuation marks, and common special

characters. The extended ASCII character set extends to 255

characters and contains special mathematical, graphics, and foreign

characters.

ASYNCHRONOUS COMMUNICATION. A method of transmission in which data

is sent one character is sent one bit at a time; also referred to

as serial transmission.

B

BELL STANDARDS. Refers to the domestic modulation protocol

standards developed by the former AT&T Bell Systems such as Bell

103 (300bps transmission) and Bell 212A (1200bps transmission).

BIT-MAPPED REGISTERS. An S-register which contains multiple

bit-oriented values. Example: the value of S22 is the sum of the

bit values which you select (bits 0-1: speaker volume/value: 0-3;

bits 2-3: speaker control/value: 0-3; bits 4-6: limit result

codes/value: 0, 4-6, 7).

BLIND DIALING. An automated process whereby the modem goes off-hook

and dials without waiting for a dial tone. This is prohibited in

many countries.

BLOCK TRANSFER CONTROL. Determines whether or not the modem uses

block or stream mode during an MNP connection. In stream mode, MNP

sends data frames in varying length. Block mode sends fixed data

frames of 256 characters and is selected with the MNP-based \L

command.

BREAK HANDLING. Determines how the modem responds when a BREAK

signal is received from either the DTE (Data Terminal Equipment or

computer/terminal) or the remote modem. This is controlled by the

MNP-based AT extended command \K. A break signal is represented on

the communications line by a steady space signal for a

significant length of time. Break signals may be activated from the

keyboard by pressing the BREAK key or the control (CTRL) and C

keys.

C

CCITT. Consultative Committee for International Telephone and

Telegraph. This advisory organization is part of the ITU

(International Telecommunication Union) which is an agency of the

United Nations.

COMMAND MODE. The modem is in command mode when it is turned on or

reset, when it loses its connection to a remote modem, when it is

in on-line mode, or when escape characters (+++) are typed. To

transmit data, the modem must be in data mode. The modem does not

transmit data when in command mode.

COMMUNICATIONS PROTOCOL. A set of procedures which controls how a

data communications network operates.

D

DCD. Data Carrier Detect. Indicates to the terminal device that the

modem is receiving a valid carrier signal from a remote modem. The

carrier is a tone at a specified frequency.

DCE. Data Communications Equipment. The local and/or remote modem.

A DCE is usually connected to a DTE.

DTE. Data Terminal Equipment. The computer or terminal, either

local (yours), or the remote (the one you're communicating with).

A DTE is usually connected to a DCE.

DTR. Data Terminal Ready. The computer issues this signal to the

attached modem indicating that it is ready to receive data.

DATA COMPRESSION. A technique that examines transmitted data for

redundancy and replaces strings (groups) of characters with special

codes which the receiving modem interprets and restores to its

original form. Transmission of compressed data results in shorter

connect times and hence cost savings for connect charges.

Data compression is sometimes called source encoding.

DATA MODE. The modem is in data mode when a connection has been

established with a remote modem and sends a CONNECT response

confirming the connection. User data may then be transmitted or

received.

DIAL MODIFIER. Dial modifiers are special characters appended to

the ATD command which instruct the modem how to place a call.

DIGITAL SIGNAL. A discrete signal which can only take on one of

several (usually only two) discrete levels in contrast to analog

signals which can take a continuous range of levels.

E

ERROR DETECTION AND CORRECTION. The transmitting modem attaches a

special pattern (called a frame check sequence) calculated

according to a prescribed algorithm from user-defined data to the

end of a block of data. The receiving modem performs the same

algorithm and compares it to the one with the transmitted data. If

these match, then the block of data has been received correctly. If

not, the block of data is re-transmitted until no errors are

detected.

ESCAPE SEQUENCE. Also referred to as the escape command. This

special command is entered as three plus symbols (+++) and places

the modem in command mode and interrupts user data transmission,

but does not terminate the data connection. This allows the

entering of commands while the connection is maintained.

EXTENDED AT-COMMAND. Extended commands were developed to provide

greater functionality and control over modem operations than is

available from the basic AT command set.

F

FAX MODE. The modem is in fax mode when, through use of fax

communications software, it can send and receive faxes, print and

display fax files, convert files to fax-files, and set certain

fax-related features. Note: the modulation protocol used by the

modem in fax mode is also different from the usual data mode

modulation.

FLOW CONTROL. Compensates for the difference between the rate at

which data reaches a device and the rate at which the device

processes and transmits. This is controlled by the extended AT

command &K. The two common types of flow control are RTS/CTS

signaling (a hardware based method, employing an electrical signal)

and XON/XOFF (a software-based method using standard ASCII control

characters to pause or resume transmission).

FULL-DUPLEX. Two-way simultaneous transmission between modems,

which may occur via a four-wire circuit on a leased line, or with

a two-wire connection when the frequency bandwidth is divided into

two distinct channels, or when echo cancellation is employed (e.g.,

Bell 103, 212, and V.22 use frequency division, while V.32 uses

echo cancellation.

G

GUARD TONE. Guard tones are used in the United Kingdom and other

countries. This requires that the modem transmit an 1800-Hz tone

after it sends an answer tone. The guard tone is controlled by the

&G command. Guard tones are not used in the U.S.A.

H

HALF-DUPLEX. Signal flow in both directions, but only one way at a

time with each modem alternating between send and receive.

HAYES-COMPATIBLE. Hayes Microcomputer Products, Inc. developed the

AT command set which has become a de facto industry standard. Hayes

commands are always initiated with an AT (attention code) prefix.

HOOK FLASH. The dial modifier ! causes the modem to go on-hook

(hang-up) for one-half second. Also controlled by the ATH command.

L

LAPM. Link Access Protocol Modem. A V.42 ARQ type of error

correction protocol which is controlled by S-Register 46, where

LAPM may be activated with or without V.42bis data compression.

LEASED LINE. Also referred to as a private line. A leased line is

obtained from a communications company (carrier) to provide a

transmission medium between two points. The line consists of a

permanent dedicated circuit between two points, or to set of

previously arranged points. The cost of the line is usually based

on the distance between locations. This is in contrast to switched

or dial-up lines, which can be connected to any point on the

network.

LINE MODULATION. The means by which a carrier is varied to

represent a signal carrying information. In a modem, the users

digital data is used to modulate the modem's transmitters carrier

or carrier to allow the digital signal to be carried over analog

facilities.

LONG SPACE DISCONNECT. Determines whether or not modem disconnects

when it receives a continuous break from a remote modem. Controlled

by the ATY command.

LOOPBACK TESTS. There are four types of loopback tests which are as

follows: (1) the local digital loopback tests the operation of the

DTE, including whether or not data is leaving the terminal or

computer port; (2) the local analog loopback tests the digital and

analog circuits of the modem; (3) the remote digital loopback

checks the operating condition of the line and remote modem; (4)

the remote analog loopback tests the line to the remote modem.

M

MNP. Microcom Networking Protocol. A series of data communications

protocols developed by Microcom for full-duplex, error-free

communications.

MAKE/BREAK RATIO. The &P command controls the ratio of the off-hook

(make) to on-hook (break) interval used by the modem when it pulse

dials. &P0 selects a 39%/61% make/break ratio for use in the U.S.

&P1 selects a 33%/67% make/break ratio for use in the United

Kingdom and Hong Kong.

MODULATION HANDSHAKE. Also referred to as Automode Enable and is

controlled by the ATN command. This determines whether or not the

modem must connect at a particular speed, or allow connection at

any speed supported by both modems.

N

NEGOTIATION FALLBACK. Controlled by S-Register 36 as part of the

V.42 protocol. Setting this register indicates what action to take

when a desired connection cannot be made (e.g., hang-up, direct

mode connect, normal mode connect).

NON-VOLATILE RAM. Also NVRAM. Random access memory whose data is

retained when power is turned off. This is especially useful for

modems to store user-defined default configuration settings. These

settings would be loaded into modem RAM at power-up.

O

ON-LINE STATE. Same as data mode. To transmit or receive data, the

modem must be in the on-line state. When placing a call, the modem

is put on-line with the dial command.

P

PBX. Private Branch Exchange. A telephone switch at a customer

site.

PULSE DIALING. Also referred to as rotary dialing, i.e., dialing

with the older-style rotary dial wheel. The dial modifier ATP sets

the modem to pulse dialing, which is the default method as opposed

to tone dialing (push-button touch-tone) which is enabled with ATT.

All telephone exchanges will accept older-style pulse dialing and

most exchanges will accept modern tone-dialing. Tone dialing is

faster and more reliable since mechanical relays and their inherent

failure mechanisms are avoided.

R

RESULT CODE. A response sent by the modem after executing a

command. The response reports the modems status or the progress of

a call and can take the form of either digits (numeric) or words

(verbose). Issuing a V1 command enables word responses. A V0

(V-zero) command enables numeric responses. The Q1 command

disables their use entirely. Example: OK  (word), or 0 (numeric)

indicates that the modem successfully executed a command.

RETRAIN. An adjustment process performed when one of the modems

detects signal distortion or line noise which threaten data

integrity.

RTS/CTS.  Request to Send/Clear to Send. RTS and CTS are two

control signal lines between the modem (DCE) and terminal (DTE)

which allow the terminal to control the flow of information. See

also flow control.

S

SLEEP INACTIVITY TIMER. Determines the length of time the modem

operates in normal mode with no activity before entering low-power

sleep mode.

SPLIT-SPEED DIRECTION. Determines which direction (transmit or

receive) has the 75bps channel and which has the 1200bps channel,

but is only active if the \W command is set to 1. Used with V.23

modulation only.

STANDARD AT-COMMAND. The basic AT command set, originated by Hayes

Microcomputer Products, Inc.

SYNCHRONOUS CLOCK SOURCE. Applies to synchronous modem operations

and is set with the &X command. The command specifies the clocking

source referred to as the transmit signal element timing. This can

be controlled by either the modem or the terminal.

SYNCHRONOUS COMMUNICATIONS. A method of transmission in which data

bits are sent continuously at the same rate under the control of a

fixed frequency clock signal.

T

TOUCH-TONE DIALING. Push-button tone dialing as used on

contemporary phone sets. The dial modifier ATT sets the modem to

tone  mode. Tone dialing is faster and more reliable than

older-style pulse dialing.

X

XON/XOFF. XON and XOFF are the names of two different control

characters. See also flow control.

Acknowledgements

The Complete Modem Reference. Gilbert Held. John Wiley & Sons, Inc.

1991.

Upgrading and Repairing PCs. (Second Edition). Que Corporation.

1992.

###############################################################################################################